Title: Update to ATN Network-to-Mobile Subnetwork priority table to add VDL Mode 3 and VDL Mode 4

PDR Reference:                              M1030005

Originator Reference:

SARPs Document Reference:               Core: Table 3-3 and SV-1: Table 1-3

Status:

PDR Revision Date:                    6/03/01 (

Submitting State/Organization:        ATNP WGA/B

Submitting Author Name:               Jones, R

Submitting Author E-mail Address:     ronnie.jones@faa.gov

Submitting Author Supplemental

Contact Information:

SARPs Date:                           ATNP/3 Approved, 2/00

SARPs Language:                       English

Summary of Defect:

The ICAO AMC Panel has approved SARPs for VDL Mode 3 and VDL Mode 4 mobile subnetworks.  The ATN network to mobile subnet priority mapping needs to be defined in the ATN core SARPs and Doc 9705 Sub-Volume I.

Impact on Interoperability:

None

Impact on Safety:

None

Assigned SME:                              Sub-Volume I SME

Proposed SARPs amendment:

Modify the ATN network to mobile subnetwork priority mapping table in the Core SARPs and SV-1 to add columns for the VDL Mode 3 and VDL Mode 4 subnetworks.  Also add a note to indicate that “VDL Mode 4 provides mobile subnetwork support for surveillance applications (e.g., ADS)”.  Proposed update to the priority mapping table is attached.

SME Recommendation to CCB:

CCB Decision:

ATTACHMENT:  Proposed updated ATN Network to Mobile Subnetwork priority table.

ATTACHMENT

Revised ATN Network Priority to Mobile Subnetwork Priority Table

Core ATN SARPs Table 3-3

Doc 9705 SV-1 Table 1-3

| Message categories | ATN network layer priority | Corresponding mobile subnetwork priority (see Note 5) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AMSS-1 (see Note 4) | VDL Mode 2 | VDL Mode 3 | VDL Mode 4 | SSR Mode S | HFDL |
| Network/systems management | 14 | 14 | see Note 1 | 3 | high see Note 7 | high | 14 |
| Distress communications | 13 | 14 | see Note 1 | 2 | high see Note 7 | high | 14 |
| Urgent communications | 12 | 14 | see Note 1 | 2 | high see Note 7 | high | 14 |
| High-priority flight safety messages | 11 | 11 | see Note 1 | 2 | high see Note 7 | high | 11 |
| Normal-priority flight safety messages | 10 | 11 | see Note 1 | 2 | high see Note 7 | high | 11 |
| Meteorological communications | 9 | 8 | see Note 1 | 1 | medium see Note 7 | low | 8 |
| Flight regularity communications | 8 | 7 | see Note 1 | 1 | medium see Note 7 | low | 7 |
| Aeronautical information service messages | 7 | 6 | see Note 1 | 0 | medium see Note 7 | low | 6 |
| Network/systems administration | 6 | 5 | see Note 1 | 0 | medium see Note 7 | low | 5 |
| Aeronautical administrative messages | 5 | 4 | restricted - see Note 1 | not allowed | not allowed | not allowed | 4 restricted |
| <unassigned> | 4 | not assigned | not assigned - see Note 1 | not allowed | not allowed | not allowed | not assigned |
| Urgent-priority administrative and U.N. Charter communications | 3 | 3 | restricted - see Note 1 | not allowed | not allowed | not allowed | 3 restricted |
| High-priority administrative and State/Government communications | 2 | 2 | restricted - see Note 1 | not allowed | not allowed | not allowed | 2 restricted |
| Normal-priority administrative communications | 1 | 1 | restricted - see Note 1 | not allowed | not allowed | not allowed | 1 restricted |
| Low-priority administrative communications & Aeronautical Passenger Communications | 0 | 0 | restricted - see Note 1 | not allowed | not allowed | not allowed | 0 restricted |
| Note 1.— VDL Mode 2 has no specific subnetwork priority mechanisms. Note 2.— The AMSS SARPs specify mapping of message categories to subnetwork priority without explicitly referencing ATN network layer priority. Note 3.— The term “not allowed” means that only communications related to safety and regularity of flight are authorized to pass over this subnetwork as defined in the subnetwork SARPs. Note 4. — The term AMSS-1 refers to the first generation Aeronautical Mobile Satellite Service. Note 5. — Only those mobile subnetworks are listed for which subnetwork SARPs exist and for which explicit support is provided by the ATN Boundary Intermediate System technical provisions. Note 6. — The term “restricted” means for this message category the use of this subnetwork may not be allowed in certain States and/or regions based on ITU radio frequency spectrum allocation. Note 7. —VDL Mode 4 provides mobile subnetwork support for surveillance applications (e.g., ADS). |  |  |  |  |  |  |  |