Title: 					Symmetry of Mobile SNDCF APRL and Route Initiation APRL

PDR Reference:				98060007

Originator Reference:

SARPs Document Reference: 		ICS SARPs, Section 5.7.7.8 and 5.3.5.2.14

Status: 					PROPOSED

PDR Revision Date:

PDR Submission Date:			22 June 1998

Submitting State/Organisation: 		DFS/Germany

Submitting Author Name: 		Klaus-Peter Graf

Submitting Author E-mail Address: 	klaus.graf@unibw-muenchen.de

Submitting Author Supplemental		Tel: +49/89/6004 4123

Contact Information:			Fax: +49/89/680 735 13

SARPs Date:				ICAO Version 2.2

SARPs Language: 			English

Summary of Defect:

The Mobile SNDCF ARPLs as specified in section 5.7.7.8 are symmetric. However, operational implementations may not implement the SNDCF symmetrically. For example, an A/G Router that only supports VDL will never perform connection initiation. The APRL needs to allow for this case.

Furthermore, the item "csReset" in the last entry of Table 5.7.7.8.2 is named inappropriately; a more appropriate name for this item is "csClear".

Assigned SME:				Sub-Volume V SME (K.-P. Graf)

SME comment:

The entries in the ATN Support column of the Table 5.7.7.8.2 have to be complemented by appropriate conditional compliance predicates. These predicates have to be introduced and defined in Table 5.7.7.8.1 or 5.7.7.8.2 respectively.

Proposed SARPs amendment:

1) In Table 5.7.7.8.2:

a) Add new line after the table headline with the following entries:

Item = clInit,  Function = Call Initiator,  ATN SARPs Reference = 5.7.6.2,  ATN Support = O.1

b) Add new line after the line added in 1) above with the following entries:

Item = clRspd,  Function = Call Responder,  ATN SARPs Reference = 5.7.6.2,  ATN Support = O.1

c) Change the entry in the ATN Support column of the item “csDynam” from “M” to “clInit:O.2”

d) Change the entry in the ATN Support column of the item “csSys” from “M” to “clInit:O.2”

e) Change the entry in the ATN Support column of the item “csDef” from “M” to “clInit:M”

f) Change the entry in the ATN Support column of the item “csCol” from “M” to “clInit:M”

g) Change the entry in the ATN Support column of the item “csAcp” from “M” to “clRspd:M”

h) Change the entry in the ATN Support column of the item “csRej” from “M” to “clRspd:M”

i) Change the entry in the ATN Support column of the item “csDiag” from “M” to “clInit:M”

- Change the entry in the Item column of the item “csReset” from “csReset” to “csClear”

The above changes are reproduced below in the SARPs table 5.7.7.8.2 using strike-out text and revision marks:

| Item | Function | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| clInit | Call Initiator | 5.7.6.2 | O.1 |
| clRspd | Call Responder | 5.7.6.2 | O.1 |
| csDynam | Dynamic Call Setup | 5.7.6.2.1.1.1 | clInit:O.2 |
| csSys | Call Setup by Systems Management | 5.7.6.2.1.1 | clInit:O.2 |
| csDef | Use of non-standard Default packet size | 5.7.6.2.1.3 | clInit:M |
| csFast | Use of Fast Select | 5.7.6.2.1.4 | M |
| csM/I | Local Reference directory maintenance request/acceptance | 5.7.6.2.1.5.13 5.7.6.2.2.2.3 | ^csFast: - McM/I:M |
| csOther | Use of other optional User Facilities and CCITT-specified DTE facilities | 5.7.6.2.1.1.3 | O |
| csCol | Call Collision Resolution | 5.7.6.2.2.1.2 | clInit:M |
| csAcp | Call Acceptance Procedures | 5.7.6.2.1.6 | clRspd:M |
| csRej | Call rejection Procedures | 5.7.6.2.1.7 | clRspd:M |
| csOrd | Order of compression Procedures | 5.7.6.2.3.2 | M |
| csDiag | Use of call rejection diagnostic codes | 5.7.6.2.1.7.3 | clInit:M |
| csClear | Call Clearing Procedures | 5.7.6.2.4 | M |

2)   Change the existing para 5.7.6.2.1.1.1 to read:

5.7.6.2.1.1.1 When it has been determined that a virtual circuit is to be made available, the calling SNDCF shall establish the virtual circuit using the procedures specified in ISO/IEC 8208, either

- dynamically, on receipt of ….., or

- by the explicit intervention of ….

3)  Delete the existing para 5.7.6.2.1.1.2 and renumber the existing para 5.7.6.2.1.1.3 and 5.7.6.2.1.1.4 to 5.7.6.2.1.1.2 and 5.7.6.2.1.1.3 respectively.

SME Recommendation to CCB:		ACCEPT proposed SARPs amendment

CCB Decision: