Title:     Support of IDRP by Airborne Routers implementing optional

non-use of IDRP

PDR Reference:                          98060004

Originator Reference:

SARPs Document Reference:               ICS SARPs, Section 5.2.4.1.2

Status:                                 RESOLVED

PDR Revision Date:

PDR Submission Date:                    22 June 1998

Submitting State/Organisation:          DFS/Germany

Submitting Author Name:                 Klaus-Peter Graf

Submitting Author E-mail Address:       klaus.graf@unibw-muenchen.de

Submitting Author Supplemental          Tel: +49/89/6004 4123

Contact Information:                    Fax: +49/89/680 735 13

SARPs Date:                             ICAO Version 2.2

SARPs Language:                         English

Summary of Defect:

Item b) of para 5.2.4.1.2 mandates the support of the ISO/IEC 10747

Inter-Domain Routing Protocol (IDRP) for the all ATN Inter-Domain

Routers, i.e. Router Classes 4 to 7 inclusive. However, Router Class 7

describes an Airborne Router without IDRP, and consequently should not

be included in the list of routers concerned by para 5.2.4.1.2.

Assigned SME:                 Sub-Volume V SME (K.-P. Graf)

Proposed SARPs amendment:

1)  Change para 5.2.4.1.2 to read:

"5.2.4.1.2    All ATN Routers (i.e. Router Classes 1 to 7 inclusive)

shall support the ISO/IEC 8473 Connectionless Network Protocol (CLNP) as

specified in 5.6, including the use of the CLNP options security

parameter, and shall interpret and obey the Routing Policy Requirements

expressed therein, whilst routing the packet in accordance with any

restrictions placed on the traffic types that may be carried over a

given ATN Subnetwork, by forwarding CLNP NPDUs."

2)  Add new para 5.2.4.1.3 to read:

"5.2.4.1.3    With the exception of Airborne Routers that implement the

procedures for the optional non-use of IDRP (i.e. Router Class 7), all

ATN Inter-Domain Routers (i.e. Router Classes 4 to 6 inclusive) shall

support the ISO/IEC 10747 Inter-Domain Routing Protocol (IDRP) as

specified in 5.8 for the exchange of inter-domain routing information

according to 5.3.6 and 5.3.7."

3)  Renumber the existing para number 5.2.4.1.3 to read 5.2.4.1.4

SME Recommendation to CCB:

CCB Decision: Proposed SARPs amendment agreed