Title: SNDCF Call Request/Confirm User Data Length Indicator

PDR Reference:  97100002

Originator Reference:

SARPs Document Reference: 5.7.6.2.2.2.5 Vs (figure 5.7-2 and

5.7.6.2.1.5.3), 5.7.6.2.1.6.1.6 Vs (figure 5.7-3 and

5.7.6.2.2.4.x)

PDR Status:     RESOLVED

PDR Revision Date:    4/12/97

PDR Submission Date: 10/10/97

Submitting State/Organization: Raytheon Electronic Systems

Submitting Author Name: Moncreaff, Greg

Submitting Author E-mail Address: Gregory_D_Moncreaff@res.raytheon.com

Submitting Author Supplemental Contact Information:

Phone: (508) 490 - 2048

Fax:   (508) 490 - 2086

Mail:  MS 2-2-2507

1001 Boston Post Rd

Marlborough, MA 01752

USA

SARPs Date:    ICAO Version 2.0 (27 June 1997)

SARPs Language: English

Summary of Defect:

Call request:

The length indicator in the figure and its corresponding text are not

consistent with the description in the call acceptance section.  The

former excludes any subsequent NPDU after the parameter block, while the

latter seems to include such a PDU.

Call confirm:

The figure and its corresponding text are not consistent with the

description in the "receipt of call confirm" section.  The former does

not indicate the presence of a length indicator, while the latter

implicitly does.

Assigned SME: Subvolume V SME (K.-P. Graf)

Proposed SARPs amendment:

1.)     Change:

5.7.6.2.1.6.1.6          If the length indicator in the SNDCF header of

the received Call Confirm Packet exceeds two, and if the User Data field

is a recognized NPDU SPI, then the User Data field contains an NPDU and

the calling SNDCF shall pass this NPDU in an SN-UNITDATA indication to

the appropriate ATN SN-Service user.

to:

5.7.6.2.1.6.1.6           If the length of the User Data of the received

Call Confirm Packet is greater than one, then the remaining part of the

received Call Confirm User Data contains an NPDU, and the calling SNDCF

shall pass this NPDU in an SN-UNITDATA indication to the appropriate

SN-Service User.

2.)     Add new section 5.7.6.2.1.6.1.7:

The first octect of this NPDU (i.e. the SPI) shall be used by the

calling SNDCF in order to identify the network layer protocol, and hence

which SN-Service User is responsible for handling this NPDU.

3.)     Add new section 5.7.6.2.1.6.1.8:

If no such SN-Service user exists, then the NPDU shall be discarded.

4.)     Change:

5.7.6.2.2.2.5  If the length indicator in the SNDCF header of the

received incoming Call Packet exceeds seven (or nine if Local Reference

Compression is offered), and if the User Data field is a recognized NPDU

SPI, then the User Data field contains an NPDU and the called SNDCF

shall pass this NPDU in an SN-UNITDATA indication to the appropriate ATN

SN-Service user.

to:

5.7.6.2.2.2.5  If there is additional User Data beyond the SNDCF

Parameter Block (see Figure 5-7.2) in the received Incoming Call Packet

and the first octet of this additional user data is a recognized NPDU

SPI, then the remaining part of the received Incoming Call User Data

contains an NPDU,and the called SNDCF shall pass this NPDU in an

SN-UNITDATA indication to the appropriate SN-Service User.

5.)     Add new section 5.7.6.2.2.2.6:

The first octect of this NPDU (i.e. the SPI) shall be used by the called

SNDCF in order to identify the network layer protocol, and hence which

SN-Service User is responsible for handling this NPDU.

6.)     Add new section 5.7.6.2.2.2.7:

If no such SN-Service user exists, then the NPDU shall be discarded.

7.)     Change:

5.7.6.2.2.2.1  The combination of compression techniques acceptable to

the SNDCF, out of those offered by the by the Calling SNDCF, shall be

indicated by including the one-octet field, shown in Figure 5.7-3, in

the ISO/IEC 8208 Call Accept User Data.

to:

5.7.6.2.2.2.1  The combination of compression techniques acceptable to

the SNDCF, out of those offered by the by the Calling SNDCF, shall be

indicated by setting the appropriate bits in the first octet of the

ISO/IEC 8208 Call Accept User Data as shown in Figure 5.7-3.

8.)     Change:

5.7.6.2.2.4.4  The Second octet of the Call Accept User Data shall be

the first octet of the User Data field.

Note.- The User Data field may be used to convey the ISO/IEC 9542 ISH

PDU as part of the routing initiation sequence.

to:

5.7.6.2.2.4.4  In case that the Call Accept Packet will be used to

convey an NPDU, the second octet of the Call Accept User Data shall be

the first octet of this NPDU.

Note 1.- An ISO/IEC 9542 ISH PDU may be conveyed as part of the routing

initiation procedure.

Note 2.- Since the negotiated compression procedures apply to the data

transfer phase (see 5.7.6.2.3.1), the optional NPDU in the Call Accept

User Data, if present, is sent uncompressed.

9.)     Rename "User Data Field" to "NPDU" in the second box of Figure

5.7-3.

10.)    Change:

5.7.6.2.1.5.1  The call user data format shall be as illustrated in

Figure 5.7-2.

to:

5.7.6.2.1.5.1  The Call Request User Data format shall be as illustrated

in Figure 5.7-2.

SME Recommendation to CCB:  Accept proposed SARPs amendment

CCB Decision: RESOLVED