Title:                                  LREF Directory Management

PDR Reference:                          97100048

Originator Reference:                   UK-SV5-01

SARPs Document Reference:               ICS SARPs, Section 5.7.6.3.4.5

PDR Status:                             RESOLVED

PDR Revision Date:                      4/12/97

PDR Submission Date:                    31/10/97

Submitting State/Organisation:          NATS, UK

Submitting Author Name:                 Cardwell, B

Submitting Author E-mail Address:       cardwellb@natsgcs.co.uk

Submitting Author Supplemental          Tel: +44-1293-576 401

Contact Information:                    Fax: +44-1293-576 399

SARPs Date:                             ICAO V2.1, Oct. 97

SARPs Language:                         English

Summary of Defect:

There is scope for non-interoperability in Section 5.7.6.3.4.5 of the

ICS SARPs (Incoming SNDCF Error Report).  The relevant SARPs are

reproduced below.

Para 5.7.6.3.4.5.1   On receipt of an SNDCF Error Report, the virtual

circuit shall be reset (see 5.7.6.3.7), unless the reason is "compressed

PDU with unrecognized local reference".

Para 5.7.6.3.4.5.2   In this case, the directory entry corresponding to

the local reference returned in the SNDCF Error Report shall be reset to

the unused state.

Note: Section 5.7.6.3.7 (Call Reset Provisions) states the actions

required on RECEIPT of a Reset Indication.

1)  When para 5.7.6.3.4.5.1 applies, it is not clear that it is also

necessary to clear the LREF directory associated with that virtual

circuit as well as initiating the Reset procedure.

2)  Para 5.7.6.3.4.5.2 can be mis-read, it could mean "when

5.7.6.3.4.5.1 applies" or "when the SNDCF Error Report gives the reason

'compressed PDU with unrecognized local reference'".

Assigned SME:                   Sub-Volume V SME (K.-P. Graf)

Proposed SARPs amendment:

To resolve point 1)

Change existing Para 5.7.6.3.4.5.1 from

5.7.6.3.4.5.1  On receipt of an SNDCF Error Report, the virtual circuit

shall be reset (see 5.7.6.3.7), unless the reason is "compressed PDU

with unrecognized local reference".

to

5.7.6.3.4.5.2  On receipt of an SNDCF Error Report with reason other

than "compressed NPDU with unrecognized local reference" (see Table

5.7-7), the virtual circuit shall be reset (see 5.7.6.3.7) and the local

reference directory associated with the virtual circuit shall be cleared

to its initial state.

Note.- If the virtual circuit on which the error has been reported

belongs to a connection group which shares the same LREF directory,

there is no need to reset the remaining virtual circuits of that group.

To resolve point 2)

Change existing Para 5.7.6.3.4.5.2 from

5.7.6.3.4.5.2  In this case, the directory entry corresponding to the

local reference returned in the SNDCF Error Report shall be reset to the

unused state.

to

5.7.6.3.4.5.1  On receipt of an SNDCF Error Report with reason

"compressed NPDU with unrecognized local reference", the directory entry

corresponding to the local reference returned in the SNDCF Error Report

shall be reset to the unused state.

SME Recommendation to CCB:  Accept proposed SARPs amendment

CCB Decision:  RESOLVED