CAMAL Part I, Section 4.2.3.1.4, add a NOTE after paragraph as follows:

NOTE:  Since the 1970s, commercial airlines have exchanged company operations data over various air/ground subnetworks through international network service providers (commonly called ACARS).  A limited number of States have made use of ACARS for some air traffic services (e.g., pre-departure clearance, digital airport terminal information service, oceanic & route clearances, and position reporting).  There is no expectation that the use of ACARS for these air traffic services will transition to and/or be accommodated by States implementing CNS/ATM, specifically, the ACARS based air traffic services will either be phased out or handled separately from ATN based applications.  The continuation of these other ACARS-based air traffic services, after the availability of ATN, should be discouraged as the ATN is standardized by ICAO

CAMAL Part II, Chapter 12 (new)

12	Other ACARS-Based Air Traffic Services

12.1	General

12.1.1	This section provides guidelines to States that implement ATN based applications, and continue to provide ACARS-based air traffic services other than those supported by FANS 1/A.

12.1.2	There is no expectation that the use of ACARS for these non-FANS 1/A air traffic services will transition to and/or be accommodated by States implementing CNS/ATM, specifically, the non-FANS 1/A ACARS based air traffic services will either be phased out or handled separately from ATN based applications.

12.1.3	It is recognized that there is some mixture of both non-ATN aircraft and ATN aircraft in common airspace, and that non-ATN aircraft may or may not have other ACARS-based air traffic services.

12.1.4	The continuation of these other ACARS-based air traffic services, after the availability of ATN, should be discouraged as the ATN is standardized by ICAO and, therefore, is internationally recognized as the aeronautical telecommunication network serving the needs of the aviation industry.

12.2	Considerations for other ACARS-based air traffic services

12.2.1	The temporary operational use of other ACARS-based air traffic services is the responsibility of the States and/or regional planning and implementation groups.

12.2.2	The operational safety, performance, security, and interoperability requirements need to be defined by the State or region for specific operational objectives.

12.2.3	The operational use of other ACARS-based air traffic services should take into account:

a)	Non-ATN and ATN aircraft operating in a common ATN airspace.

b)	Non -ATN aircraft operating on the same flight in both ATN and non-ATN airspace.

c)	Different levels of operational requirements in different kinds of ATN airspace.

12.3	Consideration on technical issues on continued support for other ACARS-based air traffic services.

12.3.1	The technical implication of the use of other ACARS-based air traffic services should be evaluated to the acceptance criteria as defined by the operational requirements for intended operational objectives.  The evaluation should consider the applications and the communication services.

12.3.2	Consideration should be given to avoid having two applications for the same operational purpose interfacing the pilot or controller in different ways.  ‘Operationally significant differences’ can not be tolerated for either the pilot or the controller.

12.3.3	Communication services should meet specific communication performance requirements, including requirements on integrity in order to satisfy operational requirements on ATS data link applications.  As it is not expected that ACARS can meet the operational communication performance requirements (see para. 12.2.2), the use of ACARS for air traffic services should be operationally constrained.

12.3.4	The continued use of other ACARS-based air traffic services should consider the feasibility associated with the certification and regulatory criteria for each option.