ATNP/WG1 Appendix F

Initial Application Set for the CNS/ATM-1 Package

At its meeting 17-20 October 1994 in San Diego, ATNP WG1 agreed the following list of ATN Applications to be developed as SARPS and Guidance Material by ATNP WG3 for the CNS/ATM-1 package (1996).  This is based on:

·	User need (airline-perceived cost/benefit)

·	Availability of Operational Requirements

·	Availability of supporting technical material

·	Technical feasibility (to build and validate in the time scale)

| ATN Application (A/G or G/G) | ICAO sourced Operational Requirements | Supplementary Material | Technically Feasible |
| --- | --- | --- | --- |
| ADS Contract Est. & Position Reporting | ADS Manual | ARINC 745-2 RTCA DO 212 | Yes |
| Controller-Pilot Communication (Note 1) | ADS Manual | RTCA DO 219 | Yes |
| Context Management (Note 2) | Implicit in ADS Manual | RTCA DO 223 | Yes |
| Flight Information Services (Note 3) | None at present (with ADS panel) | SC169 working draft | ? |
| Message Handling Services | ICAO docs for fixed services | Draft MHS Manual (in ATNP 1 report) | Yes |
| Inter-centre coordination (Note 4) | ADS Manual | ADS Manual | Yes |

Note 1

CPC is a generic term which may describe a number of closely related applications, differing only in the definition of message sets.  The terms `Controller-Pilot Data Link Communications', `Direct Controller-Pilot Communications' and `Two-way Data Link' have all been applied to this application set.   Pre-departure clearance needs to be included.

Note 2

Context Management is a systems application necessary to support logon functionality associated with applications such as ADS and CPC.

Note 3

Flight Information Services encompasses a set of related applications, some of which may be seen technically as part of the CPC set of applications, others of which may have more demanding information definitions.  For example, ATIS, whilst considered part of FIS, could be achieved using the generic approach of CPC with a new message set.  It is left to WG3 to decide what FIS applications, as advised by the ADS Panel, can be incorporated in the CNS/ATM-1 package.

Note 4

Inter-centre coordination may in the future cover a range of information interchanges, but for the CNS/ATM-1 package is seen as limited to support the ATS Inter-facility Data Communications requirements of the ADS Manual.

Disclaimer

Nothing in this flimsy should be interpreted as a directive that other sources of information are excluded, or that the identified sources are endorsed by WG1.  WG3 must make its own decisions on valid information sources for production of SARPS and Guidance Material for the applications identified above as part of the CNS/ATM-1 package.