ATNP/WG1/-WP/3-21

SUMMARY

This paper reports the progress made by WG2 since its 3rd meeting in March 1995.  Since then WG2 has held its 4th and fifth  meetings in May and July respectively.  Part of the 4th meeting took place jointly with WG3 in order to address the QoS/Priority Issues arising out of the Toulouse ATNP meetings.

(Presented By Rapporteur WG2)

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WG1

Banff, Canada,

9th - 13th  October 1995

WG2 Status Report (ATN Internet WG)

# 1.	Introduction

1.1	This report

	briefly recaps the results of the 3rd WG2 meeting (which preceded the Toulouse WG1 meeting);

	summarises the results of the 4th WG2 meeting (Fairfax, May 95);

	summarises the results of the 5th WG2 meeting (Rome, July '95);

	summarises the future work that WG2 plans  to undertake in order to discharge its responsibilities.

1.2	Formal WG2 Reports are submitted to the atn-internet-technical mailing list normally within two weeks following conclusion of the meeting.

# 2.	Brief Recap of  the 3rd WG2 Meeting (March 1995, Toulouse)

	Joint sessions were held with WG3 on QoS and Priority Issues (Note: these ultimately resulted in WG1/2 Flimsy #3 (Appendix D to Minutes of  WG1/2 Meeting).

	WG2 created the “CNS/ATM-1 Internet SARPs Editorial Committee” to expedite the development of the Internet SARPs.

	Meeting agreed that the “optional Non-Use of IDRP” would be permitted for CNS/ATM-1 and that IDRP would be recommended for implementation in avionics systems from mid 1999.

	Meeting agreed on the need detail proposals to re-structure the Internet SARPs for further consideration at the July meeting.

	Meeting resulted in Version 2.0 of the draft Internet SARPs & Guidance Material

	A summary of the Toulouse WG2 meeting was presented to WG1 in WG1\WP2-28.

# 3.	WG2 Issues from 2nd WG1 Meeting (March 1995, Toulouse)

	Continuing discussions that had taken place between WG2 and WG3 in the preceding week  on QoS issues WG1 developed a set of user requirements (titled “Requirements placed on the ATN Communications Service by Air/Ground Applications”)  documented in Flimsy #3 of the WG1 meeting  (Appendix D to the WG1/2 Meeting Minutes) and tasked WG2 and WG3 to develop the CNS/ATM-1 draft SARPs taking into account these requirements.

# 4.	Summary of  4th WG2 Meeting (May 1995, FairFax)

The May WG2 meeting:

	Adopted a solution to support the traffic type policy requirements as defined in WG1/2 Flimsy #3 (Appendix D to the WG1/2 Meeting Minutes).  For AINSC applications the solution supports the "subnetwork preference" requirements of IATA.  For ATSC applications the "subnetwork prefernce" approach was considered not to be optimal for such applications since the unavailability of the required subnetworks would result in the message being discarded.  Instead the joint meeting agreed on a  "class of communications service" approach for such applications whereby the application would specify a class of service ("a" through "h" inc.) and the internet would try its best to provide the required service.  The unavailability of the required service would result in the next best available service being offered.

	Agreed that no dynamic QoS based internet routing will be defined for the CNS/ATM-1 Package;

	Jointly agreed with WG3 a high level structure for the CNS/ATM-1 Package draft SARPs and guidance material;

	Agreed that the WG2 should endeavour to deliver stable CNS/ATM-1 draft Internet SARPs to ICAO for translation following its February 1996 meeting

	Jointly agreed with WG3 that ICAO consider the use of the “Word” word-processor to facilitate the processing of ATNP WG submitted material;

	Agreed that WG2 deliver a high level description of subjects to be considered for inclusion in the Internet component of the CNS/ATM-2 Package to WG1;

	Jointly requested with WG3 that the ADSP provide CNS/ATM-2 Package operational requirements by June 1996 to ATNP at the latest.



	Agreed that the Mobile SNCDF local reference compression technique be mandatory in the CNS/ATM-1 Internet Package;

	Agreed that no Congestion Management mechanisms/procedures be incorporated in the CNS/ATM-1 Internet definition unless current validation/simulation activities demonstrate tangible benefits;

	Revised the “Toulouse agreement” relating to IDRP support in airborne BIS’s by changing the recommendation to support IDRP in mid 1999 to a mandatory requirement.

	Conducted a detailed review of ATNP/1-WP/59 (VDL..), forwarded a set of detailed comments to the Panel Secretary for submission to AMCP and concluded that the VDL subnetwork is compliant with the ATN SARPs;

# 5.	Summary of 5thWG2 Meeting (July 1995, Rome)

The July WG2 meeting:

	Agreed to submit draft Part 1 of the CNS/ATM-1 SARPs to the October WG1 meeting following submission of the draft to the August WG3 SG2 and SG3 meetings in accordance with the Fairfax meeting resolution on high level SARPs structure;

	Thanked the CISEC for producing Version 2.1 of the draft Internet SARPs (Part V) as tasked and discontinued that Committee.

	The meeting reviewed the Fairfax WG2 decision to mandate IDRP in airborne BISs  from mid 1999 and agreed that the requirement should revert to a recommendation.  Following advice from the Panel Secretary the Version 3.0 of the applicable SARPs recommendation does not now include any date.  The following is the applicable extract:

“Recommendation. -- All ATN airborne BIS’s should support the use of ISO 10747.

Some States may require that aircraft operating in their airspace, after July 1999, and desiring ATSC services via the ATN support the use of ISO 10747, as specified in Appendix 11.”

	The meeting also reviewed the “ATN Priority”  table and agreed two significant changes which have been forwarded to the Panel Secretary for submission to SICASP and AMCP.  The first of these changes related to assigning CLNP priority values 10 and 11 as “Normal flight safety messages” and “High priority flight safety messages” respectively.  The second change related to agreeing that all message categories with CLNP priority of 5 or higher represent communications pertaining to flight regularity and safety of flight.

·	The meeting had considerable discussion on the need for a Congestion Management solution in the CNS/ATM-1 internet and the applicability of a proposed existing solution to a mobile environment such as the ATN.  The meeting agreed not to include any solution into the draft being developed and requested interested participants to conduct further simulation of the proposed solution and should results prove to be positive then propose the solution for incorporation into the draft SARPs in Banff.

	Conducted a detailed review of  Version 2.1 of the draft Internet SARPs (Part V) and agreed that the resulting draft (Version 3.0 - titled "Internet Communications Service") be baselined for validation and that any further changes to the draft be managed in accordance with CCB procedures.

	Due to their late submission the meeting deferred a limited number of proposed changes to the draft SARPs to the CCB process for further consideration.

	Due to the limited time available was not in a position to conduct a detailed review of the version 2.1 guidance material, Version 3.0 does not therefore contain any guidance material.  However, the meeting concluded that the existing content and format of the version 2.1 guidance material was not appropriate and agreed on a new high level structure.

# 6.	Future Work

	Finalise CNS/ATM-1 Internet Communications Service draft SARPs (Part V).  Note that since Version 3.0 has been baselined only changes to existing requirements that can be demonstrated to be absolutely necessary as a result of on-going validation activities will be incorporated into the SARPs.  Editorial/format changes are not bound by this restriction.

	Finalise CNS/ATM-1 Internet Communications Service guidance material (Part V).  A significant amount of effort is required to complete the guidance material.

	Finalise CNS/ATM-1 Inter SARPs Validation Report.

	The October WG2 meeting will plan its future meeting schedule which at this time includes the two meetings scheduled for January/February 1996 and June/July 1996.

·	Identify CNS/ATM-2 Internet requirements (e.g. Systems Management, Security).