ATNP/WG1

IP/2

9 to 13 October 1995

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 1

Common ICAO Data Interchange Network (CIDIN) compatibility with the Aeronautical Telecommunication Network (ATN)

Presented by Leon Sayadian (Federal Aviation Administration)

Prepared by the Center for Advanced Aviation System Development (CAASD)

SUMMARY This paper discusses the CIDIN SARPs, and incompatibles between the CIDIN architecture and the ATN architecture. It is suggested that these incompatibilities be taken into account before a WG1 recommendation for integration of CIDIN into the ATN architecture.

## 1.0 Background

In March 1995, the Aeronautical Telecommunication Network Panel (ATNP) Working Group 1 discussed a proposal to eliminate the CIDIN SARPS [1] from the ATN Manual.  AWG 1 deliverable "CIDIN as a subnetwork of ATN" was identified.  This paper provides a high level description of the CIDIN architecture, compares the CIDIN and ATN architecture, and suggested that the incompatibilities be discussed before WG 1 recommends integration of CIDIN into the ATN architecture.

## 2.0 Discussion

International Civil Aviation Organization (ICAO) work on the Common ICAO Data Interchange Network (CIDIN) was initiated to provide a new, transparent data communications with greater performance and capacity than the existing Aeronautical Fixed Telecommunication Network (AFTN).  While CIDIN was designed to support the existing AFTN services, it was also designed to provide new and enhanced services.  The ICAO Automated Data Interchange Systems Panel (ADISP) first met in February 1969, and completed the CIDIN Standards and Recommended Practices (SARPS) in October 1980.  The following subsections summarizes the CIDIN protocols and operations as described in ICAO Annex 10, Volume 1 and Volume 2 [2].

## 2.1 CIDIN architecture

Applications in centres may communicate to applications in other centres via CIDIN.  The CIDIN architecture assumes that there are three types of centres: entry, transit and exit.  Entry centres refers to the centre that has a message to transmit to another center.  Transit centres forward messages between the entry center and the destination centre.  The exit centre is the destination centre for a message. Centre applications envisioned include Air Traffic Control (ATC) processor exchanges, flight information services, flight briefing services, flight plan systems, search and rescue (SAR) systems, and fault reporting

The CIDIN architecture was based on the ISO reference model and supports the physical through transport layers.  CIDIN supports these layers with the following protocols:

1. Link Access Protocol Balanced (LAP-B) as specified in CCITT Recommendation X.25 -1980 [3]),

2. X.25 Packet Layer Protocol as specified in CCITT Recommendation X.25 -1980,

3. CIDIN Frame Protocol and

4. CIDIN Transport protocol level

These protocols, in support of entry, relay and exit centres, are shown in Figure 1.

Figure 1 CIDIN Protocols

Information concerning the CIDIN data link layer through transport layer are provided in 2.1 through 2.4.

### 2.1.1 Data Link Protocol

Annex 10 supports the link level procedures as described in ITU CCITT Recommendation X.25, Section 2, Yellow Book (1980).

### 2.1.2 CCITT X.25

CIDIN supports Permanent Virtual Circuits (PVCs).  In terms of packet types, CIDIN supports Data, Receive Not Ready, Reset, and Restart packets.  Operations concerning the receipt and transmittal of these packet types are described in Annex 10. It was recommended that the CIDIN centre select the following options, if available:

1. Maximum user data field of 256 octets,

2. Packet sequence modulo 8,

3. Local significance of packet sequencing numbers, clearing procedures, and reset procedure,

4. Default window size of seven and

5. LAPB data link control procedures.

### 2.1.3  CIDIN Frame Protocol

The CIDIN Frame Protocol was designed to enhance the functionality of the ISO network layer with specific services to support the needs of the aeronautical community.  In particular, it provides for:

1. 	Frames to be sent to multiple exit addresses

2. 	Frames to indicate CIDIN unique priority levels (low, medium, high, highest)

CIDIN Frame header formats, methodology for routing and relaying of CIDIN frames, selection of PVCs for transmission of frames, and error conditions are described in Annex 10.  Fields in the frame header include the Message Priority Indicator (MP) and the Exit Address(es) (Ax).

### 2.1.4 Transport Protocol

CIDIN provides a unique transport layer protocol which supports:

=	Messages length 2119

=	Message Acknowledgments

=	Recovery of lost messages

=	Error procedures for invalid messages

=	Conversational traffic (dialogues)

=	Network management messages

CIDIN transport headers are defined, as well as associated transport related procedures for both entry and exit centres in the CIDIN SARPS. Fields in the transport header include the Message Identification Number (MIN), CIDIN frame sequence number (CFSN), Entry Address (AE), and network acknowledgment indicator (NA).

## 2.2 Comparison of CIDIN to ATN architectures

Figure 2 compares the ATN Architecture to the CIDIN architecture.

Figure 2: CIDIN and ATN Architectures

Note 1:	The ATN Internetwork SARPS does not specify requirements for the SNaCP, but requires ISO/IEC 8473-3 if ISO 8208 is supported.

Note 2:	When ISO 8208 is supported, then ISO 7776B should be supported for the link layer.

From this table, it can be readily observed that:

=	ATN and CIDIN transport protocols are incompatible

=	ATN and CIDIN network layer protocols are incompatible

The detailed areas of incompatibility between the ATN and CIDIN architectures are summarized in Table 1.

|  | ATN | CIDIN | Compatible? |
| --- | --- | --- | --- |
| Transport Layer | ISO/IEC 8072, ISO/IEC 8073 | CIDIN unique Transport Protocol | No |
| Network Layer SNICP SNDCF SNAcP | SNICF: ISO/IEC 8473-1 ISO/IEC 9542 (ES-IS) ISO/IEC 8648 ISO/IEC 10747 (IDRP is required for BISs only) ISO/IEC 8473-3 (SNDCF) Note 1 | CIDIN Frame Protocol Not supported Not Supported Not supported Not supported CCITT X.25 - 1980 (PVCs only) No security or priority features supported | No 1984, 1988 are backward Compatible to 1980, but new option/facilities were provided |
| Link Layer | Not specified in ATN SARPS | LAP-B | N/A |
| Transport Layer Addressing | ATN-Unique | CIDIN unique (See Note 2) | No |
| Network Layer Addressing | ATN unique | CIDIN Unique (See Note 3) recommended | No |
| Security | ATN Unique (Traffic Types) | None | No |
| Priority | 14 values supported in ATN SARPS | 4 values are supported: 00 Low CIDIN Priority 01 medium CIDIN priority 10 high CIDIN priority 11 highest CIDIN priority | Priority values must be mapped |

Table 1: Comparison of CIDIN and ATN Architectures

Note 1: While the ATN SARPS does not require the support of X.25, if ISO 8208 is supported, then support for switched virtual circuits (SVCs) are required as part of the SNDCF.

Note 2: In the CIDIN transport protocol, the addressing field is variable in length.   The number of octets used is indicated in bits 1-4 of the first octet in the address.

Note 3: It is recommended that the CIDIN frame protocol support a hierarchical addressing scheme based on the following:

First Octet:  CIDIN Centre Code

Second Octet:	National Subcentre Code

Third Octet	Organization code

Fourth Octet	Station Code

These incompatibilities must be taken into account to support the integration of CIDIN over ATN.  To integrate CIDIN into the ATN architecture, SICASP has recommended that the CIDIN SARPS be modified to support an interface that complies with ATN protocols.  However, to support this recommendation as documented by the SICAS Panel,

1. The CIDIN SARPS should support an interface to the subnetwork layer,

2. CIDIN SARPS should support SVCs.

However, this proposal does not consider the forward compatibility concerns with the standard conforming to Recommendation X.25 (1980) versus the later versions (1984, 1988).  Specific differences between these X.25 versions are discussed in 2.2.1.

### 2.2.1 Subnetwork Differences

The CCITT X.25 (1984 and 1988) standards are backwards compatible to the CCITT X.25 (1980) standard.  However, the ATN Internet may require newer X.25 options and facilities that are not supported in CIDIN.  Table 1 shows the differences between the 1980, 1984, and 1988 standards.

| Feature | CCITT X.25 1980 | CCITT X.25-1984 | CCITT X.25 1988 |
| --- | --- | --- | --- |
| Datagram service | Yes | No | No |
| Fast Select option for SVCs | No | Yes | Yes |
| Calling and Called Address Extension | No | Yes | Yes (See Note 1) |
| Support for Modulo 128 frame sequence numbering | No | Yes | Yes |
| Support for 32768 bits in an I Frame (N1) | No | Yes | Yes |
| Maximum User Data Field lengths in Data packets of 2048 and 4096 octets | No | Yes | Yes |
| Facility Field lengths from 64 to 109 octets | No | Yes | Yes |
| On-line Facility Registration Facility | No | Yes | Yes |
| Local Charging Prevention Facility | No | Yes | Yes |
| Network User Identification Facility | No | Yes | Yes |
| Charging Information Facility | No | Yes | Yes |
| Hunt Group Facility | No | Yes | Yes |
| Call Redirection and Call Redirection Notification Facility | No | Yes | Yes |
| Called Line Address Modified Notification Facility | No | Yes | Yes |
| Transit Delay Selection and Indication Facility | No | Yes | Yes |
| CCITT-specified DTE facilities to support priority and protection | No | No | Yes |
| Addition of the call deflection capability consisting of two optional user facilities (call deflection subscription and call deflection selection | No | No | Yes |
| Revision of the Network User Identification (NUI) capability to explicitly distinguish two optional user facilities (NUI subscription, NUI selection), | No | No | Yes |
| Revision of the Network User Identification (NUI) capability to add a new optional user facility for NUI override | No | No | Yes |
| Revision of the RPOA text to explicitly distinguish two optional user facilities for RPOA subscription and RPOA selection | No | No | Yes |
| Modification of the CUG and CUGOA selection facilities use of basic and extended formats | No | No | Yes |
| Completion of the specification of actions to be taken by the DTE on receipt of errored packets. | No | No | Yes |
| Addition of the throughput class value of 64 kbits/s | No | No | Yes |
| Addition of three new diagnostic codes | No | No | Yes |
| Modifications to diagnostic code table covering application of diagnostic codes to packet types | No | No | Yes |
| Modification to the state table covering call collisions | No | No | Yes |
| Addition of text indicating DTEs may be tolerant to received unassigned or reserved code points in parameter fields to facilitate possible latter extensions | No | No | Yes |

Table 1: Differences between CCITT X.25 Standards

Note 1: Coding of the Calling and Called Address Extension facilities were modified in the 1988 version from the 1984 version.

Depending on the features supported in the ATN internet, an X.25 gateway may be required between the ATN internet and CIDIN .  ISO 8878 provides the requirements for an SNDCF for X.25 (1980).  Appendix B provides guidance on providing internetworking with a relay system between X.25/PLP-1984 and X.25/PLP-1980. The operation of the relay is shown in Figure 3.  Note that to support interactions between an ATN application and a CIDIN application, since the layer 3c and 4 protocols are different, a gateway to allow interaction between these protocols would be required.  Methods to support CIDIN to ATN application interactions are not discussed in this paper.

Figure 3: X.25 Gateway

Given these factors, the SICASP recommendation may require more investigation than originally envisioned.

## 3.0 conclusion

There are incompatibilities between the CIDIN and ATN architecture in the transport layer, network layer, addressing, security and priority areas.  To support the CIDIN architecture, there are a number of outstanding technical issues addressed in this paper associated with these incompatibilities. It is suggested that these incompatibilities be taken into account before a WG1 recommendation for integration of CIDIN into the ATN architecture is given. This will ensure a seamless interaction between CIDIN and the ATN.

# List of References

1.	ICAO Standards and Recommended Practices (SARPS) for the Common ICAO Data Interchange Network (CIDIN), dated 1981.

2.	ICAO Annex 10 Aeronautical Telecommunications, Fourth Edition, April 1985.

3.	CCITT Recommendation X.25, Interface between Data Terminal Equipment (DTE) and Data Circuit Terminating Equipment (DCE) for Terminals operating in the Packet Mode and Connected to Public Data Networks by Dedicated Circuit, dated 1980.

# Glossary

Ax		Exit Address

AE		Entry Address

AFTN	Aeronautical Fixed Telecommunication Network

CFSN	CIDIN frame sequence number

CIDIN	 Common ICAO Data Interchange Network

ISO	International Civil Aviation Organization

LAPB	Link Access Protocol -Balanced

MP		Message Priority Indicator

NA		Network Acknowledgment Indicator

PVC	Permanent Virtual Circuit

SARPS	Standards and Recommended Practices

SICASP	SSR Improvements and Collision Avoidance Systems Panel

SSR	Secondary Surveillance Radar