ATNP WG-1/3  WP-6

Aeronautical Telecommunications Network Panel

A meeting of the Systems Planning and Concept

Working Group 1

October 9 - 13, 1995

(Banff, Canada)

WORLD WIDE PLANNING DOCUMENT

FOR THE EVOLUTIONARY

IMPLEMENTATION OF THE ATN

(Prepared by: Benoît Gosselin and Sherry Cosgrove)

(Presented by Benoît Gosselin)

The attached paper contains a proposed draft of a World Wide Planning document for the implementation of the ATN.  It follows the guidelines from Package-1 SARPs and FANS II/4 documents for the systems to be used, concepts and schedules.  The meeting is invited to comment on the proposed draft document and suggest modifications and improvements to the document.

1. Introduction

At WG1/2 in Toulouse, an expanded Table of Contents for the World Wide Planning document for the implementation for the ATN was presented and accepted as a WG1 deliverable WG1-06.

2. Discussion

The attached appendix contains a draft of the World Wide Planning document for the implementation of the ATN.  It has been developed following the proposed Table of Contents.

3. Recommendation

The meeting is invited to comment on the proposed draft and suggest modifications and improvements to ensure that the final document is within its scope and acceptable to all member States.

1. Foreword - Document Overview

The objective of this document is to provide a high level planning reference for the worldwide evolutionary implementation of the Aeronautical Telecommunications Network (ATN).

## Document Overview.	  This document is in eleven (11) parts:

Chapter 1:  	the document overview.

Chapter 2:  	an executive summary of the topics presented in the World Wide Plan.

Chapter 3:  	provides background information on the ATN, the shortcomings and the future outlook.

Chapter 4: 	describes the planning considerations necessary for a successful transition to the ATN - includes the objectives of the plan and research, development, trials and demonstrations.

Chapter 5: 		describes the current and future ATM data requirements.

Chapter 6: 	outlines the ATN architecture based on the OSI protocols. router and gateway functions are also discussed.

Chapter 7: 	describes the evolution process from present day ATM including all affected systems.

Chapter 8:  	looks at the transition considerations from a systems approach.

Chapter 9:  	describes the different alternatives available for migration to the ATN.

Chapter 10:  	the four (4) different transition paths (unilateral, bilateral, trilateral and mixed).

Chapter 11:  	describes the administrative and management issues including ownership, security and routing.

EXECUTIVE SUMMARY

The Aeronautical Telecommunications Network Panel (ATNP) is responsible for the development and technical aspects of the Aeronautical Telecommunication Network (ATN).  This World-Wide Planning document has been developed with the guidance of members of ATNP to define the migration procedures from the existing systems and methods of Air traffic Management (ATM) to the robust internetwork of the ATN.  The internetwork is comprised of the interconnection of computers with routers and gateways via real subnetworks.

To overcome the present limitations in Air Traffic Service (ATS) and to optimize airline operations on a global basis, the ATN is necessary for future ATM and ATS services.  This network has been designed to be capable of  ground/ground networking systems as well as air/ground mobile networking solutions.  The overall purpose of this document is to provide individual States and groups responsible for the development and implementation of the ATN with an overall planning document to promote the harmonious migration towards the ATN internetwork.

The strategic objectives of the ATN network infrastructure will be to provide state of the art communications for the following user groups: 1) Air Traffic Services (ATS), 2) Aeronautical Operational Control (AOC), 3) Aeronautical Administrative Communications (AAC), and 4) Aeronautical Passenger Communications (APC).  The total benefit of the ATN will only be attained once all States and regions have implemented the network infrastructure.  This will occur on a flexible timetable as the World Wide Plan allows each State and region to implement the ATN in a series of steps at their own pace. The document is not intended to substitute for regional plans, but to provide general guidelines that serve as the basis for ATN related regional planning activities and the production of the detailed implementation plans.

The realization of a system capable of satisfying the above principals requires the adaptation of existing procedures and facilities, as well as the development of new ones.  This document identifies the structure of the future data communication requirements for air traffic management (ATM) systems.  The Standards and Recommended Practices (SARPs) for the ATN are due to be completed during the middle term of the Future Air Navigation System (FANS) Evolution plan.  The SARPs will provide guidance for a variety of alternative configurations for the planned network and application environments to support the transition, and interoperability between the existing and future ATM systems.  These guidelines should be used by Regional Planning Groups and individual States to migrate to the ATN.

There are several provisions in the SARPs to produce a graceful transition to the ATN, such as gateways.  The gateway provides each State the flexibility for a mixed AFTN and ATN environment and the freedom to implement its own individual transition schedule.  This document illustrates the various gateway configurations available to States.

The ATN router is another component to be utilized for the expansion of the ATN internetwork.  Ideally States should have the ability to inter-connect new OSI compliant ATM processors directly to the ATN with the deployment of these routers, by-passing the need to use any non-ATN or proprietary protocols.  Policy routing principals need to be established between administrations when ATN routers are deployed between States.  The inter-domain routing exchange protocol controls which inter-domain exchanges are permitted.  ATN connectivity, which is dependent upon which strategy (i.e. Unilateral, Bilateral, Trilateral, or Mixed ) is selected between neighboring States, is an example which requires early consideration and coordination.

Alternative considerations, administrative matters and network management information essential for each State to address the numerous aspects of this development effort are also identified.  While implementation details may vary around the world, the different elements of the ATM systems must be treated as integral parts of the global systems.

3. Introduction

3.1. Background

## In the early 1980's, civil aviation providers and users recognized the increasing limitations of the present Communications, Navigation and Surveillance (CNS) and Air Traffic Management (ATM) systems.  They also realized the need to make improvements to overcome the present limitations and also to meet the future needs.  Thus the council of ICAO established the Special Committee on Future Air Navigation Systems (FANS) in 1983 to study new concepts and to recommend a system that would overcome the present foreseen problems and take aviation into the 21st century.

The concept of the Aeronautical Telecommunication Network (ATN) as a world-wide data network enables aircraft to communicate via satellite, VHF, ADS and Mode-S SSR air-ground data links with ground Air Traffic Control (ATC) and Aeronautical Operational Control (AOC) centres.  In addition, high capacity data exchange is provided between ATM systems, and also between systems operated by airline operating agencies and service providers, and ATM systems.  A number of panels of technical experts were appointed by ICAO to develop the standards and recommended practices for the various parts of the ATN.  Figure 1-1 shows the ATN aeronautical data communications environment conbcept that consists of avionics, air/ground and ground/ground subnetworks.  The separate subnetworks offer the greatest flexibility in design, management, and control of each independent subnetwork and allow each subnetwork to be optimized for use in its own environment.  This document covers planning and transition aspects of the ATN global network.

The present ground-based data communications system comprises discrete Aeronautical Fixed Telecommunications Network (AFTN) circuits linking States plus dedicated point to point data circuits between air traffic control and flight data processing systems.  The AFTN provides a messaging service between fixed locations using character oriented protocols, or in some parts of the world the Common ICAO Data Interchange Network (CIDIN) protocols.

Figure 1-1 ATN Data Communications Environment

The existing Aeronautical Fixed Telecommunication Network (AFTN), although implemented globally, has limitations in being able to support the ground communication demands required to support the ICAO CNS/ATM systems in the future.  An upgrade path has been identified by the Aeronautical Fixed Service (AFS) Systems Planning for data interchange Panel (ASPP) which will enable the AFTN to be embraced within the ATN.   This will be accomplished through a gateway setup that will be required by the new ATN systems.

The ATN has adopted the OSI-based data communication system that has several benefits such as:

1) inter-operability between different computer systems and peripherals;

2) exchange of information on a global basis; and

3) freedom to choose innovative products.

These benefits will aid the ATN in eventually operating as a homogeneous global network carrying all forms of aeronautical communications.  As the full implementation of the ATN will be a complex process,  in order to minimize risks, the ATN will start up in small transitory steps.   These steps will consist of cost efficient ATN setups by States that have no current network solutions.  They will be able to communicate with existing AFTN sites through recognized gateways.

3.2. Purpose of the Document

The purpose of this document is to provide an evolutionary implementation planning document for the ATN.

This document is intended to provide guidance to States, Regional Planning Groups, Air Carriers and other interested parties.  This will assist them with the detailed  implementation planning necessary to upgrade the current ATM system to a fully OSI compliant bit-oriented network capable of meeting the requirements of the Future Air Navigation System (FANS).

3.3. Shortcomings of the Present System

## ATC is a primary component of the ATM.  Automation of ATC, which is necessary to handle the continuously increasing volume of traffic, has until now been based mainly on a better utilization of the limited information available on:

1) the flight intention as provided in the flight plan and its updates; and

2) the position of the aircraft as provided by current surveillance means.

The shortcomings of the present system can be explained in two categories; Air/Ground and Ground/Ground Domains.

### Air/Ground Information Flow	.  Despite the introduction of Secondary Surveillance Radar (SSR), there still remains a severe gap in the information flow between air and ground.  In addition, large parts of the world lack reliable coverage by the present CNS systems.  Without proper measures, the efficiency further decreases in the future because of predicted growth in the traffic and the widening gap between airborne and ground capabilities.

### Ground/Ground Information Flow	.  The implementation of medium speed AFTN circuitry has provided improvements to data integrity and speed of service for traditional message traffic.  Current automation activities employ direct physical links or X.25 based networks for connectivity.  This structure does not support the effective interconnection of ATM processors requiring direct logical connectivity between applications.  The current configurations lack the flexibility and logical connectivity required to support planned CNS system implementations.  Essentially, the ground/ground networks are progressing along two or more parallel paths;  one for store and forward message exchange, and one for interactive data exchanges.  The later capability should be employed as the initial building block for progressing ultimately toward the ATN infrastructure.  An interactive data exchange environment needs to be supported for G/G prior to deployment of the initial ATN A/G elements.

3.4. Future Environment

What must happen next to assure that the global implementation proceeds at a brisk pace?

The ATN is quickly gaining momentum.  It is now the responsibility of the States, International Organizations and ICAO to ensure that the CNS/ATM implementation happens in a timely and cost effective manner.  The Standards and Recommended Practices (SARPs) are to be completed June 1996.  These SARPs answer all technical questions and dilemmas to help the transfer to, or the setup of, the  ATN within any State.

It is necessary that States design plans not only on what is to be done,  but how and when it must be accomplished.   Those who will approve the investments will first need to be convinced that the investments will be returned via benefits achieved, and in a reasonable time period.  ICAO already has machinery which can contribute to the implementation of the global plan which should be used as far as possible so as to avoid unnecessary duplication.

3.5. Description of the ICAO CNS Concept

The ICAO concept for CNS overcomes the shortcomings of the current system. It builds on existing systems and technologies without making them obsolete.  Its improvements can be implemented in an evolutionary transition on a local scale as requirements dictate, while at the same time meeting the needs for global coordination.

In the early 1980s, the International Civil Aviation Organization (ICAO) recognized the increasing limitations of the present air navigation systems and the need for improvements to take civil aviation into the 21st Century.  In 1983, ICAO established a Special Committee on Future Air Navigation Systems (FANS) with the task of studying, identifying and assessing new concepts and new technology, and making recommendations for the coordinated evolutionary development of the air navigation for the next twenty-five years.

The original FANS committee completed its task in 1988 with a system concept for an improved CNS capability supported primarly by satellite data transfer functions.  CNS requires a high level of international cooperation and effort between the civil aviation administrators, international organizations, service providers and users.

The ICAO concept allows for a judicious mix of satellite technology and the best of line-of-site system to achieve an over-all optimum result.  The essential elements of the new CNS/ATM system are:

Communications:  It is expected that air-ground and ground-ground data communications will be increasing rapidly.  This will allow for quicker, more efficient linkages between ground and cockpit systems.  ICAO has adopted a communication system architecture which provides a range of capabilities to suit the needs of the air traffic service providers and their users, from basic low speed data to high speed data and voice, through globally integrated air-ground and ground-ground networks.

Navigation:  ICAO adopted the concept of Required Navigation Performance (RNP), which defines a capability rather than a system, necessary for an aircraft to navigate in an airspace segment.  This capability could be met by the implementation of the Global Navigation Satellite System (GNSS).

Surveillance: ICAO adopted the concept of Automatic Dependent Surveillance (ADS), where the aircraft automatically (or on manual command) transmits its position and other relevant information, such as manoeuver intent, speed and weather via satellite or other communication link to the air traffic centre.  The aircraft position can then be displayed in a manner similar to present radar displays.  It also has the potential for use in higher traffic densities as back-up to SSR.

3.6. Air Traffic Management (ATM) Improvements

## In order to fully utilize automated ATM systems, a global data networking infrastructure must be implemented which supports the internetworking of state-of-the-art computer systems operating in fixed ground-based and mobile aircraft-based locations.  The key to success in developing and implementing this new internetwork infrastructure is the recognition that:

- increased use of distributed ATM automation requires an increased level of computer-to-computer data communications, including data communication between aircraft-based and ground-based computers serving fixed and mobile users;

- increased levels of distributed ATM automation requires a more richly-connected and more flexibly-configured data network infrastructure than exists today, both in aircraft-based and ground-based environments; and

- real success in ATM automation can only be achieved when aircraft-based computer systems are designed and implemented as data processing and networking peers to their respective ground-based computers, rather than continuing in their current role as aircraft terminals attached to ground-based hosts.

3.7. Benefits

The benefits of the CNS/ATM system are dependent on several different issues such as the ground infrastructure, implementation of global standards, changing operating procedures, and, above all, the ability to achieve a timely payback for investment.  There are significant improvements that are introduced by the ATN.  The inherent enhancements in communication, navigation, and surveillance will result in improved data handling and transfer among operators, aircraft and air traffic service providers, global navigation and non-precision approach capabilities through GNSS, extended surveillance by ADS, and advanced ground-based data processing.

Indeed, the Air-Traffic Service (ATS) providers will enjoy the superb benefits of the ICAO CNS/ATM system.  They will be able to take advantage of the improved four-dimensional navigation accuracy in providing air traffic services as well as be able to better accomodate an aircraft’s preffered flight profile.   ATS providers and airspace users alike will benefit from improved conflict detection and resolution, automated generation and transmission of  conflict free clearances and adaptation to changing traffic patterns.  They will also benefit from a level of standardization that will facilitate connectivity between ATC systems and support the seamless transition between Flight Information Regions (FIRs).  These benefits will assure a continuing high level of safety for ATC operations, and provide higher traffic density.

In reality, the CNS/ATM system cannot be implemented everywhere on a common time schedule.  In fact, the CNS/ATM systems implementation schedule may well be driven by the communications protocols schedule.  Furthermore, additional benefits in the use of the CNS/ATM system can be summarized as cost savings.  The various areas where costs are improved are:

1) cost avoidance to the service provider;

2) cost avoidance to the aviation industry;

3) efficiency improvements to the aviation industry; and

4) efficiency improvements to passengers.

The benefits in both the functionality and cost areas prove the ICAO CNS/ATM system to be worthwhile and beneficial to the States.

Chapter 4  		Planning Considerations

4.1. Need for an Implementation Plan

The ATN is an extensive global concept involving standards, agreements and international cooperation.  In order for this extensive network to be implemented effectively, a set plan should be designed and followed by the participants.  This can be accomplished through an implementation plan.   The plan will not only provide guidelines and solutions, but it will eliminate overlapped work and promote idea and resource sharing.

The issues that will require planning and coordination for the ATN on a global scale are:

1)   routing;

2)   Quality of Service (QOS);

3)   network congestion and network flow control;

4)   network monitoring;

5)   problem resolution;

6)   technical advice, educational service, and registry;

7)   product certification and conformance testing;

8)   security;

9) accounting management; and

10) regulatory issues.

4.2. Objective of the plan

The objective of the plan is to provide guidance and information to States in the process of implementing the ATN.

4.3. Research, Development, Trials, and Demonstrations (RDT&D)

“To monitor the nature and direction of research and development programmes, trials and demonstrations in CNS and ATM so as to ensure their coordinated integration and harmonization”  - One of the terms of reference of the FANS (Phase II) Committee.

RDT&D must be directed towards making the transition period as effective as possible so that short and medium term benefits can also be achieved.   Currently there are Ground/Ground and Air/Ground trials and demonstrations being tested by different States.  In addition to the tests, the Standards and Recommended Practises (SARPs) have been undergoing updating and review.  It has been agreed that the SARPs for the CNS/ATM-1 package should be validated by May 1996 in order to be adopted by the second ATNP meeting in the fall of 1996.

4.4. Evolution and Transition

On a global scale, the transition to the ICAO CNS/ATM systems will be paced by a series of key events that must be completed for the implementation to proceed.  Several activities are currently being pursued by committees and personnel from different States that will aid in a smooth transition.  The benefits must be presented to the aviation community to be a stimulus for the transition to the new ICAO CNS/ATM systems.  The need for an evolutionary transition is critical for all of the new CNS/ATM systems:

The transition should be carefully planned so as to avoid degradation in performance.  The safety and performance that is apparent today should not be compromised at any point during the transition.

The planning process should ensure that the aircraft will not be burdened with a multitude of electronics during a lengthy transition.  i.e. equipment for both AFTN and ATN.

It is important that the pace of the implementation of the different systems does not vary extensively between States so that it leads to incompatibility between users.  The world-wide coordination of these activities is necessary if the ATN is to be optimized.

The services provided by the CNS should relate closely to the particular level of ATM being used by the State.

Guidelines for transition to the new Communications system

1) The ATN should be implemented in phases;

2) If new application message processors and data link systems are implemented, they should support code- and byte-independent data transmission and be fully compatible with the ATN;

3) During the transition, States should cooperate on a bilateral and multilateral basis to ensure operation of the ATN meets the needs of international aviation and the States; and

4) States should establish procedures to ensure that both security and interoperability aspects of the ATN are not compromised.

4.5. Key Events

The key events with regards to the planning considerations for the ATN are as follows:

1) Validation and Approval of the SARPs (Fall 1996);

2) Detailed information on the functionalities and methodologies of implementation strategies distributed to the States;

3) System implementation by the individual States; and

4) Global use of ATN.

4.6. Institutional arrangements for ATN

The FANS Committee has developed institutional guidelines to ensure that the States and regions maintain the institutional capability of implementing the CNS/ATM system.  The guidelines have been endorsed by the ICAO Legal Committee and appear in the Global Plan.  The guidelines in the global plan are in four sections:

1) general considerations applicable to all systems;

2) considerations specifically applicable to AMSS;

3) considerations specifically applicable to ATN; and

4) considerations specifically applicable to GNSS.

The considerations with regards to the ATN on the institutional factors and aspects from the technical point of view:

1) endorsement of the ATN guidelines and scenarios;

2) acceptance of ARINC specification 622 to provide a migration path to the ATN; and

3) documentation supporting acceptable performance and efficient use of Aeronautical Mobile Satellite Service (AMSS).

4.7. Relationship to other ICAO Planning Activities

The relationship to other activities that are being planned through ICAO are significant.  ICAO has several projects which fall under the three categories of Communications, Navigation and Surveillance.   The milestones for all these activities are scheduled quite closely between now and the year 2000.   Although detailed planning continues to be a regional responsibility, global and regional ICAO activities must be coordinated.

Chapter 5	ATS ATM DATA COMMUNICATIONS REQUIREMENTS

5.1 Current ATM Data Requirements

The primary objective of ATM is to expedite and maintain the safe and orderly flow of air traffic.  An ATM data communications network is required that meets the needs for the interchange of data and allows interoperability of applications for a diverse end user community.  The specific applications , both Ground-Ground and Air-Ground which the ATM environment needs to support are:

- Air Traffic Services (ATS)

- Air Traffic Flow Management (ATFM)

- Operational Meteorological Traffic (OPMET)

- Search and Rescue (SAR)

- Surveillance Systems (Radar, ADS)

5.1.1	 Ground-Ground

ATFM Service.  ATFM is a function of ATS and is a service for air-traffic management, rather than a control function.  ATFM involves adjusting the demand to the system capacity.  This ensures safety standards are maintained, congestion is prevented and available capacity is used as efficiently as possible.  Information to be distributed includes:

a)   Flight Plans;

b)   ATFM Notification Messages (ANM); and

c)   Slot allocation and associated messages.

OPMET Services.  OPMET data needs to be exchanged between originators, database

systems and users.  This information includes:

a)   Meteorological Observations and Reports;

b)   Forecasts;

c)   SIGMET Information;

d)   Aerodrome Warnings; and

e)   Wind Shear Warnings.

Search and Rescue Environment.  Messages are needed to support efforts to locate aircraft

that are missing or overdue.  Although these messages are not broadcast, they have a wide

distribution pattern, and are of a high priority.

Surveillance Data.  There is a requirement to exchange radar data between some States for

operational purposes.  When these data are used for control purposes, the ATN should provide

a high level of performance (e.g., low transit time, high throughput, etc.).  Data can be:

a)   digitized radar data transmitted from radar sensors to radar data processing

system(s); or

b)   processed radar data exchanged between radar data processing systems.

In the future, it can be anticipated that there will be a requirement to transmit ADS originated data.

5.1.2	Air-Ground

Air Traffic Services (ATS).  To ensure the safety and regularity of aircraft movements, the current ATS environment includes two types of safety related services, air traffic control (ATC) and flight information services (FIS) as described in the Procedures for Air Navigation Services - Rules of the Air and Air Traffic Services (PANS-RAC, Doc 4444).

a)   Air Traffic Control (ATC).  This is a safety-related service used to manage the

flow of aircraft movements; to separate and space aircraft in closing situations; and to

expedite aircraft movements as required.  Requirements for ATC services are described

in Doc 4444, Part VIII, paragraph 1.  This service requires prompt delivery of the

following:

1)   Emergency Messages:

i)   distress messages;

ii)  urgency messages;

iii) other messages concerning known or suspected emergencies;

2)   Movement and Control Messages:

i)   filed flight plan and associated update messages;

ii)  co-ordination messages;

iii) supplementary messages;

iv)  control messages; and

3)   Flight Information Messages:

i)   traffic information;

ii)  meteorological information;

iii) operation of aeronautical facilities;

iv)  aerodrome information.

b)   Flight Information Services (FIS).  As stated in Doc 4444, Part VI   Flight Information Service and Alerting Service, information is used to track the progress of flights.  This is a safety-related service designed to aid pilots in anticipating and planning for flight conditions en route (both before and after departure).  These requirements become more important with adverse weather conditions and increased traffic activity.  This service requires prompt delivery of information, but it is not always considered time critical.  The information is as follows:

1)   weather reports and forecasts;

2)   notices to airmen (NOTAMs);

3)   automatic terminal information service (ATIS);

4)   arrival delay advisories; and

5)   pilot reports (PIREPs).

5.2  Future ATM Data Requirements

The operational requirements for a future ATM system should be developed based on the framework of the current operational ATS concept.  Development plans for the future ATM should be closely co-ordinated and take into account user requirements between both the air/ground and ground/ground ATM components.

The operational requirements allow the definition of a detailed functional specification of the ATM system, identifying the necessary ATS functions with related automated applications.  Subsequently the specification and standardization for these applications can be carried out.  The ATM network consists of air/ground and ground/ground components, which are required for all phases of ATM operations and applications.  With the implementation of the ATN internetwork, the information exchanges supported include the ones listed below.

It should be noted that these are examples only and have been used as models to derive data

communication requirements.

5.2.1 Ground-Ground

The additional automated Ground/Ground Scenarios are:

a)   Instrument Flight Rules (IFR) Flight Plan (FP);

b)   FP Correction;

c)   FP Amendment; and

d)   Transfer of Control.

A general purpose aeronautical message exchange protocol with the ability to meet today's requirements, as well as future requirements is required with the following characteristics:

a)   Protocols and procedures which allow existing AFTN applications to make use of

ATN connectivity, and provide for exchange of other types of ATM data over the

ATN.

b)   An operational data transfer protocol which enables character-oriented data and

code and byte independent data exchange.

c)   Capable of communicating with peer protocol entities in various types of ATM

processors (ATS, weather, etc.).

d)   Allows for operational ground-ground data message transfers between aeronautical

host processors interconnected via the ATN internetwork.

e)   May be used for air/ground (AMS) and ground-based (AFS) ICAO data exchanges

if appropriate.

f)   May also be used for ATM data exchanges with non-ATM organizations.

g)   Definition of application protocol data units and associated service data structures

should be defined using abstract syntax notation one (ASN.1).  Transfer syntax

must be implicit within a particular association; thus, this protocol need not

support transfer syntax negotiation in the near term.

Note 1.  Transfer syntax negotiation may be a

future requirement.

Note 2.  This protocol should not preclude use of

application process encoding schemes such as Data

Link Application Coding (DLAC) or Packed

Encoding Rules (PER), while not endorsing such

techniques.

h)   Security mechanisms must be provided, at least as an option, for aeronautical

application service elements.

5.2.2 Air-Ground

Additional Air-Ground scenarios are:

a)   Predeparture Clearance Delivery  Processor generated clearance

delivered to an aircraft prior to departure;

b)  ADS Reporting:  A service for use by Air Traffic Services (ATS) in

which aircraft automatically provide, via a data link, data derived from on-board

navigation and position-fixing systems.  As a minimum, the data include aircraft

identification and four dimensional position.  Additional data may be provided as

appropriate;

c)  Meteorological Reporting:  The report from aircraft of wind speed,

wind direction, static air temperature and turbulence; and

d)  Manoeuvre Control:  Interaction of controller and aircraft which

comprises uplink and downlink messages.

1) List of uplink messages:

INTERROGATIONS

ALTITUDE REQUESTS

LATERAL REQUESTS

SPEED REQUESTS

CONTACT REQUESTS

ROUTE REQUESTS

REPORTS

NEGOTIATION REQUESTS

MISC

2) List of downlink messages:

RESPONSES

ALTITUDE

CROSSING CONSTRAINTS

LATERAL

ROUTE

SPEED

CONTACT

REPORTS

NEGOTIATION REQUESTS

ADVISORY

MISC

## CHAPTER 6	THE ATN ARCHITECTURE

## 6.1 ISO Protocol Architecture Overview

### A) General

The ISOPA is intended to permit heterogeneous data communication systems to be interconnected so as to allow the reliable exchange of messages without regard to the implementation of the networks and physical media through which the messages pass.  The goal is achieved by structuring the interconnected data communications systems so that messages traverse hierarchical protocol layers, with each layer offering a well-defined service to the layer above.  Each layer within an originating communication system performs a function that is paralleled within its peer layer in the destination communication system.  Differences in protocol and media implementations within communicating systems are thus masked, allowing for the open interconnection of those systems. [c3 t 0140]

The ISO OSI reference model (ISORM) defines seven functional layers with responsibilities ranging from control of data transfer on the physical (radio, wire, fibre, etc.) media to control of the application user interface.  Each layer incorporates well defined operations designed to minimize control and information flow across the layer boundaries.  The seven layers are referred to as physical, data link, network, transport, session, presentation, and application.  To carry out its function, each layer may add protocol control information fields to the service-data-unit (SDU) supplied by the layer above.  However, each layer leaves the control information added by previous layers intact, treating it as data to be passed on unaltered.  Figure 6-1 shows the organization of the seven ISO protocol architecture layers.

[c3 t 0150]

### B) Lower Layers

The ISOPA lower layers include the physical, data link, network and transport layers.  These layers form the network-dependent portion of the ISO protocol architecture, and provide an application-independent service interface at the boundary between the network and transport layers. [c3 t 0160] The ISOPA demands that a uniform level of network service be presented to all transport layer processes at this boundary; this requires that a single network layer service interface be adopted throughout a given domain, supported by a common internetwork protocol. [c3 t 0170]

The physical layer controls access to the transmission media which forms the basis of the communication system.  The data link layer manages the operations of the physical layer, and may utilise special error detection or retransmission techniques to achieve acceptable error rates.

The network layer manages the data link layer in order to transfer data between transport layer entities based on global address and Quality Of Service (QOS) information.  The network layer makes routing decisions when redundant data link connectivity is available and segments or reassembles data units as required by underlying data link technology.  The network layer is composed of several subnetwork layers.

The transport layer provides a reliable, end-to-end data transfer service between applications (supported by their upper layer services) residing on separate OSI end systems, irrespective of the quality and nature of the underlying networks and subnetworks.  The transport layer can operate in either connection-oriented or connectionless mode.  The transport service provides the interface to the communication system in the host processor. [c3 t 0190] It is common practice to view the OSI stack as being split at the transport service boundary, such that any implementation of the upper layer (application-related) functionality can operate over any implementation of the lower layer (data transfer-related) functionality.

[c3 t 0200]

### C)Upper Layers

The ISOPA upper layers comprise the session, presentation and application layers.  These layers provide common support functionality to applications which require to interchange information with peer applications residing on remote OSI End-Systems.  The session layer provides dialogue control, checkpointing and synchronisation services.  The presentation layer provides negotiation and control of the format and encoding rules for information exchanged between applications.  The application layer includes common application functions such as controlling the establishment and termination of associations between peer applications, remote operations services, reliable transfer services and common management information services.

### D) Application Processes

In the ISOPA, the application layer is populated by application entities (AEs), which deliver some specific service to applications. [c3 t 0220]  The AEs are the components of the Application Processes (APs) which relate to communication functions within the OSI environment.  The AP implements end-user services, which may themselves be standardised in terms of the structure of information, the procedural use of the communications service, and operational and performance parameters. [c3 t 0230]

[c3 t 0240]

Figure 6-1

ISO BASIC REFERENCE MODEL

6.2 Operational Use of the ATN

For the design of the aeronautical telecommunication network  it is essential to understand how data communications can be integrated into both the airborne and ground systems.  Therefore the operational use of data communications must be defined. The use of data communications for ATS purposes can vary widely. [c2 t 0010] Data transfer from one end user to another end user can be beneficial, or even essential, to certain ATS functions.  In particular, those functions which can increase Air Traffic Control (ATC) system performance, in terms of, for instance, safety, capacity and efficiency, will certainly be incorporated in the ATN system. [c2 t 0020]

6.2.1	Connection Orientation of the Communication Systems

Communication services at any of the seven ISOPA layers may generally be categorised as utilising one of two techniques for the transfer of service data units among communicating processes.  These techniques are referred to as connectionless-mode (CL) service and connection-mode (CO) service. [c3 t 0250] The connectionless-mode approach transfers each protocol data unit (PDU) between end users with no explicit association among the various PDUs required for transfer of the user's message.  Each PDU contains a protocol control information (PCI) field (including the addresses of both source and destination entities and the QOS parameters) and the higher layer SDU (i.e. data content) to be transferred. [c3 t 0260] The connection-mode approach establishes a relationship between communicating processes to logically associate the sequence of PDUs containing each interprocess message.  This requires one transfer of source and destination addresses and QOS parameters, in order to establish a connection.  Data PDUs are then transferred using the connection identifier in place of the addresses and QOS parameters required in the PCI field for connectionless-mode operation.  The connection may be terminated at any time via the transfer of a management PDU requesting “disconnection”.  Connection-mode operation generally requires less network bandwidth for an ongoing peer-to-peer communication session than connectionless-mode operation, since address and QOS information are transferred only once per connection.  On the other hand, connection-mode operation requires more sophistication within the communication nodes, since each node supporting a connection must retain the address and QOS information transferred at the time of connection establishment, as well as state variables indicating the relative status of the packet stream for that connection. [c3 t 0270]

6.2.2	Data Transfer

The identification of the type of data transfers performed on the ATN is important in order to design the appropriate data transfer procedures.  The different types of data transfer are determined by the different applications for which functions are managed by special applications processes.  The different types of application data can be discussed as end-user file data and end-user message data.  The particular method of data transfer used can be described in terms of dialogue types such as instructions dialogue and request dialogue.  To enable the communication network to determine and provide the required service, the end-user data has to be accompanied by quality of service (QOS) parameters and addressing information.

6.2.3	Separation of Communication Functions

The international aviation community is expected to adhere to the separation of communication functions as specified in the Open Systems Interconnection (OSI) Reference Model developed by the International Organization for Standardisation (ISO). [c2 t 0060] The OSI Reference Model permits the development of open communications protocols as a layered architecture comprising seven functionally separate layers. The lower four layers (physical, data link, network and transport) are concerned with providing code transparent communications between communicating Although all four aeronautical user groups can be supported by the ATN, there may be administrative and technical provisions to inhibit user groups from gaining access to certain subnetworks.

6. 3 Organization of the Layers

A) Upper Layers Organization

The upper layers of the ISOPA offer a rich repertoire of functionality for use by communicating applications.  The functionality derives largely from existing ISO upper layer standards, including the services offered by session layer, presentation layer and application layer protocols.

Session layer services are described in ISO standards, and include connection establishment, dialogue control, checkpointing and recovery services, data segmentation, multiplexing, and activity (data flow) management services.  These services can be selected or omitted in a particular upper layer profile. [c3 t 1270]

Presentation layer services are described in ISO standards, and include connection establishment, negotiation of transfer syntaxes for each supported abstract syntax (data encoding rules) and the management of transfer syntaxes when more than one is in use.  These services can be selected or omitted in a particular upper layer profile. [c3 t 1280]

Application service elements (ASEs) are functions within the application layer, which can be invoked by an application to achieve some particular communications function.  Some generic ASEs have been standardised by ISO, while specific ASEs can be defined to serve the needs of a particular community.  Generic and specific ASEs, together with session and presentation services, can be combined and subset to achieve some particular set of functionality, by means of the profiling process. [c3 t 1340]

B) Transport Layer Organization

The OSI Transport Layer service provides transparent transfer of data between Transport Service users.  All protocols defined in the Transport Layer have an 'End-to-End' significance, where the 'Ends' are defined as co-operating transport entities on two ATN End Systems.  The Transport protocol operates only between end systems.  Within the ATN, Transport Layer entities communicate over the ATN using the Network Service provided by the ATN Network Layer Entities.

There are two modes of the transport service, the Connectionless mode Transport Service and the Connection mode Transport Service.  The connectionless mode service allows two transport users to exchange individual datagrams, without flow control or the need to have previously established a connection, but with no guarantee of delivery.  The connection mode service allows two transport service users to negotiate a communications channel with a set of common characteristics, including reliable delivery of data units, and guaranteed (very high probability) order of delivery.

C) Network Layer Organization

The ISO network layer is responsible for providing a uniform service interface for the transfer of data among end systems and intermediate systems (ISs) utilising the ISO protocol architecture.  In order to perform this global role while accommodating the differing protocol requirements of local “real networks and systems” that are used to provide network service, the ISO standards define several sublayers within the ISO network layer.  These sublayers are referred to as the subnetwork independent convergence, subnetwork dependent convergence and subnetwork access sublayers. [c3 t 0280] The existence of these sublayers supports the notion that while a uniform network service interface is crucial to the internetworking of computer systems, diverse media and diverse local environments often demand different technologies of interconnection.

[c3 t 0290]

The subnetwork access sublayer incorporates the subnetwork access protocol (SNAcP), which provides the protocol access mechanism for data transfer between adjacent communication nodes over a particular data link.  The subnetwork independent convergence sublayer consists of an internetwork protocol, routing protocols (RPs), management protocols (MPs) and a relay function.  The internetwork sublayer is responsible for performing relaying and routing functions throughout the collection of real subnetworks forming a global internet.

The two sublayers operate together to provide the uniform ISO network service interface to ISO transport layer processes.  If the internetwork protocol cannot operate directly with the available SNAcP, then a subnetwork dependent convergence protocol (SNDCP) is required.  Depending on implementation choices, this protocol may be associated with either the SNAcP or with the internetwork protocol process. [c3 t 0310]

6.4  ATN Domain Structure

The ATN is designed to accommodate a very large number of ISs, and a structured approach is necessary in order to develop a routing strategy.  In order to simplify the design of large OSI networks, ISO has defined a Routing Framework which allows large scale networks to be designed and built in a modular fashion.  This Routing Framework is applied to the ATN in order to provide the required structured approach to routing.  The OSI Routing Framework, as applied to the ATN, is illustrated in Figure 6.2, and introduces two new functional entities: the OSI Administrative Domain and the OSI Routing Domain.

FIGURE 6.2

ATN DOMAIN STRUCTURE

A) Administrative Domains

An administrative domain comprises the End Systems, Intermediate Systems and subnetworks operated by a single organization or administrative authority.  The communications inside an administrative domain take place with a significant degree of trust between these systems.  On the other hand, inter-administrative domain communications cannot assume a comparable level of trust.

B) Routing Domains

A Routing Domain is a set of ISs and ESs bound by a common routing procedure, namely;

- they use the same set of routing metrics;

- they use compatible metric measurement techniques;

- they use the same information distribution protocol and

- they use the same path computation algorithm.

[c5 t 0170]

A Routing Domain is always wholly contained within a single Administrative Domain, while an administrative domain wholly encompasses one or more routing domains. [c5 t 0180]

Dividing an Administrative Domain into several RDs offers the following advantages:

- robustness of routing is increased because of modularity and of independence between RDs

- it is possible to distribute control between different organisational entities, along the boundaries of RDs

- it is possible to use an RD specific routing algorithm inside each RD and

- the size of the Administrative Domain is theoretically unlimited owing to the fact that each adjacent RD may be small enough to be handled by a single routing algorithm. [c5 t 0190]

6. 5 ATN Routers

Constituent subnetworks are interconnected by routers.  Each router is connected to two or more subnetworks, appearing to each subnetwork as a locally addressable entity within that subnetwork.  The routers concatenate subnetwork links in order to form an end-to-end forwarding path for ATN CLNP packets between communicating ATN host computers.

[c3 t 0520]

Forwarding an ATN CLNP packet requires each forwarding router to choose the next-hop router or, for the final hop, the destination host computer.  Forwarding choices are typically based on connectivity, security and quality of service considerations, as well as on the likelihood that the chosen next-hop will result in the ATN CLNP packet being moved closer to its eventual destination. [c3 t 0530] For the ATN CLNP, the QOS requested by the network service user is a relative (i.e. qualitative) measure, consisting of four elements:

a)	throughput;

b)	transit delay;

c)	residual error probability; and

d)	expense. [c3 t 0540]

The ATN internet uses distributed-adaptive routing procedures.  This means that each ATN router is capable of determining the ATN internet topology without reliance on a central routing data base and without the need for synchronous distribution of that data base throughout the ATN internet.  This also means that each ATN router can adapt to changes in the ATN internet topology relatively quickly, as compared to a more traditional centralised approach. [c3 t 0560] The current ATN internet topology is represented by information in the Routing Information Base (RIB) contained within each ATN router.  This RIB is initialised and maintained on a dynamic basis through the ATN IS-IS and ES-IS routing information exchange protocols.  The ATN router further reduces the RIB into a Forwarding Information Base (FIB), which contains a set of forwarding paths reflecting the various policy and QOS rankings available to reach each known destination.  A robust and flexible dynamic routing operation is made possible by the ATN internet provision of CLNS; as the routers forward ATN CLNP packets on a connectionless basis, they minimize the state information necessary to provide ATN internet service. [c3 t 0570]

6.6 ATN Protocol Operations

The ATN internet comprises a number of interconnected ATN routers and constituent subnetworks supporting packet data communication among host computers operating the ATN internet protocols.  The ATN internet protocols operate within ISO OSI Layer 3 (i.e. the network layer).   This protocol set includes the ISO 8473 Connectionless Network Protocol (CLNP), the ISO/IEC 10747 Inter-domain Routing Information Exchange Protocol (IDRP), the ISO/IEC 10589 intra-domain Intermediate System to Intermediate System (IS-IS) Routing Information Exchange Protocol, and the ISO 9542 End-System to Intermediate System (ES-IS) Routing Information Exchange Protocol. [c3 t 0340]

All CLNP protocol data units (i.e. packets) are encapsulated within appropriate subnetwork protocol data units for transfer among ATN network entities.  Each  CLNP protocol header carries a unique identification code, allowing the receiving ATN router or host computer to correctly process the protocol header and any associated data content.  Also, as all ATN internet protocols are connectionless, any information required to process a particular network protocol data unit is carried within the header of that network protocol data unit. [c3 t 0350]

Reliable end-to-end data transfer is the role of the ISO OSI transport layer; any transport protocol operating within the ATN environment must be capable of correct operation using the connectionless ISO OSI network layer service provided by the ATN internet. [c3 t 0360]

6.7 ATN Model Characteristics

6.7.1 Concept

This section is concerned with the ATN Architecture and the Service it provides to its users.  The ATN User is an air/ground or ground/ground Air Traffic Management (ATM) application, or some other application supporting an aeronautical function.  But, this is very much an end system view.  From the networking point of view, there are many interfaces over which "a user" accesses a service.  At each such interface, each user can be considered as an ATN User.  In the context of this section, the term "ATN User" is used in this sense, i.e. as a user of the network service or transport service, depending on context.

The ATN is a data communications internetwork that: [c5 t 0010]

1.	provides a common communications service for all ATSC and AINSC applications that require either ground/ground or air-ground data communications services. [c5 t 0020]

2.	integrates and uses existing communications networks and infrastructure wherever possible. [c5 t 0030]

3.	provides a communications service which meets the security and safety requirements of ATSC and AINSC applications. [c5 t 0040]

4.	accommodates the different classes of service required by each ATSC and AINSC application. [c5 t 0050]

The ATN provides its users with a robust and reliable communications service, together with the option of a datagram service. [c5 t 0060]  Formally, all communications aspects of a user's system are part of the ATN, but from a "user's point of view", the ATN is out there, separate from their own system.  The ATN is seen as an abstract "cloud" which indeed is all the user need be aware of, with its complexity hidden from view.  At this level, the ATN is a simple network that provides a datagram service to its users.

A ground based ATN user's system, which might be anything from a complete ATC system to an entry level PC, accesses ATN services via some ATN access point.  A user's access to the ATN is first via an "access subnetwork", such as an Ethernet or an X.25 PSDN, and then an ATN Router. It is then through the ATN Router that access is gained to ATN services.

An ATN user may, at any time send a CLNP formatted datagram to any valid destination address.  The user does this by passing the datagram over the access subnetwork to the ATN Router.  The ATN Router will inspect the protocol header, and it is then the ATN Router's responsibility to forward the datagram through the ATN to the ATN Router which provides ATN access to the addressed destination. Once the datagram has arrived at the ATN Router which provides ATN access to the addressed destination, it is then transferred over the destination's access subnetwork to the destination user.

When the ATN user is an organization with multiple systems to consider; has many subnetworks of its own; and also wishes to operate ATN Routers, then the organization will also be an ATN Service Provider.  The "organisational" ATN user has both Host Computers which are individual ATN users as before, but also operates a portion of the ATN cloud, with at least one link to the rest of the ATN.

The portion of the ATN operated by the organisational user may be no more than a single ATN Router.  Alternatively, a larger organisational user may operate a large number of ATN Routers, interconnected by various subnetworks also operated by the organisational user, and providing ATN access to all the systems owned by the organisational user which require ATN access.  Such Routers may also be general purpose and be part of that user's own internal network.  However, regardless of how many ATN Routers there are within an organization, the ATN Routers and the End systems (Host Computers) to which they provide ATN access, typically form what is known as a Routing Domain.  The Routing Domain is a structure specified in the ISO standards and imposed on the ATN Internet in order to enable a structured approach to be taken to solving the routing problem.

The ATN will incorporate many "mobile" subnetworks.  Examples of such subnetworks include SSR Mode S, AMSS and VDL.  If an aircraft were to attach to one mobile subnetwork only and never to any other, then even though sometimes it may be attached and at other times not attached, this has no consequence for the ATN.  This is because from the point of view of the rest of the ATN, it would be no different from a fixed system that was occasionally off-line.  However, that is not how mobile subnetworks are used.  [c5 t 0115] An aircraft will attach to many different mobile subnetworks during the course of its flight.  A long haul aircraft may move between the coverage areas of different satellites; an aircraft flying over a land mass will fly between different Mode S an/or VHF subnetworks as it passes over different countries.  And, at the same time, the applications on board the aircraft will need to maintain contact with applications on the ground.  Mobile platforms thus require special routing considerations. [c5 t 0120]

In the ATN, mobile "platforms" are treated in a similar manner as organisational users.  That is, the systems on board an aircraft are required to form a Routing Domain and hence must include an ATN Router.

6.7.2 Functional Model

### A) Relation to the OSI Model

The ATN architecture and protocols follow the International Organization for Standardisation - Open Systems Interconnection - Basic Reference Model defined in ISO 7498. [c5 t 0147] This model has been used as a framework for definition of the services and protocols that will be used in the ATN.

### B) ATN Functional Components

The ATN comprises the following functional components:

- End Systems (ESs)

- Intermediate Systems (ISs)

The OSI layer 1-4 aspects of ESs and all ATN ISs comprise the ATN Internet.  The ATN Internet provides data transfer services to ATN ESs.  Through ESs, ATM applications gain access to ATN services and are hence able to exchange application specific data.

The ATN also recognises the existence of subnetworks, which may also be viewed as being part of the ATN.  Subnetworks are characterised by the ATN as being:

- Ground subnetworks

- Mobile subnetworks

- Airline subnetworks

The function of an ATN End System is to provide the end-user applications with an OSI compliant communications interface to enable them to communicate with remote end-user applications.  ATN End System implementation of the protocols required for Layers 1 and 2 (i.e. Physical and Data Link), and subnetwork access functions in layer 3, is purely a local issue and wholly dependent on the subnetwork to which the particular End System is attached.

The functions of the Intermediate System are to perform the relaying and routing of ISO 8473 NPDUs between other Intermediate Systems and End Systems.  The protocol support required for the Physical and Data Link layers of an ATN Intermediate System are purely dependent on the subnetwork to which they are attached.

The Routing Information Base (RIB) comprises the information regarding the connectivity and topology of the ESs and ISs within a particular Routing Domain and path information pertinent to paths interconnecting Routing Domains, and is maintained by way of the information received by a routing information exchange protocol.  Each Routing Information Exchange Protocol has its own RIB specification.  The information contained in a RIB may include:

- Next-Hop Routing tables

- Lists of known End Systems and Intermediate Systems

- QOS of Subnetwork Hops (as specified by a System Manager)

- Network Topology Maps

The list of known ISs and ESs allows determination of the local subnetwork topology, while the subnetwork QOS tables provide information for selection of an acceptable path based on requested QOS.  Network topology maps provide an overall view of the global network connectivity, and are used in path computations by the operative routing algorithm.

6.7.3 Management and Management Domains

In the ATN, it is expected that each Administrative Domain will coincide with a Management Administrative Domain.  That is, each ATSC and AINSC organization is responsible for managing its own portion of the ATN.  The sub-division of each such Management Administrative Domain into subsidiary Management Domains is then the responsibility of that organization. [c5 t 0226] However, the ATN is very much concerned with co-operation between its constituent parts in order to achieve an agreed objective.  A model for the organization of a Management Administrative Domain is provided, which all ATN participants are recommended to adopt.  This model identifies the management functionality necessary to support the Quality of Service demanded by the ATN.

### A) Management Domains

Management domains are resources which for systems management purposes are represented by managed objects.

A management domain possesses at least the following properties:

- a name which uniquely identifies that management domain

- identification of a collection of managed objects which are members of the domain

- identification of any inter-domain relationships between this domain and other domains.

Management domains may or may not overlap.  When management domains created for the same purpose do overlap, special conditions may apply.

### B) Management Administrative Domains

In addition to the organisational requirement, administrative requirements exist.  These include mechanisms to:

- establish and maintain the respective authorities of each management domain, to apply modification to their boundaries, and to organise the way in which some of them overlap

- manage the transfer of control of resources from one management domain to another. [c5 t 0224]

To meet such requirements, a special kind of management domain called a management administrative domain is defined.  A management administrative domain is a management domain where the managed objects in the domain are all under the responsibility of one and only one administrative authority. [c5 t 0225]

6.7.4 System Management, Framework and Protocol

All networks need management.  The most obvious reason is to configure the various components to reflect their role in the network. Management is also necessary for fault reporting and diagnosis; accounting for use of resources; and for managing security services.  The ATN Systems Management Approach is based on the OSI Management Framework (ISO 7498-4). [c5 t 0221]  There are various mechanisms by which a system (or subnetwork) may be managed.  It may be via a local interface to each system, either by a directly coupled terminal, or remotely via some proprietary protocol. [c5 t 0222]

Alternatively, a more open approach may be implemented, but separately for each layer using a "layer management protocol".  However, the OSI approach employs an application layer protocol for Systems Management.  This is a general purpose protocol which may be used for all OSI Management and not just Network Management.  The application protocol is called the Common Management Information Protocol (CMIP), and this provides the Common Management Information Service (CMIS).  This approach is specified for the ATN.

### A) Systems Management Model

The ATN Systems Management Model is derived from the OSI Systems Management Model, presented in ISO/IEC 10040.

Management of a communications environment is an information processing application.  Because the environment being managed is distributed, the individual components of the management activities are themselves distributed.  Management applications perform the management activities in a distributed manner.  Two or more systems management application entities may associate to provide an instance of a distributed management application.

Management activities are effected through the manipulation of managed objects, which are OSI abstractions of real-world resources.  For the purposes of systems management, management applications are categorised as MIS-Users.  For a management interaction an MIS-User is allowed to take on one of two possible roles - either an agent role or a manager role.

Roles are not assigned permanently to MIS-Users.  Some MIS-Users can be restricted to only taking an agent role, some to only taking a manager role, while others are allowed to take an agent role in one interaction and to take a manager role in another.

### B) Shared Management Knowledge

It is not possible for any manager to manage just any agent.  There needs to be a common knowledge base before management can be invoked.  Management knowledge for systems management communications includes:

- Protocol knowledge (e.g. application context)

- Function knowledge (e.g. functions and functional units)

- Managed object knowledge (e.g. classes and instances and identification of managed objects and their attributes)

- Constraints on functions supported and relationships between those functions and managed objects.

This Management knowledge must be established prior to management taking place, and can be established at any time, specifically:

- prior to any communication taking place (e.g. configured in)

- during association establishment

- during the lifetime of the association.

### C) Management Protocol

The Common Management Information Protocol is specified in ISO/IEC 9595, and is an OSI application layer protocol; it thereby follows that any real system subject to OSI management must include a role as an OSI End System, and support the representation of its managed resources as managed objects.

The protocol provides a generic service for accessing Managed Objects, i.e. creating/deleting managed objects, reading/modifying the values of attributes; for invoking actions supported by managed objects; and for receiving notifications of events.

6.7.5 Subnetworks

ATN subnetworks are those subnetworks interconnecting ATN ISs and ESs for the purpose of transferring data between these components.  The ATN is designed to employ any subnetwork technology capable of code and byte independent delivery of data.  The most significant requirement placed on these subnetworks is that they must provide at least a connectionless-mode service.  While only the connectionless mode service is required for data transfer, a connection mode subnetwork may be used if the required service is provided through an appropriate subnetwork convergence function [c5 t 0150].

ATN subnetworks may be divided into three major categories:

- General Topology Subnetworks (e.g. ISDNs, PSDNs, etc.)

- Broadcast Subnetworks (e.g. LANs)

- Mobile Subnetworks (e.g. AMSS, VDL, Mode S)

The ATN architecture allows the use of any reasonable combination of these subnetwork types for construction of an end-to-end path between two communicating host computers, with the intervening ATN ISs performing the subnetwork address and/or subnetwork access protocol conversion necessary for successful packet forwarding.

General topology subnetworks (e.g. WANs) are often used to connect geographically-dispersed Intermediate Systems and End Systems.

Broadcast Subnetworks (e.g. LANs) are often used to connect ISs and ESs within a small geographic area with media offering relatively high data throughput with relatively low delays.

6.7.6 Routing Framework

The ATN is designed to accommodate a very large number of ISs, and a structured approach is necessary in order to develop a routing strategy.  In order to simplify the design of large OSI networks, ISO has defined a Routing Framework (ISO TR 9575) which allows large scale networks to be designed and built in a modular fashion.  This Routing Framework is applied to the ATN in order to provide the required structured approach to routing.  The OSI Routing Framework introduces two new functional entities: the OSI Administrative Domain; and the OSI Routing Domain.

The ATN is divided along organisational boundaries into Administrative Domains, in order to facilitate and optimise the routing of data communication traffic.  Each such ATN Administrative Domain may be administered by governmental and public/private aeronautical service users and providers and is defined to be consistent with organisational boundaries. [c5 t 0160]

An ATSC Administrative Domain is an ATN Administrative Domain owned and/or administered by an ATS organization.

An AINSC Administrative Domain is an ATN Administrative Domain owned and/or administered by an Aeronautical Industry Service organization.

6.7.7 Routing Domains

An ATN Administrative Domain comprises at least one Routing Domain.  Each such ATN Routing Domain is created by the authority operating an ATN Administrative Domain when that Administrative Domain is large enough to justify an internal hierarchy to optimise the exchange of data and routing information within that Administrative Domain, or when security or architectural requirements dictate the partitioning of the Administrative Domain.  While the ATN Administrative Domain structure is a mandatory part of the ATN Internet, the creation of multiple ATN Routing Domains is a local matter within a particular Administrative Domain. [c5 t 0195]

In the ATN, Routing Domains may comprise either ground systems that are part of an ATSC or AINSC Administrative Domain, or may comprise mobile systems (e.g. airborne systems).

An ATSC Routing Domain comprises ESs and ISs that are part of an ATSC Administrative Domain.

An AINSC Routing Domain comprises ESs and ISs that are part of an AINSC Administrative Domain.

6.7.8 Routing Areas

In OSI, RDs may be subdivided into Routing Areas.  A Routing Area is a routing subdomain comprising one or more ISs, and optionally, one or more ESs.  Structuring a Routing Domain into multiple Routing Areas enables very large Routing Domains to be constructed without serious impact of the routing overhead.  The Routing Domain and Routing Area are logical and not geographical notions.  The structuring of a Routing Domain into multiple Routing Areas may be performed in response to many considerations.  However, an important consideration is data link bandwidth.  Typically, all systems within a Routing Area are interconnected by high bandwidth data links (e.g. LANs), while inter-Routing Area data links are lower bandwidth (e.g. WANs).  For such reasons, Routing Areas will often coincide with physical sites, but this is due to practical rather than technical constraints.

An ATSC Routing Domain may comprise multiple ATSC Routing Areas.  For example, an ATSC Routing Area may correspond to a physical location such as an airport.

An AINSC Routing Domain may comprise multiple AINSC Routing Areas.  For example, an AINSC Routing Area may correspond to a physical location such as an airline's systems located at an airport.

6.7.9 Routing Domain Confederations

Although the structuring of an internetwork into Administrative and Routing Domains enables a structured approach to routing to be developed, this is not in itself readily scaleable.  Once there exists a large number of Routing Domains, the structuring problem re-asserts itself, and there is a need to provide another level to the Routing Framework, and so on.  This problem is resolved in a recursive fashion by the Routing Domain Confederation.

A Routing Domain Confederation (RDC) is a set of Routing Domains and/or RDCs which have agreed to join together and form a Routing Domain Confederation.  The formation of a RDC is done by private arrangement between its members without any need for global co-ordination.  From the outside, an RDC appears exactly like a single Routing Domain.  Routing Policies can also refer to entire RDCs in the same way that single RDs are referred, which enables the straightforward specification of routing policy rules that apply to whole classes of RD.  There is, however, no requirement for there to be co-ordination of routing strategies or the adoption of any common routing policy rules.  However, efficiencies can result from the co-ordination of Addressing Plans and policies.  RDCs are simply groupings of Routing Domains.  The organization of ATSC, AINSC and Mobile Routing Domains into RDCs is essential if the ATN is to have a scaleable architecture.

6.7.10 Protocols

The ATN protocols follow the reference model specified by ISO in ISO 7498-1.  The ATN protocol architecture is illustrated in Figure 6.3, and specifically illustrates the use of a mobile subnetwork for air-ground communications.  This protocol model has been derived directly from the OSI Reference Model.

FIGURE 6.3

ATN PROTOCOL ARCHITECTURE

6.7.11 Naming and Addressing Model

The ATN Naming and Addressing Model is based on part 3 of the OSI Reference Model (ISO 7498-3) , which provides for the unambiguous identification of applications within an End System, and other OSI functional entities.

Names are necessary to uniquely and unambiguously identify some object, while addresses are necessary to locate an object.  Following ISO 7498-3, a Name stays with an object as long as it exists, while an address may change.  Therefore, a Name can always be used to refer to an object, and may be used to find its current address, while an address can be used to find the object (e.g. the use of network layer addresses by routing algorithms).

In the ATN, there is a need to name the following objects:

- ATM Application Entities (AEs)(Application Entities are the representations in the OSI Reference Model of applications), so that they may identify and communicate with each other.  (Application Entities represent the communications aspects of applications in the OSI Reference Model).

- Network Layer Entities in order to support the routing protocols

- Systems Management Application Entities (SMAEs)

Routing Domains and Routing Domain Confederations, in order to control route propagation.

[c5 t 0205]

Furthermore, there is a need to be able to locate (i.e. provide addresses for) AEs, upper-layer, and Network Layer Entities. [c5 t 0206]

According to ISO 8348, Network Layer Entities are both named and located by their Network Entity Title (NET), while Routing Domains and RDCs simply require some mechanism for assigning unique names.

An OSI Presentation Address comprises:

- A Presentation Service Selector (PSAP Selector)

- A Session Service Selector (SSAP Selector)

- A Transport Service Selector (TSAP Selector)

- An NSAP Address

The first three components are locally assigned and any or all may be null.  In many End Systems, an NSAP Address is sufficient to identify the transport entity (and effectively identifies the ES), and then a transport service selector is sufficient to identify each application within the ES.  The combination of TSAP Selector and NSAP Address is known as a transport address.  In the ATN, presentation addresses must, as a minimum, include an NSAP Address and TSAP Selector.

6.7.12 Quality of Service Framework

An application, in combination with the supporting upper layer protocols, is an ATN user.  Each application will have defined requirements for service characteristics such as class of communications service (based on transit delay), residual error rate, availability or cost.  Some applications, for example, may have strict transit delay requirements with little consideration for cost, while others may have very low needs for transit delay but high sensitivity to the cost.

The network layer entity in ATN ISs and ESs must provide QOS requirements to a subnetwork service provider whenever a subnetwork connection is established, whether this be in support of a routing protocol, or in support of ISO 8473. [c5 t 0280]  However, the ATN Network Service is only provided in the connectionless mode.  As each invocation of the connectionless network service is a distinct event, it is not possible to derive subnetwork QOS requirements from an NPDU that caused the subnetwork connection to be opened. [c5 t 0290]  Instead, the QOS requirements must either be based on a priori knowledge of user or routing protocol requirements (e.g. using subnetwork connection QOS requirements specified by a Network Manager), or should be determined by extrapolation from previous behaviour. [c5 t 0300]

During initial network design, it is necessary to model the internetwork operation and identify the expected QOS requirements of each such connection.  These expected QOS requirements provide the initial values for use as discussed above. [c5 t 0310]  Thereafter, as part of the Performance Management function, Network Managers and network entities will monitor network performance, and compare achieved QOS with required QOS.  Using such information and through refinement of the initial network model, revised values for the QOS parameters can be determined, and indeed the overall topology of the internet may also be revised, in order to match requested with actual QOS.

Systems management techniques may be used at the ATN user level to determine feasibility of a path as a first step to establishing communications between two applications.  For example, a path needs to exist between the ground ATC host computer and the aircraft, and this path should be able to support the required communications characteristics.  In order to obtain this information, the user can use QOS Management techniques to determine the expected end-to-end QOS.

The actual communications characteristics in operation between two ATN Users can only be determined by measured end-to-end QOS.  Measurement is used to confirm that the actual QOS provided is sufficient to support the user's communications needs.  Measurement may be done either by the systems management function or directly between two ATN users.  Typical measurement techniques include time tags on data or "probes", which can test one-way or round trip path characteristics such as transit delay or error rates.

6.7.13 Security Model

ISO/IEC 11577 specifies a Network Layer Security Protocol (NLSP), which may be used in conjunction with ISO 8473 to provide the identified network layer security services that are appropriate for the connectionless mode. However, this is not to say that there are no known ATN security requirements. Requirements for the control over access to resources reserved for air-safety related applications, and the separation of traffic serving operational and non-operational users, are already known. Furthermore, commercial service providers will also operate ATN Routing Domains, and the use of their resources will be restricted to paying customers.

In order to implement such  requirements it is necessary to tag, or label, data and routes through the ATN, so that each  type of traffic may be identified,  rejected if  it is passed over a network resource that is not available to that traffic type, but otherwise routed via resources that are available to it. A label used for such a purpose is generally known as a Security Label, and a suitable Security Label format is specified for use in the ATN.

The syntax and semantics of a Security Label are specified by the Security Policy under which  it is defined, and hence it is necessary to specify a suitable Security Policy in this manual, in order to specify a suitable Security Label. There are three ATN-wide Security Policy, which may be viewed as a "shell" Security Policies as they exist solely for the purposes of specifying the ATN Security Label. ATN Systems will operate under their organisation's Security Policy which will then reference the ATN Security Policies for the purposes of  recognising the syntax and semantics of the ATN Security Label.

A Security Policy is specified for each class of traffic believed applicable for the ATN; i.e. Systems Management Traffic, Operational Traffic, Administrative Traffic, and General Communications (Default) .

In  order to readily support security requirements that may be derived from a full risk analysis, the Security Label format that has been adopted, is  extendible, and, in addition to the identification of the Security Policy consists of  a classification field to enable the later extension to hierarchical security, if required, zero, one or more categories, and some privacy marking. The only categories so far specified are used to ensure that traffic only passes through service providers that will accept the data. No privacy markings have yet been specified.

6.7.14 Priority Model

Congestion occurs primarily because more data is submitted to a network than it can process.  The result is that ISs run out of capacity and either ignore packets received over connectionless subnetworks, or give flow control over connection mode subnetworks.  Once several ISs become congested, it no longer becomes possible to transfer data between them, the "lifetime" of individual packets expires and they are discarded.  Catastrophic congestion occurs when users continue to bombard the network with data during this period, typically by retransmitting packets, the congestion spreads, more users start retransmitting, and eventually the whole network becomes congested.  The risk of catastrophic network congestion is a known problem of connectionless networks.

The ATN requires a strategy to minimize the risk of congestion, and to prevent catastrophic congestion. [c5 t 0385]  Congestion will always be avoided as long as there is adequate capacity within the network to deal with the applied load, and capacity planning, as part of performance management, is an essential part of the management of a connectionless network.  However, even with the best capacity planning, outages and transitory peak loads may still cause congestion.

ISO 8473 permits a sender to identify the priority of the data contained within the packet and also to indicate Quality of Service preferences, the latter feature, however, is prohibited for CNS/ATM-1.  ISs may reference a packet's priority in order to speed high priority data on its way, but its most important use is during periods of congestion.  Without prioritization of data, a congested router may choose data to discard on a random basis, or may choose to discard oldest packets first.  When data is prioritized, the low priority data is discarded first.  Furthermore, communications resources both within and between ISs can be allocated separately to different priorities of data, such that communications resources can be diverted to high priority data when outages occur.  Through the use of priority, congestion can be managed such that the network service degrades in a predictable manner with lower priority data affected first, and the most critical data affected last.

### A) Prioritization of ATN Data

ICAO has defined several categories of application traffic which may require prioritization where resources (e.g. channel bandwidth, computer processing power) are limited.  Fourteen application categories have been identified as requiring separate transmission queues for their message traffic when resource limitations arise.  These application categories have been derived from various sources, primarily from within ICAO and the ITU.

### B) Relationship to Subnetwork Priority

Within a single subnetwork, prioritization of data is not required to arbitrate resource access amongst data travelling between the same pair of systems, as such arbitration is done in those systems.  It is only required, and indeed is only effective for arbitrating between the claims to communications resources by data travelling between different pairs of systems.

6.7.15 System Certification

Certain ATM Applications may be relevant to flight safety.  For the ATN is to support such applications, it is necessary to provide a safety case presenting the basis for eventual certification of these applications for use via the ATN. [c5 t 0410]  A safety case is an analysis presenting an overall justification for the declaration that a particular system satisfies its safety requirements.  The safety case defines the safety requirements applicable to the system and presents the evidence, arguments and assumptions used to show the degree of compliance with these requirements. [c5 t 0420]

A safety case provides assurance that the following objectives have been met:

- That potential hazards of the system have been identified and appropriate controls provided.

- That the safety management procedures developed for the system are adequate to ensure that the design and operation of the system meet the safety requirements.

- That the evidence available to show compliance with the safety requirements is sufficient to allow the system to proceed to the next life cycle phase or continue in operation, as appropriate. [c5 t 0430]

During the design phase for ATN implementations, work should begin on the preparation of the eventual safety case for each part of the ATN, and for the applications that use the ATN. [c5 t 0440]  The user requirements documents should include preliminary hazard information, identifying initial safety requirements. [c5 t 0450]  For those functional areas for which safety requirements are applicable, a preliminary hazard analysis should be conducted during the design work and will be part of the resulting functional specification. [c5 t 0460]

Chapter 7 		Strategic Objectives

7.1. The Present Day ATM

## In the present day ATM, ATC plays a primary role.  Automation of ATC, which is necessary to handle the continuously increasing volume of traffic, has until now been based mainly on a better utilization of the limited information available on:

1) the flight intention as provided in the flight plan and its updates; and

2) the position of the aircraft as provided by current surveillance means.

### 7.1.1.	Air/Ground Information Flow

### Despite the introduction of SSR, there still remains a severe gap in the information flow between air and ground.  In addition, large parts of the world lack reliable coverage by communications, navigation and surveillance (CNS) systems.  Without proper measures, the efficiency further decreases in the future because of predicted growth in the traffic and the widening gap between airborne and ground capabilities.  The examples as presented below are applicable to some extent to even the most developed ATC systems environments:

1) the information flow within ATC units and between ATC units and aircraft under their control is insufficient to support further significant improvements.  However, it is recognized that the utilization of flight plan data processing and radar data processing are implemented with various degrees of refinement. Also, automated assistance to traffic flow management already exists;

2) ATC needs improved data exchange procedures for surveillance, management and optimization of the air traffic flow;

3) the most advanced ATC conflict predictions systems are still working on the basis of limited data representing aircraft performance and environmental conditions.  Hence, only limited accommodation of optimized flight profiles is achieved;

4) the capability of advanced airborne equipment in the field of planning and optimization of flight paths has outstripped that of the ground systems.  Operators are pressing to be able to fully exploit such capabilities; and

5) route structure are generally inflexible although, increasingly, direct routings are allowed when controller work load permits.

### 7.1.2. Ground/Ground Information Flow

### The implementation of medium speed AFTN circuitry has provided improvements to data integrity and speed of service for traditional message traffic.  Current automation activities employ direct physical links or X.25 based networks for connectivity.  This structure does not support the effective interconnection of ATM processors requiring direct logical connectivity between applications.  The current configurations lack the flexibility and logical connectivity required to support planned CNS system implementations.  Essentially, the ground/ground networks are progressing along two or more parallel paths;  one for store and forward message exchange, and one for interactive data exchanges.  The later capability should be employed as the initial building block for progressing ultimately toward the ATN infrastructure.  An interactive data exchange environment needs to be supported for G/G prior to deployment of the initial ATN A/G elements.

## 7.2. The Evolution of the ATM

The CNS/ATM system will be evolving within each of the groups: Communications, Navigation and Surveillance.  Indeed, the improvements to be found in airborne and ground systems capabilities could be used in a complimentary fashion to maximize the efficient use of airport and airspace resources.  The systems that are to be implemented in the future ICAO system (as detailed in figure 7-1) are items that may be implemented as the need for that airspace environment arises.

Communications		Current System			ICAO System

* Very High-Frequency 		* VHF Voice/Data

Voice (VHF)			* AMSS Voice/Data

* HF Voice			* SSR Mode S Data Link

* ATN

Navigation			Current System			ICAO System

* MNPS				* Area Navigation/RNP

* Omega/Loran-C			* GNSS

* Nondirectional beacon		* Barometric Altimetry

* VOR/DME			* GNSS Altitude

* Barometric Altimetry		* INS/IRS

* INS/IRS				* MLS

* ILS

Surveillance		Current System			ICAO System

* Primary Radar/SSR		* ADS

* Voice Position Reports		* SSR Mode A/C or

* SSR Mode A/C			Mode S

Figure 7-1 CNS System Evolution

7.2.1. Air-Ground System Direction

In the respect of the future ATM the use of an Air-Ground data link plays an essential role.  The elements of the Air-Ground system will be evolving as per figure 7-1.  The direction that this particular system concept is taking will lead to the following improvements:

1) flexible and efficient use of airspace;

2) enhanced safety and predictability of flight;,

3) provide greater economies of operation;

4) improvement in conflict detection and resolution;

5) improvement in the ability to take advantage of the improved navigation accuracy in four dimensions of modern aircraft; and

6) improved accommodation of a flight's preferred profile in all phases of flight, based on operator's objectives.

### 7.2.2. Ground System Direction

### For the ground system, the following directions for change are envisaged:

1) improved handling and transfer of information between operators, aircraft and ATC centres; and

2) improved and more accurate planning, including use of improved weather information;

### 7.2.3. Constraints and Conditions

### The evolution should take place within an environment which puts a number of constraints and conditions on the process, in particular:

1) a future system can only be implemented by means of an evolutionary process;

2) improvements in weather forecasting may be needed to enhance overall system performance;

3) although the system must be designed to accommodate normal peak traffic demands, and should be easily expanded to meet anticipated future growth, it is accepted that it may not be practicable to provide for excessive peak levels of air traffic demand; and

4) There will be a requirement to retain the ability to provide an acceptable level of service to aircraft not equipped to take full advantage of the planned CNS improvements.

## 7.3. Applications and Environment

## The future ATM applications will operate over the ATN internetwork.  This OSI environment will allow additional applications such as directory traffic, network management traffic, as well as file transfer and graphics.  Ultimately, this infers that all information carried by the ATM must be converted and recognized by ATN standards.

### 7.3.1. Future ATS, ATFM, OPMET and SAR Environments

Applications in the future are expected to be supported using standard off-the-shelf OSI upper-layers and application program interfaces.  Application program interfaces to support application over transport environments are expected to meet present, and in some cases, long-term ATM needs.  Continued analysis of performance, reliability, security and other communications requirements helps determine applicable standards.

Chapter 8 		Transition Considerations

8.1. Considerations

There are six elements to be considered in planning the transition to the ATN.  These elements are:

1) configuration;

2) operational;

3) funding;

4) participation;

5) security; and

6) communication standards.

8.1.1. Configuration

The following types of interfaces and configurations must be supported:

1) the operation of AFTN applications in the AFTN environment;

2) the operation of AFTN message exchange over the ATN internetwork using a defined application gateway; and

3) the operation of applications over the ATN internetwork using any available and suitable subnetwork connectivity.

8.1.2. Operational

Operational considerations should be made for planning in the following areas:

1) types of data traffic;

2) expected technology;

3) operational functions; and

4) technical operating considerations.

Types of Data Traffic.  The communications system in a sepcific domain should be capable of carrying the ATN network management function and at least one of the following types of traffic:

1) Civil Aviation Authority (CAA);

2) Air Traffic Control(ATC);

3) Aeronautical Operations Control (AOC); and,

4) Aeronautical Passenger Communications (APC).

Expected Technology.  Technology that either has to be supported for an interim period or is expected to be used for the next generation of communication capabilities must be considered.

Operational Functions.  Application gateways provide the necessary services for two or more dissimilar protocol architectures to intercommunicate.  This type of gateway may be constructed for transition purposes, or for long-term use.  Two operational functions of the AFTN/ATN gateway include:

AFTN/ATN gateways must relay and route AFTN messages across the ATN in a manner so that end systems understand what is delivered to them; and

The AFTN/ATN gateway must relay and route messages transparently across the ATN internetwork to either an end system which can interpret the message or to another AFTN/ATN gateway that can send the message onward to an AFTN terminal.

Technical Operating Considerations.  A migration path is required from the current environment that is upwardly compatible.  It is desirable to progress in an evolutionary direction rather than one that forces a change in technology.  This course must allow organizations to support different configurations — on different timescales — while maintaining interoperability and increasing functional utility for each step.

8.1.3. Funding

It is possible that groups of States might propose group funding for systems that are jointly developed, owned or operated.  For example, joint funding might be proposed to solve a regional problem.  Each Contracting State should manage its own ATN integration.

8.1.4. Participation

Participants:  Any organization that sends, relays, or receives ATN traffic.

Roles of participants:  Organizations which participate in the process outlined by this plan have six on-going and contiguous roles.  They should participate through feedback, comments and technical contributions in the:

1) network requirements assessment;

2) strategic planning;

3) network design and SARPs development;

4) configuration development, implementation/ deployment;

5) inter-region issues resolution through appropriate ICAO bodies; and

6) network operations.

8.1.5. Security

There are no requirements for telecommunication/protocol security mechanisms in the existing AFTN.  The FANS II committee has indicated a need for security for any future ATM communications capability.

The OSI architecture allows for the integration of security mechanisms into the OSI protocols used at multiple layers or by the addition of security protocols.  Security mechanisms provide security by defining specific features and parameter information.  The security function that is currently under development is in Chapter 6 (version 3.0) of the SARPs (Internetwork Service and Protocol Specification).  It states that the Security function shall be supported by ATN End Systems (ES) or Intermediate Systems (IS) Network entity implementations receiving or transmitting inter-domain traffic other than General Communications.  ATN Network entities shall therefore provide the Globally Unique Security format for all created NPDUs. There is one exception: General Communications traffic where no security parameter information is required to be encoded in created NPDUs.  Security parameters can be found in the ATN SARPs under chapter 6.

8.1.6. Communications Standards

The Communications Standards plan and architecture should be based on the applicable air/ground and ground/ground communication standards defined in the SARPs.

The ATN should use ISO standards and protocols wherever possible.  This approach leads to a standardized technical solution at all layers.

Additional Assumptions.  It should be noted that due to the limitations of technology, some technical solutions for air/ground communication systems are constrained by limited throughput.  The ground/ground environment must not be unnecessarily constrained to accommodate the air/ground environment, although it would appear desirable to make use of a single solution for both mobile and fixed communication systems.

8.2 System Transition

System transition planning requires the development of an implementation strategy.  System transition is described as being:

1) the process of developing a system to the state of full functionality; and

2) system modifications and enhancements.

### 8.2.1. Transition Planning

### Below is a list of planning topics that could be considered in each site strategy.  Communications transition issues may be less visible where an integrated AFTN/ATN gateway is implemented:

- resource scheduling;

- configuration data/general data/site specific data/test data/interface control documents;

- procedures;

- transition;

- all operational phases and functionalities;

- module/interoperability testing procedures;

- software;

- modules;

- system software and tools;

- scheduling/testing/operations;

- hardware;

- equipment components;

- scheduling/testing/operations;

- personnel;

- training;

- roles/responsibilities per site or group of sites;

- sites/facilities;

- physical equipment layouts;

- security;

- environment and energy requirements;

- logistics and maintenance planning;

- testing;

- operations;

- external dependencies;

- site/system test plan;

- site/region transition plan; and

- post implementation assessment.

## 8.3. Transition Guidelines

### 8.3.1. Regional Implementation Interdependencies

### The interdependencies of implementation tasks create inherent technical and operational risks, and the ICAO regions require transition/implementation guidance for individual and multiple site conversions.

An explanation of the ATN Routing Domain (RD) will aid in the explanation of the interdependencies of the regions.  The ATN RDs shall be interconnected by real subnetworks permitting communication between ATN Routers for each of the interconnection scenarios below:

- interconnection between members of an ATN island backbone RDC;

- interconnection between members of an ATN island backbone RDC and other ATN RDs within the ATN island;

- interconnection of an ATN island;

- interconnection of mobile and fixed RDs; and,

- interconnection of ATN RDs and non-ATN RDs.

### 8.3.2. Implementation Tasks

### The migration strategy for the AFTN is designed to allow States to implement the AFTN/ATN gateway on a timescale without impacting interoperability.  With a flexible strategy that provides interoperability for a range of configurations, each State or group of States should ensure the following implementation tasks are achieved in system transition:

- creation of new system procedures;

- system and site conversion;

- system tests and system acceptance;

- scheduling implementation components for each site;

- software;

- hardware;

- firmware;

- configuration data;

- personnel/personnel training;

- maintenance;

- procedures/documentation;

- organizing site requirements versus resources; and

- maintenance/logistics transition.

### 8.3.3. AFTN/ATN Gateway Factors

### Each State is responsible for decisions leading to either a procurement of an integrated ATN product or the development and integration of the AFTN/ATN gateway. In either case, after system development, the operational variables and factors associated with system transition must be integrated and planned for a change towards ATN configurations.

## 8.3.4. AFS Data Network Changes/Modifications

### Conventional AFTN Support	.  The AFS needs to continue to support AFTN traffic and services during transition.  The transfer capability provides an interface to and from the ATN and the AFTN/CIDIN.

#### Some improvement to AFTN data integrity and speed of service can be achieved by upgrading existing AFTN circuits.  This presumes that the AFTN centres are modern computer based systems.  Centres could support a reliable data link protocol, such as HDLC or Systems Category B to provide the data integrity.  Implementation of HDLC provides a code and byte independent path, while Systems Category B is constrained to IA-5.  The addition of 1200 to 9600 bits per second medium speed data circuits provide significant improvement to the message transit time.

### AFTN Interoperability	.  In the migration strategy, the message transfer capability of the AFTN/ATN gateway provides the basic interoperability.  Specific requirements of the gateway that provide interoperability with the AFTN environment include:

- provision of an OSI protocol environment between AFTN switching systems and the ATN internetwork;

- enveloping or conversion of AFTN messages; and

- mapping of AFTN addresses and priority information to ATN addresses and priority.

### CIDIN Implementation	.  The CIDIN protocols provide a parallel path to improved service, and eventual ATN implementation.

8.3.5. Pre-ATN Improvements

The measures to achieve improvements discussed, provide a path for States to migrate to the ATN. In the future, HDLC links could be used to interconnect ATN boundary routers, instead of AFTN centres. AFTN centres could be reterminated to reach routers via the proposed AFTN/ATN gateway.

Chapter 9	Migration and Implementation Planning Alternatives

9.1.	Migration to Improved Data Networks

Migration to the end-state ATN environment can occur in a minimum of three steps.  Each step should be fully backward compatible from the migrating system, since each State may migrate at its own pace.  All new proposed systems or services should converge upon the desired end-state or transitional steps to the end-state.  The three migration steps have been identified as:

1) Evaluating the current operations and planning for the future.

2) Transition to use ATN gateways, and implementation of ATN Routers.

3) Implementation of ATN in support of the new CNS/ATM environment.

9.2.	Migration Steps

9.2.1	Step 1:  Current ATM Operations and Planning for the Future

This step involves examining current communications and surveillance operations.  It must take into account the current and planned inter-regional network configurations, the international communication centres, the reliability of existing circuits and channels, and the responsibilities for routing and distribution of information.  The ATN internet is made up of routers, subnetworks and an addressing plan that is unique to the ATN.  It is necessary to acknowledge the functionalities of the current system in order to create a migration path to the future ATN environment.  Regional planning groups should co-ordinate with their neighbouring regions in order to facilitate complimentary efforts for the future plans within their areas.  This step can be divided into the following task groups:

1) Management: The final analysis of current ATM technologies and the migration plan will be handled within the individual regions.  The key issues are:

a) Creation of Admin Domains for networks and subnetworks:  Users will be grouped into defined routing domains centered on policy based routing controls provided by the ATN.  To further the explanation, the Admin Domain consists of one or more Routing Domains (RD) and within each RD there is one or more subnetwork.  The Admin Domain itself must consist of one or more ATN routers and one or more real subnetworks.  All addressing domains are designed to be hierarchical in structure.  It is necessary to define these domains and areas to proceed with the design of the future ATN infrastructure.

b) Development of ATN routers:  The routers to be implemented in the ATN have been designed to be dynamic.  The routing principles are defined in the SARPs and have specified Network Service Access Points (NSAP) for the source and destination routers.

c) Development and application of SARPs:  The first package of the Standards and Recommended Practises for the Communications side of the CNS/ATM system are scheduled to be completed at the beginning of the middle term (1996-2000).  The second package is scheduled to be completed at the end of the middle term (1996-2000) of the FANS Communications System Evolution.  The systems included in this time table are: AMSS, VHF data, Mode S and the ATN.  On the Surveillance side, the SSR Mode S Standards and procedures have been completed while the ADS is awaiting the completion of only the procedures.

The outputs from step #1 will be:

- complete awareness of current operations with detailed information on current network configuration;

- co-ordinated transition plans of network configuration and operations;

- agreements with neighbouring regions; and

- scheduling of any necessary modifications required to support future plans.

9.2.2	Step 2:  Transition to the use of Gateways and the Implementation of Routers

This step is to occur during the middle term (1996-2000) of the FANS evolution plan.  The gateways will provide compatibility for the ATN systems.  The ACARS and AFTN gateways are currently undergoing development and will be tested on performance.  They have been defined for two types of implementations i) stand alone (as in a PC platform) and ii) integrated into current systems. The routing considerations for the ATN gateways are based on the idea that the end user will be unaware if their communicating partners are on a different system other than the ATN.

1) Management: The ATN Gateways provide access between the two systems.  Both Type A and Type B are to be considered part of the ATN.  The management concerns are based on negotiating traffic routes and using the available routers (Admin Domain and service providers).

2) Building the ATN Backbone:  The main concern in the construction of the ATN backbone is the compatibility between States.  The protocols and interfaces are to be uniquely ATN so that ATN routers and gateways will be interoperable.  The transition and operation of the ATN backbone service is to be transparent to the other systems.  A defined protocol architecture will have to be implemented so that ESs converting to the ATN will be able to reach the ATN gateway and remote users.  Other concerns and guidelines are as follows:

a) Development of the SARPs: The SARPs contain the regulations for the implementation of the ATN gateways;

b) Implementation of the first routers based on the Routing Domains; and

c) First exchange of ATN traffic:  The communications systems for FANS are scheduled for the middle term.  These trials will test the AMSS, VHF data, Mode S and the ATN.

3) Ground/Ground Activities: The transition of Ground/Ground (G/G) activities to ATN systems will  affect many user environments.  Systems such as AIDC, OPMET data bank, AIS and slot allocations will be of concern.  The issue of transition to the use of AFTN/ATN gateways is also necessary.

4) Air/Ground Activities: The transition of Air/Ground (A/G) activities to ATN activities is an involved process.  The systems such as ADS, CPDLC, PDC and ATIS are issues of concern in the A/G environment.  The ACARS and AMSS gateways will be of great concern once developed.

The outputs from step #2 will be:

- Implementation of ATN gateways;

- post implementation network performance reviews;

- continued planning from step #1 implementation, transition and operations; and

- development of OSI CNS/ATM products.

9.2.3	Step 3: Implementation of ATN in support of the new CNS/ATM environment

This step provides the ability for data exchange between OSI compliant ATM processors.  Some of these systems also have the capability to exchange traditional store and forward messages among themselves, and in some cases with user stations.  The major difference is the additional protocol architecture needed for data transfer between ATM processors operating within the OSI environment.  This step is the last step in the development and testing stages of the ATN.  The infrastructure should be capable of performing with virtually no error at this point.

1) Management: The management of the implementation (or final transition) must take several important factors into account.  The systems backward compatibility is necessary as a “fail safe” in the event that the newly implemented infrastructure fails during this step.  Alternate routes must be available for all systems.

2) Ground/Ground and Air/ Ground: The G/G and A/G environments will be expected to operate without the use of gateways at this step.  These activities will be fully integrated into the CNS/ATM system.

The outputs from step #3 will be:

- an ICAO standardized OSI upper layer protocol architecture operating over the ATN;

- an ATM environment that supports network configurations as defined in this document for each migration step; and

- post-implementation review of performance and support.

Chapter 10 		ATN Transition Paths

10. ATN Transition Paths

When a State desires to make improvements to its communications infrastructure it can do so unilaterally, or in cooperation with one or more neighboring communication partners. In the Common ICAO Data Interchange Network (CIDIN) implementation case, planning must include the cooperation of one or more additional States to allow the improvements to be completed.  Improvement to the communication service between users, and with the format of the data to be exchanged between the users should be considered.  Neither service or data format concerns are trivial and neither can be resolved independently.  Communication between different Admin Domains and Routing Domains requires the structured architecture of the ATN.

10.1. Unilateral Approach

A State wishing to begin unilateral transition to the ATN must do so with two concepts in mind.  The first being that the ATN implementation must be done correctly to preclude problems when an adjacent State implements ATN.  Secondly, the transition must not affect the systems of adjacent states.

### Figure 10-1.  Unilateral ATN Deployment

#### AFTN Connectivity.   Figure 10-1 depicts a way to approach the unilateral type of implementation.  In the example the ATN subnetwork is a packet network and provision has been made to connect with an adjacent State using the AFTN/ATN gateway.  Note that as long as AFTN terminals are used within State B it is necessary to maintain some type of AFTN message switching center to support a AFTN message exchange.  Another variant of this approach would be the extension of a X.25 circuit to an adjacent State to provide connection to one or more ATM systems, such as a ground ATC processor.

#### Application Layer.  If an ISO application layer protocol is used to exchange messages within the State it would be possible to phase out international connections to the AFTN switching center and use AFTN/ATN gateways for all international AFTN connectivity requirements.  OSI protocols could also be used to connect ATM systems supporting higher levels of automation (e.g. flight plan data, radar data, and OPMET data).

### 10.2. Bilateral Approach

### Figure 10-2 depicts a way to approach bilateral deployment between two adjacent States (A and B).  The concerns identified in paragraph 10.0 (ATN Transition Paths) apply.  In this case each State has agreed to upgrade their subnetworks and implement ATN routing between the States.  At least one ATN router is required for each State, which provides the inter-domain data exchanges required for the ATN. Based on network topology and cost considerations, the routers can be connected as shown or, alternately, a point to point circuit could be established between the two ATN routers.  A Permanent Virtual Circuit (PVC) is used to provide the logical equivalent of a point-to-point circuit between ATN routers to ensure that all data exchanged between the two subnetworks are subject to the policy based routing controls.  AFTN/ATN gateways are also needed to provide access to existing AFTN message switching centers.  Once the gateways are implemented, the international AFTN circuit can be discontinued.

### Figure 10-2 Bilateral ATN Deployment

#### Application Layer.  If an ISO application layer protocol is used to exchange messages between the States, the transition can be accomplished without regard to the AFTN.  AFTN message exchange would be accomplished via the AFTN/ATN gateway. OSI protocols could also be used to connect ATM systems supporting higher levels of automation (e.g. flight plan data, radar data, and OPMET data).

#### Routing Policy.  Deployment of ATN routers between administrations requires implementation of the appropriate routing information exchange protocol.  This protocol exchanges policy information concerning which end systems are allowed to communicate in the respective States.  Intra-domain routing, which is not policy based, is required when a routing domain is implemented within a State.

### 10.3. Trilateral Approach

This approach, as shown in Figure 10-3, depicts one possible way three States could be interconnected using ATN routers.  The same concerns as identified in the bilateral approach apply.  In this example of a trilateral approach, state C has become a Domain.  Another possibility is the implementation of a triangular trunk circuit interconnection.  It should be noted that ATN router implementations are based on policy routing principals that control which inter-domain exchanges are permitted.

### Figure 10-3 Trilateral ATN Deployment

### 10.4. Mixed ATN Approach

### It is possible that two or more non-adjacent States independently transition to the ATN (i.e. decide to implement a national ATN consistent ground subnetwork) as described in Chapter 9 paragraph 9.1.  This creates "islands" of OSI subnetworks with or without direct ATN-to-ATN connectivity.

#### AFTN Connectivity.  For message transfer services only, one connectivity option between these two States is the AFTN.  Each State would have to use an AFTN/ATN gateway to gain access to the AFTN.

##### Routing Policy.  The gateway static routing tables allow for address mapping of ATN end system through the AFTN, as well as AFTN end systems through the ATN.  This configuration does not require routers and only supports the routing of information that can be handled over the AFTN.

##### Application Layer.  OSI peer-to-peer connectivity in this case is not possible.  However, migration to a pure OSI environment requires only that direct ATN router-to-router connectivity be established.

Chapter 11	ATM/ATN ADMINISTRATION AND MANAGEMENT ISSUES

11.1. Administrative Issues

11.1.1 Ownership and Responsibility

ATM systems, routers, network interfaces, ground/ground connections, air/ground connections and AFTN/ATN gateways of the Aeronautical Telecommunications Network (ATN) are to be owned and managed by civil aviation authorities or their chartered representatives.

11.1.2. Regional Considerations

To simplify the overall implementation of the ATN ICAO has designated 6 World Regions.  However, coordination is required between communicating States to achieve initial ATM/ATN interoperability.  This effort may in some cases be undertaken without inter-regional coordination.  In other cases, inter-regional coordination may be required at the onset of the planning process.  Once system implementation has been planned the actual implementation and on-going operation may require continuing inter-regional coordination.  As an example, one of the inter-regional activities would be coordinating the alternate communication paths with neighboring regions.

11.1.3. Interface Documentation Considerations

Documentation of the technical and functional details of interfaces between ATM processors, between ATM processors and the ATN, and between ATN routers is necessary for proper management and operation of these complex arrangements.  Regional planning groups should develop and maintain documentation to capture the progress of their implementation and associated technical details.  As the work progresses there may be a need to apply this approach to inter-regional efforts as well.  In this way the requirement for a "globally seamless" CNS/ATM system can be supported by management functions.  The “user driven regional plans” are documents (one per region) that have been created and can be purchased from ICAO.  These plans have been created in the aid of CNS/ATM developers, integrators and users.

11.2. Network and System Management of OSI Resources

The OSI management approach has been proposed for the ATN and has significant impact on system and administrative resources.  Figure 11-1 depicts the interactions between the six management functions and the management information data base, all of which make up a system of network management.  The OSI approach provides for a distributed architecture, which could be used to implement integrated network management arrangements in various configurations on a multi-lateral basis.  Thus a more responsive management function can be realized that takes advantage of the inherent dynamic nature of the ATN structure.  Figure 11-2 depicts the progression of implementation steps necessary to manage ATN resources.

11.2.1 Configuration Management

Configuration management procedures need to be established and maintained for the proper management and operation of ATN resources.

11.2.2. Security Management

Security services and mechanisms need to be established to account for the reporting and distribution of security related events and information.  The SARPs (version 3.0) have established partial ATN network security functions.  These functions will help to maintain data integrity and data security over ground/ground and air/ground links.

Figure 11-1	Network Management - OSI Management Function Interaction

11.2.3. Accounting Management

Accounting management needs to be established to capture the cost and usage of ATN resources.  There are several cost considerations: i) bandwidth, ii) line usage, iii) data links.

### 11.2.4. Performance Management

### The performance of various network resources should be monitored and measured on a continuing basis.  These measurements provide management with the information needed to evaluate and determine network efficiency and reliability, and also may be used for evaluating various system modes in order to identify optimum operational network configuration(s).

Figure 11-2	Management of ATN Resources

### 11.2.5. Fault Management

### Fault management capabilities need to be established for the detection, isolation, and correction of abnormal network conditions.  Notification procedures should be developed to inform inter-connected facilities when a fault has occurred which has an adverse affect on normal operations.  These procedures should also be used for coordination when network configuration changes are required due to system fault.

### 11.2.6. Routing Management

### Routing management procedures need to be implemented to maintain control over the ATN.  According to routing policies on some portions of the ATN, the classes of messages allowed may be restricted.  Neighboring States may form agreements in order to route data through different domains in the event of a failure.  Routing tables, message routes and alternate routes must be created.

## 11.3. Naming and Addressing

## All ATM application names, and the associated addressing needed for connectivity between applications, are required to be assigned by a registry function.  This requirement is basically fulfilled by administrative actions that provide procedures for applying, registering and distributing unambiguous names and addresses.  Names are fundamentally associated with objects, thus each object that must be reachable must have a unique name in the communications environment.  The addressing describes the path, or route, needed to reach the named object.  ISO standards are available that describe the naming and addressing environment that needs to be applied to the CNS/ATM.  These standards require the establishment of naming and addressing authorities to serve the users of the ATN.  The authorities should distribute naming and addressing information on a routine basis so system managers can meet operational system requirements.

### 11.3.1 ICAO Naming

### ATM systems communicating over the ATN require the use of an unambiguous naming structure that is also consistent with the AFTN.  The 8-character AFTN address structure makes a suitable model for this purpose during transition.  Using this structure, the eighth character can be assigned to indicate an application process running on a particular end system denoted by the first seven characters.

### 11.3.2. NSAP and SNPA Address Assignments

### The addressing authority making ATN NSAP and SNPA address assignments should be aware of the need to conserve addresses to the extent operationally possible.  A single ATN NSAP address should be sufficient for a given ATM processor or terminal computer system.  A corresponding SNPA address needs to be assigned for each real subnetwork connected to an ATM system.  ATN NSAP and SNPA addresses are dynamically distributed within the routing domain to indicate the logical path to any attached ATM system.  Unnecessary assignments have a negative impact on system resources and traffic loading, resulting from the routine updating process used by the ATN routing information exchange protocols.

11.4. Security

## Security	.  Security in the ATN is necessary to prevent unauthorized access to data processing and data communications resources.  Security control is required in order to protect the information exchanged between the application processes.  The security for data processing is required in end systems and the security for data communications is required in end systems and network resources.

### 11.4.1. OSI Security

### ISO 7498-2 is the basis for the development of an ATN security model, insofar as communications security is concerned.  It is anticipated that some security mechanisms will be incorporated into the future ATM communications environment.  Inclusion of security mechanisms in SARPs is dependent upon continued analysis.  This security analysis relates the level and type of security for the different traffic types.  The OSI architecture allows for the integration of security mechanisms into the OSI protocols used at multiple layers or by the addition of security protocols.  Security mechanisms provide security by defining specific features and parameter information.

### 11.4.2.  Network Access Control

### The ATN environment may include private and public subnetworks.  The management of security for public subnetwork resources can be more difficult than for private subnetwork resources.  In ATN subnetworks, the detection of unauthorized communication is desirable at the entrance point of a domain.  This can be achieved by:

- Rejection of call from unauthorized end systems or users;

- Restriction to authorized origin NSAP addresses;

- Restriction to authorized destination NSAP addresses; and

- Restriction of communication traffic type between a specific combination of origin and destination NSAP addresses.

### 11.4.3. End System Access Control

### Unauthorized use of resources can be prevented by:

- Formal procedures for introducing new users;

- Controlling user access to relevant systems/facilities; and

- Authentication and passwords with controls to manage their use.

### 11.4.4. Security Mechanisms

### Security mechanisms can be used to provide combinations of the basic security services.  Systems may be implemented using combinations of the following mechanisms:

- Encipherment;

- Digital Signature;

- Access Control;

- Data Integrity;

- Authentication;

- Traffic Padding;

- Routing Control; and

- Notarization.

### 11.4.5. Security Management

### Security management functions for the protection of ATN resources will be developed as the requirements are defined, and will be based on ISO 7498-2.

## 11.5. Testing

## ATM and ATN elements need to be tested prior to connection to operating networks.  This testing can be divided into that needed to validate the application processing functions and that needed to validate the communication functions.  In the latter case, the implemented protocols must be tested in two, if not three different scenarios.  The first scenario ensures that the implemented protocol(s) conform to the standard.  The second scenario ensures that the protocol(s) between two communicating systems can interoperate correctly and properly exchange the required data.  The third scenario involves testing to ensure that the necessary performance is being achieved by the implementation.  Once the testing has been completed, the system element(s) can be attached to a subnetwork for operational testing and initial provision of services.

## 11.6. Configuration Control

## Each aeronautical authority should adopt configuration control procedures for managing the system configuration and interfaces associated with AFTN/ATN transition and final configuration.  These procedures are often associated with some type of document coordination and approval process to ensure accuracy and control of the technical parameters.  A single document is needed for each interface between systems operated by aeronautical authorities, and between systems operated by aeronautical authorities and other organizations.  Following initial implementation, the documents controlled by configuration management provide a stable reference for future planning, troubleshooting, and training activities.  As in all cases of technical documentation, the value of a document is directly proportional to its accuracy.  Configuration management is a process to achieve that goal.