ATNP/WG1/-WP/

SUMMARY

The Joint ATNP WG2/WG3 meetings in May 1995 (Fairfax USA) agreed on a five part structure to be adopted for the development of  draft SARPs and guidance material for the CNS/ATM-1 Package.

It was agreed that Part I (Introduction and System Level Requirements) would be developed by WG1 with input from WG2 and WG3 as appropriate.  It was further agreed that WG2 would develop the first draft of the document.  The first draft was presented at the WG2 meeting in Rome (July 1995) and the WG3 SG2 & 3 meetings in August ‘95 (Canberra).  A revised draft is attached to this WP.

PROPOSAL

It is proposed that WG1:

a. - Confirm that it will undertake the further development of Part 1;

b. - Review the attached draft in detail;

c. - Appoint an Editor for the future development of the draft;

d. - Notify WG2 & WG3 accordingly of its considerations of the above.

(Presented By Rapporteur WG2)

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WG1

Banff, Canada,

9th - 13th  October 1995

Part I of the CNS/ATM-1 Package SARPs

DRAFT

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

CNS/ATM-1 SARPs & Guidance Material

Part I - CNS/ATM-1 SARPs

Introduction and System Level Requirements for the CNS/ATM-1 Package

1. Introduction	4

1.1 Overview of  CNS/ATM-1 Package	4

2. CNS/ATM-2 and Beyond	5

3. CNS/ATM-1 System Requirements	5

3.1 Systems Management	5

3.2 Security	5

3.3 Quality of Service	5

3.4 Application Requirements	5

Table 1-1: CNS/ATM-1 Air/Ground Application General Design Requirements	6

Table 1-2: CNS/ATM-1 Air/Ground Application Priority Allocation	7

4. Formal Specification Techniques	7

4.1  Implementation Conformance Statements	8

4.2  PICS and PICS Proformas	8

4.3  Profiles, ISPs and Requirements Lists	8

4.4  ATN Profile Requirements Lists (APRLs)	9

1.	Introduction

The CNS/ATM-1 Package SARPs and Guidance Material are  documented in  “Parts” as indicated in Table 1-1 below.  Each Part will comprise the applicable SARPs and supporting guidance material.

| Part Reference | Title |
| --- | --- |
| Part I | Introduction and System Level Requirements for the CNS/ATM-1 Package |
| Part II | CNS/ATM-1 Air/Ground Applications |
| Part III | CNS/ATM-1 Ground/Ground Applications |
| Part IV | CNS/ATM-1 Upper Layer Architecture |
| Part V | CNS/ATM-1 Internet Communications Service |

This document comprises Part I of the CNS/ATM-1 Package SARPs.

## 1.1	Overview of  CNS/ATM-1 Package

The CNS/ATM-1 Package SARPs define the provisions for the following components of the CNS/ATM System:

Air/Ground Applications 				(Part II)

Ground/Ground Applications 			(Part III)

Upper Layer Architecture				(Part IV) and

Internet	Communications Service			(Part V).

### 1.2	CNS/ATM-1 Air/Ground Applications

Standards for the  following Air/Ground Applications are specified in Part II:

Automatic Dependent Surveillance 		(ADS)

Controller Pilot Data Link Communications 	(CPDLC)

Flight Information Services			(FIS)

Context Management 				(CM)

Note. - The Aeronautical Industry may define additional AINSC air/ground applications which will also require on the use of the Upper Layers and the CNS/ATM-1 Internet Communications Service.

### 1.3	CNS/ATM-1 Ground/Ground Applications

Standards for the  following Ground/Ground Applications are specified in Part III:

Message Handling Services			(MHS)

Inter Centre Co-ordination 			(ICC)

Note. - The Aeronautical Industry may define additional AINSC ground/ground applications which will also rely on the use of the Upper Layers and the  CNS/ATM-1 Internet Communications Service.

### 1.4	CNS/ATM-1 Upper Layers

Standards for the Upper Layer Architecture for both the  CNS/ATM-1 Package Air/Ground and Ground/Ground Applications are defined in Part IV.

### 1.5	CNS/ATM-1 Internet Communications Service

Standards for the Internet Communications Service are defined in Part V.

Note. - Part V  is based on the Second Edition of the ICAO ATN Manual.

# 2.	CNS/ATM-2 and Beyond

The CNS/ATM-1 comprises the provisions for the initial implementation of the ICAO CNS/ATM System.  It is expected that further air/ground and ground/ground applications will be standardised in  the CNS/ATM-2 and subsequent Packages.  It is further anticipated that the Upper Layer  and Internet Communications Service will  be enhanced in CNS/ATM-2 in order to provide meet any additional application and emerging overall system requirements (e.g. systems management, security).

# 3.	CNS/ATM-1 System Requirements

## 3.1	Systems Management

3.1.1	No overall Systems Management framework and/or standards/requirements are defined for the CNS/ATM-1 Package.

Note 1. - This does not preclude implementation of Systems Management functions on a local and/or bi-lateral basis.

Note 2. - It is expected that a Systems Management framework and applicable standards will be specified for the CNS/ATM-2 Package.

## 3.2	Security

3.2.1	No overall Security framework and/or standards/requirements are defined  for the CNS/ATM-1 Package.

Note 1. - This does not preclude implementation of Security functions on a local and/or bi-lateral basis.

Note 2. - It is expected that a Security framework and applicable standards will be specified for the CNS/ATM-2 Package.

## 3.3	Quality of Service

3.3.1	Dynamic Quality of Service (QoS) based routing in the internet  is prohibited on an inter-domain basis.

Note 1. - This does not preclude intra-domain QOS based routing which is considered to be a local matter.

Note 2. - Dynamic QOS based routing may be a requirement for the CNS/ATM-2 Package.

3.3.2	Real-time QOS monitoring is not required to be provided in the CNS/ATM-1 Package.

Note. - Inclusion of this capability in future CNS/ATM packages is not precluded.

## 3.4	Application Requirements

### 3.4.1	Air/Ground Application Requirements

3.4.1.1	The  allowable Transit Delay  (TD)  for each CNS/ATM-1 Application is specified in Table 1-1 below

Note. - The transit delay requirements will be satisfied through appropriate network design.

3.4.1.2	The Residual Error Rate (RER)  for each CNS/ATM-1 Application is specified in Table 1-1 below.

Note. - For the CNS/ATM-1 Package all applications  have the same value and will be satisfied through appropriate network design.

3.4.1.3	The Service Loss Reporting period for each CNS/ATM-1 Application is specified in Table 1-1 below.

Note. - For the CNS/ATM-1 Package all applications  have the same value.

3.4.1.4	The Availability for each CNS/ATM-1 Application is specified in Table 1-1 below.

Note. - Application availability requirements will be satisfied through appropriate network design.

3.4.1.5	The Service Restoration Time for each CNS/ATM-1 Application is specified  in Table 1-1 below.

|  | Transit Delay (TD) | Residual Error Rate (RER) | Service Loss Reporting | Availability | Service Restoration Time |
| --- | --- | --- | --- | --- | --- |
| ADS | TBD | TBD | TBD | TBD | TBD |
| CPDLC | TBD | TBD | TBD | TBD | TBD |
| ATIS (FIS) | TBD | TBD | TBD | TBD | TBD |
| CM | TBD | TBD | TBD | TBD | TBD |

## Table 1-1: CNS/ATM-1 Air/Ground Application General Design Requirements

3.4.1.6	For CNS/ATM-1 Air/Ground Applications sequentially ordered message delivery capability shall be  provided  (e.g., where succeeding message delivery is dependent upon the successful delivery of preceding messages).

Note 1: - This requirement is satisfied through use of the ISO Connection Oriented Transport Protocol within the ATN Internet End Systems as specified in Part V of the SARPs.

Note 2. - The CNS/ATN Internet Communications Service as specified in Part V of the SARPs also provides a connectionless transport service through use of the ISO Connectionless Transport Protocol which may be applicable for ground/ground applications such as radar data exchange.

3.4.1.7	a.	The communications service shall provide an orderly termination of service upon indication by the application (e.g. if messages  have been passed to the communications service and then a termination of service is requested, the preceding messages are to be delivered as per normal operations before the service is terminated).

Note: - This requirement is satisfied through the service provided by the ATN Upper Layer Architecture as defined in Part IV of the SARPs.

3.4.1.7	b.	Upon failure of orderly termination an indication shall be provided to the application.

Note: - This requirement is satisfied through the service provided by the ATN Upper Layer Architecture as defined in Part IV of the SARPS.

3.4.1.8.	CNS/ATM-1 Air/Ground Applications shall use priority in a manner consistent with ITU radio regulations.  The priority relationship (WG3 to Confirm) between CNS/ATM-1 Air/Ground Applications and the ITU categories  is documented in Table 1-2 below.

Note. -  The detailed provisions relating to priority handling within the internet  are contained in Part V of the SARPs.

| ITU Category | Description | CNS/ATM-1 Application |
| --- | --- | --- |
| 1 | Distress calls, distress messages and distress traffic |  |
| 2 | Communications proceeded by the urgency signal |  |
| 3 | Communications relating to radio direction finding | ADS |
| 4 | Flight safety messages | CPDLC |
| 5 | Meteorological messages |  |
| 6 | Flight regularity messages | FIS (ATIS) CM |
| 7 | Messages relating to the application of the United Nations Charter |  |
| 8 | Government messages for which priority has been expressly requested |  |
| 9 | Service communications relating to the working of the telecommunication service or to communications previously exchanged |  |
| 10 | Other aeronautical communications |  |

## Table 1-2: CNS/ATM-1 Air/Ground Application - ITU Priority Mapping

3.4.1.9.	CNS/ATM-1 Air/Ground  Applications shall  specify  routing policies based on the class of communications service required (class “A” through “H” inclusively), each class being based on the transit delay of the available mobile subnetworks.

Note 1. - Aeronautical Industry  Air/Ground applications (AINSC) will specify routing policies based on mobile subnetwork selection and/or mobile subnetwork preference lists.

Note 2. - The detailed provisions relating to handling the routing policy required by both CNS/ATM-1 air/ground Applications and AINSC air/ground applications are defined in Part V of the SARPs.

3.4.1.10	A message delivered to the communications service shall not be delivered more than once to its peer entity.

Note. - This requirement is satisfied through the use of the ISO Connection Oriented Transpiort protocol as defined in Part V of the SARPs.

### 3.4.2	Ground/Ground Application Requirements

TBD by WG3

# 4.	Formal Specification Techniques

Parts IV & V of the CNS/ATM-1 SARPs have adopted an ISO based technique to support the  specification of requirements for both the Internet and Upper Layer protocols.  These  requirements are documented in tabular fashion in  “ATN Protocol Requirements Lists” (APRLs) .  This section provides the background information necessary to correctly interpret these APRLs.

Text has long been recognized as being inexact and potentially ambiguous with regard to the specification of protocols and their profiling.  In the CNS/ATM-1 SARPs, the decision was reached to conform the formal description techniques used for profiling within the ISO community.  These techniques are defined by OSI Conformance Testing Methodology and Framework: Implementation Conformance Statements (ISO 9646-7) and are referenced and used by Framework and taxonomy of International Standardized Profiles (TR-10000).  Within the SARPs , APRLs and MORTS are used to further specify or profile the standards as defined for use within the ATN.

## 4.1  Implementation Conformance Statements

Initial basis for APRLs and MORTS come from ISO 9646-7 which specifies the purpose and notational conventions for the writing of Implementation Conformance Statements (ICSs).  ICSs consist of tables organized by individual features (questions, functions, PDUs, parameters, components, etc.) which define implementation conformance constraints on some reference specification.  In the context of the SARPs  these specifications are for protocols, profiles and information objects (such as managed objects).  Features designate rows in these tables to a detailed enough level to allow conformance to the specification referenced to be verified or tested for conformance depending upon the purpose of the specific ICS.

No generic format or proforma for an ICS is provided by the standards because of the wide variety of specifications for which conformance requirements are stated.  Nevertheless 9646-7 and the SARPs state general rules, guidance and notational conventions needed to allow the ICSs (APRLs and MORTs) to be defined and interpreted unambiguously.

## 4.2  PICS and PICS Proformas

The development of ICSs grew out of the introduction into ISO standards of conventions to allow conformance testing of protocols.  ISO 9646-2 states that in order to provide an effective basis for conformance testing of implementations ISO protocol standards must include Protocol Implementation Conformance Statement (PICS) Proforma.  A PICS Proforma is considered a necessary pre-condition for the generation of an abstract test suite for a standard and in general PICS Proforma have been produced for all ISO protocol standards and are typically published as a normative part or annex to the protocol standard.

PICS Proformas are questionnaires or templates which cover all the known protocol features which apply to the process of static conformance testing for the standard.  These templates contain one or more empty columns which indicate the support for and if appropriate the nature of support for the feature corresponding to that row.  Once an implementor has filled in the support material for the support column(s), the PICS Proforma becomes simply a PICS which is specific to the implementation the conformance statement is created for.

## 4.3  Profiles, ISPs and Requirements Lists

As various regional and application profiles were developed to further constrain the options within the ISO standards, it was recognized that text was no better a specification tool for profiling than it was for the definition of the standards themselves.  As efforts within ISO came together to harmonize and standardize the various functional sets of profiles in progress, the definition of ICS became broadened to include more than PICS Proforma and PICS.

Initially the effort to create PICS Proforma for profiles that further constrained the PICS Proforma for a standard, but were still not specific to an implementation was attempted.  The theory was that a Proforma could be used to define PICS and conformance testing for the profiles being specified.

PICS Proformas are concerned with static conformance requirements (i.e. what capabilities have to be or are allowed to be implemented in a conforming system).  Profiles are more concerned with the use of implemented capabilities in order to meet requirements for interworking.  A PICS Proforma only defines static conformance and does not reflect dynamic behavior, but the standard and profiles can.  In an effort to allow conformance and constraints on dynamic behavior to be specified, Requirements Lists (RLs) were defined as another of the ICS types.

As International Standardized Profiles (ISPs) were developed, ISP Requirements Lists (IPRLs) were developed to allow the requirements for interoperability in accordance with these profiles to be specified.  In particular notation is added to the RL notation to specify both status of features for both implementation and use (e.g. optional for implementation, prohibited for use).

An RL expresses the constraints upon allowed answers in a corresponding PICS.  Profile RLs are derived from the PICS Proforma of the base standard in question with its entries enabled, disabled, or pre-selected according to the profile's choices.  Any special requirements that do not originate in the base standard should be documented in the profile RL.

It should be noted that ISO has prepared a copyright release with respect to all PICS Proforma stating that: "Users of this International Standard may freely reproduce this ICS Proforma so that it may be used for its intended purpose and may further publish the completed ICS."

Many variations exist in ICS Proforma format and conventions.  Almost all contain the four columns in the following sections. These columns are generically identical in meaning, but often use differing headings to be more specific to the features or items contained in each specific table.  The various names for these headings are called out below within the corresponding column section below.  Other details which relate to the conventions (of the many options within ISO 9646-7) used in these SARPs  are described within the sections specifically describing APRLs and MORTS.

### 4.3.1  Item

Usually the first column in an ICS Proforma, this entry contains a short mnemonic to indicate the feature for reference.  This column is also referred to as index in some ICS Proforma.

### 4.3.2  Description

This column can have many names including Capability, Feature, Function, (N)PDU, Parameter, Dependency, Timer, etc. It is usually the second column in the ICS Proforma and specifies the row item in a longer and hopefully more common sense way than the item mnemonic.  If the feature is a question the question is written out in this column.

### 4.3.3  Reference

This column references the feature back to the base standard (or other references, as appropriate).  The entry should be as specific as possible and reference to the section number in the base standard.  In some cases this column will contain references to specific sections in these SARPs, particularly for tables needed that are not based on ISO PICS Proforma tables.

### 4.3.4  ISO Status

This column indicates the ISO status of the feature in the base standard.  Typically this uses a single character indicator for mandatory, optional, conditional, excluded, out of scope or not applicable.

## 4.4  ATN Profile Requirements Lists (APRLs)

Within these SARPs  the conventions developed and used to specify IPRLs are used to define APRLs in appendices 8, 9, and 11.  Most of the component tables in APRLs are produced by copying selected tables from the relevant base standards PICS Proformas, removing the column(s) to be completed by the supplier and adding a new column giving the CNS/ATM-1  requirements or support, both in terms of status and value ranges.  Where appropriate ATN specific tables and features have been created to document what has been agreed to and is best documented in this form.

Both "ISO Status" and "CNS/ATM-1 Support/Requirement" columns should contain whether the feature indicated for that row is mandatory (M), conditional (C), optional (O), excluded (X), out of scope (I) or not applicable (-).

SARPs material has traditionally been divided into Standards containing mandatory features indicated by statements containing the word "shall", and recommendations which contain the word "recommended".

This approach is not completely consistent with ISO ICS notation.  While Mandatory (M) is considered to be synonymous within the SARPs  material to "shall" standard statement, the category Optional (O) contains both items that are recommended and options that are not precluded or recommended.

In order to maintain conformance to the ICS notation and provide information on the recommendations within the APRLs, tables of recommendations which are cross referenced to point to optional items elsewhere within the APRLs using the Item or Index value have been created.  In order for users of these SARPs  to take standards and recommendations into account, the appendix material should be read in conjunction with the APRLs, as there are both standards and recommendations which do not correspond to items in the APRLs.

### 4.4.1  Footnotes

Where explanations are needed they can be added at the bottom of the page, often referencing back to the item identifier in the table.  Footnotes referencing status and support column items can be indicated by the status symbol immediately followed by an integer (e.g. C2).  This notation is further extended for grouped items.

For groups of mutually exclusive or selectable options among a set, footnotes are created by placing after the "O" (for optional) a period followed by an integer.  The following table shows an example of a group of three related options and means that the implementation shall support at least one of the group of options numbered 4.

| Item | Feature | Reference | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| Item_1 |  |  | O.1 |  |
| Item_2 |  |  | O.1 |  |
| Item_3 |  |  | C2 |  |
| Item_4 |  |  | O.3 |  |
| Item_4 |  |  | O.3 |  |

Example wording below might be:

O.1: support for at least one of these options is required

C2:  Item_2:M  - If Item_2 is supported then Item_3 is mandatory.

O.3: support for exactly one of these options is required.

#### 4.4.2  Dynamic Requirements

In the case where the static requirements differ from the dynamic requirements, ATN Support/Requirement columns can contain a two character notation that specifies the static (implemented) and dynamic (used) requirements.  If static and dynamic requirements are identical, a single character should be used.  The three simple cases shown below:

Item 1: optional to be implemented, optional to be used if implemented;

Item 2: mandatory to be implemented, optional to be used;

Item 3: optional to be implemented, prohibited for use if implemented;

are expressed in tabular form as:

| Item | Feature | Reference | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| Item 1 |  |  | O | O |
| Item 2 |  |  | O | MO |
| Item 3 |  |  | O | OX |

Note that this two character representation only applies to the ATN Support column where dynamic requirements can be expressed.  Note also that in the example above, the profile made Item 2 mandatory for implementation, while the ISO Status reflected in the PICS Proforma only had Item 2 optional.

#### 4..4.3  Predicates

Within these SARPs  a predicate is typically denoted using the item mnemonic associated with a feature in the same APRL.  Predicates in general can grow in complexity from a simple YES/NO entry, to a relational expression involving a reference within the APRL which gives a value as an answer, or to a predicate expression (i.e. a Boolean expression involving multiple predicates).  Whatever the nature of the predicate, it should resolve to a YES/NO or TRUE/FALSE value which can be used elsewhere in the APRL.

#### 4.4.4  Conditional Requirements

Any conformance requirement in the base standard or APRL may be made conditional upon some predicate.  If the base standard includes a conditional requirement, then the APRL must use the same predicate, but it may be possible to partially or fully evaluate it given the conditions that are known to apply in the APRL.  If such a predicate is fully evaluated in the APRL, then the requirement becomes unconditional and should be specified as such in the ATN Support column.

#### 4.4.5  Logical Negation

The logical negation symbol used in the SARPs  is the circumflex symbol, "^", due to its wide availability in character sets.  This symbol can also be used within predicates.

#### 4.4.6  Flagging Predicate References

An asterisk may be used to prefix the mnemonic reference in the item column for any feature that is referenced by a predicate or conditional expression elsewhere in the APRL.

#### 4.4.7  Conditional Expressions

A "c" followed by an integer can be placed in the status column, providing a reference to a conditional status expression defined elsewhere in the APRL.  These conditional status expressions are "if .. then .. else" expressions depending on the predicate following the IF.   If a conditional requirement does not specify an "else" case, then the implied else case is optional.   As an example, in the following table:

| Item | Feature | References | ISO Status | ATN Support |
| --- | --- | --- | --- | --- |
| P1 |  |  | C1 | C1 |
| P2 |  |  | C2 | C2 |

footnotes C1 and C2 might be defined as:

C1: if P0 then M

C2: if P1 then (if (P4 AND ^P3) then M) else O

where c1 implies that for ^PO, P1 is optional.  The use of AND and OR in predicate expressions should be consistent with boolean logic.