WG1/IP 3-2____

JWG/WP____

Aeronautical Telecommunication Network (ATN) Panel

Working Group 1

Joint Working Group Meeting

Banff, Canada

October 9 - 12, 1995 (WG1)

October 13, 1995 (JWG)

EVALUATION OF ATN/AMSS END-TO-END PERFORMANCE

BY COMPUTER SIMULATION

presented by:

Ron Jones

Prepared by:

Sinisa Maravic, Wassim Ferzali and Vassilis Zacharakis

This paper presents a comprehensive set of simulation results obtained using a high-fidelity ATN/AMSS Computer Model developed by the U.S. Federal Aviation Administration (FAA).  The simulation results presented herein encompass the ATN/AMSS end-to-end packet data transit delay performance for two AMSS channel rates, 600 and 10,500 bps, sensitivity of transit delay to variation in network configuration parameters, and routing initiation (BIS-BIS) and transport connection establishment delay performance.

EVALUATION OF ATN/AMSS END-TO-END PERFORMANCE

BY COMPUTER SIMULATION

1. Introduction

This paper is a follow on to a previous two papers titled “Evaluation of ATN/AMSS End-to-End Transit Delay performance” [1] and “Assessment of BIS-BIS and Transport Connection Establishment Delays” [2], presented on ICAO ATNP Working Group 2 meetings in March and May of 1995.  The results on the end-to-end transit delays are updated since the traffic model used for the simulation runs was changed to correspond to the latest traffic model developed by RTCA [3].  In addition, this paper presents new results obtained for the highest AMSS channel rate at 10,500 bps.  It also contains assessesment of the sensitivity of the end-to-end transit delay in terms of  two additional parameters, COTP acknowledgment and retransmission timers.  To obtain these results, we used the high-fidelity ATN/AMSS Computer Model developed by Mayflower Communications Company, Inc. and the MITRE Corporation with sponsorship of the U.S. Federal Aviation Administration (FAA).

2. Description of the ATN/AMSS Computer Model and Simulation Scenarios

ATN/AMSS Simulation Architecture. The ATN/AMSS model was developed using the Optimized Network Engineering Tool (OPNET), developed by MIL-3, Inc.  The principles of the International Organizations for Standardization (ISO) Open Systems Interconnection (OSI) architecture are used as the basis for development of ATN.  The configuration analyzed in this paper is illustrated in Figure 1. The following ISO protocols are included: ISO 8073 Connection Oriented Transport Protocol (COTP) class 4, ISO 8473 Connectionless mode Network Protocol (CLNP) and the AMSS-specific  Subnetwork Dependent Convergence Function (SNDCF) over ISO 8208, ISO 10747 Inter-Domain Routing Protocol, and ISO 9542 End System to Intermediate System Routing Information Exchange Protocol. The ATN/AMSS computer model that simulates the end-to-end system is comprised of the Air and Ground ATN Routers and End Systems.

As depicted in Figure 1, the ATN applications interface with the ATN Router's CLNP Layer via the Transport Layer.  The remaining host system's protocol stack is circumvented at this time since it is expected not to significantly affect the system performance.

The interface between the SNDCF and the underlying subnetwork is the ISO 8208, via the exchange of primitives with the ISO 8208 Data Terminal Equipment.  As specified in the ATN Manual [4] and CNS/ATM-1 Package SARPs and Guidance Material [5], the existence of the connection mode SN-Service relieves the SNDCF from the need of synchronizing its actions with the peer SNDCF.  It is left to the underlying service to preserve the order of packet transmission and recover from information loss.  The SNDCF module assumes that the interfacing sub-network provides this service.

Figure 1. ATN/AMSS OPNET Model Protocol Stack

As shown in Figure 1, the ATN end-to-end data path consists of an application traffic generator interfacing directly with the transport layer (COTP), which is interfacing directly with the ATN Router, thus excluding at this time any intervening ground or aviation subnetworks.  The ATN routers' DTE directly interfaces with the DCE of the air-ground subnetwork, in the case of the AMSS model.  The topology of the ATN is reduced to multiple aircraft routers communicating with one ground router.  The delays obtained with this simulation effort represent the bulk of the true ATN end-to-end delay since AMSS is thought to be the subnetwork with the least data bandwidth capability.

The ATN/AMSS simulation encompasses the complete functionality of the AMSS data communication service including the following process models: Physical Layer Model, Link Layer Model, Subnetwork Layer Model, and AES/GES Management Model.

Simulation Scenarios. The ATN/AMSS computer model uses the traffic model that was developed by RTCA SC-165 WG-3.  Table 1 in Appendix I presents the developed traffic model.  For AMSS, one P channel, one T channel, and a variable number of R channels are selected to maintain a desired loading level.  Separate simulations were performed for the 600 bps and 10,500 bps AMSS channel rates, at the desired AMSS channel occupancy levels of 60 - 70 % on the P or T channel, and 10 - 12 % on the R channel.  Specific to the BIS-BIS connection establishment experiments, one aircraft was used to repeatedly perform the BIS-BIS connection establishment procedure, while all other aircraft were transmitting and receiving user data to achieve the desired AMSS channel loading levels.  Similarly, for the transport connection establishment experiments, one aircraft was used to repeatedly perform transport connection establishments at all priority levels, without exchanging user data, while all other aircraft were exchanging user data.

3. BIS-BIS Connection Establishment Delays

The BIS-BIS connection establishment delay is measured as the time elapsed from the transmission of the AMSS log-on request from the aircraft until the receipt of the first IDRP KEEPALIVE PDU at the aircraft’s BIS.  Defined this way, the BIS-BIS connection establishment delay encompasses the delays associated with the AMSS log-on procedure and the establishment of the required Virtual Circuit between the communicating SNDCF entities of the ground and aircraft ATN routers, in addition to the delay incurred by the exchange of the required protocol data units.  The results for the BIS-BIS connection establishment delay experiment, together with its components, AMSS log-on procedure and Virtual Circuit establishment for the 600 bps AMSS channel rate are presented in Table 2.

|  | BIS-BIS Establish. | Log-on | VC Establish. |
| --- | --- | --- | --- |
| # of points | 800 | 800 | 800 |
| Mean | 112 | 40 | 42 |
| Median | 93 | 21 | 31 |
| Std Dev. | 66 | 46 | 30 |
| 95% | 233 | 133 | 106 |

Table 2.  	Delay statistics for the BIS-BIS Connection Establishment, AES Log-on, and Virtual Circuit Establishment, 600 bps (in seconds).

4. COTP Connection Establishment Delays

The transport connection establishment delay is defined as the elapsed time from the transmission of a Connection Request (CR) TPDU from the local Transport Layer Entity until the receipt of a Connection Confirm (CC) TPDU from the remote Transport Layer Entity.  In other words, this is the time that elapses from the instant a request to establish a connection is made by the application until the instant the Transport Layer Entity is able to send data.  This assumes that any subnetwork (AMSS) virtual circuit (VC) required to convey the Transport Entities TPDUs is available prior to the request to establish a Transport Connection.  Figure 2. presents results for COTP connection establishment delays for both 600 and 10,500 bps channel rates.  It shows that measured delays do not depend on the initiating side, i.e. they are almost equal in both directions.  It is true, since the SNDCF module multiplexes more than one ATN priority to one subnetwork (AMSS) priority, as specified in the ATN SARPs, and maintains this connection independently of the COTP connections.  Since the connection establishment phase entails essentially the exchange of two TPDUs of equal size (CR and CC), one in each direction, the delay to complete the procedure is essentially the same regardless of whether the connection was initiated from the aircraft or from the ground.

Figure 2. Transport Connection Establishment Delay vs. Priority

5. End-to-end Performance

5.1. Performance Metrics

We selected three parameters for the purpose of the end-to-end performance assessment: capacity, end-to-end data transit delay, and throughput.

Capacity is defined as the number of aircraft that allow the satellite link (AMSS) to operate at or near the desired P (TDM uplink) and T (TDMA downlink) channel loading.  The desired operating load levels for the AMSS P and T channels are 60 to 70 percent.  Operating at such loading levels will still allow the AMSS delay specification of the ICAO SARPS to be met.

The end-to-end data transit delay for ATN is defined as the elapsed time from the submission of a T-DATA.Request primitive from the application to the transport layer entity, until the receipt of a T-DATA.Indication primitive by the receiving application.  We register data transit delay after the required COTP and AMSS connections are established.  In the simulation, a COTP connection is established per priority level.

Throughput per user is defined here as user bits received per second at a given destination based on the receipt of the last 9 of 10 test user-data packets of length 128 octets each.  This throughput is the measure of the maximum sustained user’s bit flow per second between an aircraft user and a peer ground user per a specific priority per one network connection while the system is operating near the expected capacity.  Throughput is computed as:

Throughput(bs)=(N−1)´128´8T2−T1

where N = 10 (number of packets generated for experiment purpose), T1 is the time (in seconds) of receipt of the first packet of the subject N packets and T2 is the time of receipt of the final packet of the subject N packets.

5.2. Performance Results

Numerous simulation runs were executed under varying scenarios in order to collect results pertinent to the analysis presented in this paper. In the following sections we present these results for each of the defined performance metrics.

5.2.1. Capacity

In Figure 3. we present capacity measured for both 600 and 10,500 bps AMSS channel rates.  As depicted, required loadings are obtained with 40 AES operating at 600 bps, and 720 AES operating at 10,500 bps.  Loading for 720 AES is reached in a network model of 90 AES with each AES generating the nominal traffic load multiplied by a factor of 8, i.e., each AES in the simulation represent the equivalent of 8 separate AESs with nominal traffic. A network simulation with 720 separate AESs was not feasible due to the limitations of the computer workstation we used, i.e., memory and processing power.  This required some modifications to some of the transport layer parameters chosen for the experiment, as further explained in the following text.

For both rates, 600 and 10,500 bps, measured AMSS transit delays even at the highest capacity level used met the ICAO AMSS SARPS specifications.

Figure  3. 	AMSS Channel Load versus number of Aircraft, 600 bps (left) and 10,500 bps (right) AMSS channel rates

5.2.2. End-to-end Transit Delay

For the simulation results presented herein we considered four priority groups in ATN, as shown in Table 3.  They are segregated after the mapping of ATN to subnetwork priorities specified in the ATN SARPs.  The application traffic model that we used was provided by RTCA SC-165 WG-3, and is described in Appendix I.

| Traffic Type | Priority Level |
| --- | --- |
| Urgent | 0 - 2 |
| Flight Safety | 3 - 4 |
| Other Safety | 5 - 7 |
| Normal | 9 - 14 |

Table 3. ATN Priority Grouping

We analyzed four network parameters and their effect on the end-to-end transit delays. These parameters are: AMSS channel loading, maximum TPDU size, COTP acknowledgment timers and COTP retransmission timers.  End-to-end transit delays were measured per priority group for network with 40 AES logged on at 600 bps and 720 AES at 10,500 bps, and maximum TPDU size of 512 octets.  The decision factors for selecting the values for COTP acknowledgment and retransmission timers are: transit delay performance, and subnetwork reliability and bandwidth.  According to these factors, we selected values for acknowledgment timers and retransmission timers as presented in Table 4.  The selection of these parameters was mainly based on simulation runs conducted to assess the sensitivity of transit delay performance to variation in these parameters.  The sensitivity results and the justification for the parameter selection are provided in Section 5.2.3.  The end-to-end transit delay for uplink and downlink messages are given in Figures 4 to 7.  The Figures depict results for downlink messages with length greater than 15 octets (those transmitted on the T channel in AMSS).  Transit delays performance for short messages that are transmitted on the R channel in AMSS are given separately in Table 5.

| COTP Timer Values | Priority Group |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urgent | Flight Safety | Other Safety | Normal |
| Acknowledgment: 600 bps AC / Ground | 40 / 70 | 50 / 80 | 60 / 90 | 70 / 100 |
| Retransmission: 600 bps AC / Ground | 290 / 310 | 310 / 340 | 390 / 420 | 480 / 510 |
| Acknowledgment: 10,500 bps AC / Ground | 160 / 280 | 200 / 320 | 240 / 360 | 280 / 400 |
| Retransmission: 10,500 bps AC / Ground | 190 / 310 | 230 / 350 | 280 / 400 | 330 / 450 |

Table 4. COTP Timer Values in seconds

Note that the “jumps” in the transit delay performance are due to the effect of COTP segmentation of data messages in accordance with the set Max TPDU size.  Also note that, since the effect of message prioritization is more prominent in the uplink direction, the uplink delays tend to have large variation going from high to low priority than in the downlink direction, where no such spread is shown.  Due to this effect, we repeatedly notice better performance for higher priority messages in the uplink than in the downlink direction, and vice versa for low priority messages.

Figure 4.	Mean end-to-end transit delay in seconds, from-aircraft (left) and to-aircraft (right) 		direction for 600 bps AMSS channel rate.

Figure 5.	95 % end-to-end transit delay in seconds, from aircraft (left) and to-aircraft (right) direction for 600 bps AMSS channel rate.

Figure 6.	Mean end-to-end transit delay in seconds, from-aircraft (left) and to-aircraft (right) direction for 10,500 bps AMSS channel rate.

Figure 7.	 95 % end-to-end transit delay in seconds, from aircraft (left) and to-aircraft (right) direction for 10,500 bps AMSS channel rate.

| channel rate | message length | mean delay (sec) | 95 % delay (sec) |
| --- | --- | --- | --- |
| 600 bps | 0 - 15 octets | 10.7 | 25.4 |
| 10,500 bps | 0 - 15 octets | 2.4 | 7.8 |

Table 5.  Average transit delays for short messages transmitted via R channel.

The difference in the behavior with the respect to priority level is observed between the from- and to- aircraft directions in Figures 4 to 7 above.  This occurs because of the prioritization mechanisms used by AMSS in the two directions.  In the from-aircraft transmissions, prioritization can only take place for data sent by the same aircraft; data of two different priorities transmitted from two different aircraft will be treated in exactly the same way.  On the other hand, in the to-aircraft direction the prioritization scheme is applied to all packets regardless of the aircraft to which they are addressed.

5.2.3. End-to-end Transit Delay Sensitivity Analysis

5.2.3.1.  AMSS Channel Loading

Different levels of AMSS channel loading conditions were simulated using a variable number of aircraft. The sensitivity of the mean end-to-end transit delay of flight safety traffic for variation in logged on number of AESs is given in Figure 8 for the 600 bps case and in Figure 9 for the 10,500 bps case.  From both figures we observe that end-to-end delays increase more in the downlink than in the uplink direction when some additional number of AESs log on.  This is caused by the nature of the R and T channel, which are more prone to the effect of loading than the time-multiplexed P channel. However, loading eventually impacts P channel, after the operating channel load is exceeded.  We have selected to measure the end-to-end transit delay performance at the maximum channel loading levels that maintain a normal behavior without violating the performance specification of the ICAO AMSS SARPs.  The maximum loading levels that meet these constraints are obtained with 40 and 720 aircraft for 600 bps and 10,500 bps, respectively.

Figure 8.	Mean end-to-end transit delay in function of number of the AESs, from-aircraft (left) and 	 	       to-aircraft (right) direction, for flight safety messages and 600 bps AMSS channel rate.

Figure 9.	Mean end-to-end transit delay in function of number of the AESs, from-aircraft (left) and 			to-aircraft (right) direction, for flight safety messages and 10,500 bps AMSS channel rate.

5.2.3.2.  Maximum TPDU Size

Maximum TPDU size defines the maximum allowable size that TPDUs submitted to the CLNP can have.  This parameter controls the degree of segmentation performed by the Transport Layer and the AMSS Subnetwork Layer for a given traffic profile.  Segmentation in the AMSS occurs for packets arriving at the AMSS with size greater than 503 octets.  ATN SARPs specify several allowable maximum TPDU sizes, with 128 octets as a minimum and 8192 octets as a maximum.  Four lowest permitted Maximum TPDU sizes are used in this set of simulation runs: 128, 256, 512 and 1024 octets.  We collected end-to-end transit delays for the flight safety messages.  Results are presented in Figures 10 and 11.

Figure 10.	Mean end-to-end transit delay in function of Max. TPDU size, from-aircraft (left) and to-aircraft (right) direction, for flight safety messages and 600 bps AMSS channel rate.

Figure 11.	Mean end-to-end transit delay in function of Max. TPDU size, from-aircraft (left) and to-aircraft (right) direction, for flight safety messages and 10,500 bps AMSS channel rate.

Through this experiment, our intent was to demonstrate the benefit of using higher maximum TPDU size, and emphasize the degradation of service if the lowest of the COTP allowed values are used.  However, when using higher TPDU sizes, retransmissions resulting from virtual circuit resets within AMSS may cause inefficient use of available, limited AMSS bandwidth.  Thus, we selected 512 octets for the maximum TPDU size as a compromise.

5.2.3.3. Retransmission Timer Values

Retransmission time is defined (ISO 8073) as the duration of time during which the local transport entity awaits an acknowledgment before retransmitting the TPDU.  The probability of LSDU loss in the AMSS is very low, since AMSS is considered a highly reliable subnetwork.  The transfer delay, however, fluctuates mainly due to the variation in the packet lengths and instantaneous loading on the channels coupled with bit error rates and retransmissions within AMSS.  Thus, given that the bandwidth in AMSS is limited, it is useful to select retransmission timer values large enough to avoid unnecessary retransmissions.  The retransmission timer values are calculated using the following formula (ISO 8073):

T=ELR+ERL+AR+x,

where

ELR is the expected local-to-remote transit delay;

ERL is the expected remote-to-local transit delay;

AR is the remote acknowledgment time;

x is the local processing time for a TPDU.

The main contributing terms in this formula are the expected packet transit delays ELR and ERL.  The retransmission timer value for a transport connection must be selected based on the worst transfer delays and allow some additional time to prevent unnecessary retransmissions.  For the chosen set of COTP acknowledgment timer values AR, given in Table4 we calculated three different sets for the retransmission timer values, for each channel rate, by varying the expected delays ELR and ERL.  The values for the retransmission timers are provided in Table 6.  The end-to-end delays for flight safety messages in function of the three sets of retransmission timer values are presented in Figure 12.  Through this set of experiments we demonstrate that the retransmission timer value does not impact the model performances much when running on the either 600 bps or 10,500 bps channel rate.  Thus, the selection of the retransmission timer values was mainly dictated by the selection of the acknowledgment timer values which will be discussed in the next section.  The selected values are given in set I for both 600 and 10,500 bps cases shown in Table 6.

|  |  | Priority Group |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Urgent | Flight Safety | Other Safety | Normal |
| AC/GND 600 bps, set I |  | 290/310 | 310/340 | 390/420 | 480/510 |
| AC/GND 600 bps, set II |  | 220/260 | 240/280 | 270/310 | 320/360 |
| AC/GND 600 bps, set III |  | 170/210 | 200/240 | 230/270 | 290/330 |
| AC/GND 10.5 kbps, set I |  | 190/310 | 230/350 | 280/400 | 330/450 |
| AC/GND 10.5 kbps, set II |  | 175/295 | 218/338 | 263/383 | 310/430 |
| AC/GND 10.5 kbps, set III |  | 169/289 | 213/333 | 257/377 | 302/422 |

Table 6. COTP Retransmission Timer Values in seconds

Figure 12.	Mean end-to-end transit delay in function of COTP retransmission timer value, from-aircraft (left) and to-aircraft (right) direction, for flight safety messages and 600 and 10,500 bps AMSS channel rate.

5.2.3.4. COTP Acknowledgment Timer Values

Network performance as a function of COTP acknowledgment timer values is studied in this section.  Acknowledgment time is the maximum time which elapses between the receipt of a TPDU from the Network Layer and the transmission of the its acknowledgment.  Depending on traffic model, i.e. type of messages and their distribution function, there are different requirements for the receiving transport entity to acknowledge one or more messages with one acknowledgment PDU.  We measured end-to-end transit delay for the flight safety message for three different sets of acknowledgment timers.  For the 600 bps case these values are presented in Table 7.  A timer value of 1 second is an adequate choice in the case when each message needs to be acknowledged immediately after it is received.  Values in Set II reflect expected transit delays from the remote to the local entity, while the Set III contains values much bigger than the expected transit delays.  Results from the experiments at the 600 bps AMSS channel rate are depicted in Figure 13.  We selected set II timer values in this case.

|  | Priority Group |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urgent | Flight Safety | Other Safety | Normal |
| Set I: AC / Ground | 1 / 1 | 1 / 1 | 1 / 1 | 1 / 1 |
| Set II: AC / Ground | 40 / 70 | 50 / 80 | 60 / 90 | 70 / 100 |
| Set III: AC / Ground | 140 / 400 | 180 / 460 | 220 / 500 | 260 / 600 |

Table 7.  COTP Acknowledgment Timer Values in seconds (600 bps)

Figure 13.	Mean and 95 % end-to-end transit delay in function of COTP acknowledgment timer value, from-aircraft (left) and to-aircraft (right) direction, for flight safety priority and channel rate 600 bps.

In the 10,500 bps channel rate experiment, in order to achieve the required loading levels on the P and T channels, we used a network of 90 AESs with the traffic per AES multiplied by a factor of 8.  This approximates a network of 720 AESs.  Under these conditions, a large number of flight safety messages, particularly ADS messages, in the downlink direction were generated.  If we were to allow that each message gets acknowledged separately, the resultant uplink traffic increases to a level at which the AMSS P-channel Link Layer flow control mechanisms will start effecting performance of particularly  the flight safety traffic.  We noticed that the transit delays for this type of messages increase and become higher than the transit delays of the lower priority message.  Therefore, we needed to reduce this uplink traffic on P-channel due to COTP acknowledgment. One solution was to increase the acknowledgment timer values, particularly in this case for the ground COTP, in order to enforce multiple message acknowledgment with a single acknowledgment TPDU.  In order to maintain the flow of traffic in the downlink in this case, the initial transmitting window size had to be increased also.  The intent here was to remedy the transit delay problem while maintaining the same service capacity, i.e. 720 AESs.  Simulation results for the various sets of acknowledgment timer values shown in Table 8 are presented in Figure 14.  We selected Set II of Table 8 for timer values, as shown earlier in Section 5.2.2, to provide the end-to end transit delay statistics.  As shown in Figures 6 and 7 in Section 5.2.2, the effect of message prioritization scheme is preserved with this selection of timer values.  Figure 14 shows the improvement of safety traffic transit delay performance going from set I to set II of timer values, specifically in the to aircraft direction and the 95% statistics.  Further increase in acknowledgment timers, Set III, did not realize any appreciable improvement.  The potential drawback using this remedy may be the impact of delayed acknowledgment on the operations of the higher layers (above COTP), although this cannot be assessed at this time.  If this proves to be a problem, solutions other than increasing acknowledgment timer values could be devised.  One simple such solution would be to reduce the load on the AMSS by operating at lower capacity, i.e. less number of logged on AESs.  Further investigation will be required to determine the maximum capacity at which acceptable results could be obtained in this case.

It is worth noting that the actual values for the acknowledgment timers were suitable for this particular scenario, i.e. 90 AES at 8 times normal traffic factor which may not be so in reality when actual number of AESs is present.  However, this exercise point to potential problematic area in the ATH operations.

|  | Priority Group |  |  |  |
| --- | --- | --- | --- | --- |
|  | Urgent | Flight Safety | Other Safety | Normal |
| Set I: AC / Ground | 40/70 | 50/80 | 60/90 | 70/100 |
| Set II: AC / Ground | 160/280 | 200/320 | 240/360 | 280/400 |
| Set III: AC / Ground | 320/560 | 400/640 | 480/720 | 560/800 |

Table 8.  COTP Acknowledgment Timer Values in seconds (10500 bps)

Figure 14.	End-to-end transit delay (mean and 95%) in function of COTP acknowledgment timer value, from-aircraft (left) and to-aircraft (right) direction, for flight safety priority and 10,500 AMSS channel rate.

5.2.4. Throughput

Using the selected set of network parameters, established in previous sections, we conduct simulation runs to assess the throughput (bit/sec) that could be observed by the users of ATN.

Results from this experiment are given in Tables 9 and 10.  Note that the to-aircraft direction has worse throughput performance than the from-aircraft direction at 10,500 bps.  This is due to the AMSS P-channel Link Layer protocol, responsible for uplink transmission, which permits only one message per aircraft per priority level to be in transmission at any given time.  On the other hand, in the from-aircraft direction, up to eight messages per aircraft per priority level may be transmitted simultaneously for 10,500 bps operation.  Also, note that message prioritization has a prominent effect in the uplink direction whereas the results show that this is not true in the downlink direction.  This is consistent with the internal operation of AMSS.

|  | Load [%] |  | From Aircraft |  | To Aircraft |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Priority | P | T | Mean [bits/s] | 95% Conf. Int. |  | Mean [bits/s] | 95% Conf. Int. |
| urgent | 82 | 77 | 60 | + 2.5 |  | 63 | + 1 |
| flight safety | 68 | 60 | 62 | + 3 |  | 57 | + 2 |
| other safety | 73 | 64 | 58 | + 3 |  | 48 | + 2 |
| normal | 71 | 62 | 56 | + 3 |  | 27 | + 2 |

Table 9. ATN Throughput for 600 bps AMSS channel rates

|  | Load [%] |  | From Aircraft |  |  | To Aircraft |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Priority | P | T | Mean [bits/s] | 95% Conf. Int. | Mean [bits/s] |  | 95% Conf. Int |
| urgent | 59 | 66 | 612 | + 95 | 208 |  | + 24 |
| flight safety | 58 | 66 | 631 | + 150 | 167 |  | + 35 |
| other safety | 55 | 62 | 519 | + 90 | 160 |  | + 27 |
| normal | 57 | 64 | 565 | + 63 | 153 |  | + 25 |

Table 10. ATN Throughput for 10,500 bps AMSS channel rates

6.  Conclusion and Remarks

In this paper we intend to assess the performance of ATN when AMSS is used as the underlying subnetwork and study the interactions between the two systems.  This work is driven by the need to establish both the expectations from this system, as well as the configuration requirements that may be used in future studies. As it is shown in this paper, the performance depends on many factors that are related to AMSS, the traffic profile, and ATN configuration.

At the beginning, results for the establishment delay of BIS-BIS and COTP connections were presented. These can be used to assess the behavior of the system during AMSS log-on and GES or satellite handover procedures or ATN Ground Router handover procedures.

The performance analysis was examined through observation of three salient network metrics: capacity, end-to-end transit delay and throughput.  Comparative analysis for the lowest and the highest AMSS channel rates, 600 and 10,500 bps respectively, shows the differences in both network parameter values and performance.  A summary of these observations is as follows:

- Nominal capacity is reached on a network with 40 AESs operating at 600 bps, and approximately 720 AESs.  Simulation runs with the 720 AESs were accomplished by using 90 AESs each operating at 10,500 bps and generating the nominal traffic load multiplied by a factor of 8.  For both AMSS channel rates, measured AMSS transit delays, even at the highest capacity level used, met the ICAO AMSS SARPs specifications.

- The influence of four network parameters on the end-to-end transit delays was analyzed.  These parameters are: AMSS channel loading, maximum TPDU size, COTP acknowledgment timers, and COTP retransmission timers.  The effect of these parameters is as follows:

-	Delay variations due to AMSS channel loading are observed to be higher in the from-aircraft direction.  This is mainly because both the R channel (slotted Aloha) and T channel (TDMA with on-demand slot assignment) of AMSS are more prone to the effect of loading, than the time-multiplexed uplink P channel.

-	Four maximum TPDU sizes were analyzed in this paper: 128, 256, 512, and 1024 octets.  Significant degradation of service is observed when the lower Maximum TPDU sizes (128, 256) are used.  This is due to: (1) increase of ATN overhead due to segmentation; (2) increase of the number of segmented packets into AMSS resulting in greater impact of the signaling and flow control mechanisms employed by the AMSS Link Layer protocols; and (3) increase of the probability to generate Subnetwork Layer resets.  However, when using higher TPDU sizes, retransmissions resulting from virtual circuit resets within AMSS may cause inefficient use of the available, limited AMSS bandwidth.  Thus, we selected 512 octets as a maximum TPDU size rather than the 1024 octets.

-	The selection of re-transmission and acknowledgment timer values is mostly influenced by: (1) the fact that AMSS is a reliable subnetwork; (2) the high fluctuation in the TPDU transit delays within the same Transport Connection; and (3) the limited AMSS channel bandwidth.  It is shown by the simulation results that the values of the re-transmission timers do not have substantial effect on transit delay statistics, as long as they are long enough to avoid premature timer expiration, since message retransmissions are very infrequent. The low bound of these timer values are determined by the acknowledgment timer values; more so for the 10,500 bps case than for the 600 bps case.  The acknowledgment timer values for 10,500 bps operations were set high enough to acknowledge multiple TPDUs in the same AK TPDU in order to minimize the number of AK TPDUs.  Otherwise, for the simulated scenarios, an excessive AMSS channel load would be generated due to the transmission of AK TPDUs in the uplink direction at the flight safety priority level.  This is because the downlink flight safety traffic (e.g. ADS reports) represents the highest traffic percentage.  If it is not rectified, this situation causes service degradation for the uplink safety traffic to the extent that its transit delay statistics become worse than those of the lower priority other safety traffic.  However, high acknowledgment timer values mean that message retransmissions, when they occur, will be substantially delayed.  Furthermore, such higher values might have adverse effect on the operation of the applications.  This aspect was not addressed in this paper.

Possible solutions, other than increasing the acknowledgment time values, to address the problem of loading at the safety traffic level are currently under investigation.  These include:  (1) the reduction of the number of aircraft that will be allowed to operate on the same AMSS channel set, and (2) the the reduction of excessive traffic input at the AMSS flight safety priority level by transferring some portion of this traffic to the next lower priority levels of other safety.  The latter solution will come at very small cost in the transit delay of the messages that will be transmitted at lower priority, specifically for the case of 10,500 bps.

- The lack of prioritization in the from-aircraft direction, and the flow control schemes used to regulate the to-aircraft transmissions, lead to different performance  of transit delay and throughput in the from and to aircraft direction. The transit delay is better in the to-aircraft direction, while the throughput is worse in the same direction.  It is recommended that this fact, reflecting the AMSS behavior, be considered when defining specifications for ATN applications.

We expect that future work on the ATN performance and ATN/AMSS interactions analysis will identify other system bottlenecks and point to solutions for suitable configuration parameters in order to optimize the performance.

References

[1] 	Mayflower Communications Co. Inc., “Evaluation of ATN/AMSS End-to-end Transit Delay Performance”, ICAO ATNP Working Group 2, Toulouse, France, 13-24 March 1995.

[2]	Mayflower Communications Co. Inc., “Assessment of BIS-BIS and Transport Connection Establishment Delays”, ICAO ATNP Working Group 2, Fairfax, VA, 15-19 May 1995.

[3] 	RTCA, “SATCOM/HFDL Traffic Model”, SC165 WG-3 Working Paper 308, 22 May 1995.

[4]	 ICAO, “ATN Manual”, Second Edition, 1 September 1993.

[5] 	ICAO, “CNS/ATM-1 Package SARPs and Guidance Material’, Version 3.0, 7 August 1995.

Appendix I

Table 1. Application Traffic Model developed by RTCA SC-165 WG-3

Downlink Traffic Model - Oceanic Flight, Per Aircraft

| Name | Type [Note] | Description | Application Message Length (octets) | Interarrival Time (minutes, mean) | AMSS Priority or % mix | ATN Priority or % mix |
| --- | --- | --- | --- | --- | --- | --- |
| DN1 | ATC[1] | Request | 23 | 54.5 | Q = 11 | 3, 4 |
| DN2 | ATC[2] | Wilco | 3 | 54.5 | Q = 11 | 3, 4 |
| DN3 | ATC[3] | ADS = Basic + Earth Ref. | 17 | 15 | Q = 11 | 3, 4 |
| DN4 | ATC[3] | ADS = Basic + Earth Ref.+Met. | 22 | 60 | Q = 11 | 3, 4 |
| DN5 | AOC[4,5] | Various | mean = 50 | 30 | 0/50/50/0 | 0/50/50/0 |
| DN6 | AAC[5] | Various | mean = 50 | 300 | Q = 5 | 9 |
| DN7 | APC[6] | C-ch signal (air-orig) | N/A | 60 | Q = 9 | N/A |
| DN8 | ALL[7,8] | Background Traffic Exponential Distribution | mean = 160 Truncated [1-600] | 72 | 2/18/60/20 | 2/18/60/20 |

Uplink Traffic Model - Oceanic Flight, Per Aircraft

| Name | Type [Note] | Description | Application Message Length (octets) | Interarrival Time (minutes, mean) | AMSS Priority or % mix | ATN Priority or % mix |
| --- | --- | --- | --- | --- | --- | --- |
| UP1 | ATC[1,2] | ALT Assignment + Xing Restriction + Report Reaching | 16 | 60 | Q = 11 | 3, 4 |
| UP2 | ATC[1,2] | DARPS Route Clearance, 15 WPS | 194 | 480 | Q = 11 | 3, 4 |
| UP3 | AOC[4,9] | Various | mean = 128 | 30 | 0/50/50/0 | 0/50/50/0 |
| UP4 | AAC[9] | Various | mean = 128 | 300 | Q = 5 | 9 |
| UP5 | APC[6] | C-Ch Signal (air original) | N/A | 60 | Q = 9 | N/A |
| UP6 | APC[6] | C-Ch Signal (gnd original) | N/A | 120 | Q = 9 | N/A |
| UP7 | ALL[8,10] | Background Traffic Exponential Distribution | mean = 160 Truncated [1-600] | 81 | 2/18/60/20 | 2/18/60/20 |

Notes:

1.	DN1 precedes UP1 and UP2 by 30 second mean, exponential distribution.

2.	DN2 follows UP1 and UP2 by 30 second mean, exponential distribution.

3.	Every fifth ADS report is a DN4; others are DN3's.

4.	UP3 follows a DN5 by 60 second mean, exponential distribution.

5.	Data length for AOC & AAC downlink messages are generated using a Gaussian process having a mean of 50 octets, s= 283, then truncated for a minimum of 1 octet.  The resultant distribution has a mean of 245 octets.

6.	Call traffic offered load is per 100 passengers for both speech and fax calls, and should be scaled for various passenger loads.

7.	DN8 traffic adjusted by scaling interarrival time to be 20% of the total DN1-DN7 traffic.

8.	Priority mix to be read as percentages, Urgent/Flight Safety/Other Safety/Normal AAC & APC.

9.	Data length for AOC & AAC uplink messages are generated using a Gaussian process having a mean of 128 octets, s=242, then truncated for a minimum of 1 octet.  The resultant distribution has a mean of 248 octets.

10.	UP7 traffic adjusted by scaling interarrival time to be 20% of the total UP1 - UP6 traffic.

As mentioned earlier, 720 AES’s simulation is achieved by actually using 90 different AES with the nominal traffic per AES (Appendix I) multiplied by a factor of 8.