# Appendix 12	Systems Management Provisions

## A12.1  Introduction

Note.-  ATN Systems Management (SM) applies OSI Systems Management principles as defined in ISO 7498 OSI Basic Reference Model (Part 4 - Management Framework).

### A12.1.1  Overview

Note.¾  This Appendix specifies the minimum set of functions, services, protocols and information required to manage ATN systems.

Recommendation.¾  ATN managed resources should be grouped into management domains in order to assign responsibility for control of the resources.  An organization should assign an administrative authority to establish and maintain the respective management authorities of each of its management domains, and to manage the transfer of control of resources from one management domain to another.

### A12.1.2  General Considerations

Systems Management Applications (SMAs) shall be MIS Users operating in either the Agent or Manager role as defined in ISO/IEC 10040.

An SMA operating in an Agent role shall be provided in ATN Boundary Intermediate Systems.

Recommendation.¾  SMAs operating in the Agent role should be provided in ATN end-systems and intermediate-systems where operational requirements for Systems Management exist.

Manager SMAs shall communicate with other Manager and/or Agent SMAs to exchange management information.

Management Information shall exist and Management Functions shall operate independently of the ATN Systems Management Profile and vice versa.

Note 1.¾ The Systems Management Profile defined in A12.3 and A12.4  specifies the Application, Presentation and Session Layers for the purposes of management.

SMAs shall use the services provided by the ATN Systems Management Profile as defined in A12.3 and A12.4 to associate and communicate. These services shall be made available to the SMAs by the Systems Management Application Entity (SMAE).

The ATN Systems Management Profile shall use the services provided by the ATN Internet Profile for the Transport and Network Layers defined elsewhere in this manual to relay management communications.

Note 2.¾  The ATN Internet Profile is a multi-part profile as defined in ISO 10608. It consists of the Connection Oriented  Transport (defined in Appendix 8) and Connectionless Network layers (defined in Appendices 9, 10 and 11).

## A12.2  Systems Management Functions

The SMA shall provide systems management functions to enable the management of ATN resources.

Note.-  Specific Systems Management Functions (SMFs) are defined by ISO in ISO/IEC 10164 and its parts..

SMF functionality shall be distributed between the Manager SMAs and Agent SMAs as specified in the following sections.

The following SMFs shall be implemented as part of the SMA to provide the required functionality.

a)	Object Management Function (ISO/IEC 10164-1)

b)	State Management Function (ISO/IEC 10164-2)

c)	Alarm Reporting Function (ISO/IEC 10164-4)

d)	Event Report Management Function (ISO/IEC 10164-5)

e)	Log Control Function (ISO/IEC 10164-6)

f)	Security Alarm Reporting Function (ISO/IEC 10164-7)

g)	Objects and Attributes for Access Control (ISO/IEC 10164-9)

Note 1.¾  The set of functions given above were chosen to address requirements for network and transport layer systems management developed from analysis of problems encountered in certain existing data networks.  Items a) and b) are fundamental requirements for object-oriented systems management.  Items c) and d) allow the reporting of unusual occurrences and exceptions (i.e., events and alarms) encountered during network operation.  Item e) allows the logging of information related to normal network operations which require historical tracking or which require later analysis.  Item f) allows the reporting of unusual security-related occurrences. Item g) adds the object management capabilities required for support of security management services.

Note 2.¾  The list of management functions given above represents the minimum required management functionality to support the management scope defined in A12.1.2 above.  Other management functions may be implemented as needed based on operational requirements.

### A12.2.1  Manager SMA Functions

#### A12.2.1.1  Object Management Function

The object management function shall be implemented as defined in ISO/IEC 10164-1.  The Manager SMA shall implement both the specific object management services and the object management pass-through services as defined in ISO/IEC 10164-1.

Note.¾  This function enables a Manager SMA to create, delete, examine and change the set of managed objects that are located on the managed resource for which the Manager has management responsibility.  It also enables a Manager SMA to receive notifications of object creation, deletion, and modifications.

##### A12.2.1.1.1  Specific Object Management Services

A Manager SMA shall implement the following specific object management services:

a)	reporting of creation and deletion of managed objects

b)	reporting of name changes to managed objects

c)	reporting of changes to attribute values of managed objects

##### A12.2.1.1.2  Object Management Pass Through Services

Note.¾ The pass through services provide the basic Common Management Information Service (CMIS) management services defined in ISO 9595.

A Manager SMA shall implement the following object management pass through services:

a)	creation and deletion of managed objects

b)	performing actions upon managed objects

c)	attribute changing

d)	attribute reading

e)	event reporting

For all pass-through services,  the Manager SMA shall use the corresponding CMIS service defined in ISO 9595 and perform a one-to-one mapping between the pass-through service parameters and the CMIS service parameters as specified in ISO/IEC 10164-1.

#### A12.2.1.2  State Management Function

The state management function shall be implemented as defined in ISO/IEC 10164-2.

Note.¾ This function enables a Manager SMA to examine and be notified of changes in the state of managed objects, to monitor overall operability and usage of resources, and to control the general availability of specific resources.

The Manager SMA shall implement the State Change Reporting service.

A Manager SMA shall have the ability to receive reports from Agent SMAs conveying changes in the value(s) of state attributes of managed objects.  The Manager SMA shall use the services and procedures defined in ISO 9595 for the confirmed or non-confirmed event report performer to accept object state change reports from the Agent SMAs.

The Manager SMA shall use the pass through services to manage the state attributes of managed objects.

#### A12.2.1.3  Alarm Reporting Function

The Alarm Reporting Function shall be implemented as defined in ISO/IEC 10164-4.

Note.¾  This function enables a Manager SMA to receive alarm reports from Agent SMAs which convey information about faults or abnormal conditions of ATN resources.  This function will aid in the early detection of network faults before significant effects have been felt by the end users.

The Manager SMA shall implement the Alarm Reporting Service.  The alarm reporting functional unit defined in ISO/IEC 10164-4 shall be supported and may be negotiated over a management association.

#### A12.2.1.4  Event Report Management Function

The Event Report Management Function shall be implemented as defined in ISO/IEC 10164-5.

Note.¾  This function enables a Manager SMA to control the types and frequency of transmission of event reports from Agent SMAs, and to specify the destinations to which selected reports are sent.

The Manager SMA shall be comprised of the following event report management services:

a)	Initiation of Event Forwarding

b)	Termination of Event Forwarding

c)	Suspension of Event Forwarding

d)	Resumption of Event Forwarding

e)	Modification of Event Forwarding Conditions

f)	Retrieval of Event Forwarding Conditions

#### A12.2.1.5  Log Control Management Function

The Log Control Management Function shall be implemented as defined in ISO/IEC 10164-6.

Note.¾  This function enables a Manager SMA to control the creation, deletion, retrieval, log scheduling and logging criteria..

The Manager SMA shall be comprised of the following log control services:

a)	 Log creation

b)	 Log deletion

c)	Log attribute modification

d)	Suspension and termination of  logging

e)	Initiation and resumption of logging

f)	Deletion and retrieval of log records

#### A12.2.1.6  Security Alarm Reporting Management Function

The Security Alarm Reporting Management Function shall be implemented as defined in ISO/IEC 10164-7.

Note.¾  This function enables a Manager SMA to receive security alarm reports from Agent SMAs and issue management operations to control security alarm reporting.

The Manager SMA shall implement the Security Alarm Notification Service.  The security alarm reporting functional unit defined in ISO/IEC 10164-7 shall be supported and may be negotiated over a management association.

#### A12.2.1.7  Objects and Attributes for Access Control

The objects and attributes for access control defined in ISO/IEC 10164-9 shall be supported.

Note.¾  This SMF does not define any management services.  Instead, it defines managed objects and attributes that may be used by systems that implement access control functions.

A Manager SMA shall implement an access control function for establishing management associations.  The managed object classes for access control defined in ISO/IEC 10164-9 and ISO/IEC 10165-2 shall be supported.

### A12.2.2  Agent SMA Functions

The Agent SMA shall implement the following SMFs as defined in  ISO/IEC 10164 and as defined in the following subparagraphs.

#### A12.2.2.1  Object Management Function

The object management shall be implemented as defined in ISO/IEC 10164-1.

Note.¾  This function enables an Agent SMA to send notifications of object creation, deletion, and modifications to a Manager SMA.

The Agent SMA shall implement both the specific object management services and the object management pass through services.

##### A12.2.2.1.1  Specific Object Management Services

An Agent SMA shall implement the following specific object management services:

a)	reporting of creation and deletion of managed objects

b)	reporting of name changes to managed objects

c)	reporting of changes to attribute values of managed objects

##### A12.2.2.1.2  Object Management Pass Through Services

An Agent SMA shall implement the following object management pass through services:

a)	creation and deletion of managed objects

b)	performing actions upon managed objects

c)	attribute changing

d)	attribute reading

e)	event reporting

#### A12.2.2.2  State Management Function

The state management function shall be implemented as defined in ISO/IEC 10164-2.

Note.¾  This function enables an Agent SMA to notify one or more Manager SMAs of changes in the state of managed objects.

An Agent SMA shall implement the State Change Reporting service.

An Agent SMA shall have the ability to send reports conveying changes in the value(s) of state attributes of managed objects.  The Agent SMA shall use the services and procedures defined in ISO 9595 for the confirmed or non-confirmed event report invoker to send object state change reports to Manager SMAs.

#### A12.2.2.3  Alarm Reporting Function

The Alarm Reporting Function shall be implemented as defined in ISO/IEC 10164-4.

Note.¾ This function enables an Agent SMA to send alarm reports to Manager SMAs which convey information about faults or abnormal conditions of ATN resources.  This function will aid in the early detection of network faults before significant effects have been felt by the end users.

The Agent SMA shall implement the Alarm Reporting Service.

#### A12.2.2.4  Event Report Management Function

The Event Report Management Function shall be implemented as defined in ISO/IEC 10164-5.

Note.¾  This function enables an Agent SMA to discriminate among the types and frequency of transmission of event reports which it sends to Manager SMAs, and to send reports to specific destinations which have been selected by a Manager SMA.

The Agent SMA shall accept management operations which request the following management services:

a)	Initiation of Event Forwarding

b)	Termination of Event Forwarding

c)	Suspension of Event Forwarding

d)	Resumption of Event Forwarding

e)	Modification of Event Forwarding Conditions

f)	Retrieval of Event Forwarding Conditions

The following management operations shall be supported for the event forwarding discriminator in order to support the event report management services listed above:

i)	creation of a discriminator

ii)	deletion of a discriminator

iii)	modification of discriminator attributes

iv)	suspension of the activity of the discriminator

v)	resumption of the discriminators activity

The Agent SMA shall use the pass-through services (in the role of performer) of the Object Management Function to perform the discriminator management functions listed above.   The Agent SMA shall also use the specific notification services of the Object Management Function to send notifications of discriminator object creations, deletions, and attribute value changes.  The State Management Function shall be used to send notifications of state changes to discriminator objects.

The event report management functional unit defined in ISO/IEC 10164-5 shall be supported, and may be negotiated over a management association between a Manager SMA and an Agent SMA.

#### A12.2.2.5  Log Control Management Function

The Log Control Management Function shall be implemented as defined in ISO/IEC 10164-6.

Note.¾  This function enables an Agent SMA to control the creation, deletion, retrieval, log scheduling and logging criteria..

The Agent SMA shall be comprised of the following log control services:

a)	 Log creation

b)	 Log deletion

c)	Log attribute modification

d)	Suspension and termination of  logging

e)	Initiation and resumption of logging

f)	Deletion and retrieval of log records

#### A12.2.2.6  Security Alarm Reporting Management Function

The Security Alarm Reporting Management Function shall be implemented as defined in ISO/IEC 10164-7.

Note.¾ This function enables an Agent SMA to send security alarm reports to Manager SMAs.

The Agent SMA shall implement the Security Alarm Notification.

#### A12.2.2.7  Objects and Attributes for Access Control

The objects and attributes for access control defined in ISO/IEC 10164-9 shall be supported.

Note.¾  This SMF does not define any management services.  Instead, it defines managed objects and attributes that may be used by systems that implement access control functions.

An Agent SMA shall implement an access control function for establishing management associations.  The managed object classes for access control defined in ISO/IEC 10164-9 and ISO/IEC 10165-2 shall be supported.

## A12.3  Systems Management Services

The ATN SMA shall use a set of application services to exchange the management information processed by the SMA.  This set of communication services shall be incorporated into a Systems Management Application Entity (SMAE).  The SMAE shall provide access to the OSI communications services that will be utilized by the ATN SMA to exchange management information.

### A12.3.1  Common Management Information Service Element (CMISE) Requirements

The Common Management Information Service Element (CMISE) shall be used to exchange the management information generated by the SMFs.  The following CMISE information transfer services shall be implemented as defined in ISO 9595:

a)	M-GET

b)	M-SET

c)	M-ACTION

d)	M-CREATE

e)	M-DELETE

f)	M-CANCEL-GET

g)	M-EVENT-REPORT

The functional units implemented for the communication of management information over the ATN shall be as specified in ISO/IEC 11183 Part 2 (Profile AOM 12: Enhanced Management Communication):

i)	Kernel functional unit

ii)	Multiple Object Selection (scoping) functional unit

iii)	Filter functional unit

iv)	Cancel Get functional unit

v)	Multiple Reply functional unit

### A12.3.2  ACSE Requirements

The ACSE shall be used to establish peer ATN SMA associations.  The following ACSE services shall be implemented as defined in ISO 8649:

a)	A-Associate

b)	A-Release

c)	A-Abort

d)	A-P-Abort

The SMA shall establish a peer application association using the A-Associate service.  The ACSE shall provide an interface to allow the SMA to use the ACSE services.  The SMA shall always include an application context name in the service request.

Recommendation.¾  The application context names defined in Appendix A of ISO 10040 should be used for ATN SMA associations.

Agent SMAs shall use the Agent application context, and Manager SMAs shall use the Manager application context.

Note.-  The Manager-and-Agent application context is not required.

## A12.4  Systems Management Protocols

The protocols implemented for the communication of management information among ATN systems shall conform to the requirements of ISO/IEC 11183 Parts 1 and 2 (Profile AOM 12: Enhanced Management Communication).

### A12.4.1  Application Layer Protocols

The SMA functions shall use the CMIP protocol defined in ISO 9596-1.  In addition, the ROSE protocol, ISO 9072-2, and the ACSE protocol, ISO 8650 shall be required.  The use of protocol elements to provide the CMISE and ROSE services shall be as specified in ISO/IEC 11183 Part 2 (Profile AOM 12: Enhanced Management Communication).  The use of the ACSE protocols shall be as specified in ISO/IEC 11183 Part 1 (Profiles AOM nn: Management Communication Protocols: ACSE, Presentation and Session protocols for use by ROSE and CMISE).

### A12.4.2  Presentation Layer Protocol

A minimal set of Presentation Layer protocols corresponding to the Kernel functional unit defined in ISO 8823 shall be implemented.

The use of Presentation protocols shall be as specified in ISO/IEC 11183 Part 1 (Profiles AOM nn: Management Communication Protocols: ACSE, Presentation and Session protocols for use by ROSE and CMISE).

### A12.4.3  Session Layer Protocol

A minimal set of Session Layer protocols corresponding to the Kernel and Duplex functional units defined in ISO 8327 shall be implemented.  Session for the use of unlimited user data, as specified by Addendum 2 to ISO 8327 shall be selected.

The use of Session protocols shall be as specified in ISO/IEC 11183 Part 1 (Profiles AOM nn: Management Communication Protocols: ACSE, Presentation and Session protocols for use by ROSE and CMISE)

A12.4.4  Transport Layer Protocol

Figure A12.1: Managed Object Containment Hierarchy

The ATN Systems Management Profile shall use the Connection-mode Transport protocol defined in Appendix 8.

## A12.5  Systems Management Information

### A12.5.1  ATN Managed Object Containment Hierarchy

The containment relationship shall be used for naming managed objects.  Names shall be unambiguous in a specified context; for management this context is determined by the containing object.  Objects named in terms of another object shall be subordinate objects of that object.  The object that establishes the naming context for other objects shall be the superior object of these subordinate objects.

A subordinate object shall be named by the combination of:

-	the name of its superior object

-	the unique identity of this object within the scope of its superior object

An object name shall be unambiguous within the context of its superior object, within a wider context its name shall be qualified by the names of its superior objects.

Containment naming and the existence of managed objects shall be related as follows:

-	a managed object exists if its superior object exists

-	every managed object has a name which is derived from its containment

Note 1.¾  This hierarchical naming scheme can lead to potentially long names. implementations needing to utilize bandwidth efficiently will need to define a common and efficient naming scheme within the options given in 10165-1. It is likely that a combination of both local and global naming will be needed to manage the ATN as a whole. The actual naming schema to be used in managed ATN Systems is outside the scope of this manual although global naming may be standardized elsewhere.

Note 2.¾  The transfer syntax for management application data units required in 10165-1 for static conformance requirements is that defined in ISO/IEC 8825 (basic encoding).

The top level of the naming tree is referred to as "root" and is a null object (i.e. with no properties) that shall always exist. For each object class defined below the attributes and superior object classes whose instances may be used in constructing the name of the object shall be identified.

Note 3.¾  The relationship that identifies the possible superior object class is known as a "name binding".

Figure A12.1, Managed Object Containment Hierarchy, illustrates the containment hierarchy for managed objects in ATN Systems that shall apply to all managed objects instances. The managed objects that shall be provided in ATN Systems are those used by the management functions required in A12.2 as well as those relating to the OSI communications layers supported by ATN Systems (i.e. Network and Transport Layers).

Note 4.¾  In Figure 12.1 the echoed boxes indicate that there may be multiple instances of that object.

Note 5.¾  The detailed definition of management information relating to the Network and Transport layer entities is given elsewhere in the relevant sections of this Manual.

Note 6.¾  The foregoing does not imply that ATN Routers must physically provide logging facilities, these facilities may be provided by other ATN Systems responsible for management. ATN Routers may just provide information to be logged by other physical systems.

### A12.5.2  ATN Managed Object Instances

At least one instance of all the managed objects defined in this section and shown in Figure A12.1 shall exist in ATN Systems.

Note 1.¾  ATN System Managed Object instances are instantiated from standard class definitions as identified below.

Note 2.¾  How instances of managed objects are actually created and deleted is a local issue.

#### A12.5.2.1  System

Instances of this object class shall be instantiated from the class definition in ISO/IEC 10165-2 to represent sets of hardware and software that form an autonomous whole capable of performing information processing and/or information transfer.

#### A12.5.2.2  Access Control Policy

An instance of this object class shall be instantiated from the class definition in ISO/IEC 10164-9 (Objects and Attributes for Access Control) to identify:

-	the access control policy for the establishment of associations which specify the use of systems management functions,

-	the access control policy for the protection of management information from unauthorized user of management operations,

-	the access policy for preventing management notifications from being sent to unauthorized recipients.

#### A12.5.2.3  Targets

Instances of this object class shall be instantiated from the class definition in ISO/IEC 10164-9 (Objects and Attributes for Access Control) to identify the set of managed objects and associated operations that may be performed by authorized initiators.

Note.¾  Multiple instances are required when different access restrictions to parts of a managed object apply to different initiators.

#### A12.5.2.4  Authorized Initiators

Instances of this object class shall be instantiated from the class definition in ISO/IEC 10164-9 (Objects and Attributes for Access Control) to identify a set of initiators that have been authorized by access management information and targets they are allowed to access.

Note.¾  Instances of this managed object class are referenced by instances of the targets managed object class.

#### A12.5.2.5  Event Forwarding Discriminator

Instances of this object class shall be instantiated from the class definition in ISO/IEC 10164-5 (Event Report Management Function) to identify a set of conditions to be satisfied before the event report is forwarded to a particular destination(s).

Note.¾  The event forwarding discriminator is a subclass of the discriminator class.

#### A12.5.2.6  Logs

Instances of this object class shall be instantiated from the class definition in ISO/IEC 10165-2 as required by the specified management functions specified in A12.2.

Instances of  Log MOs shall contain records instantiated from the object classes defined in ISO/IEC 10165-2 (illustrated in Figure A12.1) as follows:

-	Object Creation Record

-	Object Deletion Record

-	Attribute Value Change Record

-	State Change Record

-	Security Alarm Record

-	Alarm Record

-	Event Log Record

Note.¾  Logs need not be present on all physical ATN equipment (e.g. Routers).  Notifications from local systems not supporting logging that inform remote systems of local events may result in the remote system logging those events.

## A12.6  Managed Objects for Communications Subsystems

The Transport and Network Layer Subsystems managed object classes shall be derived from the Subsystem class definition in ISO/IEC 10165-5 to represent a containment point for all the managed objects of a given communications layer.  ATN Systems requiring communication shall instantiate instances of the relevant subsystem class definitions.

Note.¾  The Data Link Layer Subsystem MO is outside the scope of this manual, refer to ISO/IEC 10742.

### A12.6.1  Network Entity Managed Objects

The Agent System Management Application Process (SMAP) shall support each of the MOs listed in this specification.

Note 1.-  This section specifies the Network Entity managed objects that the SMAP application will manage.

Note 2.-  The following subsections describe ATN management requirements in terms of Managed Object (MO) definitions, which can be seen as the specifications of interfaces between ISO defined Systems Management and the resources to be managed within the Network Layer.

#### A12.6.1.1  Network Entity Managed Object Containment

ATN Network Entity Managed Objects shall conform to the containment tree shown in A12.2.

Note 1.-  Shadowed boxes indicate that multiple instances may exist in a given managed system. The dashed boxes surround MOs which are only implemented in ATN routers, depending on the router's role (intra-domain or inter-domain routing).

Note 2.-  In the templates that follow, if not otherwise mentioned, the definitions given make an implicit reference to the ISO 10733 standard.

#### A12.6.1.2  Network Entity Managed Object Classes

Note.-  Two groups of Network Entity Managed Object Classes are defined for the ATN.  The first group is closely related to OSI management standards, i.e. ISO 10733 for Network Layer MOs, ISO 10589 for intra-domain routing MOs, and ISO 10747 for inter-domain routing MOs.  The second group consists of specific ATN MOs, which are not reflected in ISO standards.

Instances of the managed objects (MOs) from the following two groups of object classes, denoted Group 1 and Group 2, shall form the Network Entity Management Information Base (MIB).

Group 1 Managed Objects:

	Network Subsystem managed object class;

	Network Entity managed object class;

	NSAP managed object class;

	Connectionless Network Protocol Machine managed object class;

	Linkage managed object class;

	IDRP Configuration managed object class;

	Adjacent BIS managed object class;

	Destination managed object class;

	Destination System managed object class;

	Destination Area managed object class;

	Adjacency managed object class;

	Reachable Address managed object class.

Group 2 Managed Objects:

Figure A12.2: ATN Network Layer Managed Object Containment Hierarchy

	PIB managed object class;

	Adj-RIB managed object class;

	Loc-RIB managed object class;

	FIB managed object class.

##### A12.6.1.2.1  The Network Subsystem Managed Object (NSMO) Class

There shall be one such MO within each ATN system.

Note 1.-  The network subsystem MO exists to provide a container for all Network Layer MOs.

Note 2.-  The network subsystem MO cannot be created or deleted by management operation. It exists inherently in a managed system and is created when the system is being initialized.

##### A12.6.1.2.2  The Network Entity Managed Object (NEMO) Class

There shall be one such MO within each ATN system.

Note.-  The network entity MO cannot be created or deleted by management operation. It exists inherently in a managed system and is created when the system is being initialized.

##### A12.6.1.2.3  The NSAP Managed Object (NSAP-MO) Class

There shall be one NSAP MO for each set of NSAPs supported by the network subsystem that are associated with a single Transport Layer client.

Note.-  The definition of the NSAP-MO permits it to be created and deleted explicitely by management operation or to be created and deleted automatically as part of systems operation. Automatic creation and deletion is the recommended practice.

##### A12.6.1.2.4  The Connectionless Network Protocol Machine Managed Object (CL-NPM-MO) Class

There shall be one such MO within each ATN system per Network entity MO. This MO represents an implementation of the ISO 8473 protocol.

Note.-  The CL-NPM-MO cannot be created or deleted by management operation. It exists inherently in a managed system and is created when the system is being initialized.

##### A12.6.1.2.5  The Linkage Managed Object (LINK-MO) Class

There shall be one of these MOs associated with each separate provision of the underlying service to the CLNS protocol machine, that is for each linkage emanating from the system or each subnetwork the system is attached to.

An instance of the linkage MO shall be created when the system is being initialized, when the system is being attached to a new subnetwork and/or the system is configured with a new SNDCF/SNAcP.

An instance of the linkage MO shall be deleted when the system is no longer attached to the subnetwork identified by that MO.

Note 1.-  Creation and deletion of a linkage MO are of local matter and neither operation can be performed through systems management.

Note 2.-  The following notations are used in  Templates NE-18a and NE-18b:

	C1:	this feature shall be present if the managed system is an IS supporting the ISO 9542 protocol.

	C2:	this feature shall be present if the managed system is an ES supporting the ISO 9542 protocol.

	C3:	this feature shall be present if the managed system supports an SNDCF over a connection-oriented SN.

	C4:	this feature shall be present if the managed system is a level 1 IS.

	C5:	this feature shall be present if the managed system is a level 2 IS.

##### A12.6.1.2.6  The IDRP Configuration Managed Object (IDRP-CFG-MO) Class

There shall be one instance of the IDRP-CFG-MO on each BIS. It represents the IDRP protocol machine on that system.

An instance of the IDRP-CFG-MO shall be created with the initialization of the IDRP protocol. It exists inherently within the system and neither creation nor deletion through systems management is possible.

Note.-  The following notation  is used in Template NE-22:

	C6:	this feature shall be present if the managed system is a ground or an air/ground BIS.

##### A12.6.1.2.7  The Adjacent BIS Managed Object (ADJ-BIS-MO) Class

This MO shall represent a single BIS to BIS connection. There shall be exactly one instance of ADJ-BIS-MO per BIS-BIS connection used on a BIS.

Note.-  There is one instance of ADJ-BIS-MO for every adjacent BIS, i.e. a BIS of a neighbouring Routing Domain, that can be reached via a single subnetwork. It may be created and deleted and deleted explicitly by management operation. In that case it may be created before a new BIS-BIS connection is being established and may be deleted when the BIS-BIS connection which will no longer be maintained has been closed. If not created or deleted by management operation, an instance of ADJ-BIS-MO shall be created or deleted automatically by the IDRP protocol machine.

##### A12.6.1.2.8  The PIB Managed Object (PIB-MO) Class

There shall be one instance of the PIB-MO on each ATN BIS. This MO represents the Policy Information Base used by the IDRP protocol.

The PIB-MO shall exist inherently for each BIS with neither creation nor deletion possible by Systems Management.

##### A12.6.1.2.9  The Adj-RIB Managed Object (ADJ-RIB-MO) Class

There shall be two instance of the ADJ-RIB-MO on each ATN BIS. These MOs represent the Adj-RIB-In and Adj-RIB-Out records used by the IDRP protocol on that BIS.

The ADJ-RIB-MOs shall be created and deleted automatically by the IDRP protocol machine. Neither creation nor deletion is possible by Systems Management.

##### A12.6.1.2.10  The Loc-RIB Managed Object (LOC-RIB-MO) Class

There shall be one instance of the LOC-RIB-MO on each ATN BIS. This MO represents the local Routing Information Base used by the IDRP protocol.

The LOC-RIB-MO shall exist inherently for each BIS with neither creation nor deletion possible by Systems Management.

##### A12.6.1.2.11  The FIB Managed Object (FIB-MO) Class

There shall be one instance of the FIB-MO on each ATN IS. This MO represents the Forwarding Information Base used by the IDRP protocol.

The FIB-MO shall exist inherently for each ATN IS with neither creation nor deletion possible by Systems Management.

#### A12.6.1.3  Network Entity Managed Object Requirements Templates

ATN Network Entity Managed Objects shall conform with the MORTs in the following templates.

Index	Operations on Object Class	Ref.	Status

None

Template NE-1: Operations on NSMO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.2.1	Network subsystem id	ISO 10165-5:	S	GET	M

(subsystem)

Template NE-2: NSMO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

None

Template NE-3: NSMO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.		Status

NL.4.1	subsystem id	system	ISO 10165-5	M

Template NE-4: NSMO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

None

Template NE-5: Operations on NEMO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.6.1	Network entity id	ISO 10165-5:	S	GET	M

communications

Entity

NL.6.2	Set of NETs identifying the network		R/W	GET,		M

entity in a managed system			REPLACE,

ADD, REMOVE

Template NE-6: NEMO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

None

Template NE-7: NEMO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.8.1	communicationsEntityId	networkSubsystem	ISO 10165-5	M

Template NE-8: NEMO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

NL.9.1	CREATE			O

NL.9.2	DELETE			O

Template NE-9: Operations on NSAP-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.10.1	NSAP id	ISO 10165-5:sap2	S	GET	M

NL.10.2	Set of addresses associated with the NSAP	ISO 10165-5:sap2	R	GET	M

NL.10.3	Names of (N-1) layer entities which provide	ISO 10165-5:sap2	R	GET	O

services to this SAP

NL.10.4	Names of (N+1) layer entities which use	ISO 10165-5:sap2	R	GET	R

services offered by this SAP

Template NE-10: NSAP-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

NL.11.1	Object creation	ISO 10164-1,	M

ISO 10165-2

NL.11.2	Object deletion	ISO 10164-1,	M

ISO 10165-2

Template NE-11: NSAP-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.		Status

NL.12.1	sapId	network		Subsystem		ISO 10165-5	M

Template NE-12: NSAP-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

NL.13.1	ACTION - activate	ISO 10165-5:	O

to be implemented if the instance	activate

supports the administrative state.

NL.13.2	ACTION - deactivate	ISO 10165-5:	O

to be implemented if the instance	deactivate

supports the administrative state.

Template NE-13: Operations on CL-NPM-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.14.1	Connectionless protocol machine id	ISO 10165-5:	S	GET	M

(clProtocolMachine)

NL.14.2	Operational state	ISO 10165-5:	R	GET	M

(clProtocolMachine)

NL.14.3	Administrative state		R	GET	O

NL.14.4	The set of connectionless network protocols		R	GET	M

supported by this instance.

NL.14.5	Counter of user data octets sent in valid		C	GET	M

data packets

NL.14.6	Counter of user data octets received in valid		C	GET	M

data packets

NL.14.7	Counter of the total number of data and error		C	GET	M

report NPDUs received by the CLNS protocol

machine

NL.14.8	Counter of the segments discarded. This is the		C	GET	M

number of the data and error report NPDUs

discarded without being delivered to a network

service user or forwarded. This includes

segments discarded for any reason except

reassembly time expiry.

NL.14.9	Counter of segments (i.e. data and error PDUs)		C	GET	M

discarded by the CLNS protocol machine due to

reassembly timer expiry.

NL.14.10	Counter of received error reports. This is the		C	GET	M

number of error report NPDUs received which

were addressed by the local network entity.

NL.14.11	Counter of PDUs (data and error report PDUs)		C	GET	M

discarded for any reason except for congestion.

NL.14.12	Counter of PDUs (data and error report PDUs)		C	GET	M

discarded due to congestion.

NL.14.13	Maximum value (in half seconds) which may be		R/W	GET, 	M

placed in the lifetime field of any NPDU that			REPLACE

is generated by the local network entity.

NL.14.14	Checksum generation flag. When true, the		R/W	GET,	M

generation of checksums is enabled			REPLACE,DEFAULT

Template NE-14: CL-NPM-MO Object Class Attributes

Index	Object Class 	Notifications	Ref.	Status

NL.15.1	Object creation		ISO 10164-1,	M

ISO 10165-2

NL.15.2	Object deletion		ISO 10164-1,	M

ISO 10165-2

NL.15.3	Change administrative or	stateChange	ISO 10165-2	M

operational state

NL.15.4	Report discarded data NPDU	communications	ISO 10165-5:	M

Information

Template NE-15: CL-NPM-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.16.1	clProtocolMachineId	networkEntity	ISO 10165-5	M

Template NE-16: CL-NPM-MO Object Class Name Types

Index	Operations on Object Class		Ref.		Status

NL.17.1	ACTION - activate		ISO 10165-5:	O

to be implemented if the instance	activate

supports the administrative state.

NL.17.2	ACTION - deactivate		ISO 10165-5:	O

to be implemented if the instance	deactivate

supports the administrative state.

Template NE-17: Operations on LINK-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.18.1	Linkage Id		S	GET	M

NL.18.2	Operational state		R	GET	M

NL.18.3	Administrative state		R	GET	O

NL.18.4	Set of subnetwork dependent protocols		R	GET	M

NL.18.5	SNPA address	None	R/W	GET,REPLACE	M

NL.18.6	The factor to derive a holding timer from a		R/W	GET,REPLACE	M - C1

configuration timer.

This value, when multiplied by a configuration

timer, yields the value of the holding timer

parameter issued with configuration information.

NL.18.7	Value for the ISO 9542 IS configuration timer.		R/W	GET, REPLACE,	M - C1

It is used to determine how often an IS reports			DEFAULT

configuration information to ESs.

NL.18.8	Configuration timer to be used in ESs which is		R/W	GET, REPLACE,	M - C1

suggested by an IS and sent in an ISH PDU			DEFAULT

generated by the IS network entity.

NL.18.9	The holding time (in seconds) to be specified		R/W	GET, REPLACE,	M - C1

in Redirect PDUs (RD PDUs) generated by the IS.		DEFAULT

NL.18.10	Counter of the number of changes in the		C	GET	M - C1

reachability of ESs from this IS.

NL.18.11	Counter of the number of ISO 9542 PDUs		C	GET	M - C1

received which are discarded as result of the

PDU Header Error Detection or Protocol Error

Processing Functions specified in ISO 9542.

NL.18.12	The factor to derive a holding timer from a		R/W	GET, REPLACE	M - C2

configuration timer.

This value, when mulitplied by a configuration

timer, yields the value of the holding timer

parameter issued with configuration information.

NL.18.13	Set of SNPA addresses to which calls		R/W	GET DEFAULT	M - C2

associated with the SNDCF are to be 			ADD, REMOVE

established in the absence of any other

information, that isno routing information has

been explicitely given. This attribute can then

be used to find the router.

NL.18.14	Default value for the ISO 9542 ES configuration	R/W	GET, REPLACE,	M - C2

timer. This value is used when the ES has not			DEFAULT

received, or not chosen to accept, a suggested

configuration timer value from an IS.

NL.18.15	Counter of changes in reachability of ISs		C	GET	M - C2

from this ES.

NL.18.16	Counter of the number of ISO 9542 PDUs 		C	GET	M - C2

received which are discarded as result of the

PDU Header Error Detection or Protocol Error

Processing Functions specified in ISO 9542.

NL.18.17	Timer that determines the interval (in seconds)		R/W	GET, REPLACE,	M - C3

during which a call shall remain connected			DEFAULT

after being established, irrespective of traffic.

This timer shall be set small enough so that

the call is cleared before the start of next

charging interval.

NL.18.18	Time (in seconds) to reserve resources for		R/W	GET, REPLACE,	M - C3

call re-establishment. This timer determines			DEFAULT

the interval for which an attempt shall be made

to retain those resources, as determined by the

implementation, whose retension will increase

the probability of successfull re-establishment

of an idle virtual call.

Template NE-18a: LINK-MO Object Class Attributes (1 of 2)

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.18.19	Time (in seconds) before release of an idle		R/W	GET, REPLACE,	M - C3

call, i.e. the interval for which a call is			DEFAULT

permitted to remain idle (no data traffic in

either direction) before being released by the

SNDCF.

NL.18.20	Counter of the number of VCs successfully		C	GET	M - C3

established by the SNDCF.

NL.18.21	Counter of the number of call failures while		C	GET	M - C3

attempting establishment by the SNDCF.

NL.18.22	The default metric value of this	ISO 10589	R/W	GET, REPLACE,	M - C4

linkage for level 1 traffic			DEFAULT

NL.18.23	The delay metric value of this	ISO 10589	R/W	GET, REPLACE,	M - C4

linkage for level 1 traffic. The value of zero			DEFAULT

is reserved to indicate that this metric is not supported.

NL.18.24	The error metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C4

level 1 traffic. The value of zero is reserved			DEFAULT

to indicate that this metric is not supported.

NL.18.25	The expense metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C4

level 1 traffic. The value of zero is reserved			DEFAULT

to indicate that this metric is not supported.

NL.18.26	The default metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C5

level 2 traffic			DEFAULT

NL.18.27	The delay metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C5

level 2 traffic. The value of zero is reserved			DEFAULT

to indicate that this metric is not supported.

NL.18.28	The error metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C5

level 2 traffic. The value of zero is reserved			DEFAULT

to indicate that this metric is not supported.

NL.18.29	The expense metric value of this linkage for	ISO 10589	R/W	GET, REPLACE,	M - C5

level 2 traffic. The value of zero is reserved			DEFAULT

to indicate that this metric is not supported.

Template NE-18b: LINK-MO Object Class Attributes (2 of 2)

Index	Object Class	Notifications	Ref.	Status

NL.19.1	Object creation		ISO 10164-1,	M

ISO 10165-2

NL.19.2	Object deletion		ISO 10164-1,	M

ISO 10165-2

NL.19.3	Change administrative or		ISO 10165-2	M

operational state	stateChange

NL.19.4	IS Reachability change,		ISO 10165-5	M

generated when an ES or	communications

an IS detects a change in the	Information

reachability of a neighbouring IS.

NL.19.5	ES Reachability change,		ISO 10165-5	M

generated when an IS	communications

detects a change in the	Information

reachability of a neighbouring ES.

NL.19.6	Reception of an ISO 9542 PDU		ISO 10165-5	M

which is discarded as invalid	communications

by either the PDU Header	Alarm

Error Detection Function or the Protocol

Error Processing Function

Template NE-19: LINK-MO Object Class Notifications

Index	Object Class Name Type	Superior        Ref.	Status

NL.20.1	linkageId	cLNS	M

Template NE-20: LINK-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

NL.21.1	StartEvent action - see IDRP	ISO 10747	M

NL.21.2	StopEvent action - see IDRP	ISO 10747	M

Template NE-21: Operations on IDRP-CFG-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.22.1	IDRP config id	ISO 10747	S	GET	M

NL.22.2	Set of NETs which identify the BISs in an	ISO 10747	R/W	GET, REPLACE	M

adjacent routing domain that are reachable via a

single subnetwork hop.

NL.22.3	Set of NETs and NSAPs identifying the	ISO 10747	R/W	GET, REPLACE	M

systems in this routing domain, which are used

by the BIS to construct network layer

reachability information.

NL.22.4	Routing Domain Identifier (RDI) for the remote	ISO 10747	R/W	GET, REPLACE	M

routing domain where the BIS is located.

NL.22.5	Routing confederations to which the RD of this	ISO 10747	R/W	GET, REPLACE	M

BIS belongs to and the nesting relationships that

are in force between them. The nesting

relationships are described as a sequence of sets

of RDC identifiers.

NL.22.6	List of SNPAs of this BIS	ISO 10747	R/W	GET, REPLACE	M

NL.22.7	Set of path-attribute combinations supported	ISO 10747	R	GET	M

by the BIS.

NL.22.8	Version of the IDRP protocol used by default.	ISO 10747	R	GET	M

NL.22.9	Maximum time (in seconds) between checking of	ISO 10747	R	GET	M

the Adj-RIBs-In by a local mechanism. If a

corrupt Adj-RIB-In is detected, the BIS shall

purge the offending Adj-RIB-In.

NL.22.10	Set of NETs which identify the BISs in this	ISO 10747	R/W	GET, REPLACE	M - C6

routing domain.

NL.22.11	Set of NETs of the ISs to which the local BIS	ISO 10747	R/W	GET, REPLACE	M - C6

may deliver inbound NPDUs whose destination

lies within the BIS's routing domain and is

attached to the same common subnetwork.

NL.22.12	Estimated average delay across a routing domain,	ISO 10747	R/W	GET, REPLACE	M - C6

in units of 500 milliseconds.

NL.22.13	Average error rate of a routing domain in units	ISO 10747	R/W	GET, REPLACE	M - C6

of an integer which if divided by 2**32-1 will

provide the actual probability of an error.

NL.22.14	Monetary expense of transiting this routing	ISO 10747	R/W	GET, REPLACE	M - C6

domain. Contains an indication of cost and the

unit in which it is calculated.

NL.22.15	Traffic carrying capacity of this routing domain.	ISO 10747	R/W	GET, REPLACE	M - C6

NL.22.16	Lowest value of ISO 8473 priority parameter	ISO 10747	R/W	GET, REPLACE	M - C6

this routing domain will provide services for.

NL.22.17	Indication whether this BIS will use the	ISO 10747	R/W	GET, REPLACE	O

MULTI_EXIT_DISC attribute to decide between otherwise identical routes.

NL.22.18	Indication whether this BIS may set the	ISO 10747	R/W	GET, REPLACE	O

IDRP_Server_Allowed field in the NEXT_HOP attribute to X"FF" for BIS-BIS UPDATE BISPDUs.

NL.22.19	Maximum number of octets that this BIS is able	ISO 10747	R	GET	O

to handle.

NL.22.20	Maximum number of seconds that may elapse	ISO 10747	R	GET	O

between the receipt of two successive BISPDUs

of any of the following types: KEEPALIVE, UPDATE, RIB CHECKSUM or RIB REFRESH.

NL.22.21	Maximum number of BISPDUs that may be sent	ISO 10747	R	GET	O

to this BIS without receiving an acknowledgement.

NL.22.22	Indication of which authentification mechanism	ISO 10747	R	GET	O

will be used.

NL.22.23	Number of seconds between KEEPALIVE 	ISO 10747	R	GET	O

messages if no other traffic is sent.

NL.22.24	Number of seconds the local system shall stay	ISO 10747	R	GET	O

in the CLOSE-WAIT state prior to changing to the CLOSED state.

Template NE-22: IDRP-CFG-MO Object Class Attributes

Index	Object Class	Notifications	Ref.	Status

NL.23.1	The FSM used to communicate with		ISO 10165-5:	M

another BIS is started.	communications

Information

NL.23.2	Occurrence of a transition from one		ISO 10165-5:	M

state to another in the FSM used to	communications

communicate with a remote BIS.	Information

NL.23.3	A BISPDU is received with an error		ISO 10165-5:	M

in its format.	communications

Alarm

NL.23.4	An OPEN BISPDU is received from		ISO 10165-5:	M

another BIS in the same RD, and the	communications

remote BIS is not a member of	Alarm

identically the same confederations

as the local BIS.

NL.23.5	An ERROR BISPDU has been		ISO 10165-5:	M

received from a remote BIS.	communications

Alarm

NL.23.6	The local method of checking the		ISO 10165-5:	M

Adj-RIB-In has found an error.	communications

All Adj-RIBs-In are being purged.	Alarm

NL.23.7	The local BIS has been presented		ISO 10165-5:	M

with a BISPDU whose source is not	communications

one of the BISs adjacent to the	Alarm

local BIS.

Template NE-23: IDRP-CFG-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.24.1	idrpConfigId	networkEntity	ISO 10747	M

Template NE-24: IDRP-CFG-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

NL.25.1	CREATE	None	O

NL.25.2	DELETE	None	O

Template NE-25: Operations on ADJ-BIS-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.26.1	NET of the remote BIS of this	ISO 10747	S	GET	M

BIS-BIS connection.

NL.26.2	Routing Domain Identifier (RDI) of	ISO 10747	S	GET	M

the remote BIS of this BIS-BIS

connection.

NL.26.3	Routing Domain Confederation (RDC)	ISO 10747	S	GET	M

the remote BIS belongs to in this

BIS-BIS connection.

NL.26.4	SNPAs announced by the remote BIS of	ISO 10747	S	GET	M

this BIS-BIS connection.

NL.26.5	Current state of the BIS-BIS	ISO 10747	R	GET	M

connection in the local BIS.

NL.26.6	Last sequence number sent to the	ISO 10747	C	GET	M

remote BIS from this local BIS on

this BIS-BIS connection.

NL.26.7	Last sequence number received from	ISO 10747	C	GET	M

the remote BIS by this local BIS on

this BIS-BIS connection.

NL.26.8	Number of the last ack sent to the	ISO 10747	C	GET	M

remote BIS from this local BIS on

this BIS-BIS connection.

NL.26.9	Number of the last ack received from	ISO 10747	C	GET	M

the remote BIS by this local BIS on

this BIS-BIS connection.

NL.26.10	Counter of the number of UPDATE	ISO 10747	C	GET	M

BISPDUs received by this BIS on this

BIS-BIS connection.

NL.26.11	Counter of the number of UPDATE	ISO 10747	C	GET	M

BISPDUs sent by this BIS on this

BIS-BIS connection.

NL.26.12	Counter of the number of BISPDUs	ISO 10747	C	GET	M

received by this BIS on this BIS-BIS

connection.

NL.26.13	Counter of the number of BISPDUs	ISO 10747	C	GET	M

sent by this BIS on this BIS-BIS

connection.

NL.26.14	Number of KEEPALIVE BISPDUs received	ISO 10747	C	GET	M

from the remote BIS since the last

UPDATE BISPDU.

NL.26.15	Negotiated version of the IDRP protocol this	ISO 10747	R	GET	O

BIS-BIS connection is using.

NL.26.16	Authentification type the remote BIS sent in the	ISO 10747	R	GET	O

OPEN BISPDU in this BIS-BIS connection.

Template NE-26: ADJ-BIS-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

None

Template NE-27: ADJ-BIS-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.28.1	bisNET	idrpConfig	ISO 10747	M

Template NE-28: ADJ-BIS-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

None

Template NE-29: Operations on PIB-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.30.1	PIB id	None	S	GET	M

NL.30.2	Full information contained in PIB records, with	None	R	GET	M

a structure to be defined in accordance with the

policy syntax.

Template NE-30: PIB-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

NL.31.1	Object creation	ISO 10164-1,	M

ISO 10165-2

NL.31.2	Object deletion	ISO 10164-1,	M

ISO 10165-2

Template NE-31: PIB-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.32.1	pibId	idrpConfig	None	M

Template NE-32: PIB-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

None

Template NE-33: Operations on ADJ-RIB-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.34.1	Adj-RIB id	None	S	GET	R

NL.34.2	Information contained in the Adj-RIB-In and	None	R	GET

Adj-RIB-Out records.

Template NE-34: ADJ-RIB-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

NL.35.1	Object creation	ISO 10164-1,	R

ISO 10165-2

NL.35.2	Object deletion	ISO 10164-1,	R

ISO 10165-2

Template NE-35: ADJ-RIB-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.36.1	AdjRibId	idrpConfig	None	M

Template NE-36: ADJ-RIB-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

None

Template NE-37: Operations on LOC-RIB-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.38.1	Loc RIB id	None	S	GET	R

NL.38.2	Information contained in the Loc-RIB records.	None	R	GET	R

Template NE-38: LOC-RIB-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

NL.39.1	Object creation	ISO 10164-1,	R

ISO 10165-2

NL.39.2	Object deletion	ISO 10164-1,	R

ISO 10165-2

Template NE-39: LOC-RIB-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.40.1	locRibId	idrpConfig	None	R

Template NE-40: LOC-RIB-MO Object Class Name Types

Index	Operations on Object Class	Ref.	Status

None

Template NE-41: Operations on FIB-MO Object Class

Index	Object Class Attributes	Ref.	Type	Operation	Status

NL.42.1	FIB id	None	S	GET	R

NL.42.2	Information contained in the FIB records.	None	R	GET	R

Template NE-42: FIB-MO Object Class Attributes

Index	Object Class Notifications	Ref.	Status

NL.43.1	Object creation	ISO 10164-1,	R

ISO 10165-2

NL.43.2	Object deletion	ISO 10164-1,	R

ISO 10165-2

Template NE-43: FIB-MO Object Class Notifications

Index	Object Class Name Type	Superior	Ref.	Status

NL.44.1	fibId	networkEntity	None	R

Template NE-44: FIB-MO Object Class Name Types

### A12.6.2  Transport Entity Managed Objects

The Transport Entity managed objects, their attributes and operations shall be commonly available to implementations of either Systems or Layer Management via the described mechanisms.  Any local or distributed management implementation shall exercise Transport Layer management via these mechanisms.

Note 1.-  The definitions given in this section concern the managed objects for the Transport Layer, their attributes, the operations that may be performed on them, and generic mechanisms for access to them.

Note 2.-  The definitions are derived from ISO/IEC 10737. The Ref. column in the subsequent tables can be assumed to refer to this standard unless otherwise stated.

#### A12.6.2.1  Transport Entity Managed Object Containment

ATN Transport Entity Managed Objects shall conform to the containment tree shown in A12.2.

Note 1.-  Shadowed boxes indicate that multiple instances may exist in a given managed system.

Note 2.-  In the templates that follow, if not otherwise mentioned, the definitions given make an implicit reference to ISO 10737.

#### A12.6.2.2  Transport Entity Managed Object Classes

Instances of the managed objects (MOs) from the following set of object classes shall form the Transport Layer Management Information Base (MIB).

·	Transport Subsystem managed object class;

·	Transport Entity managed object class;

·	Connection-Oriented Transport Protocol Machine managed object class;

·	Connectionless Transport Protocol Machine object class;

·	Transport TSAP managed object class;

·	Transport Connection Initial Value Managed Object (IVMO) class;

·	Transport Connection managed object class.

##### A12.6.2.2.1  The Transport Subsystem Managed Object (TSMO) Class

The transport subsystem managed object shall contain those functions of an Open System which provides the OSI Transport Service subject to management operation. There shall be exactly one of these MOs within a System.

##### A12.6.2.2.2  The Transport Entity Managed Object (TEMO) Class

One TEMO shall exist for each Transport Entity, that is, for each Transport Layer user of the Network Service (associated with an NSAP or set of NSAPs).

Note.-  This class is derived from ISO/IEC 10165-5: communicationsEntity.

##### A12.6.2.2.3  The Connection-Oriented Transport Protocol Machine Managed Object (CO TPM MO) Class

There shall be only one such object for each Transport Entity associated with an NSAP (or set of NSAPs). When the protocol machine is operable the Operational State shall have the value 'enabled'; otherwise it shall have the value 'disabled'.  Transitions of Operational State shall be reported using the State Change notification.

Note 1.-  The use/non use of checksums is selectable on a connection basis (see TC IVMO).

Figure A12.3: Transport Managed Object Hierarchy

Note 2.-  This class is derived from ISO/IEC 10165-5: coProtocolMachine.

##### A12.6.2.2.4  The TSAP Managed Object (TSAP MO) Class

There shall be one such MO for each TSAP currently recognized by the containing Transport Entity.

The Transport Address of a TSAP shall be formed by concatenating its TSAP selector with the NSAP address of the NSAP being used by the TEMO.

##### A12.6.2.2.5  The Transport Connection Managed Object (TCMO) Class

The Transport Connection Object Managed Object class shall contain the set of attributes characterizing the manageable aspects of a Transport Layer Protocol Connection.

An MO of this class shall exist corresponding to each active Transport Connection, i.e., for which a CR TPDU has been sent or received and which has not yet been terminated.

Note 1.-  This specification defines an object class for Transport Class 4 service over a Connectionless Network service.

Note 2.-  There is currently no attribute defined by ISO 10737 for the maximum window size on a connection although it is in the Initial Value MO (TE.18.29).

Note 3.-  There is currently no attribute defined by ISO 10737 for the current receive and transmit window size in use on the connection (TE.18.30 - TE.18.31).  These may be needed for a system-wide solution to congestion (e.g., to dynamically adapt congestion control algorithms to network load).

##### A12.6.2.2.6  The Connectionless-Mode Transport Protocol Machine Managed Object (CL TPM MO)

Note 1.-  This object is a member of the Transport Layer Management Information Base (MIB) as shown in the containment hierarchy in Figure A12.3.  It shall be created according to ATN Application requirements for its use.

There shall be no more than one of these MOs per Transport Entity. When the protocol machine is operable the Operational State shall have the value 'enabled'; otherwise it shall have the value 'disabled'. Transitions of Operational State shall be reported using the State Change notification.

Note 2.-  The use/non use of checksums is selectable upon object creation.

Note 3.-  This class is derived from ISO/IEC 10165-5: coProtocolMachine.

##### A12.6.2.2.7  The Transport Connection Initial Value Managed Object (TC IVMO) Classes

##### A12.6.2.2.8  The ATN Transport Layer Superset MO Classes

Note.-  These are objects and events identified for use specifically in the ATN environment. They have not been standardized by ISO but they are still contained in  the Transport Subsystem managed object hierarchy.

###### A12.6.2.2.8.1 The Transport Connection QOS Initial Value Managed Object (TCQ-IVMO).

There shall be an additional object in the hierarchy to the Transport Connection Initial Value Managed Object to support default QOS selection by transport service users called the Transport Connection QOS Initial Value Managed Object (TCQ-IVMO).

#### A12.6.2.3  Transport Entity Managed Object Requirements Templates

ATN Transport Entity Managed Objects shall conform with the MORTs in the following templates.

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-1 Operations on TEMO Object Class

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-2 Operations on TSMO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.2.1 | Transport Subsystem Id |  | S | GET | M |

Template TE-3 TSMO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-4 TSMO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.4.1 | subsystemId | system | iso/iec 10165-5 | M |

Template TE-5 TSMO Object Class Name Bindings

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.6.1 | Transport Entity Id | iso/iec 10165-5 | S | GET | M |
| TE.6.2 | Local TSAP(s) names | iso/iec 10165-5 | R | GET | M |
| TE.6.3 | Operational State | iso/iec 10165-5 | R | GET | M |
| TE.6.4 | The MO name of the NSAP(s) to be used by this T-entity |  | R/W | GET, REPLACE ADD, REMOVE | M |
| TE.6.5 | The MO name of the NSAP(s) used by this T-entity |  | R | GET | R |
| TE.6.6 | The number of NSDUs undecoded |  | C | GET | R |
| TE.6.7 | The number of PDUs Reived with incorrect checksum |  | C | GET | R |
| TE.6.8 | The number of protocol errors |  | R | GET | R |

Template TE-6 TEMO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
| TE.7.1 | Notification of a protocol error detected by the local TE. | iso/iec 10165-2 Communications Alarm | R |

Template TE-7 TEMO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.8.1 | communinicationsEntityId | TransportSubsystem | iso/iec 10165-5 | M |

Template TE-8 TEMO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.9.1 | CREATE | iso/iec 10165-2 | M |
| TE.9.2 | DELETE | iso/iec 10165-2 | M |
| TE.9.3 | ACTION - Activate. Cause entity to enter operation mode, if successful the Operational State of the TE will eventually become Enabled. | iso/iec 10165-5, 10164-2 | M |
| TE.9.4 | ACTION - De-activate. Cause entity to enter operation mode, if successful the Operational State of the TE will eventually become Disabled. | iso/iec 10165-5, 10164-2 | M |

Template TE-9 Operations on CO-TPM-MO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.10.1 | CO TPM Identity | iso/iec 10165-5 | S | GET | M |
| TE.10.2 | Operational state (enable, disabled) | iso/iec 10165-5 | R | GET | M |
| TE.10.3 | Administrative state (locked, unlocked, shutting down) |  | R | GET | M |
| TE.10.4 | The number of octets sent |  | C | GET | R |
| TE.10.5 | The number of user data octets received |  | C | GET | R |
| TE.10.6 | The number of TCs currently in an open state |  | G | GET | R |
| TE.10.7 | The max. number of simultaneously open TCs allowed (more con. requests will be refused) |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.10.8 | No. of locally initiated TCs that reached open state |  | C | GET | R |
| TE.10.9 | No. of remotely initiated TCs that reached open state |  | C | GET | R |
| TE.10.10 | No. of locally initiated TCs that did not reach open state |  | C | GET | R |
| TE.10.11 | No. of remotely initiated TCs that did not reach open state |  | C | GET | R |
| TE.10.12 | No. of locally initiated T_Disconnects due to error |  | C | GET | R |
| TE.10.13 | No. of remotely initiated T_Disconnects due to error |  | C | GET | R |
| TE.10.14 | No. of TPDUs that could not be associated with a TC |  | C | GET | R |
| TE.10.15 | Highest no. of TCs to simultaneously be in an open state |  | T | DEFAULT, REPLACE, GET | R |

Template TE-10 CO-TPM-MO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
| TE.11.1 | Object creation | iso/iec 10165-2, 10164-1 Object Creation | M |
| TE.11.2 | Object deletion | iso/iec 10165-2, 10164-1 Object Deletion | M |
| TE.11.3 | Change administrative or operational state. | iso/iec 10165-2, 10164-2 State Change | M |
| TE.11.4 | Report rejected connection | iso/iec 10165-5, Communications Information | M |

Template TE-11 CO-TPM-MO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.12.1 | coProtocolMachineId_Management OR coProtocolMachineId_Automatic | Transport Entity | iso/iec 10165-5 | M |

Template TE-12 CO-TPM-MO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.13.1 | CREATE | iso/iec 10165-2 | M |
| TE.13.2 | DELETE | iso/iec 10165-2 | M |

Template TE-13 Operations on TSAP-MO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.14.1 | TSAP Address |  | S | GET | M |

Template TE-14 TSAP-MO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-15 TSAP-MO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.16.1 | tSAP-transportEntity-Automatic OR tSAP-transportEntity-Management | Transport Entity | iso/iec 10165-5 | M |

Template TE-16 TSAP-MO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.17.1 | CREATE | iso/iec 10165-2 | M |
| TE.17.2 | DELETE | iso/iec 10165-2 | M |

Template TE-17 Operations on TC-MO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.18.1 | Connection Identity | iso/iec 10165-5 | S | GET | M |
| TE.18.2 | Local connection endpoint (may be TSAP id or some other identifier) | iso/iec 10165-5 | R | GET | M |
| TE.18.3 | Underlying connection name (distinguished MO name of underlying connection or pysical medium) | iso/iec 10165-5 | R | GET | M |
| TE.18.4 | Local TSAP Address being used | iso/iec 10165-5 | R | GET | M |
| TE.18.5 | Remote TSAP Address being used | iso/iec 10165-5 | R | GET | M |
| TE.18.6 | Protocol class being used |  | R | GET | M |
| TE.18.7 | The max. TPDU size that was negotiated |  | R | GET | M |
| TE.18.8 | The local reference number |  | R | GET | M |
| TE.18.9 | The remote reference number |  | R | GET | M |
| TE.18.10 | The calling TSAP selector specified during connection establishment |  | R | GET | M |
| TE.18.11 | The called TSAP selector specified during connection establishment |  | R | GET | M |
| TE.18.12 | The calling NSAP address specified at Network Service interface |  | R | GET | M |
| TE.18.13 | The called NSAP address specified at Network Service interface |  | R | GET | M |
| TE.18.14 | The responding NSAP address specified at Network Service interface |  | R | GET | M |
| TE.18.15 | Indicator for connection direction |  | R | GET | M |

Template TE-18a TC-MO Object Class Attributes (1/2)

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.18.16 | Total no. of TPDUs transmitted (excluding re-transmission) |  | C | GET | M |
| TE.18.17 | Total no. of TPDUs received |  | C | GET | M |
| TE.18.18 | Total no. of user data octets sent on the TC |  | C | GET | M |
| TE.18.19 | Total no. of user data octets received on the TC |  | C | GET | M |
| TE.18.20 | No. of TPDUs re-transmitted due to incoming protocol error |  | C | GET | M |
| TE.18.21 | Value used for the local acknowledge time |  | R | GET | M |
| TE.18.22 | Indicator for checksum use/non-use |  | R | GET | M |
| TE.18.23 | Indicator for normal, extended format use/non-use |  | R | GET | M |
| TE.18.24 | Value used for the inactivity timer |  | R | GET | M |
| TE.18.25 | The max. no of TPDU Transmissions |  | R | GET | M |
| TE.18.26 | Value used for the local re-transmission timer |  | R | GET | M |
| TE.18.27 | Indicator for expedited data use/non-use |  | R | GET | M |
| TE.18.28 | Value used for the window timer |  | R | GET | M |
| TE.18.29 | The max. window permitted on this connection | no ISO 10737 ref. | R | GET | M |
| TE.18.30 | Current receive window size | no ISO 10737 ref. | R | GET, REPLACE | O |
| TE.18.31 | Current transmit window size | no ISO 10737 ref. | R | GET, REPLACE | O |

Template TE-18b TC-MO Object Class Attributes (2/2)

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
| TE.19.1 | Object creation containing: TC name, protocol class, max. TPDU size, called and calling T Selector, called, calling and responding NSAP addresses, connection direction. | iso/iec 10165-2, 10164-1 Object Creation | M |
| TE.19.2 | Object deletion containing: TC name, protocol class, max. TPDU size, called and calling T_Selector, called, calling and responding NSAP addresses, deletion cause. | iso/iec 10165-2, 10164-1 Object Deletion | M |
| TE.19.3 | Successful Connection Establishment containing: TC name, protocol class, max. TPDU size, called and calling T_Selector, called, calling and responding NSAP addresses, deletion cause. | iso/iec 10165-2, 10164-2 State Change | M |

Template TE-19 TC-MO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.20.1 | connectionId | Connection Oriented Transport Protocol Machine | iso/iec 10165-5 | M |

Template TE-20 TC-MO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.21.1 | CREATE | no ISO 10737 ref. | M |
| TE.21.2 | DELETE | no ISO 10737 ref. | M |

Template TE-21 Operations on TC-IVMO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.22.1 | TC IVMO Identity |  | S | GET | M |
| TE.22.2 | The preferred/alternative set of protocol classes required |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.3 | The max. TPDU size to be negotiated |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.4 | Enables negotiation of checksum use/non use |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.5 | Enables negotiation of extended format use/non use |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.6 | Value to use for the inactivity timer |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.7 | The max. number of transmissions of a TPDU |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.8 | Value used for the local re-transmission timer |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.9 | Value used for the window timer |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.10 | The max. window permitted on this connection |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.22.11 | Indicator for Transport expedited data use/non-use |  | R/W | DEFAULT, REPLACE, GET | M |

Template TE-22 TC-IVMO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-23 TC-IVMO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.24.1 | transportConnectionIVMOId | Connection Oriented Transport Protocol Machine | iso/iec 10165-5 | M |

Template TE-24 TC-IVMO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.25.1 | CREATE | iso/iec 10165-2 | M |
| TE.25.2 | DELETE | iso/iec 10165-2 | M |
| TE.25.3 | ACTION - Activate. Cause entity to enter operation mode, if successful the Operational State of the TE will eventually become Enabled. | iso/iec 10165-5, 10164-2 | M |
| TE.25.4 | ACTION - De-activate. Cause entity to enter operation mode, if successful the Operational State of the TE will eventually become Disabled. | iso/iec 10165-5, 10164-2 | M |

Template TE-25 CL-TPM-MO Operations on Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.26.1 | CL TPM Identity | iso/iec 10165-5 | S | GET | M |
| TE.26.2 | Operational state (enable, disabled) | iso/iec 10165-5 | R | GET | M |
| TE.26.4 | Checksum on/off option |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.26.5 | The number of octets sent |  | C | GET | R |
| TE.26.6 | The number of user data octets received |  | C | GET | R |
| TE.26.7 | The number of undeliverable PDUs |  | C | GET | R |

Template TE-26 CL-TPM-MO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
| TE.27.1 | Object creation | iso/iec 10165-2, 10164-1 Object Creation | M |
| TE.27.2 | Object deletion | iso/iec 10165-2, 10164-1 Object Deletion | M |
| TE.27.3 | Change administrative or operational state. | iso/iec 10165-2, 10164-2 State Change | M |
| TE.27.4 | Report rejected connection | iso/iec 10165-5, Communications Information | M |

Template TE-27 CL-TPM-MO Object Class Notifications

| Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.28.1 | clProtocolMachineId_Management OR clProtocolMachineId_Automatic | Transport Entity | iso/iec 10165-5 | M |

Template TE-28 CL-TPM-MO Object Class Name Bindings

| Index | Operations on Object Class | Ref. | Status |
| --- | --- | --- | --- |
| TE.29.1 | CREATE | iso/iec 10165-2 | M |
| TE.29.2 | DELETE | iso/iec 10165-2 | M |

Template TE-29 Operations on TCQ-IVMO Object Class

| Index | Object Class Attributes | Ref. | Type | Operation | Status |
| --- | --- | --- | --- | --- | --- |
| TE.30.1 | Name of the TCQ IVMO |  | S | GET | R |
| TE.30.2 | Connection establishment delay |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.3 | Connection establishment failure probability |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.4 | Throughput |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.5 | Residual error rate |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.6 | Transfer failure probability |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.7 | Connection release delay |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.8 | Connection release failure probability |  | R/W | DEFAULT, REPLACE, GET | R |
| TE.30.9 | Protection |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.30.10 | Priority |  | R/W | DEFAULT, REPLACE, GET | M |
| TE.30.11 | Resilience |  | R/W | DEFAULT, REPLACE,GET | M |

Template TE-30 TCQ-IVMO Object Class Attributes

| Index | Object Class Notifications | Ref. | Status |
| --- | --- | --- | --- |
|  | None |  |  |

Template TE-31 TCQ-IVMO Object Class Notifications

| Appendix 12 Systems Management Provisions 1 Index | Object Class Name Type | Superior | Ref. | Status |
| --- | --- | --- | --- | --- |
| TE.32.1 | transportConnectionQOS_IVMOId | Connection Oriented Transport Protocol Machine | iso/iec 10165-5 | M |

Template TE-32 TCQ-IVMO Object Class Name Bindings

A12.1  Introduction	1

A12.1.1  Overview	1

A12.1.2  General Considerations	1

A12.2  Systems Management Functions	1

A12.2.1  Manager SMA Functions	2

A12.2.1.1  Object Management Function	2

A12.2.1.1.1  Specific Object Management Services	2

A12.2.1.1.2  Object Management Pass Through Services	2

A12.2.1.2  State Management Function	3

A12.2.1.3  Alarm Reporting Function	3

A12.2.1.4  Event Report Management Function	3

A12.2.1.5  Log Control Management Function	3

A12.2.1.6  Security Alarm Reporting Management Function	4

A12.2.1.7  Objects and Attributes for Access Control	4

A12.2.2  Agent SMA Functions	4

A12.2.2.1  Object Management Function	4

A12.2.2.1.1  Specific Object Management Services	4

A12.2.2.1.2  Object Management Pass Through Services	4

A12.2.2.2  State Management Function	4

A12.2.2.3  Alarm Reporting Function	5

A12.2.2.4  Event Report Management Function	5

A12.2.2.5  Log Control Management Function	5

A12.2.2.6  Security Alarm Reporting Management Function	6

A12.2.2.7  Objects and Attributes for Access Control	6

A12.3  Systems Management Services	6

A12.3.1  Common Management Information Service Element (CMISE) Requirements	6

A12.3.2  ACSE Requirements	7

A12.4  Systems Management Protocols	7

A12.4.1  Application Layer Protocols	7

A12.4.2  Presentation Layer Protocol	7

A12.4.3  Session Layer Protocol	7

A12.4.4  Transport Layer Protocol	8

A12.5  Systems Management Information	8

A12.5.1  ATN Managed Object Containment Hierarchy	8

A12.5.2  ATN Managed Object Instances	9

A12.5.2.1  System	9

A12.5.2.2  Access Control Policy	9

A12.5.2.3  Targets	9

A12.5.2.4  Authorized Initiators	10

A12.5.2.5  Event Forwarding Discriminator	10

A12.5.2.6  Logs	10

A12.6  Managed Objects for Communications Subsystems	10

A12.6.1  Network Entity Managed Objects	10

A12.6.1.1  Network Entity Managed Object Containment	10

A12.6.1.2  Network Entity Managed Object Classes	11

A12.6.1.2.1  The Network Subsystem Managed Object (NSMO) Class	12

A12.6.1.2.2  The Network Entity Managed Object (NEMO) Class	12

A12.6.1.2.3  The NSAP Managed Object (NSAP-MO) Class	12

A12.6.1.2.4  The Connectionless Network Protocol Machine Managed Object (CL-NPM-MO) Class	13

A12.6.1.2.5  The Linkage Managed Object (LINK-MO) Class	13

A12.6.1.2.6  The IDRP Configuration Managed Object (IDRP-CFG-MO) Class	13

A12.6.1.2.7  The Adjacent BIS Managed Object (ADJ-BIS-MO) Class	13

A12.6.1.2.8  The PIB Managed Object (PIB-MO) Class	13

A12.6.1.2.9  The Adj-RIB Managed Object (ADJ-RIB-MO) Class	14

A12.6.1.2.10  The Loc-RIB Managed Object (LOC-RIB-MO) Class	14

A12.6.1.2.11  The FIB Managed Object (FIB-MO) Class	14

A12.6.1.3  Network Entity Managed Object Requirements Templates	14

A12.6.2  Transport Entity Managed Objects	32

A12.6.2.1  Transport Entity Managed Object Containment	32

A12.6.2.2  Transport Entity Managed Object Classes	32

A12.6.2.2.1  The Transport Subsystem Managed Object (TSMO) Class	32

A12.6.2.2.2  The Transport Entity Managed Object (TEMO) Class	32

A12.6.2.2.3  The Connection-Oriented Transport Protocol Machine Managed Object (CO TPM MO) Class	32

A12.6.2.2.4  The TSAP Managed Object (TSAP MO) Class	33

A12.6.2.2.5  The Transport Connection Managed Object (TCMO) Class	33

A12.6.2.2.6  The Connectionless-Mode Transport Protocol Machine Managed Object (CL TPM MO)	34

A12.6.2.2.7  The Transport Connection Initial Value Managed Object (TC IVMO) Classes	34

A12.6.2.2.8  The ATN Transport Layer Superset MO Classes	34

A12.6.2.2.8.1 The Transport Connection QOS Initial Value Managed Object (TCQ-IVMO).	34

A12.6.2.3  Transport Entity Managed Object Requirements Templates	35