From: Tom Kraft/Catherine Gandolfi/Rachel Daeschler

Date: 29 July 1996

Subject: Discussion, Comments, and Considerations for ICAO ANNEX 10, Part I, Chapter 3, Appendix A Technical Requirements for the Aeronautical Telecommunication Network

This working paper contains 1) a presentation of the appendix layout, 2)  a summary of the review of slr (section 1) with remaining questions, 3) general comments about sections 2 and 3, ATN applications and ATN communication services, 4)comments on section 2.1.1 (cm), 5) comments on section 2.2.1 (cpdlc), 6) comments on section 2.2.2 (ads), 7) comments on section 2.2.3 (fis), 8) comments on section 2.3.1 (ats-mhs), 9) comments on section 2.3.2 (aidc), 10) comments on section 3.1 (ula), 11) comments on section 3.2 (ics), 12) a list of acronyms.

1 Layout of the appendix

The following table describes the appendix structure and contents, and provides cross referencing to old references. The left column indicates the short name used to designate the sections in this document.

| ANNEX 10, Part I, Chapter 3 |  |  |
| --- | --- | --- |
|  | Description | Old References |
| atn-core | Main Body Text | New |

| Appendix for ANNEX 10, Part I, Chapter 3 |  |  |  |
| --- | --- | --- | --- |
|  | Apx Ref | Description | Old References |
| atn-slr | 1 | Introduction and System Level Requirements (SLR) | Sub-Volume 1 |
| atn-app | 2 | ATN Applications |  |
| sys-app | 2.1 | System Applications |  |
| cm | 2.1.1 | Context Management (CM) | Sub-Volume 2, Part 1 |
| ag-app | 2.2 | Air-Ground Applications |  |
| cpdlc | 2.2.1 | Controller-Pilot Data Link Communications (CPDLC) | Sub-Volume 2, Part 3 |
| ads | 2.2.2 | Automatic Dependent Surveillance (ADS) | Sub-Volume 2, Part 2 |
| fis | 2.2.3 | Flight Information Service (FIS) | Sub-Volume 2, Part 4 |
| gg-app | 2.3 | Ground-Ground Applications |  |
| ats-mhs | 2.3.1 | Air Traffic Services Message Handling Services (ATSMHS) | Sub-Volume 3, Part 1 |
| aidc | 2.3.2 | ATS Interfacility Data Communication (AIDC) | Sub-Volume 3, Part 2 |
| atn-cs | 3 | ATN Communication Services |  |
| ula | 3.1 | Upper Layers Communication Service | Sub-Volume 4 |
| ics | 3.2 | Internet Communication Service | Sub-Volume 5 |

2 Review of atn-slr - List of definitions and references - System level requirements.

This part contains 2.1 What has been done, 2.2 Additional suggestions and questions about the list of definitions, and 2.3 Changes or suggested changes to the system level requirements.

2.1 What has been done with the list of definitions

All the sections have been reviewed in order to update the list of acronyms and definitions. Most of the acronyms that do not appear in the text have been deleted, and all acronyms that are used should now appear in the list of acronyms provided at the end of this document. Not all of these acronyms have been selected to appear in the master definitions section, only the ones that are “stared” and “double-stared”, please see the legend part 10 of this document. The logic applied to the selection of acronyms in the master list is the following: Popular acronyms have an alphabetized entry. In many cases the “spelled-out” term is also listed with a formal definition. Not so popular acronyms are identified in parenthesis after the “spelled-out” term. Obscure acronyms are not included and should be defined in the text where they are used.

A similar process has been applied to the terms that need to be defined with more than a spelled-out expression. Some definitions have been suppressed from the list because they were not used in the text  and were not particularly relevant regarding the general understanding of the appendix. On the other hand, some terms that are used in the text and were considered important enough were added to the list and defined (for example: Next Data Authority, FIS contract)

2.2 Additional suggestions and questions about the list of definitions

2.2.1 Many terms that were capitalized have been made lower case. A guideline should be to write sentences using normal English syntax and structure, with little “capitalization”, e.g., proper names of organizations, Standards and Recommended Practices, acronyms, and the first word of sentences.

2.2.2 Three definitions are still to be done: aeronautical stakeholder, CIDIN (Common ICAO Data Interchange Network) and Inter-Centre Communications (ICC).

A suggestion would be to delete the “aeronautical stakeholder” entry, to delete all references to “stakeholder” and to clarify the “Aeronautical industry service communications definition”, especially with respect to what is encompassed by “aeronautical industry”. About AINSC, what is the use of it? Is this old terminology?

A suggestion would be to delete all references to ICC (ICC appears only in slr) and to replace them with AIDC  (ATS interfacility data communications). Otherwise, the definition of Inter-centre communications should be exactly the same as AIDC.

2.2.3 A follow-up review of the master list of definitions should focus on the following questions: Is the list of terms complete? Are there terms that should be deleted? Are the terms consistent with ICAO publications, ISO publications, other documents? Are differences clearly delineated?

2.2.4 It is a little confusing that both ATS MHS (ATS Message Handling Service) and AMHS (ATS Message Handling System) co-exist. Are there occurrences of AMHS where AMHS stands for ATS Message Handling Service?

2.2.5 Definitions for “XF-address”, “YR-EDTU-NR”, and “YR-TU-NR” seem to need clarification. Suggestions for this:

XF-address: Translated address (used in MHS). The XF-address is related to XF-addressing scheme used as the single common (common is here opposed to “locally defined”) ICAO AMHS addressing scheme in the present version of the mhs document. An XF-address is composed of an AMHS management domain identifier, an organization-name attribute and an organizational-unit-names attribute.

YR-EDTU-NR Expected ED TPDU sequence number in EA(‘your ED TPDU number’).

YR-TU-NR Expected TPDU sequence number (‘your TPDU number’)

These two last definitions come from an old version of ics, internet, communication services. ED stands for Expedited Data, and EA stands for ED acknowledge. But these definitions don’t seem to appear in the latest version of ics. They might be deleted from the list?

2.2.6 Definitions of “user” and “user requirement:” These two definitions provide a definition of the term “user” that are not equivalent. In the text of these SARPS the term “user” is used in the sense defined by the “user” definition and not in the sense defined by the “user requirement” definition.

Moreover, the definition of “user requirement” does not correspond to the sense in which “user requirement” is used in the text. In each application section, there is a subsection numbered 7 called “user requirements” that outlines the requirements that the user (that abstract part of the aircraft or ground system that performs the non-communications related functions of the application) must meet especially with respect to the interfacing with the application ASEs, and not the requirements that the user expects to obtain from the system.

2.2.7 “ICAO” might be spelled out the first time it is used in slr.

2.3 Changes or suggested changes to the system level requirements (slr, section 1.3)

Ref 1.3.5: “Authorized ATSU” has been changed to “controlling ATSU.” “ATC instructions has been changed to “ATC clearances.”

Ref 1.3.8: “Traffic classes” has been changed to “traffic types.”

Ref 1.3.11 and 1.3.12 have been revised to make consistent and more precise. “One or more authorized paths.”

Ref 1.3.14: “Within a dialogue” has been deleted.

Ref 1.3.17 Should there be a “shall” for “ICAO standardized fixed subnetworks” to bound the validation effort?

Ref 1.3.23: It has been reordered with 1.3.22.

Ref 1.3.27: “AMHS” has been changed to “ATS MHS.”

3 General comments about sections 2 and 3, ATN applications and ATN communication services

### 3.1 Conventions for Expressing Requirements

#### The following conventions apply for expressing requirements in this Sub-Volume:

- shall - is used  to state a mandatory requirement;

- should - is used to state a recommended practice.

#### In tables which list parameters of service primitives, and fields of PDUs, the presence of each parameter or field is described (using the notation specified in Annex B of ISO/IEC 10731) by one of the values given in the following table:

Symbols Used in Tables

| Symbol | Meaning |  |
| --- | --- | --- |
| blank | the parameter is not applicable |  |
| C | Conditional - the presence or absence of the parameter is dependent upon some stated condition |  |
| M | Mandatory - the parameter shall be present |  |
| U | the presence of the parameter is a User option |  |
| O | the feature is optional to implement |  |
| X |  | eXcluded - the parameter shall not be present |
| (=) |  | indicates that a parameter value is semantically equal to the value to its left in the table |

#### In time sequence diagrams, dotted lines indicate the time sequence between primitives.  A wavy line is used to represent the case in which the service description does not imply any time relationship between the primitives.

#### 3.2 The industry should review these SARPs... But, their validation would require an awful amount of work, since, despite the intent to describe the concepts, some details like the state transition diagrams would almost require a simulation in order to perform a review...

3.3 The modeling conventions for the primitives and the associated parameters are repeated in all the documents describing the applications (such as CPDLC, ...).

3.4 Description of the abstract service: The abstract service is a functional model used to describe the application. It is generic for every application. Hence, a generic explanation illustrated on a “simple” example could be provided (where?) instead of being repeated for every application. This generic description should then be referenced by each application section.

3.4 Year coding: Numerous problems will be experienced by various software programs including GPS receivers at year 2000. Why limiting the coding of the date (in CPDLC, ADS, MHS...messages) to 2 digits and shifting the date reference? What is the advantage of this coding? Does it not complicate the analysis of the data link recorded messages (the reference of 1996 needs to be known and is likely to differ from other practices in some existing programs...)

3.5 ASE version number management (This comment is based on the cpdlc section, p12, but it might be relevant for all the applications)

The document does not explicit the underlying assumption that the version number for ASE increases with additional functionality and defines a rather complicated procedure in order to check compatibility of versions between applications. This specification in fact dictates the design for the version number implementation (1, 2, 3, ...) and limits the range to 15.

Furthermore, the document specifies a dedicated procedure requiring the sending ground-ASE to transmit the version number only if its version number is greater than the version number of the receiver ASE. What is the advantage or goal of such an asymmetric procedure, which seems to unnecessarily add a test on the version number ? Should a service giving a result on compatibility of versions be provided or should this verification be accomplished by each application ?

3.6 Data types description (This comment is based on the cpdlc section, but it is relevant for other applications like fis or ads)

Because ASN.1 precise grammar is provided and in order to avoid duplication, the data types should not be described in the core of the document but rather referenced to the ASN.1 abstract syntax. This is the case for most of data types, except for aircraft identification and ICAO facility designator in some instances (see Called Peer Identifier and Calling Peer Identifier parameters for CPDLC-start Service p7, DSC-start Service p8). Similarly, the "Result" parameter as "accepted" or "rejected" should have been specified in the ASN.1 grammar in order to define the complete set of parameters and provide a consistent description (there are other "simple" data types specified in the ASN.1 format like "LogicalAck" which is either required or "notRequired").

3.7 First implementation of ATN: From the notes retranscripted below (located in mhs section), we conclude that the first implementations will provide gateways from the existing AFTN entities and that the security will be specifically handled by each organization ... The first implementation looks like the interconnection of existing networks with local adaptations.

Note 10.-  Applicability

a)	The implementation of the ATS Message Service is mandatory for conformance with 3.1. However, as a matter of organisations’ policy, interim conformance may be achieved with the implementation of the ATN Pass-Through Service. The choice to implement the ATN Pass-Through Service as an interim solution does not replace the requirement to implement the ATS Message Service at the earliest possible date.

Note 1.-  In the Basic ATS Message Service, the security at each ATS Message Server or AFTN/AMHS Gateway is deemed a local issue to be addressed by the authority in charge of the system.

3.8 The ASN.1 descriptions for common data types do not have a unique definition across the various SARPs documents and the various definitions are not always consistent.

For example the "altitude" data are described slightly differently. This makes the correctness of the data more difficult to check like for example the upper range for AltitudeFeet correctly defined as 70000 in CPDLC and ICC documents but with an incorrect associated comment of 700000 in CPDLC document.

AltitudeType in CPDLC document is equivalent to Altitude in ICC document.

In CPDLC application:

Altitude ::= CHOICE

{

noAltitude		[0]	NULL,

singleAltitude		[1]	AltitudeType,

blockAltitude		[2]	SEQUENCE SIZE (2) OF AltitudeType

}

AltitudeType ::=  CHOICE

{

altitudeFeet			[0]	AltitudeFeet,

altitudeMeters			[1]	AltitudeMeters,

altitudeFlightLevel		[2]	AltitudeFlightLevel,

altitudeFlightLevelMetric	[3]	AltitudeFlightLevelMetric

}

AltitudeFeet ::=  INTEGER (-600..70000)

--unit = Feet, Range (-600..700000), resolution = 10

AltitudeMeters ::= INTEGER (-30..25000)

--unit = Meter, Range (-30..25000), resolution = 1

AltitudeFlightLevel ::= INTEGER (30..700)

--unit = Level (100 Feet), Range (030..700), resolution = 1

AltitudeFlightLevelMetric ::= INTEGER (100..2500)

--unit = Level (10 Meters), Range (100..2500), resolution = 1

In ICC document:

Altitude::= CHOICE

{

altitudeFeet				[0]	AltitudeFeet,

altitudeMeters				[1]	AltitudeMeters,

altitudeFlightLevel			[2]	AltitudeFlightLevel,

altitudeFlightLevelMetric		[3]	AltitudeFlightLevelMetric

}

AltitudeFeet, AltitudeMeters, AltitudeFlightLevel and AltitudeFlightLevelMetric are equivalent to the CPDLC definitions.

There are other examples of discrepancies like for DistanceKm:

- defined in ICC document as:

DistanceKm::= INTEGER (0..2000)

-- unit = kilometer, Range (0..2000), resolution = 1

- specified in CPDLC document as:

DistanceKm ::= INTEGER (0..8000)

--	unit = Kilometer, Range (0..1000), resolution = 0.25

The coding of the date also differs between the ICC and CPDLC documents, with a coding limited to 2 digits but with a different reference.

In ICC document:

Year::= INTEGER (0..99)

-- unit = Year, Range (0..99), resolution = 1

In CPDLC document:

Year ::= INTEGER (0..99)

-- unit = Year, Range (1996..2095), resolution = 1

#### 4) Comments on cm section

#### Acronyms that are not explained the first time they are used in the cm section:

CM-ASE, PDU, TSAP, RDP, ARS, APDU, ID, QOS, PER, RER, ATSC, ADS, CPDLC, FIS, IDP, LOC, SYS, SEL.

In CM, the following indications are used: “D-data, D-end, D-abort, D-P abort, D-start.”

#### It would be helpful to clarify the meaning of “D” which stands probably for dialogue, and the meaning of “D-P”.

#### 5) Comments on cpdlc section

5.1 Remark on the references

The references in the CPDLC document like [1], [4], ... do not directly match with the list of references provided (no numbering or cross-reference).

# 5.2	Compatibility/Differences with RTCA DO-219

The document does not explain nor reference the message attributes meaning for urgency, alert, uplink response and downlink response. The message presentation recommendations contained in Tables 2.3.7-5 to 2.3.7-28 (from subpart 5.8) for a possible means of presenting the information should be referenced.

In order to facilitate the transition from FANS-1/A RTCA DO-219/ARINC622 based systems to ATN, compatibility and differences between this CPDLC specification and RTCA DO-219 message description should be provided.

For example, although the descriptions of ATCuplinkmessage in CPDLC SARPS and DO-219 use a different ASN.1 representation, the defined structures are equivalent. But, CPDLC SARPS adds some alternatives in the "CHOICES" for the constituent elements of ATCUplinkMsgElementId from uM183 to uM236. The expected behavior of the ground or airborne applications not implementing these additional messages should be documented. Were these additional messages justified by some overview from operational experts ? Are these messages urgently missing from DO-219 and should they be integrated in future upgrades of FANS 1/A packages ? In many conferences, operational users from both the ground and the airborne communities constantly report that the FANS CPDLC data link is complex to use compared to the previous ACARS operations and that the major problem is the presentation of the information... Finally, the DO-219 recall attributes for uplinks are not present in CPDLC SARPS.

The defined structures for ATCdownlinkmessage in CPDLC SARPS and DO-219 are equivalent although their ASN.1 descriptions are slightly different. The constituent elements of ATCDownlinkMsgElementId in DO-219 were only provided a "response" attribute whereas CPDLC SARPS also associates an "alert" and "urgency" attribute. CPDLC SARPS adds some alternatives in the "CHOICES" for the constituent elements of ATCDownlinkMsgElementId from dM80 to dM113 and suppresses dM128NULL (reserved by DO-219 to ensure eight bit message element identification field).

Additionally. some data structures like AircraftEquipmentCode (not defined in DO-219) are not used in the CPDLC SARPS. Some basic data structures like "Altitude" in DO-219 and "Altitude"/"AltitudeType"/"AltitudeFlightLevel" in CPDLC SARPS do not completely match. Differences in boundaries can be observed sometimes like for the altitude in Flight Level unit (upper range of 600 in DO-219 and 700 in CPDLC SARPS) or in FL metric unit (2000 in DO-219 and 2500 in CPDLC SARPS). Some structures in CPDLC SARPS are more restrictive than in DO-219 like for example "temperature" for which the Fahrenheit choice has been suppressed.

#### 6) Comments on ads section

6.1 2.2.1.1.1.1 Note 2 and 2.2.1.9.1.1 Note 2: “It will be necessary for an implementation to provide information which is both accurate and timely in the ADS reports, however, quantification of the age and accuracy of the information is beyond the scope of 2:” It might be useful to clarify the reference “2” here: is it a reference to section 2 of the appendix, or to note 2, or to the ads section?

6.2 When ASN.1 is mentioned, it would be appropriate to provide the relevant references at least the first time, like ISO/IEC 8825-2 (OSI- Specification of Basic Encoding Rules for Abstract Syntax Notation One -Part 2: Packed encoding rules).

6.3 List of invalid references found in ads document, that correspond to the old reference numbering system of these SARPS:

- in 2.2.1.3.1.1, note 8, and in 2.2.2.3.1 note 8, reference to [SV1/ULA].

- in 2.2.1.4.1.1 and in 2.2.2.4.1, reference to [SV1/ASN].

Maybe the references list in slr should be numbered so that they can be easily referenced in other sections?

6.3 List of acronyms that are not explained the first time they are used in the ads document:

ETA (2.2.1.4.1), PDU (2.2.1.4.1), PER (2.2.1.6.1.1), RER (2.2.1.5.3.15.2.1), FU (2.2.1.8.1.1), RF(2.2.1.9.1.1)

6.4 Comparison to ADSP document (Draft ICAO Manual of ATS Data Link Applications, Preliminaries [3] and Part III, ADS [2], version 0.3 dated 29 March 1996)

6.4.1 Comment on [2].

There are some inconsistencies between the core text of [2] and its appendix 4, ADS Message Data Glossary:

- There are some types of event contracts that are specified by the SARPS and by appendix 4that are not mentioned in the core text of the ADSP document (1A.6.3): altitude range event, Way-Point change event, Airspeed event, Track angle change, Extended projected profile change.

- The list of optional ADS information in [2] 4.1.2.1 is not fully consistent with the list provided by appendix 4, which one is, however, consistent with the SARPs. Particularly, the core document of [2] mentions an “intermediate intent” information group.

6.4.2 [1] versus [2] (especially in [2]: Appendix A (ADS Message Data Glossary) and Appendix B (ADS  variables range and resolution)).

- In [2] 2.2.2, there is a requirement about the prioritization scheme for the handling of emergency, event/demand and periodic ADS reports. I am not sure that an equivalent requirement is stated by the SARPs ?

- The Aircraft Identification is referred to as being the Field 7 of the ICAO flight plan in ADSP (see [2] 4.1.2.2 and [2] appendix B 4B.1), and as the 24-bit ICAO identifier in the SARPs. I am not sure these two definitions are equivalent.

- There are many differences between the definitions, range and resolution of variables as they are defined in [2] appendix B and in [1]:

| Variable | Unit | Unit | Range | Range | Resolution | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
|  | ADSP | SARPs | ADSP | SARPs | ADSP | SARPs |
| Aircraft Id | IA5 | 24-bit ICAO id | 2-8 characters | N/A | N/A | N/A |
| Altitude | Alt. units | Alt. units | -10 +10000 | -10 +6640 | 1 | 1 |
| airspeed | *Mach *Speed Units | * Mach 0.001 * Speed Units | * 0.5 -4.0 * -110 -+4200 | * 0-4 * -110 - 4200 | * 0.001 * 1 | * 0.001 * 1 |
| date (year) | year of century | year/1996 | 0-99 | 1996-2095 | 1 | 1 |
| distance | dist units | NM | 1- 2000 | 1 - 8000 | 1 | 1 |
| FOM | Integer | Sequence |  |  |  |  |
| Lateral change | dist units | 0.125 NM | 1-200 | 0 - 31.875 NM | 1 |  |

I am still not sure of the status of ADSP material relatively to the SARPs, but it would be preferrable to have consistent data definitions between both documents, and also among the ATN documents (although I have not checked for consistency among the ATN documents themselves). A means of ensuring consistency would be to define this data in only one place, like a “data dictionary”, or to reference one set of documents like the documents related to ASN.1.

#### Emergency mode has been deleted from sv-1 definitions per configuration control sheets of ADS; however, emergency mode appears in ADS chapters 5 and 7, should be purged.

#### 7) Comment on fis section

List of acronyms that are not explained the first time they are used in the ads document:

SigMet (2.4.1.1.5), APDU (2.4.5.3.3), PER (2.4.6.1.1), IA5 (2.4.6.3.1), ATSC (2.4.6.2.2.3), SARPs (2.4.2.1.1)

#### 8) Comments on mhs section

8.1 In paragraph 3.1.2.3.4.2.1.2. on p24, should "clause referred" be substituted to "clausereferred" ?

8.2 What does "-eit" mean such as in "conversion-eits" in paragraph 3.1.2.3.4.3.3.5. ?

ORDescriptor is explained in paragraph 3.1.2.3.4.3.3.7 but is used earlier (Table 3.1.2-1. p7) and is not detailed in the glossary.

8.3 List of acronyms and terms that should be explained the first time they are used

- SOH (Start of Heading), LF (line feed), FT (Filling time), PRI (Priority prompt) are used in Table 3.1.2-2. on p12. Also, in Table 3.1.2-2 p12, does CR mean connection request as listed in the glossary or carriage return?

- MT-Elements (Message Transfer?) of Service is used for the first time in paragraph 3.1.2.2.1.1.2. and is not defined.

- EDI-messaging is used in Note 2 in paragraph 3.1.2.3.5.1.1. but is not defined.

- IA5RV is first used in paragraph 3.1.2.3.5.2.1.4. on p54 but is not defined.

- "oda" in "oda-body-part" is first used in Table 3.1.2-11. on p63 but is not defined.

- MT-EoS (Message Transfer Elements of Service) is first used in paragraph 3.1.2.3.5.2.4.2. on p65, but is not defined.

- D/X (Discarded/Excluded) is first used in paragraph 3.1.2.3.5.2.4.2. on p65, but is not defined.

- IPM (p3), PRL (note 8 p3), BER (paragraph 3.1.2.1.7. p5), nrn (Table 3.1.2-1. p7 Part 3 : AMH21/A.1.5 Common data types), ADS (paragraph 3.1.3.3.3.3.3. page 9), MTS (3.1.2.3.3.1.3. page 20), MD (should be detailed in paragraph 3.1.2.3.3.2. instead of paragraph 3.1.2.3.3.2.1), RN (to be explained on page 3 and not in paragraph 3.1.2.3.4.3.1.3. on p41), ASN.1 (to be detailed in paragraph 3.1.2.1.7. on p5 instead of paragraph 3.1.2.3.4.3.2.4. on p43)

# 8.4	Acronyms used in the core of the text and explained more than once (the following explanations should not be repeated)

ISPs (International Standardized Profiles) (note 9 p4 and note 2 a) p1)

XF-address should be explained in 3.1.2.1.5.1.4.1. and not in the following paragraph 3.1.2.1.5.1.4.2.

PRL is already detailed in paragraph 3.1.1 note 1 (p3) and should not be explained in paragraph 3.1.2.2.1.1.1. (p6).

IPM is already explained earlier in the text and the detail in paragraphs 3.1.2.2.1.1.1. note 3 (p6), 3.1.2.2.3.2. (p9) and Table 3.1.2-5. (p27) should be deleted.

IPN is already explained earlier in the text and the detail in Table 3.1.2-5. (p27) and paragraph 3.1.2.3.4.3.1.3. (p41) should be deleted.

MS is already detailed in paragraph 3.1.2.1.2.1.1. (p1) and the explanation should be deleted from Note 2 of paragraph 3.1.2.2.2.1.1. p9.

#### 9) Comments on aidc section

# 9.1 Acronyms used in the core of the text and not explained for their first occurrence

From IDC201.DOC file: AIDC (p1, IDC201.DOC file, Introduction), AIDC-AE, AIDC-ASE, ACSE (Association Control Service Element, Figure 3.2.1-1)

# 9.2	Abbreviations

The following abbreviations (used in the definition of primitives or variables) may be easier to understand with a local definition in the text.

From IDC204.DOC file: The following primitives are defined but the acronyms are not explained in the glossary: AIDC-ucf, AIDC-nfy, AIDC-crd-start, AIDC-crd-end, AIDC-crd-ngtt, AIDC-crd-stndby, AIDC-tfr-init, AIDC-tfr-rqst, AIDC-tfr-prpsl, AIDC-tfr-accept, AIDC-tfr-cntrl, AIDC-tfr-comm, AIDC-tfr-comm-assm, AIDC-inf-tfr, AIDC-end, AIDC-usr-abrt, AIDC-pvd-abrt

From IDC205.DOC file: atn-ac (ATN Accept ? from Table 3.2.0-1).

The following variables are only used in this document. Should the naming convention be explained in the glossary or should be taken as an identifier for the variable ? nfy-assc (notify association ?), crd-assc (coordination association ?

From IDC206.DOC file: AIDC-nfy-apdu, AIDC-crd-start-apdu, AIDC-crd-end-apdu, AIDC-crd-stndby-apdu, AIDC-tfr-init-apdu, AIDC-tfr-rqst-apdu, AIDC-tfr-prpsl-apdu, AIDC-tfr-accept-apdu, AIDC-tfr-comm-assm, AIDC-tfr-cntrl-Req-apdu, AIDC-tfr-cntrl-Rsp-apdu, AIDC-inf-tfr, AIDC-inf-tfr-apdu, AIDC-ucf-apdu, AIDC-tfr-end-apdu, AIDC-end-apdu, TBD (To be defined; Table 3.2.6-1), Req (Request; Table 3.2.6-1), Ind (Indication; Table 3.2.6-1), +ve (?; Table 3.2.6-1), -ve (?; Table 3.2.6-1), rcv (State Table)

Should the following variables be explained in the glossary ? trns-prpsl, coord-start, coord-stndby, trns-comm, trns-assm, trnsf-start or trns-start, trns-accept, trns-reject, coord-negot, coord-end, trns-init

# 9.3 General remarks

The modelling conventions for the primitives and the associated parameters (see paragraph 1 p2 in IDC201.DOC file) are repeated in all the documents describing the applications (such as CPDLC, ...).

The reference to the Upper Layer Architecture ([4]) mentionned in IDC202.DOC file is not specifically defined.

The definitions of the parameters for the AIDC-AE ABSTRACT SERVICE (see IDC203.DOC file) should all reference the ASN.1 abstract syntax like MessageNumber instead of detailing the definition like for the "Called ICAO Facility Designator parameter" defined as the "eight letter ICAO facility designator". Moreover, such data types common accross all the applications should only be defined once and referenced by the other documents. Ideally, a data dictionary would ensure overall consistency which is critical for interoperability of the various applications. Any change in these data types would gratly impact all the applications.

I could not find the primitive "AIDC-usr-abort"of IDC203.DOC file in any of the other ICC files. Does it correspond to "AIDC-usr-abrt request" referenced in other ICC documents ?

#### File IDC203.DOC (p18) mentions the "Message Number" parameter, which is provided by the AIDC-ASE user except when invoking the AIDC-usr-abort or AIDC-ucf request primitives and conforms to the ASN.1 abstract syntax MessageNumber, whereas the other ICC files mention "Msg Number". Then, in IDC205.DOC file (p32), for the Transfer-initiate Request Primitive", the CF shall invoke an AIDC-tfr-init request primitive with the Transfer-initiate request primitive Message Number parameter as the AIDC-tfr-init request primitive Msg Number parameter".

AIDC-info-tfr Request Primitive (see IDC206.DOC file p60) references both AIDC-info-tfr-apdu and AIDC-inf-tfr-apdu, whereas the rest of the document mentions AIDC-inf-tfr-apdu.

File IDC206.DOC mentions twice "trnsf-start" and uses "trns-start" in other instances. Are these variables identical ?

#### How do the "{" close with "}" in the if clauses work in the State Event Table for example for AIDC-ucf REQ ?

#### 10) Comment on ula section: list of acronyms that should be explained the first time they are used:

#### OSI, ADS, CM, CPDLC, FIS(ATIS), MHS, PRL, PER, M, U, C, ATSC, IATA,ADV,ACPM, AARQ, TSAP, CLNP, SPDU, SCN, SAC, SRF, SPM, PSAP, TPDU, TS, PPM, PPDU, PCI, AARE, ITU, PRL, FU, HD, CDSE, EX, FD.

#### 11) Comments on ics  section

# 11.1 Redundant definitions

The following expressions are defined several times: End Systems (ESs), ATSC, AINSC, Boundary Intermediate Systems (BISs), Inter-Domain Routing Protocol (IDRP),  Subnetwork Independent Convergence Function (SNICF),  Subnetwork Access Protocol (SNAcP)

# 11.2 General remarks

In Chapter 3, shouldn't "compliancy" be replaced by "compliance" (This chapter is not concerned with compliancy with ...)? Shouldn't Mobile Rds be replaced by Mobile RDS ?

Some local definitions of variables and services might clarify the text for: Adj-RIB-In, RD_HOP_COUNT, RIB-Att, Adj-RIB-Out, Loc-RIB, aLloc-RIB ?, DIST_LIST_INCL and DIST_LIST_EXCL

12) List of acronyms

Legend

-	Indicates proposal to include acronym definition within text the first time it is used or it may not be used.  No impact on master definitions section.

* 	Indicates that the acronym is defined within the definitions section, but appears together with the spelled-out term that is the line entry.

** 	Indicates that the acronym has a specific entry within the definitions section with the phrase, “The symbol used to designate [spelled out term].  The spelled out term also appears in the definition section with a full definition of the term.

?- 	What does the acronym mean?.

nothing - indicates not reviewed.

| Term | Where used | Description |
| --- | --- | --- |
| -A-FU | ULA | Authentication Functional Unit |
| -A-G | ICS | Air-ground |
| -AA | ULA | Abort Accept |
| *AAC | SLR, ICS | Aeronautical Administrative Communications |
| -AARE | ULA | ACSE Associate Response APDU |
| -AARQ | ULA | ACSE Associate Request APDU |
| -AB | FIS, ADS | Abort |
| -ABRT | ULA | ACSE Abort APDU |
| -AC | ULA | Accept |
| -ACA | ICS | Address compression algorithm |
| *ACAS | ADS | Airborne Collision Avoidance System |
| -ACPM | ULA, ICC | ACSE Protocol Machine |
| *ACSE | ULA | Application control service element |
| -ACT | ULA | Activity Management |
| -ADM | ICS | Administrative identifier |
| -ADMD | MHS | Administration management domain |
| -ADMF | ICS | ADM Flag |
| **ADS | SLR, ADS, CM, MHS, ULA, ICS, P1 | Automatic Dependent Surveillance |
| *ADS-AE | ADS | ADS Application Entity |
| *ADS-CF | ADS | ADS Control Function |
| -ADS-RF-AE | ADS |  |
| -ADS-RF-ASE | ADS |  |
| -ADS-RF-CF | ADS |  |
| *AE | SLR, ADS, CM, MHS, ULA, ICS, P1, CPDLC, ICC | Application Entity |
| *AE-Qualifier | ULA | Application Entity Qualifier |
| *AE-title | ULA | Application Entity title |
| *AF-Address | MHS | AFTN-form address |
| -AFI | ICS | Authority and format identifier |
| **AFTN | SLR, MHS, ULA, P1 | Aeronautical fixed telecommunication network |
| **AIDC | ICC, SLR | ATS Interfacility Data Communications |
| -AIDC-AE | ICC | AIDC Application Entity |
| -AIDC-ASE | ICC | AIDC Application Service Element |
| -AINSC | ICS | Aeronautical Industry Service Communication |
| -AK | ADS, CPDLC, MHS, ICS | Data acknowledgement |
| *ALS |  |  |
| **AMHS | SLR, ULA, P1 | Automatic message handling system |
| -AMSS | ICS | Aeronautical mobile satellite service |
| -ANC | ICS | Air Navigation Commission |
| *AOC | ULA, ICS, SLR | 1. Aeronautical Operational Communications 2. Aeronautical Operational Control |
| *AP | CPDLC, MHS ULA, ICC | Application process |
| -AP title | ULA | Application process title |
| *APC | ICS | Aeronautical Passenger Communications |
| *APDU | ULA | Application Protocol Data Unit |
| -API | ULA | Application Program Interface |
| -App | ULA | Application |
| *APRL | ICS, ICC | ATN profile requirements list |
| -ARP | ULA | Abnormal Release Provider |
| -ARS | CM, ICS | Administrative Region Selector |
| -ARSD | ICS | ARS Default [Flag] |
| -ARU | ULA | Abnormal Release User |
| *ASE | ADS, CM, CPDLC, FIS, ULA, P1, ICC | Application Service Element |
| *ASN.1 | ADS, CM, CPDLC, FIS, MHS, ULA, ICS, ICC | Abstract Syntax Notation One |
| *ASO | ULA, P1 | Application Service Object |
| *ATC | SLR, ADS, CPDLC, FIS, ICS, P1, ICC | Air Traffic Control |
| **ATIS | SLR, CPDLC, FIS, ULA, P1 | Automatic Terminal Information Service |
| *ATM | SLR, ADS, CM, CPDLC, FIS, MHS, ULA, ICS, P1 | Air Traffic Management |
| **ATN | SLR, ADS. CM, FIS, MHS, ULA, ICS, P1, ICC | Aeronautical Telecommunication Network |
| -ATN App | ULA, P1 | A generic name for an ATN application. |
| -ATN App AE | ULA | A generic name for an ATN application application entity |
| -ATN App ASE | ULA | A generic name for an ATN application application service interface |
| *ATNI | ULA | ATN internet |
| *ATNPA | ICS | ATN presentation address |
| *ATNSM |  | ATN systems management |
| **ATS | SLR, CM, CPDLC, MHS, ULA, ICS, P1, ICC | Air Traffic Services |
| **ATSC | SLR, ADS, ULA | Air Traffic Services Communications |
| **ATS MHS | MHS, P1 | ATS Message Handling Service |
| *ATSU (ATS Unit) | CPDLC, ICC | Air Traffic Services Unit |
| -AU | MHS, ULA, ICS | Access unit |
| -BCD | ICS | Binary Coded Decimal |
| -BER | MHS | Basic Encoding Rules (of ASN.1) |
| *BIS | ICS | Boundary intermediate system |
| -BISPDU | ICS | BIS PDU |
| -C | FIS, ADS, ULA, CM, MHS | Conditional |
| -CAN | ICS | Cancellation |
| *C-ATSU | ICC | Controlling ATSU |
| -CC | ICS | Connection confirm |
| -CCITT | MHS, ICS | International Telegraph and Telephone Consultative Committee |
| -CDO | ULA | Connect Data Overflow |
| -CDSE | ULA | Confirmed Data Service Element |
| -CDT | ICS | Credit |
| -CE | ICS | Congestion experienced flag |
| *CF | ADS, CM, CPDLC, FIS, MHS, ULA, ICC | Control Function |
| **CIDIN | ICS | Common ICAO data interchange network |
| -CL | ICS | Connectionless mode |
| -CL | FIS | Cancel Contract |
| -CLNP | SLR, ULA, ICS, ICC | CL network protocol |
| -CLNS | ICS | CL network service |
| -CLTP | ICS | CL transport protocol |
| **CM | SLR, CM, ULA, ICS, ICC | Context Management |
| -CM-AE | CM | CM-Application Entity |
| -CM-ASE | CM | CM-Application Service Element |
| -CNLP | ICS | Connectionless Network Protocol |
| -CN | ULA | Connect |
| -Cnf | ULA | Confirm |
| -CNS | SLR, ADS, CM, FIS, MHS, ULA, P1 | Communications, Navigation, and Surveillance |
| -CNS/ATM | SLR, CM, FIS, MHS, ULA | Communications Navigation Surveillance / Air Traffic Management |
| -CO | ICS | Connection mode |
| -COTP | ICS | 1. CO transport protocol 2. Connection-oriented transport protocol |
| -CP | ULA, ICS | Connect Presentation PPDU |
| -CPA | ULA | Connect Presentation Accept PPDU |
| *CPC | CPDLC, ULA | Controller-Pilot Communications |
| **CPDLC | SLR, CM, CPDLC, ULA, ICS, P1 | Controller-Pilot Data Link Communications |
| -CPR | ULA | Connect Presentation Reject PPDU |
| -CR | ADS, MHS, ULA, ICS | 1. Connection request 2. Context Restoration |
| -CVER | ICS | Compressed VER |
| -DAM | ULA | Draft Amendment |
| *D-ATSU | ICC | Downstream ATSU |
| *DC | ADS, FIS, ICS | Demand Contract |
| -DCE | ICS | Data circuit terminating equipment |
| *DCPC |  | Direct controller pilot communications. |
| -DIR | MHS | Use of Directory |
| -DL | MHS | Distribution List |
| -DN | ULA | Disconnect |
| -DR | ULA, ICS | Disconnect request |
| *DS | ADS, CM, CPDLC, FIS, MHS, ULA | Dialogue Service |
| **DSC | CPDLC | Downstream clearance |
| *DSP | FIS, ICS | Domain specific part |
| -DT | ULA, ICS | 1. Data 2. Data Transfer SPDU |
| -DTE | ICS | Data terminal equipment |
| -E/C | ICS | Error probability over cost flag |
| -E/R | ICS | Error report requested |
| -E/T | ICS | Error probability over transit delay flag |
| -EA | ICS | ED acknowledge |
| -ED | ICS | Expedited data |
| *EM | ADS | Emergency Mode |
| -EoS | MHS | Element of Service |
| -ER | CM, ICS | 1. Error [TPDU] 2. Error report [NPDU] 3. Error report requested flag |
| *ERD | ICS | End routing domain |
| -ERP | ICS | Echo Response [NPDU] |
| -ERQ | ICS | Echo Request [NPDU] |
| *ES | SLR, ICS, P1 | End System |
| -ES-IS | ICS | End-System to Intermediate System |
| *ETA | ADS | Estimated Time of Arrival |
| -EX | ULA | Expedited Data SPDU |
| -EXCEP | ULA | Exceptions |
| -EXP | ULA, ICS | LOCREF extension flag |
| -F/M | ICS | Fixed/Mobile |
| -FANS | ICS | Future Air Navigation Systems |
| -FD | ULA | Full Duplex |
| -FG | MHS | Functional Group |
| *FIB | ICS | Forwarding information base |
| *FIR | ICC | Flight Information Region |
| **FIS | SLR, CM, FIS, ULA, ICS, P1 | Flight Information Services |
| -FIS-AE | FIS | FIS Application Entity |
| -FIS-ASE | FIS | The FIS-air-ASE and the FIS-ground-ASE. |
| -FIS-user | FIS | The FIS-air-user or the FIS-ground-user. |
| *Fixed ATN RDC | ICS | The Fixed ATN RDC consists of all ground-based RDs that form the ATN. |
| *flight id | ADS, CM, ULA | An identifier, to an ICAO approved format, for a particular flight. |
| *FOM | ADS | Figure Of Merit |
| -FP | ICS | Full/Prefix |
| -FSM | ICS | Finite state machine |
| -FU | ADS, CPDLC, FIS, ULA, ICS | Functional Unit |
| -GA | MHS | General Aviation |
| *GES | ICS | Ground earth station |
| -GT | ULA | Give Tokens SPDU |
| *GWA | ULA | Type A Gateway (See AFTN/ATN Type A gateway) |
| -HD | ULA | Half Duplex |
| *HF | ICS | High Frequency |
| -HI | ADS, FIS | High Interface |
| -IA5 | ADS, CM, CPDLC, FIS, MHS, ULA | International Alphabet Number 5 |
| -IATA | ULA, ICS | International Air Transport Association |
| -ICAO | SLR, ADS, CM, CPDLC, FIS, MHS, ULA, ICS, P1, ICC | International Civil Aviation Organization |
| **ICC | SLR, ULA, ICS, P1 | 1. Inter-Centre Coordination 2. Inter-Centre Communications |
| -ICD | ULA, ICS | International code designator |
| **ICS |  | Internet communication services |
| -ID | ADS, CM, CPDLC, MHS, ULA | 1. Identification 2. Identifier |
| -IDC | ICC | ? |
| -IDI | ICS | Initial domain identifier |
| -IDP | CM, ICS | Initial Domain Part |
| -IDRP | ICS | Interdomain routing routing protocol |
| -Ind | ULA | Indication |
| *IP | ULA, ICS | Internetwork protocol |
| -IPI | ICS | Initial protocol identifier |
| -IPM | MHS | Interpersonal message |
| -IPMS |  | Interpersonal Messaging System |
| -IPN | MHS | Interpersonal notification |
| *IS | SLR, ADS, CM, MHS, ICS, P1 | Intermediate system |
| -IS-SME | ICS | IS SME |
| -ISDN | ICS | Integrated Services Digital Network |
| -ISH | ICS | IS hello |
| **ISO | SLR, MHS, ULA, ICS, P1 | International Organization for Standardization (Note that definition of spelled out term is not provided for this acronym) |
| -ISP | MHS | International standardized profile |
| -ISPICS | MHS | Intermediate System Profile Implementation Conformance Statement |
| -ITA-2 | MHS | International Telegraph Alphabet No. 2 |
| -ITU | MHS, ULA, ICS | International Telecommunication Union |
| **ITU-T | MHS, ICS | ITU Telecommunication Standardization Sector (Note that definition of spelled out term is not provided for this acronym) |
| -IUT | ICS | Implementation under test |
| -LI | ADS, FIS | 1. Low Interface 2. Length indicator |
| -LOC | CM, ICS | Location Identifier |
| -LOCREF | ICS |  |
| -LOCD | ICS | LOC Default [Flag] |
| -M | FIS, ADS, ULA, CM | Mandatory |
| *MD | MHS, ICS | Management Domain |
| -MD4 | ICS | Message Digest Algorithm |
| *MF-Address | MHS | MHS-form address |
| -MO | ICS |  |
| *Mode S | ULA, ICS | Mode Select |
| -MS | MHS, ICS | 1. More segments flag 2. Message store |
| -MTA | MHS | Message transfer agent |
| -MTS | MHS | Message Transfer System |
| -N/A | SLR, CM, CPCLC, FIS, ULA | Not applicable |
| *NE | ULA | Network entity |
| *NET | ULA, ICS | NE title |
| *NL |  | Network layer |
| -NLPI | ICS | NL protocol information |
| -NLRI | ICS | NL reachability information |
| -NLSP | ICS | NL security protocol |
| *NM | ICS | Network management |
| -No | ICC | Number |
| -NOR | ICC, ULA | No orderly release |
| *NOTAM | FIS | Notice to Airmen |
| -NPDU | ICS | Network protocol data unit |
| -NR | ULA | Negotiated Release |
| -NRN | MHS | Non-Receipt Notification |
| -NS | ULA, ICS | Network service |
| *NSAP | ULA, ICS | Network Service Access Point |
| -NSDU | ICS | NS data unit |
| -O | FIS | Optional |
| -O/R | MHS | Originator/recipient |
| -OA | ADS, ULA | Overflow Accept |
| -OHI | MHS | Optional Heading Information |
| -OID | ULA | Object Identifier |
| -OR | MHS, ICS | Operational Requirement |
| **OSI | P1, SLR, ULA, ICS | Open Systems Interconnection |
| -OSIE | ICS | OSI environment |
| -P | ULA | Provider |
| *P | ADS, ULA, FIS, CM, CPDLC, SLR | Priority |
| *PC | ADS | Periodic Contract |
| *PCI | ULA | Protocol control information |
| -PD | ULA | Presentation Data |
| -PDAI | MHS | Predetermined address indicator |
| *PDU | ADS, CM, CPDLC, FIS, ICC, ULA, ICS | Protocol Data Unit |
| *PDV | ULA | Presentation Data Value |
| -PER | ADS, CM, CPDLC, FIS, ICC, ULA | Packed Encoding Rules (of ASN.1) |
| *PICS | ULA, ICS | Protocol implementation conformance statement |
| -PIREP | FIS | Pilot Report |
| -PPDU | ULA | Presentation PDU |
| -PPM | ULA | Presentation Protocol Mechanism |
| -PR | ICC, ULA | Prepare |
| -PRL | MHS, ULA, ICS | Profile Requirements List |
| -PRMD | MHS | Private management domain |
| -PSAP | MHS, ULA | Presentation service access point |
| -PSDN | ICS | Packet switched data network |
| -PT | ULA | Please Tokens |
| *QoS | ADS, CM, CPDLC, FIS, MHS, ULA, ICS | Quality of Service |
| *R-ATSU | ICC | Receiving ATSU |
| *RD | ICS | Routing domain |
| *RDC | ICS | Routing domain confederation |
| -RDF | ICS | Routing domain format |
| *RDI | ICS | Routing domain identifier |
| -RDP | CM | Router Domain Part (How different from DSP) |
| -RED | MHS | Redirection |
| -REL | ULA | Release |
| -Req | ULA | Request |
| *RER | ADS, CM, CPDLC, FIS, ICC, MHS, ULA, ICS | Residual Error Rate |
| -RESYNC | ULA | Resynchronisation |
| -RF | ADS, CPDLC,ULA | 1. Radio frequency 2. Refuse |
| -RFC | ICS | Request for comments |
| *RIB | ICS | Routing information base |
| -RL | ICC | Requirements list |
| -RLRE | ULA | ACSE Release Response APDU |
| -RLRQ | ULA | ACSE Release Request APDU |
| -RLS | ULA | Release |
| -RN | MHS | Receipt Notification |
| -ROA | ICS | Request of Acknowledgement |
| -Rsp | ULA | Response |
| -RTSE | MHS | Reliable transfer service element |
| *RVR | FIS | Runway Visibility Range |
| -S/T | ICS | Segmentation over transit delay flag |
| -SAC | ULA | Short Accept |
| -SACC | ULA | Short Accept Continue |
| -SARPs | P1, SLR, ADS, CM, CPDLC, FIS, ICC, MHS, ULA, ICS | Standards and Recommended Practices |
| -SCN | ULA | Short Connect |
| -SCNC | ULA | Short Connect Continue |
| -SDT | ULA | Short Data Transfer |
| *SDU | ICS | Service data unit |
| -SEL | CM, MHS, ICS | (Transport) Selector |
| -SFN | ULA | Short Finish |
| *Short TSAP | CM, ULA | Composed of the ARS, the LOC, the SYS, and the SEL. |
| -SHORT-CP | ULA | Short Connect PPDU |
| -SHORT-CPA | ULA | Short Connect Accept PPDU |
| -SHORT-CPR | ULA | Short Connect Reject PPDU |
| -SICASP | ICS | SSR Improvements and Collision Avoidance Systems Panel |
| -SigMet | FIS | Significant Meteorological information |
| *SM | ULA, ICS | Systems management |
| -SMA | ULA | SM application |
| -SME | ICS | SM entity |
| *SN | ADS, ICS | Subnetwork |
| -SN-SME | ICS | SN SME |
| ?SNAcF | ICS | SN access function |
| *SNAcP | ICS | SN access protocol |
| ?SNCR | ICS | SN connection reference |
| *SNDCF | ICS | SN dependent convergence function |
| ?SNDCP | ICS | SN dependent convergence protocol |
| *SNICF | ICS | SN independent convergence function |
| ?SNICP | ICS | SN independent convergence protocol |
| *SNPA | ICS | SN point of attachment |
| -SNQOS | ICS | SN QoS |
| -SNS | ICS | SN service |
| -SNSDU | ICS | SN Service Data Unit |
| -SP | ICS | 1. SN Processor 2. Segmentation permitted flag |
| -SPDU | ULA, ICS | Session PDU |
| -SPI | ICS | Subsequent protocol identifier |
| -SPM | ULA | Session Protocol Machine |
| -SRF | ULA | Short Refuse |
| -SRFC | ULA | Short Refuse Continue |
| -SS | MHS | 1. Symmetric Synchronise 2. Session Service |
| *SSR | CPDLC, ICS, ULA | Secondary surveillance radar |
| -SY | ULA | Minor Synchronise |
| -SYS | CM, ICS | System Identifier |
| -SYS4 | ICS | SYS 4th Octet [Flag] |
| -SYS5 | ICS | SYS 5th Octet [Flag] |
| -SYS6 | ICS | SYS 6th Octet [Flag] |
| -T/C | ICS | Transit delay over cost flag |
| -TC | ULA, ICS | Transport connection |
| -TD | ULA | Typed Data |
| -TE | ICS | Transport entity |
| -TPDU | ICS, ULA | Transport protocol data unit |
| -TR | ICS | Technical report |
| *TRD | ICS | Transit routing domain |
| -TS | ULA, ICS | Transport service |
| -TS-user | ICC | Transport service user |
| *TSAP | CM, MHS, ULA, ICS, ICC | Transport Service Access Point |
| -TSDU | ICS | TS data unit |
| -TWS | FIS | Terminal Weather Service |
| -U | FIS, ADS, ULA, CM | User option |
| ?UA | MHS | User agent |
| *UC | FIS | Update Contract |
| *UL | ULA | Upper Layer |
| **UTC | SLR, MHS | Co-ordinated Universal Time |
| *VDL | ICS | VHF data link |
| -VER | ICS | Version |
| *VHF | CPDLC, ULA, ICS | Very high frequency |
| -WAN | ICS | Wide area network |
| -X | ICS | Hexadecimal |
| *XF-Address | MHS | Translated address |