ATNP/WG 1

WP 6-
	19 July 1996

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 1

Halifax, Nova Scotia, Canada

30. 7. - 2. 8. 1996

Proposed ATNP/2 Working Paper Summarising the

ATN Naming, Addressing and Registration Concept

Presented by K. Platz

Prepared by K.-P. Graf

SUMMARY

This paper presents in Appendix A a draft working paper which introduces and summarises the WG 1 deliverable ATN Naming, Addressing and Registration Concept. This draft working paper is proposed for presentation and discussion during agenda item 4 of the second meeting of ATNP in the context of the consideration of addressing issues involved in the implementation and operation of the ATN. The ATN Naming, Addressing and Registration Concept in its final version, revised and updated as agreed during the last WG 1 meeting, is attached to this draft working paper as Appendix B.

The working group is invited to review the proposed draft working paper for ATNP/2, amend it, as appropriate, and agree its submission to ATNP/2 as a WG 1 paper.

Appendix A	ATNP/2-WP/

24/7/96

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

SECOND MEETING

Montreal, 4 to 15 November 1996

AGENDA ITEM 4: Consideration of system management, security, system performance, addressing, and institutional issues involved in the further development, planning, implementation, and operation of the ATN

ATN Naming, Addressing and Registration Concept Summary

Working Paper

(DRAFT)

(Presented by Tom Calow, Rapporteur WG 1)

SUMMARY

This working paper provides a summary of the ATN Naming, Addressing and Registration Concept which has been developed by WG 1 as part of its work programme for ATNP/2. It presents the outline, main issues and status of this concept and provides a number of recommendations resulting from the WG 1 work on ATN naming, addressing and registration aspects since ATNP/1.

References:

[1]	ATN Naming, Addressing and Registration Concept, Version 2.0, 24 July 1996

1.	Introduction

In its first meeting ATNP WG 1 identified the deliverable WG1-16 ATN Naming, Addressing and Registration Concept as part of its work programme for ATNP/2. This deliverable has been produced for the second meeting of WG 1 and has been discussed and reviewed by subsequent meetings of WG 1. This paper summarises the outline and the main issues of the ATN Naming, Addressing and Registration Concept; it presents the status of this WG 1 deliverable, and provides a number of recommendations resulting from the WG 1 work on ATN naming, addressing and registration aspects since ATNP/1.

2.	Discussion

2.1	Scope and Purpose

The issue of naming and addressing of information objects in the ATN spans across the whole communications stack, stretching from the subnetwork specific address of the local point of attachment of an ATN system to a given transmission system to the globally unambiguous names used for the identification of ATN applications. This cross-sectional aspect of naming and addressing was the basic motivation for the development of a general framework and concept document relating to the naming, addressing and registration of information objects in the ATN. The resulting ATN Naming, Addressing and Registration Concept aimed at supporting and steering the detailed definition of the ATN naming, addressing and registration issues across the three ATNP working groups. As additional longer-term objectives, the ATN Naming, Addressing and Registration Concept is intended to serve as

r	a concise overview and compilation of the individual ATN elements which are subject to the assignment of names and/or addresses for the CNS/ATM-1 Package, as well as the general concepts and principles underlying the definition and maintenance of ATN addressing plans and the administration and registration of ATN names and addresses;

r	self-contained and stand-alone guidance material on the ATN naming, addressing and registration issues relevant for the CNS/ATM-1 Package;

r	a baseline for the refinement and further specification of the ATN naming, addressing and registration provisions required for the successful world-wide development and operational use of ATN systems and services beyond the CNS/ATM-1 Package time frame.

2.2	Outline

The ATN Naming, Addressing and Registration Concept is organised into six major sections with the following outline:

Section 1 (Introduction) presents the scope and purpose of the document, gives an overview of the document and contains the reference list and glossary.

Section 2 (Naming and Addressing Principles and Concepts) describes the general principles and features pertinent to ATN addressing and naming and introduces the concepts of OSI names and addresses as well as naming and addressing domains and authorities.

Section 3 (ATN Elements Subject to Naming and Addressing) presents the various ATN objects which require unambiguous identification and location throughout the ATN and outlines the generic ATN names or addresses which have been associated with these objects.

Section 4 (ATN Address and Name Definitions) provides a detailed compilation of the structure and semantics of the individual ATN addresses and names, including subnetwork addresses, internetwork addresses, transport addresses, upper layer and application addresses and application names. Furthermore, this section provides a cross-reference to the relevant ATN SARPs sections and ISO/IEC standards containing the detailed addressing and naming specifications.

Section 5 (ATN Name and Address Registration) outlines the general procedures for name and address registration and discusses the scope and status of registration of addresses and names in the ATN.

Section 6 (Use of Context Management as Basic Directory Service) introduces the concept of a basic name and address directory service for use by ATSC applications distributed between aircraft and ground-based systems, and explains the resolvement of fragmented address and name information by this service.

2.3	Major Issues

ATN naming, addressing and registration involves the

r	unambiguous identification of ATN elements, such as ATN applications, ATN service users, and ATN layer entities;

r	assignment of names and addresses to these information objects to achieve this identification;

r	recording of definitions of the instances to which those names and addresses are assigned;

r	propagation of registered names and addresses both on a global level and on a local level; and

r	interpretation and use of these names and addresses during ATN operation, e.g. by ATN routing algorithms and protocols, or ATN directory services.

2.3.1	ATN Addressing Plans

The unambiguous identification of ATN elements is achieved through the specification of well-defined addressing plans for the ATN. An ATN addressing plan exists for network, transport, and application addresses respectively [1]. These addressing plans define the structure, syntax and semantics of the individual ATN addresses with regard to the needs of a variety of aeronautical data communication user groups, including ATS providers, airlines and international aeronautical communication service providers. Furthermore, the ATN addressing plans support essential goals of ATN internal operation, such as efficient information reduction when exchanging address information and unambiguous and complete address reconstruction from received address fragments (in the context management application).

2.3.2		ATN Address Domains and Authorities

In order to facilitate the address assignment in the ATN, the overall ATN address space is partioned into a set of hierarchically structured address domains in a recursive fashion. Each address domain consists of a set of address formats and values which are administered and assigned by a single addressing authority. Each addressing authority is responsible for its own address domain, and may further partition it into several subordinate sub-domains, and delegate authority for these sub-domains. This principle of hierarchically structured ATN address domains and associated addressing authorities allows to de-centralise the assignment of ATN addresses with clearly identified responsibilities without the need for co-ordination between authorities in the process of address assignment.

The hierarchical address domain approach is reflected in the ATN address structure in the form of an ordered sequence of individual address fields whereby each field is associated with an address domain and addressing authority. The general philosophy that is underlying the assignment of ATN addresses is that the higher order field values of an ATN address are assigned on the international level, through international addressing authorities such as ISO, ICAO and IATA, whereas the responsibility for the administration and assignment of the lower order address parts is delegated to the regional, national and local level. Consequently, ICAO Regions, States and aeronautical organisations are expected to assume full responsibility and administrative duties related to their address space, i.e. establish the necessary administrative structure to perform assignment activities for ATN addresses.

2.3.3		ATN Address Registration

In order to ensure unambiguous assignments within the context or environment of their use and to make these assignments available to interested parties, ATN addresses and names must be registered by ATN address registration authorities. Name/address registration includes activities such as

r	receiving proposals for register entries from applicants;

r	processing received proposals for register entries;

r	recording names/addresses for each register entry that is accepted in accordance with the specified procedures for a register entry;

r	conveying the results of the registration, i.e. the decision taken on the proposal, to the appropriate applicant; and

r	promulgating the register entries within its sphere of responsibility.

The address registration function for the higher order fields of ATN addresses has already been performed to a large extent in parallel to the development of the ATN SARPs. As a result of this, the values of the ATN address prefixes for ATSC systems are registered with ISO and ICAO and use either existing, already established international registers or are published in the ATN SARPs. In this sense, the ATN SARPs constitute an international register for (portions of) ATN addresses.

However, following the principle of delegated responsibility for certain name and address portions, States and aeronautical organisations are expected to set up name and address registers  for the ATN deployment and carry out registration activities.

3.	Status

The deliverable WG1-16 ATN Naming, Addressing and Registration Concept was produced in its initial version for the second meeting of WG 1 at Toulouse and was subsequently presented for review to the WG 1 meetings in Banff, Brisbane and Munich.

Between each review cycle, it was updated and progressed along the comments and changes agreed at these meetings and in close co-ordination with the evolution of the detailed technical addressing and naming provisions developed by the ATNP working groups 2 and 3.

The final version of the deliverable was made available at the sixth WG 1 meeting in Halifax were the ATN Naming, Addressing and Registration Concept was approved by WG 1.

4.	Proposal

The meeting is invited to

a)	accept the ATN Naming, Addressing and Registration Concept as developed and approved by Working Group 1;

b)	publish the ATN Naming, Addressing and Registration Concept as attachment to the report of the second meeting of the ATN Panel, in order to make this material available to implementors of the ATN addressing and naming provisions, and to States and organisations responsible for the assignment and registration of ATN addresses and names;

c)	encourage ICAO to advise ICAO Regions and States to establish the necessary administrative structure and procedures which are required to assume full responsibility and administrative duties related to the assignment, administration, and registration of ATN addresses and names within their allocated name/address space;

d)	inform ICAO on the need for a facility that maintains an up-to-date repository of ATN addresses and names registered in the ATSC addressing domain and publishes the repository entries according to a defined schedule;

e)	include further work on ATN naming, addressing and registration issues beyond the CNS/ATM-1 Package into the work programme of the ATN Panel.