W1S2F1702

Discussion on CA Hierarchy

> Our understanding is that a State may run their own CA service or may

> designate an entity as their CA.  Also, a group of States may designate one

> CA for all the States.  For example, all States within the EU may designate

> Eurocontrol as their CA.

I agree. In fact I would add that States are encouraged to share a CA to minimize the total number of CAs and therefore lower the overhead of cross-certification.

> In the scenario where one CA acts on behalf of multiple States, who is the

> issuer of the Certificate - is it the CA (e.g. Eurocontrol) or the issuer is

> still the independent State (e.g. Eurocontrol for UK)?  If the latter is

> true, then the CA should maintain a separate key-pair for each State it is

> responsible for.  Also, Each State has to cross-certify the other State

> although a mutual trust exists through the selection of a common CA.

Okay. As I see it, each entity will maintain a database of State <-> State CA pairs.

If State_1 and State_2 both assign CA_X to act as their State CA, then this database will contain the entries:

(State_1 assigned CA_X)

(State_2 assigned CA_X)

CA_X will now issue certificates to all relevant entities within State_1 and State_2 and will therefore be identified in the issuer field of all these certificates.  CA_X will have a single key pair it uses to sign all the certs and CRLs it issues.

CA_X will cross-certify only with other State CAs. Thus if in addition State_3 and State_4 both assign CA_Y to act as their State CA, then there will only be two cross-certificates ... one from CA_X to CA_Y and one from CA_Y to CA_X.  Again the idea here is to minimze the cross-certification overhead.

> If the former is true, then the number of cross-certification can be reduced

> drastically whereas only the regional CAs needs to cross certify each other.

> Also, only a small number of key-pairs (one per regional CA) needs to be

> maintained in this case.

Right ... I think this is what I was trying to describe above :-) I should read all the email before typing :-)

> The issue in this case is the validation of

> certificate path.  If the issuer name in the certificate contains the name

> of the CA (e.g. Eurocontrol), how would the aircraft know that the CM

> certificate for UK is issued legitimately by Eurocontrol?  (A database

> maintained in the aircraft listing valid CA for each state will eliminate

> this problem if the issuer name is omitted from the compressed certificate

> and aircraft inserts the issuer field from its database before

> authenticating the certificate.)

Right ... the aircraft will know that Eurocontrol issues certificates to UK (and German and French) ATN entities by looking in its database.

As you say the issuer field could be omitted from the compressed certificate IF the aircraft knows in advance which CA to expect ... perhaps based on its location. However I guess it seems simpler to me to include the issuer name in the compressed certificate and instead indicate to the user that he’s flying over somewhere whose CA is Eurocontrol.

> This relates to the Flimsy from you that

> discussed a valid CM1 issuing certificate for application X on which it has

> no authority over.  We understand and agree that the responsibility and

> accountability for the CA will reside with the State although a group of

> State may designate one specific CA.

Right ... since CA_X issues certificates to ATN entities in State_1 and State_2 you don’t get quite the granularity of responsibility control you get if State_1 and State_2 assigned different CAs. This is the trade off ... it probably means the US and Syria are unlikely to share a CA but hopefully will not prevent the US and Canada sharing a CA.

> Finally, how would the aircraft know

> that the certificate received is for the State entity it is interested in.

> Would the State ID be part of the designated name or AE title?

The only check I have envisioned was “This certificate is issued by Eurocontrol which is designated to act as CA for everyone in Europe and the user appears to be happy that I’m flying over Europe.

> Also, a related issue is:  If a State designates a CA, can that CA be a

> subordinate CA for an organization in a different State?  Would this lead to

> some kind of third party attack?

Let me make sure I’m understanding. The case you’re worried about is the case where an AOE within State_3 assigns CA_X to act as its AOE CA.  And the problem is that CA_Y may be happy to certify CA_X as an AOE CA but not as a State CA. ie. the problem is that you can’t tell whether the certificate issued by CA_Y to CA_X is an AOE certificate or a State cross-certificate. I hadn’t investigated this possibility.  What do you think?

> Please provide rationale to your answers to avoid further confusion.  Also,

> can we reach you for a telecon tomorrow, if it becomes necessary?

Absolutely ... I should be in the office all day. Please ring through to reception and have them page me or come find me if I’m not at my desk.