ATNP/WG1/SG2	
WP/2118

Prepared by: Tom McParland, BCI

For

FAA/ACT-350

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WG2/21

Berlin, Germany

21-23 August, 2000

IDRP Authentication Analysis

Introduction

SUMMARY

The attached paper is a Sub-Volume V Validation Report for an analysis of IDRP authentication mechanisms.  It reports the results of the US FAA IDRP Security Validation Initiative (FAA_IDRP), Validation Exercise 101 (FVE_101).

  - Document Scope

This report presents the results of the validation of IDRP authentication by analysis. This report lists the ATN Validation Objectives that have been covered, and presents possible defects/omissions that create incomplete, inconsistent or ambiguous requirements contained therein.

  - Document Overview

This document comprises of the following sections:

- Section 1 is the introductory section

- Section 2 lists the objectives

- Section 3 describes the method used in the validation and the validation strategy

- Section 4 lists possible defects or omissions, and gives possible suggestions for improvement

- Section 5 gives the results and analysis of the each validation objective

- Section 6 offers the conclusion of the validation effort

- Validation Objectives

Below is a summary of the validation objectives as allocated to FVE_101 for Analysis of IDRP Authentication.  In addition to the validation objectives below, the implementation of the tests took into consideration whether the applicable requirements for IDRP authentication as specified in Sub-Volume V and Sub-Volume VIII were complete, consistent, and unambiguous.

- Table 1:  Summary of FVE_101 Validation Objectives

| Validation Objective | Validation Initiative | Validation Exercise | Exercise Title | Validation Achieved |
| --- | --- | --- | --- | --- |
| AVO-3_210 | FAA_IDRP | FVE_101 | Validation of the secure and unsecured exchange of IDRP information over a ground-ground path | g |
| AVO-3_211 | FAA_IDRP | FVE_101 | Validation of the secure and unsecured exchange of IDRP information over an air-ground path | g |
| AVO-3_212 | FAA_IDRP | FVE_100 | Validation that compliant airborne and air-ground BISs supporting optional mutual entity authentication of routing exchanges are each able to authenticate IDRP PDUs received from the peer BISs. | g |
| AVO-3_213 | FAA_IDRP | FVE_101 | Validation that compliant airborne and air-ground BISs supporting the option to request and to attach a security certificate to an IDRP OPEN-PDU will interoperate. | g |
| AVO-3_214 | FAA_IDRP | FVE_101 | Validation of the IDRP security enhancements (backward interoperability) | g |
| AVO-3_400 | FAA_IDRP | FVE_101 | Validation of a compliant ground and air-ground BISs capability to authenticate IDRP exchanges with peer BISs across a ground-ground path. | g |
| AVO-3_401 | FAA_IDRP | FVE_101 | Validation of a compliant airborne and air-ground BISs capability to authenticate IDRP exchanges in the air-to-ground direction. | g |
| AVO-3_402 | FAA_IDRP | FVE_101 | Validation of a compliant airborne and air-ground BISs capability to mutually authenticate IDRP exchanges. | g |
| AVO-3_403 | FAA_IDRP | FVE_101 | Validation that the security information exchange and processing overhead for secure exchange of IDRP routing information between ground and air-ground BISs is not excessive. | g |
| AVO-3_404 | FAA_IDRP | FVE_101 | Validation that the security information exchange and processing overhead for secure exchange of IDRP routing information between airborne and air-ground BISs is not excessive. | g |

- Table 2:  Validation Means

| Code | Description |
| --- | --- |
| a | Two or more independently developed interoperating implementations validated by two or more states/organizations. |
| b | Two or more independently developed interoperating implementations validated by one state/organization. |
| c | One implementation validated by more than one state/organization. |
| d | One implementation validated by one state/organization |
| e | Partial implementation validated by one state/organization. |
| f | Simulation, analysis using tools e.g. compiler, modeling tools. |
| g | Analysis and inspection. |

- Validation Strategy

  - Validation Method

Analysis was conducted by the systematic review of Sub-Volume V authentication requirements and where appropriate by constructing a matrix for IDRP authentication scenarios.  Distinct step-by-step scenarios were developed based on Working Group 2 Working Paper 585.  For each step in the scenario the requirement in Sub-Volume V is identified.

  - Validation Scenarios

In order to achieve the validation objectives that have been set forth, the following set of operational scenarios have been analysed.

    - Operational Scenario 1

Scenario 1 is an analysis of a secured IDRP air-ground information exchange.  In this scenario both the airborne and air-ground routers have a policy of mutual authentication.  The airborne router has not pre-stored the air-ground router certificate; therefore, the air-ground router must transmit it.  The air-ground router has access to a certificate delivery service and certificate path validation by both routers is successful.

A subset of this scenario also covers the nominal ground-ground authenticated IDRP exchange.  The difference is that in the ground-ground case there is no ISH exchange.  In this case the routers would be configured for mutual authentication.

This scenario incurs significant overhead and accordingly is specified as optional based on local policy. In addition to the nominal overhead in the authentication data field of an IDRP OPEN PDU for a 4 octet random variable and other signalling information, the air-ground router must transmit its own compressed certificate of 124 octets.  Additionally, if the airborne router does not have the same CA, it must transmit at least one compressed CA certificate of 119 octets.

    - Operational Scenario 2

Scenario 2 is an analysis of an unsecured IDRP air-ground information exchange.  In this scenario the airborne router supports type 2 authentication but the air-ground router does not.  Thus only type 1 authentication is performed.

    - Operational Scenario 3

Scenario 3 is an analysis of a secured IDRP air-ground information exchange where both routers have a policy for single entity authentication.  In single entity authentication there is no need for the air-ground router to transmit a certificate path over the air ground link.  The air-ground router has access to a certificate delivery service and its certificate path validation is successful.

There is no significant overhead in this scenario.  Only the nominal signalling information and random variable is added.

    - Operational Scenario 4

Scenario 4 is an analysis of a secured IDRP air-ground information exchange where both routers have a policy for single entity authentication.  However, access to a delivery service is not available. Because the air-ground router does not have access to a certificate delivery service, it must receive the airborne routers certificate path over the air-ground link.

The overhead in this scenario is the same as in scenario 1 but with transmission of a certificate path in the other direction.

    - Operational Scenario 5

Scenario 5 is an analysis of an attempted secured IDRP air-ground information exchange where the air-ground router certificate path validation is not successful. In this case the IDRP connection is not established.

    - Operational Scenario 6

Scenario 6 is an analysis of a secured IDRP air-ground information exchange where both routers have a policy for single entity authentication.  This scenario is similar to scenario 3 but with the air-ground router accessing a certificate delivery service prior to sending its ISH PDU.

    - Operational Scenario 7

Scenario 7 is an analysis of a secured IDRP air-ground information exchange where both routers have a policy for single entity authentication.  This scenario is a variant of scenario 7 but in this case certificate path validation is not successful.  Therefore, the ISH PDU is not transmitted and an IDRP connection is not established.

  - Validation Assumptions

None.

- Defect Report

  - FAA_IDRP_FVE_101-01

5.8.3.2.10.2.6	There is not an explicit requirement to perform type 1 authentication on an OPEN PDU when the authentication code field is set to 2.  Section 5.8.3.2.10.2.1.4 requires that it be generated and ISO/IEC requires that BISPDUs be discarded in the event that verification fails as is indicated by the note in 5.8.3.2.10.2.6.5

Recommend that a new requirement be added to as requirement 5.8.3.2.10.2.6.1 and that the extant requirements in this section be renumbered.

- FVE_101 Results and Analysis

  - AVO-3 210

Verify that ground BISs and air-ground BISs will interoperate for the secure exchange of IDRP information. The provisions to authenticate IDRP exchanges with the peer BIS across a ground-ground path will be verified.

This AVO is achieved as a subset of the first scenario.

  - AVO-3 211

Verify that compliant airborne and air-ground BISs supporting authentication of IDRP exchanges in the air-to-ground direction will interoperate.

This AVO is achieved for all major variations of local policy and delivery service access conditions.

  - AVO-3 212

Verify that compliant airborne BISs and air-ground BISs supporting the optional mutual authentication of IDRP exchanges are each able to authenticate IDRP PDUs received from the peer BIS.

This AVO is achieved in scenario 1.

  - AVO-3 213

Verify that compliant airborne BISs and air-ground BISs supporting the option to request and to attach a security certificate to an IDRP OPEN-PDU will interoperate.

This AVO is achieved for certificate delivery service being access prior to and

  - AVO-3 214

Verify that a compliant airborne BIS, air-ground BIS and ground BIS will interoperate for the unsecured exchange of routing information with a peer BIS implemented in accordance with the current baseline Sub-Volume V provisions.

This AVO is achieved in scenario 2 for the airborne and air-ground case where there is a difference in the intent to perform type 2 and type 1 authentication but where local policy permits type 1 when type 2 is not supported by the peer.  The ground-ground cases are trivial based on configuration options.

5.3	AVO-3 400

Evaluate the ground BIS and air-ground BISs capability to authenticate IDRP exchanges with the peer BIS across a ground-ground path.

This case is trivial based on configuration options.  The AVO is achieved as a subset of scenario 1.

5.4	AVO-3 401

Evaluate the compliant airborne and air-ground BISs capability for (single entity) authentication of IDRP exchanges in the air-to-ground direction.

This AVO is achieved in scenario 3.

5.5	AVO-3 402

Evaluate the compliant airborne BISs and air-ground BISs capability for the mutual authentication of IDRP exchanges in the air-to-ground direction.

This AVO is achieved in scenario 1.

5.6	AVO-3 403

Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between ground and air-ground BISs.

This AVO is achieved for information exchange in scenario 1.  Processing overhead is not significant from a ground-ground perspective since the most significant overhead of certificate path validation is performed only during connection establishment.

5.7	AVO-3 404

Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between airborne and air-ground BISs.

This AVO is achieved for information exchange in scenario 3, which is expected to be the normal air-ground case, and in scenario 1 if mutual entity authentication is selected.  Processing overhead is not significant in the normal case since it can be performed during intervals of normal transmission.  In particular, the most significant overhead of certificate path validation can be performed by the air ground-router in the interval between transmission and processing of the ISH PDU and transmission and processing of the IDRP OPEN PDU.

- Conclusions

Through the analysis conducted using the above scenarios,  the William J. Hughes Technical Center achieved level g validation of the IDRP related AVOs identified in Table 1. During analysis one minor defect in the Sub-Volume V was found and reported in section 4 of this report.

- Appendix (OPERATIONAL SCENARIOS 1 –7)

(see separate files: FVE_101_Scenario1...FVE_101_Scenario7)