ATNP WG-1/SG-2

WP-2106

Aeronautical Telecommunications Network Panel (ATNP)

WG-1 — Systems Planning and Concept Working Group

SG-2 — Security Sub-group

Meeting 20

Berlin, Germany

21-22 August 2000

Working Paper

Validation Report for the

Aeronautical Telecommunications Network (ATN)

Public Key Infrastructure (PKI) Laboratory

Prepared By: Honeywell Advanced Systems Technology Group

For

NASA Ames Research Center

SUMMARY

This report addresses the PKI Laboratory Validation program which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report on the planned activities and coverage of Sub-Volume VIII validation objectives and Sub-Volume VIII requirements.

DOCUMENT REVISION LOG

| Revision | Description | Date |
| --- | --- | --- |
| 0.1 | DRAFT – Sections 1, 2, and 4 | 03 February 2000 |
| 0.2 | DRAFT – Added detail to sections 4.3.1 and 6 | 01 March 2000 |
| 0.3 | DRAFT – Added detail to sections 3 and 6 | 04 April 2000 |
| 0.4 | First Distribution to WG1SG2 for Meeting 19 – Atlantic City | 09 May 2000 |
| 0.5 | Incorporate Comments fromWG1SG2 Meeting 19 | 15 May 2000 |
| 0.6 | Draft Distribution to WG1SG2 for SV8 Ad Hoc Meeting – Ft. Lauderdale; added detail to section 7, Results and Analysis | 19 June 2000 |
| 0.6A | Second Distribution to WG1SG2 for Meeting 20 – Toulouse, France. | 17-20 July 2000 |
| 0.7 | Reformatted section 7 Results and Analysis to meet the WG1SG2 validation report standard. | 11 August 2000 |
| 1.0 | Distribution to WG1/SG2 for Meeting 21—Berlin, Germany Includes updates to Sections 4, 6, 7, and 8 as a result of SV8 Ad Hoc Meeting (19ter) in Atlantic City | 20-21 August 2000 |

TABLE OF CONTENTS

1.	Introduction	1

1.1.	Document Scope	1

1.2.	Document Overview	1

1.3.	Reference Documents	2

1.4.	Dependencies on External Standards	2

1.4.1.	PKI	2

1.4.2.	ASN.1/PER	2

1.5.	Terminology	2

2.	PKI Laboratory Validation Obectives	3

2.1.	Relationship to ATN Validation Objectives	3

2.2.	Relationship to SARPS Requirements	5

2.3.	High-Level PKI Laboratory Objectives	5

Table 2. Validation Objectives and Means of PKI Laboratory Validation Program	6

Table 3.  Validation Means	6

3.	PKI Laboratory Architecture	7

4.	PKI Laboratory Validation Strategy	9

4.1.	Overall Strategy	9

4.2.	Assumptions and Constraints	9

4.3.	Test Scenarios	9

4.3.1.	PKI Laboratory Independent Test Scenarios	10

4.3.1.1.	Key Management and Distribution	10

4.3.1.1.1.	Test Scenarios	10

4.3.1.1.2.	Expected Output	10

4.3.1.2.	ATN PKI Certificates	10

4.3.1.2.1.	Test Scenarios	10

4.3.1.2.2.	Expected Output	10

4.3.1.2.3.	Test Scenarios	15

4.3.1.2.3.1.	Certificates for CA, Key Certification Signing Use, AP-Title Subject	15

4.3.1.2.3.2.	Certificates for non-CA, Digital Signature Use, AP-Title Subject	15

4.3.1.2.3.3.	Certificates for non-CA, Key Agreement Use, AP-Title Subject	16

4.3.1.2.3.4.	Certificates for non-CA, Key Agreement Use, NET Subject	16

4.3.1.2.3.5.	Certificates for non-CA, Digital Signature Use, AMHS Subject	17

4.3.1.3.	ATN Compressed Certificates	18

4.3.1.3.1.	Test Scenarios	18

4.3.1.3.2.	Expected Output	18

4.3.1.3.3.	Test Scenarios	21

4.3.1.3.3.1.	Compressed CA Certificates	21

4.3.1.3.3.2.	Compressed Ground Certificates	21

4.3.1.3.3.3.	Compressed Router Certificates	21

4.3.1.4.	ATN Certificate Revocation Lists	22

4.3.1.4.1.	Test Scenarios	22

4.3.1.4.2.	Expected Output	22

4.3.1.4.3.	Test Scenarios	22

4.3.2.	PKI Laboratory with NASA Ames Entrust System Support Test Scenarios	25

4.3.2.1.	ATN Cryptographic Setting	25

4.3.2.1.1.	Test Scenarios	25

4.3.2.1.2.	Expected Output	25

4.3.2.2.	Interoperability of Independent Implementations	25

4.3.3.	Protocol Validation Efforts with PKI Laboratory Support Test Scenarios	26

5.	PKI Laboratory Environment	27

6.	Defect Report	29

6.1.	FVO2, FVO3 and FVO4 Defects Found During Test Scenario Preparation	29

6.1.1.	Defects addressed during WG1SG2 Meeting 19, Atlantic City	29

7.	Results and Analysis	33

8.	Conclusions	37

Validation Report for the Aeronautical Telecommunications Network (ATN) Public Key Infrastructure (PKI) Laboratory

# Introduction

## Document Scope

This report addresses the PKI Laboratory Validation program which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report on the planned activities and coverage of Sub-Volume VIII validation objectives and Sub-Volume VIII requirements.

## Document Overview

This document is organized as follows:

- Section 1 – Introduction

The introduction identifies the scope of the document, summarizes the document organization, provides references to applicable documents, identifies dependencies on external standards, and defines terminology used throughout the document.

- Section 2 – PKI Laboratory Validation Objectives

This section documents the relationship to high-level ATN validation objectives, describes the high-level PKI Laboratory objectives, and provides a cross-reference to specific SARPS requirements.

- Section 3 – PKI Laboratory Architecture

This section describes the architecture of the PKI Laboratory with respect to the ATN communications environment.

- Section 4 – PKI Laboratory Validation Strategy

This section defines the strategy for achieving the validation objectives, including assumptions, constraints and test scenarios.

- Section 5 – PKI Laboratory Environment

This section describes the PKI Laboratory environment including tools and techniques.

- Section 6 – Defect Report

This section documents any defects uncovered during execution of this validation exercise.  The defect description includes the applicable validation objective and Sub-Volume VIII section, as well as a disposition of the defect.

- Section 7 – Results and Analysis

This section contains a summary of results with respect to the PKI Laboratory validation objectives.

- Section 8 – Conclusions

This section provides a summary conclusion of the PKI Laboratory validation.

## Reference Documents

Draft ATN SARPS Technical Provisions, ICAO Document 9705 Sub-Volume VIII, W1WP1708.

Appendix I, Doc 9705 – Sub-Volume VIII (Security), Validation Report, ATNP/3

## Dependencies on External Standards

### PKI

- ITU-T X.509

### ASN.1/PER

- ISO 8825-2

## Terminology

| ASN | Abstract Syntax Notation |
| --- | --- |
| ATN | Aeronautical Telecommunications Network |
| ATNP | Aeronautical Telecommunications Network Panel |
| CA | Certificate Authority |
| CM | Context Management |
| COTS | Commercial Off-The-Shelf |
| CRL | Certificate Revocation List |
| FVO | Functional Validation Objective |
| ICAO | International Civil Aviation Organization |
| IDRP | Inter-Domain Routing Protocol |
| PER | Packed Encoding Rules |
| PKI | Public Key Infrastructure |
| SARPs | Standards and Recommended Practices |
| SVO | System Validation Objective |
| TVO | Technical Validation Objective |
| ULCS | Upper Layer Communication Services |

# PKI Laboratory Validation Obectives

## Relationship to ATN Validation Objectives

The primary goal of the PKI Laboratory Validation program is to address the PKI portion of several Sub-Volume VIII Validation Objectives by conducting test scenarios defined in this document and documenting the objective, measurable results.  These primary Validation Objectives include FVO1, FVO6, TVO5, and TV08 as defined in Table 1.  A number of other, secondary Validation Objectives related to PKI will be addressed in a more subjective manner, during the conduct of the test scenarios defined in this document.  These secondary validation objectives are FVO2, FVO3, FVO5, TVO3, TVO4, TVO6, and TVO7.  Validation Objectives FVO7, TVO1, TVO3, and TVO4 are potential candidates for coverage by the PKI laboratory Validation program whereby the laboratory can be used to support other TBD validation programs as defined in section 4.3.3. The PKI Laboratory Validation program does not address Validation Objectives SVO1, SVO2, and SVO3.

Table 1.  High-level Sub-Volume VIII Validation Objectives Allocated to the PKI Laboratory Program

| Sub-Volume VIII Validation Objective | Description | Validation Objective Allocated to PKI Laboratory Program |
| --- | --- | --- |
| SVO 1 | To determine which system level requirements are satisfied by the Sub-Volume VIII requirements. | None. |
| SVO 2 | Validate that the Sub-Volume VIII requirements trace to other SARPS sub-volumes, where applicable. | None. |
| SVO 3 | Validate that Sub-Volume VIII includes provision for backward compatibility with prior versions of peer ATN implementations that do not incorporate security services. | None. |
| FVO 1 | Validate that Sub-Volume VIII supports implementation of local security policies and practices, within the boundaries of SARPs, as determined by States/Organizations. | Validate that the SARPs supports implementation of local PKI policies and practices, within the boundaries of the SARPs, as determined by States/Organizations. |
| FVO 2 | Validate that Sub-Volume VIII requirements are complete. | Validate that the Sub-Volume VIII PKI requirements are complete. |
| FVO 3 | Validate that Sub-Volume VIII requirements are unambiguous. | Validate that the Sub-Volume VIII PKI requirements are unambiguous. |
| FVO 4 | Validate that Sub-Volume VIII requirements are consistent. | Validate that the Sub-Volume VIII PKI requirements are consistent. |
| FVO 5 | Determine if there are any Sub-Volume VIII requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. | Determine if there are Sub-Volume VIII PKI requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. |
| FVO 6 | To determine if provision has been made to ensure that Sub-Volume VIII are implementation independent. | To determine if provision has been made to ensure that Sub-Volume VIII PKI requirements are implementation independent. |
| FVO 7 | To determine if Sub-Volume VIII includes provision for security services necessary for all security users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports security services necessary all security users. |
| TVO 1 | Validate that Sub-Volume VIII includes provisions for both mobile and fixed ATN users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports both mobile and fixed ATN users. |
| TVO 2 | Validate that Sub-Volume VIII minimizes air-ground security related protocol overhead. | Validate that the Sub-Volume VIII includes provision for a PKI that supports minimal air-ground overhead to meet required security level. |
| TVO 3 | Validate that Sub-Volume VIII supports the security provisions of the ATN Upper Layer Communications Service (ULCS). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Upper Layer Communication Services (ULCS). |
| TVO 4 | Validate that Sub-Volume VIII supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). |
| TVO 5 | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. |
| TVO 6 | To determine if the ATN security solution has any unacceptable behaviour. | To determine if the ATN security solution has any unacceptable behaviour. |
| TVO 7 | To determine if provision for future migration has been addressed. | To determine if provision for future migration has been addressed. |
| TVO 8 | To determine if the functionality described in Sub-Volume VIII is implementable. | To determine if the functionality described in Sub-Volume VIII is implementable. |

## Relationship to SARPS Requirements

The following individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements are to be validated by the PKI Laboratory effort.

8.3.1.1.4	Singular requirement regarding Public Key Certificates

8.3.1.2 [all]	Key Management and Distribution

8.3.1.9 [all]	Role of ATN Directory Servers within the ATN Security Framework

8.4.1 [all]	Certificate Policy

8.4.2 [all]	Certificate Practice Statement

8.4.3.1 [all]	ATN PKI Certificate Format – General Format

8.4.3.2 [all]	ATN PKI Certificate Format – ATN Certificates to be Signed

8.4.3.3 [all]	ATN PKI Certificate Format – Compressed Certificates

8.4.3.4 [all]	ATN PKI Certificate Format – ATN Certificate Path

8.4.4 [all]	ATN PKI CRL Format

8.4.5 [all] 	ATN PKI Certificate and CRL Validation

8.4.6 [all]	ATN PKI ASN.1 Module

8.5.2 [all]	ATN Cryptographic Setting

## High-Level PKI Laboratory Objectives

Table 2 identifies individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements cross-referenced to specific Validation Objectives that are to be addressed by the PKI Laboratory effort.  Table entries indicate the planned validation means to be applied to the intersection of the requirement(s) and validation objective as defined in Table 3.

## Table 2. Validation Objectives and Means of PKI Laboratory Validation Program

| Requirements Grouping | Sub-Volume VIII Requirements | SVO |  |  |  | FVO |  |  |  |  |  |  | TVO |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| ATN Security Strategy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Backward Compatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ULCS Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CM Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Applications Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Key Management and Distribution | 8.3.1.2, 8.3.1.9 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Certificate Authority Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN PKI Certificates | 8.3.1.1.4, 8.4.1, 8.4.2, 8.4.3.1, 8.4.3.2, 8.4.6 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Compressed Certificates | 8.4.3.3, 8.4.3.4 |  |  |  | c |  | g | g | g | g | c | * | * | g | * | * | b | c | g | c |
| ATN Certificate Revocation Lists | 8.4.4 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Cryptographic Setting | 8.5.2 |  |  |  |  |  | b | b | b | b | b |  |  |  |  |  | b | b | g | b |
| ATN Key Agreement Scheme (AKAS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Digital Signature Scheme (ADSS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Keyed Message Authentication Code Scheme (AMACS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Auxiliary Cryptographic Primitives and Functions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Object (SSO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note: “*” See section 4.3.3.

## Table 3.  Validation Means

| Code | Description |
| --- | --- |
| a | Two or more independently developed interoperating implementations validated by two or more states/organizations. |
| b | Two or more independently developed interoperating implementations validated by one state/organization. |
| c | One implementation validated by more than one state/organization. |
| d | One implementation validated by one state/organization. |
| e | Partial implementation validated by one state/organization. |
| f | Simulation, analysis using tools e.g. compiler, modeling tools. |
| g | Analysis and inspection. |

# PKI Laboratory Architecture

Figure 1.  PKI Validation Laboratory PKI Architecture

RA

CA

Sub CA

X.500 Directory Server

Peer Logic

Baltimore Technologies

Entrust

CA

RA

Legend:    CA = Certificate Authority

Sub CA = Subordinate Certificate Authority

RA = Registration Authority

This page intentionally left blank.

# PKI Laboratory Validation Strategy

## Overall Strategy

The overall strategy for the PKI Laboratory Validation effort is to validate selected ATN Validation Objectives and specific ATN Technical Provisions, ICAO Document 9705, Sub-Volume VIII requirements using Commercial Off the Shelf PKI Systems (FVO6).

## Assumptions and Constraints

TBD.

## Test Scenarios

The PKI Laboratory test scenarios fall into three major categories.  First category of tests scenarios are those which can be preformed with the PKI Laboratory resources and personnel independently.  The second category are those which the PKI Laboratory resources and personnel lead but require the resources and / or support of other agencies.  The third category are those which are primary conducted by other agencies but where the PKI Laboratory can play a support role.

### PKI Laboratory Independent Test Scenarios

#### Key Management and Distribution

##### Test Scenarios

Numerous test scenarios will be described in detail to address the key management and distribution requirements within the context of the CA architecture described in Section 3 of this document.

##### Expected Output

The results of these test scenarios will be printed copies of certificates received at a test CA representing one state’s CA from various test CAs representing CAs at differing hierarchical levels in another state. These printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08).  Any unacceptable behavior noted during the tests will be documented (TVO6).  The printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

#### ATN PKI Certificates

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPs requirements listed in Table 2 (FVO1).  The structure of an ATN Certificate is shown in Figure 2.

##### Expected Output

The results of these test scenarios will be electronic format and printed and human readable copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any defects will be documented in section 6.

| Figure 2. ATN Certificate Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| version |  | Version 3 | INTEGER | 2 | 0, 1 |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| signature | Algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| issuer | AttributeType | X.501 Distinguished Name | OBJECT IDENTIFIER | X.501 Distinguished Name Types | Non-X.501 Distinguished Name |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| validity | NotBefore | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ |
|  | NotAfter | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ (any time after notBefore) | YYMMDDhhmmssZ (any time equal to or before notBefore) |
| subject | AttributeType | X.501 distinguished name for CA or AMHS entity; Empty otherwise | OBJECT IDENTIFIER | X.501 Distinguished Name Types / Empty | Empty / Non-X.501 Distinguished Name Types |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| subjectPublicKeyInfo | Algorithm | Id-ecPublicKey | OBJECT IDENTIFIER | 1.2.840.10045.2.1 |  |
|  | Parameters | NamedCurve sect163r2 (for non-CAs) sect233r1 (for CAs) | OBJECT IDENTIFIER | 1.3.132.0.15 1.3.132.0.27 | Other OIDs |
|  | SubjectPublicKey | ECPoint | BIT STRING | 02¦¦xQ (compressed) 03¦¦xQ (compressed) 04¦¦xQ¦¦yQ (uncompressed) | Any other form |
| issuerUniqueID |  |  |  | Omitted | Any |
| subjectUniqueID |  |  |  | Omitted | Any |
| Extensions |  |  |  |  |  |
| AuthorityKeyIdentifier | ExtnID | id-ce-authorityKeyIdentifier | OBJECT IDENTIFIER | 2.5.29.35 | Other OIDs |
|  | Critical | Not Critical | BOOLEAN | FALSE | TRUE |
|  | ExtnValue - keyIdentifier |  | OCTET STRING | 0100 plus 60 least significant bits of the SHA-1 hash of the subjectPublicKey | Other than Valid Values |
|  | ExtnValue – authorityCertIssuer |  |  | Omitted | Any |
|  | ExtnValue - authorityCertSerialNumber |  |  | Omitted | Any |
| KeyUsage | extnID | id-ce-keyUsage | OBJECT IDENTIFIER | 2.5.29.15 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - Key Usage |  | BIT STRING digitalSignature (0) nonRepudiation (1) keyEncipherment (2) dataEncipherment (3) keyAgreement (4) keyCertSign (5) cRLSign (6) encipherOnly (7) decipherOnly (8) | 000000001 000010000 001100000 | All other combinations |
| SubjectAltName | extnID | id-ce-subjectAltName | OBJECT IDENTIFIER | 2.5.29.17 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - SubjectAltName | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 3: AMHS Address (See Note) 7:NET OCTET STRING SIZE (20) (See Note) 8:AP-Title (See Note) | 0: 1: 2: 4: 5: 6: |
| IssuerAltName | extnID | id-ce-issuerAltName | OBJECT IDENTIFIER | 2.5.29.18 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Algorithm OID | algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | parameters | NULL | NULL | NULL | Non-NULL |
| Signature Value | signatureValue | r | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | s | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | NOTE: |  |  |  |  |
|  | AP-Title (Airborne) | 1.3.27.1.Integer(1..(2**24-1)).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | AP-Title (Ground) | 1.3.27.2.Integer(1..?).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | NET (Fixed AINSC) | 47 0027 01 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile AINSC) | 47 0027 41 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Fixed ATSC) | 47 0027 81 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile ATSC) | 47 0027 C1 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | AMHS Address | X.400 Address |  |  |  |

This page intentionally left blank.

##### Test Scenarios

There are three certificate parameters that form the basis for the five major types of ATN Certificates:

- Is the certificate for CA or non-CA use

- Is the subject of the certificate a AP-Title, NET entity, or AMHS address

- Is the certificate used for Key Certification Signing, Digital Signature, or Key Agreement

Figure 3 illustrates the valid ATN combinations that form the basis for the test scenarios.

|  | Certificate Type |  |  |
| --- | --- | --- | --- |
| Subject | Key Certification Signing and CRL Signing | Digital Signature | Key Agreement |
| CA / AP-Title |  |  |  |
| Non-CA / AP-Title |  |  |  |
| Non-CA / NET |  |  |  |
| Non-CA / AMHS |  |  |  |
| Notes: 1. CA / NET is not a valid subject for ATN certificates. 2. Valid types apply to both standard and compressed ATN certificates |  |  |  |

Figure 3.  Valid ATN Certificates

###### Certificates for CA, Key Certification Signing Use, AP-Title Subject

This is an example of a certificate issued to State CAs by other State CAs or a certificate issued to AOE CAs by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 2, with the following specific values:

issuer							{x.501 Distinguished Name}

subject							{x.501 Distinguished Name}

subjectPublicKeyInfo

Parameters					1.3.132.0.27

KeyUsage

extnValue – KeyUsage			001100000

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Digital Signature Use, AP-Title Subject

This is an example of a certificate issued to ground CMA, aircraft CMA, AMHS, or other ground entities for their signing keys by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 2, with the following specific values:

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

KeyUsage

extnValue – KeyUsage			000000001

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Key Agreement Use, AP-Title Subject

This is an example of a certificate issued to ground CMA, aircraft CMA, or other ground entities for their key agreement keys by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 2, with the following specific values:

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

KeyUsage

extnValue – KeyUsage			000010000

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Key Agreement Use, NET Subject

This is an example of a certificate issued to ground routers or aircraft routers for their key agreement keys by State CAs.

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

KeyUsage

extnValue – KeyUsage			000010000

SubjectAlternateName

extnValue – SubjectAltName

0: 470027 xxxxxxxx00xxxxxxxxxxxxxxxxxxxxxxxx

IssuerAlternateName

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Digital Signature Use, AMHS Subject

This is an example of a certificate issued for AMHS digital signature key by State CAs.

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

KeyUsage

extnValue – KeyUsage			000000001

SubjectAlternateName

extnValue – SubjectAltName			3: AMHS x.400 address

IssuerAlternateName

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

#### ATN Compressed Certificates

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of compressed certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPs requirements listed in Table 2 (FVO1).  The structure of an ATN Compressed Certificate is shown in Figure 4.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

| Figure 4. ATN Compressed Certificate Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| SerialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| Validity | NotBefore | ATNSecurityDateTime | ATNSecurityDateTime | YYYYMMDDhhmmss | Non- YYYYMMDDhhmmss |
|  | NotAfter | ATNSecurityDateTime | ATNSecurityDateTime | YYYYMMDDhhmmss | Non- YYYYMMDDhhmmss |
| subjectPublicKeyInfo | SubjectPublicKey | ECPoint | BIT STRING | 02¦¦xQ (compressed) 03¦¦xQ (compressed) 04¦¦xQ¦¦yQ (uncompressed) | Non- 04¦¦xQ¦¦yQ |
| KeyUsage |  |  | BIT STRING digitalSignature (0) nonRepudiation (1) keyEncipherment (2) dataEncipherment (3) keyAgreement (4) keyCertSign (5) cRLSign (6) encipherOnly (7) decipherOnly (8 | 000000001 000010000 001100000 | All other combinations |
| SubjectAltName | extnValue - SubjectAltName | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 7:NET OCTET STRING SIZE (20) (See Note) 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: |
| IssuerAltName | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Signature Value | signatureValue | r | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | s | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | Note: |  |  |  |  |
|  | AP-Title (Airborne) | 1.3.27.1.Integer(1..(2**24-1)).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | AP-Title (Ground) | 1.3.27.2.Integer(1..?).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | NET (Fixed AINSC) | 47 0027 01 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile AINSC) | 47 0027 41 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Fixed ATSC) | 47 0027 81 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile ATSC) | 47 0027 C1 (3 Octets) 00 (3 Octets) (2 Octets) (6 Octets) (1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |

##### Test Scenarios

###### Compressed CA Certificates

This test consists of generating a compressed certificate version of the certificate described in section 4.3.1.2.3.1.

###### Compressed Ground Certificates

This test consists of generating compressed certificate versions of the certificates described in section 4.3.1.2.3.2 and 4.3.1.2.3.3.

###### Compressed Router Certificates

This test consists of generating a compressed certificate version of the certificates described in section 4.3.1.2.3.4.

#### ATN Certificate Revocation Lists

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of compressed certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPS requirements listed in Table 2 (FVO1). The structure of an ATN Certificate Revocation List is shown in Figure 5.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

##### Test Scenarios

TBD

| Figure 5. ATN Certificate Revocation List Structure |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| Version |  | Version 3 | INTEGER | 2 | 0, 1 |
| Signature | Algorithm | Ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| Issuer | AttributeType | X.501 Distinguished Name | OBJECT IDENTIFIER | X.501 Distinguished Name | Non-X.501 Distinguished Name |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| ThisUpdate | Time | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ |
| NextUpdate | Time | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ (any time after thisUpdate) | YYMMDDhhmmssZ (any time equal to or before thisUpdate) |
| SerialNumber | CertificateSerialNumber |  | INTEGER | Any Integer | Non-Integer |
| RevocationDate | Time | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ |
| CrlEntryExtentions |  |  |  | absent |  |
| . . . |  |  |  |  |  |
| SerialNumber | CertificateSerialNumber |  | INTEGER | Any Integer | Non-Integer |
| RevocationDate | Time | Coordinated Universal Time to seconds | UTCTime | YYMMDDhhmmssZ | Non-YYMMDDhhmmssZ |
| CrlEntryExtentions |  |  |  | Absent |  |
| CrlExtensions |  |  |  |  |  |
| IssuerAltName | extnID | Id-ce-issuerAltName | OBJECT IDENTIFIER | 2.5.29.18 | Other OIDs |
|  | critical | Non-critical | BOOLEAN | FALSE | TRUE |
|  | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Algorithm OID | Algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| Signature Value | SignatureValue | R | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | S | INTEGER | 1 to n-1 | Other than 1 to n-1 |

This page intentionally left blank.

### PKI Laboratory with NASA Ames Entrust System Support Test Scenarios

#### ATN Cryptographic Setting

##### Test Scenarios

Numerous test scenarios will be described in detail to utilize the ATN cryptographic settings in the PKI system.  Proper functionality will be verified by having keys generated and certificates signed by a CA implemented on one COTS PKI system verified by a CA implemented on a PKI system implemented on a different COTS system.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of keys and certificates generated in accordance with the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

#### Interoperability of Independent Implementations

(Ed. note: This section will contain a subset of test scenarios from the section 4.3.1 but specify the use of two different PKI systems within the test scenario architecture.)

### Protocol Validation Efforts with PKI Laboratory Support Test Scenarios

There are no firm plans to execute any test scenarios in this section, however, it likely that ATN Certificates, Compressed Certificates and CRLs generated under proceeding PKI Laboratory test scenarios could be of value in validation efforts requiring these types of data as inputs.  The variety of certificates and CRLs generated under the PKI test scenarios might provide a range of inputs to more fully exercise these security services than might otherwise be possible.

# PKI Laboratory Environment

Operating System – all workstations:	Windows NT 4.00

Entrust

Certificate Authority:		Entrust/Authority R5.0

Registration Authority:	Entrust/RA™ R5.0

Baltimore Technologies		UniCERT™ ECDSA v3.0.6

Peer Logic®

X.500 Directory Service	i500® Directory General Release 8A.3

This page intentionally left blank.

# Defect Report

## FVO2, FVO3 and FVO4 Defects Found During Test Scenario Preparation

### Defects addressed during WG1SG2 Meeting 19, Atlantic City

The following Sub-volume VIII requirements were identified as incomplete, ambiguous or inconsistent during the preparation of the test scenarios for this report and addressed, as noted during WG1SG2 Meeting 19, Atlantic City.  The disposition of each defect is indicated in bold-italics.

| Defect Number | Validation Objective | Section | Description |
| --- | --- | --- | --- |
| 1. | FVO3 | 8.4.3 | The subsection numbering, starting at 8.4.3 should be restructured to follow the major and minor components of the certificate as shown in Figure 2. (Accept: Paragraph indentation corrected.) |
| 2. | FVO2 | 8.4.3.1.3.3 | Suggest adding a reference to section 8.5.4.2.1 5) and 7) for the values of r and s respectively to reduce ambiguity and for completeness. (Accept) |
| 3. | FVO2 | 8.4.3.2.5 | The requirement does not state how the values for type and value are established. There are a great number of Distinguished Name types. Recommend that SV VIII define which are to be used within ATN. It is possible that the AE Title or NET could be added to the distinguished name, thereby eliminating the need for those two certificate extensions. For example, AE Name could be put in AttributeType “Organizational Role” and NET could be put into AttributeType “Device”. Other AttributeTypes such as “Country”, Organization”, and “Organizational Unit” could be made mandatory to make the certificates human readable. In the case of compressed certificates, only “Organizational role” or “Device” type would be mandatory (Accept: Added reference to directory scheme in Sub-Volume VII.) |
| 4. | FVO2 | 8.4.3.2.7.1 | It is not clear how the values for subject are established. Recommend adding a reference to 8.4.3.2.5 where the syntax for X.501 Distinguished Name is defined. (Accept.) |
| 5. | FVO2 FVO3 | 8.4.3.2.8.1 | Is SEC1, Standards for Efficient Cryptography Group, a valid reference for the OIDs defined in 8.4.3.2.8.2 through 8.4.3.2.8.5? Provide the correct reference for the OID secids modules used in 8.4.3.2.8.5. (Reject: Draft SV8 Ver 1.0.2 (WP1911) changed to reflect X9.62 with ATN labels.) |
| 6. | FVO2 FVO3 | 8.4.3.2.8.4 | This appears to be an incomplete OID. For example, should atnPI-arc be atnPKI-arc? Also, the OBJECT IDENTIFIER for secids modules is not defined. (Reject: Draft SV8 Ver 1.0.2 (WP1911) changed to reflect X9.62 with ATN labels.) |
| 7. | FVO2 | 8.4.3.2.8.5 | There is not a reference to how the octet string for ECPoint is derived. There does not appear to be any equations in section 8.5 defining this. The recommendation is to treat this like the base point as defined in 8.5.2.3.1.5 and providing a reference to values for the ECPoint, by adding the following to section 8.4.3.2.8.5: The value of ECPoint = 04¦¦xQ¦¦yQ, as defined in ANSI X9.62 - 1998, section 4.3.6, where 04 indicates that point compression is not being used. Values are as defined in ANSI X9.62 - 1998, section 5.2.1. (Accept: Added note with text from SEC document.) |
| 8. | FVO4 | 8.4.3.2.9 | Is “omitted” valid ASN.1 terminology? Should this say “empty” to be consistent with other requirements such as 8.4.3.2.7.2? (Reject: "omitted" is correct per PKIX.) |
| 9. | FVO4 | 8.4.3.2.10 | Is “omitted” valid ASN.1 terminology? Should this say “empty” to be consistent with other requirements such as 8.4.3.2.7.2? (Reject: "omitted" is correct per PKIX.) |
| 10. | FVO3 | 8.4.3.2.11.1.3 | Recommend adding the reference to extnValue to this requirement to make it unambiguous. (Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.) |
| 11. | FVO2 | 8.4.3.2.11.1.4 | Recommend merging with 8.4.3.2.11.1.5 as this is not a meaningful stand-alone requirement. Recommend adding a reference to section 8.5.6.3 that specifies how the value is determined. (Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.) |
| 12. | FVO3 | 8.4.3.2.11.1.5 | Recommend a requirement explicitly stating that authorityCertIssurer and authorityCertSerialNumber are not used in ATN since these are listed as optional fields in X.509. (Accept: Added text stating that these are omitted.) |
| 13. | FVO3 | 8.4.3.2.11.2.3 | Recommend adding the reference to extnValue to this requirement to make it unambiguous. (Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.) |
| 14. | FVO3 | 8.4.3.2.11.3.3 | Recommend adding the reference to extnValue to this requirement to make it unambiguous. (Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.) |
| 15. | FVO2 | 8.4.3.2.11.3.5 | Recommend adding a reference to Sub-Volume IV, section 4.3 to clarify how an AP-Title is structured to be consistent with the reference to NET in section 8.4.3.2.11.3.8. (Accept: Added reference to SV9.) |
| 16. | FVO2 | 8.4.3.2.11.3.7 | Recommend adding a reference to the ICAO assigned OIDs that are valid for use as a type-id value. (Reject: "otherName" replaced with IP address octet string.) |
| 17. | FVO3 | 8.4.3.2.11.4.3 | Recommend adding the reference to extnValue to this requirement to make it unambiguous. (Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.) |
| 18. | FVO5 | 8.4.3.3.2.1 | Recommend deleting “…” as it doesn’t appear to be meaningful. (Reject: Valid ASN syntax.) |
| 19. | FVO2 | 8.4.3.3.2.1 | Recommend clarifying how the values of subjectAETitle and issuerAETitle are specified. (Accept: Added reference to SV9.) |
| 20. | FVO5 | 8.4.3.3.3.1 | Recommend deleting “…” as it doesn’t appear to be meaningful. (Reject: valid ASN syntax.) |
| 21. | FVO2 | 8.4.3.3.3.1 | Recommend clarifying how the values of subjectAETitle and issuerAETitle are specified. (Accept: Added reference to SV9. |
| 22. | FVO5 | 8.4.3.3.4.1 | Recommend deleting “…” as it doesn’t appear to be meaningful. (Reject: valid ASN syntax.) |
| 23. | FVO2 | 8.4.3.3.4.1 | Recommend clarifying how the values of subjectAETitle and issuerAETitle are specified. (Accept: Add reference to SV9.) |
| 24. | TVO5 |  | Recommendation Sub-Volume VIII does not specify a cross-certification request protocol or procedures. With the number CAs that will need to cross-certify, specifying a protocol and possibly a procedure would be beneficial to the smooth operation of the PKI. (Reject: Guidance issue. Eurocontrol does not want to add requirements that necessitate an electronic solution.) |
| 25. | TVO8 |  | (In addition, SG2 recommended that encoding rules specified in SV8 be changed as follows: DER for uncompressed X.509 certificates and CRLs and PER for compressed certificates. This facilitates use of COTS PKI applications, which support DER, for standard certificate and CRLs, which are not sent over the air. Compressed certificates must be PER encoded since these are sent over the air.) |

# Results and Analysis

## SVO 1

To determine which system level requirements are satisfied by the Sub-Volume VIII requirements.

The PKI Validation Program did not address this validation objective.

## SVO 2

Validate that the Sub-Volume VIII requirements trace to other SARPS sub-volumes, where applicable.

The PKI Validation Program did not address this validation objective.

## SVO3

Validate that Sub-Volume VIII includes provision for backward compatibility with prior versions of peer ATN implementations that do not incorporate security services.

The PKI Validation Program did not address this validation objective.

## FVO 1

Validate that Sub-Volume VIII supports implementation of local security policies and practices, within the boundaries of SARPs, as determined by States/Organizations.

Validation activity is still in progress. As determined by PKI validation analysis and inspection completed to date, this functional objective is satisfied by Sub-Volume VIII requirements, which permit local security policies to be implemented as determined by States/Organizations.

## FVO 2

Validate that Sub-Volume VIII requirements are complete.

Defects detected during PKI validation analysis and inspection have been rectified in Sub-Volume VIII in accordance with the disposition of defects recorded in Section 6 of this report.

## FVO 3

Validate that Sub-Volume VIII requirements are unambiguous.

Defects detected during PKI validation analysis and inspection have been rectified in Sub-Volume VIII in accordance with the disposition of defects recorded in Section 6 of this report.

## FVO 4

Validate that Sub-Volume VIII requirements are consistent.

Defects detected during PKI validation analysis and inspection have been rectified in Sub-Volume VIII in accordance with the disposition of defects recorded in Section 6 of this report.

## FVO 5

Determine if there are any Sub-Volume VIII requirements that would have no effect if removed.

Defects detected during PKI validation analysis and inspection have been rectified in Sub-Volume VIII in accordance with the disposition of defects recorded in Section 6 of this report.

## FVO 6

To determine if provision has been made to ensure that Sub-Volume VIII are implementation independent.

Validation activity is still in progress. As determined by PKI validation analysis and inspection completed to date, the Key Management and Distribution and ATN PKI Certificates requirements could be implemented almost completely via off-the-shelf PKI systems from multiple vendors.  In cases where the off-the-shelf solutions do not provide certain required functionality, some vendors have indicated that those features are on their current product development schedule.  In those cases where a vendor has not yet committed to implement a required function, the nature of the requirements is such that they are not technically challenging.  The reason that they are not on a vendor’s product development schedule is due primarily to the lack of broad market demand for the functionality.  The following table summarizes the requirements that are not implemented and two vendor’s responses to implementation schedules.

| Requirement | Baltimore Technologies | Entrust |
| --- | --- | --- |
| Issuer Alternate Name | Scheduled for 2nd quarter 2001, however, will not likely support being set as critical | Supported |
| Subject Alternate Name | Supported, however, cannot be set as critical | Modifiable to permit any documented or undocumented syntax by using the E-mail (subjectAltName): field |
| Key Usage | Supported | Supported |
| User Specified Elliptic Curve Parameters | Not available, not currently scheduled, but being considered | Can be specified by providing detailed parameters, not covered by Entrust’s validation efforts |

Compressed certificates are not implementable via off-the-shelf PKI systems.  Custom software will be needed to generate the compressed certificates.

## FVO 7

To determine if Sub-Volume VIII includes provision for security services necessary for all security users.

The PKI Validation Program did not address this validation objective.

## TVO 1

Validate that Sub-Volume VIII includes provisions for both mobile and fixed ATN users.

The PKI Validation Program did not address this validation objective.

## TVO 2

Validate that Sub-Volume VIII minimizes air-ground security related protocol overhead.

As determined by PKI validation analysis and inspection, this technical objective is satisfied by Sub-Volume VIII requirements.  The ATN Compressed Certificate formats were inspected to verify that they contain the minimal data content required to implement the ATN Security.

## TVO 3

Validate that Sub-Volume VIII supports the security provisions of the ATN Upper Layer Communications Service (ULCS).

The PKI Validation Program did not address this validation objective.

## TVO 4

Validate that Sub-Volume VIII supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP).

The PKI Validation Program did not address this validation objective.

## TVO 5

Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate.

Validation activity is still in progress.  No results or analysis reported to date.

## TVO 6

To determine if the ATN security solution has any unacceptable behaviour.

Validation activity is still in progress. As determined by PKI validation analysis and inspection completed to date, PKI implementations based upon two different vendors’ PKI software packages were utilized during this validation program and no unacceptable behavior was noted during the conduct of this validation program.

## TVO 7

To determine if provision for future migration has been addressed.

As determined by PKI validation analysis and inspection, this technical objective is satisfied by Sub-Volume VIII requirements.  The ATN PKI Certificates’ ASN.1 specifications are structured such that additional data fields can be added in the future while maintaining backward compatibility with this version.

## TVO 8

To determine if the functionality described in Sub-Volume VIII is implementable.

Validation activity is still in progress. As determined by PKI validation analysis and inspection completed to date, the Key Management and Distribution and ATN PKI Certificates requirements could be implemented almost completely via off-the-shelf PKI systems from multiple vendors.  In cases where the off-the-shelf solutions do not provide certain required functionality, some vendors have indicated that those features are on their current product development schedule.  In those cases where a vendor has not yet committed to implement a required function, the nature of the requirements is such that they are not technically challenging.  The reason that they are not on a vendor’s product development schedule is due primarily to the lack of broad market demand for the functionality.

Compressed certificates are not implementable via off-the-shelf PKI systems.  Custom software will likely be needed to generate the compressed certificates.

This page intentionally left blank.

# Conclusions

For FVO1, FVO2, FVO6, TVO5, TVO6, and TVO8, the Validation Exercise achieved a level e validation for Requirement Groupings SEC3-7, 9 and 11 and a level g validation for Requirement Grouping SEC3-10 in Sub-Volume VIII.  To date, partial implementation of the ATN Public Key Infrastructure requirements has been achieved with minor exceptions. In a few cases, neither of the commercial off-the-shelf PKI systems was able to implement all applicable requirements of Sub-Volume VIII.  However, discussions with the PKI vendors indicated that the changes required bring their product into compliance with Sub-Volume VIII requirements were minimal and in some cases were already on the product development schedule of the vendor.  The PKI laboratory Validation Exercise is an ongoing activity with a goal of achieving a level b validation for SEC-11 and level c validation for SEC3-7, 9 and 10.

For FVO3, FVO4, FVO5, and TVO7, the Validation Exercise achieved level g validation for Requirement Groupings SEC3-7, 9, 10, and 11 in Sub-Volume VIII.  This was achieved during development of the test scenarios and implementation of the PKI prototype.