FAA_IDRP_FVE_101

Scenario 7

Air-Ground Secured Exchange

Access to Delivery Service Prior to Sending ISH PDU

Unsuccessful Certificate Path Validation

Conditions:

- Airborne and Air-Ground Routers support Type 2 Authentication

- Airborne Router has not pre-stored Air-Ground Router certificate

- Airborne Router policy is for Single Entity Authentication

- Air-Ground Router has access to a certificate delivery service prior to sending ISH PDU

- Certificate path validation by the Air-Ground Router is unsuccessful

| Item | Scenario Step | Requirement |
| --- | --- | --- |
|  | (Upon receipt of a joint event), the airborne router signals type 2 authentication in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.1 |
|  | The airborne router signals public key certificate not required in the ATN Data Link Capability Parameter in the ISH PDU | 5.3.5.2.16.2.2 |
|  | The air-ground router retrieves the aircraft’s public-key certificate, CertU from a supporting directory service and validates the certificate path. | 5.3.5.2.16.3.2 |
|  | If certificate path validation is not successful, the air-ground router does not send the ISH PDU. | 5.3.5.2.16.3.4 |