ATNP/WG1/SG2	
WP/2117

Prepared by: Jamie Chappell, BCI

For

FAA/ACT-350

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WG2/21

Berlin, Germany

21-23 August, 2000

Validation of ATN Cryptographic Primitives in IDRP

Introduction

SUMMARY

The attached paper is a SubVolume V Validation Report for an implementation of the ATN Cryptographic Primitives within IDRP, specifically the Ground/Ground configuration. It reports the results of the US FAA IDRP Security Validation Initiative (FAA_IDRP), Validation Exercise 100 (FVE_100).

  - Document Scope

This report presents the results of the Validation of the Cryptographic Primitives in IDRP as presented in document 9705 Sub-Volume V and Sub-Volume VIII, lists the ATN Validation Objectives that have been covered, and presents possible defects/omissions that create incomplete, inconsistent or ambiguous requirements contained therein.

  - Document Overview

This document comprises of the following sections:

- Section 1 is the introductory section

- Section 2 lists the objectives

- Section 3 describes the tools used in the validation and the validation strategy

- Section 4 lists possible defects or omissions, and gives possible suggestions for improvement

- Section 5 gives the results and analysis of the each validation objective

- Section 6 offers the conclusion of the validation effort

- Validation Objectives

Below is a summary of the validation objectives as allocated to FVE_100 for the Validation of ATN Cryptographic Primitives in IDRP.  Thus far, tests have only encompassed the ground-ground BIS interoperability scenarios.  In addition to the validation objectives below, the implementation of the tests took into consideration whether the applicable requirements for IDRP authentication as specified in Sub-Volume V and Sub-Volume VIII were complete, consistent, and unambiguous.  Requirements lacking in detail were reported in section 4 of this report.

- Table 1:  Summary of FVE_100 Validation Objectives

| Validation Objective | Validation Initiative | Validation Exercise | Exercise Title | Validation Achieved |
| --- | --- | --- | --- | --- |
| AVO-3_210 | FAA_IDRP | FVE_100 | Validation of the secure and unsecured exchange of IDRP information over a ground-ground path | e* |
| AVO-3_211 | FAA_IDRP | FVE_100 | Validation of the secure and unsecured exchange of IDRP information over an air-ground path |  |
| AVO-3_212 | FAA_IDRP | FVE_100 | Validation that compliant airborne and air-ground BISs supporting optional mutual entity authentication of routing exchanges are each able to authenticate IDRP PDUs received from the peer BISs. |  |
| AVO-3_213 | FAA_IDRP | FVE_100 | Validation that compliant airborne and air-ground BISs supporting the option to request and to attach a security certificate to an IDRP OPEN-PDU will interoperate. |  |
| AVO-3_214 | FAA_IDRP | FVE_100 | Validation of the IDRP security enhancements (backward interoperability) | e* |
| AVO-3_400 | FAA_IDRP | FVE_100 | Validation of a compliant ground and air-ground BISs capability to authenticate IDRP exchanges with peer BISs across a ground-ground path. | e* |
| AVO-3_401 | FAA_IDRP | FVE_100 | Validation of a compliant airborne and air-ground BISs capability to authenticate IDRP exchanges in the air-to-ground direction. |  |
| AVO-3_402 | FAA_IDRP | FVE_100 | Validation of a compliant airborne and air-ground BISs capability to mutually authenticate IDRP exchanges. |  |
| AVO-3_403 | FAA_IDRP | FVE_100 | Validation that the security information exchange and processing overhead for secure exchange of IDRP routing information between ground and air-ground BISs is not excessive. | e* |
| AVO-3_404 | FAA_IDRP | FVE_100 | Validation that the security information exchange and processing overhead for secure exchange of IDRP routing information between airborne and air-ground BISs is not excessive. |  |

Note - * Tests were conducted over the ground-ground paths only, thus a partial implementation of the AVO

- Table 2:  Validation Means

| Code | Description |
| --- | --- |
| a | Two or more independently developed interoperating implementations validated by two or more states/organizations. |
| b | Two or more independently developed interoperating implementations validated by one state/organization. |
| c | One implementation validated by more than one state/organization. |
| d | One implementation validated by one state/organization |
| e | Partial implementation validated by one state/organization. |
| f | Simulation, analysis using tools e.g. compiler, modeling tools. |
| g | Analysis and inspection. |

- Validation Strategy

  - Validation Tools Involved

Tests were conducted on an Intel workstation (Pentium III 800 MHz with 512 MB RAM) running multiple instances of the FAA Router software connected through simulated X.25 subnetworks.  A prototype of the elliptic curve cryptographic library, based upon the primitives specified in Sub-Volume 8 section 5, was integrated into the software to provide authentication level 2 support.

  - Validation Scenarios

In order to achieve the validation objectives that have been set forth, the following set of operational scenarios, for ground-ground exchange, have been tested.

    - Operational Scenario 1

Multiple instances of authentication level 2 compliant ground routers entered a secured exchange with mutual authentication.  Trace files for each router, B01 and B02, were generated to show each stage of the secure exchange along with timings of the cryptographic functions and results of cryptographic computations.  Successful completion of this test partially validated AVO-3_210 and AVO-3_400 as well as provided some processing overhead information for AVO-3_403.

    - Operational Scenario 2

Multiple instances of authentication level 2 compliant ground routers entered an unsecured exchange, authentication level 1.  Trace files for each router, B01 and B02, were generated to show each stage of the unsecured exchange. Successful completion of this test partially validated AVO-3_214 as well as provided some processing overhead information for AVO-3_403.

    - Operational Scenario 3

An instance of an authentication level 2 compliant ground router initiated an authentication level 1 unsecured exchange with a baseline authentication level 1 router. Trace files for each router, B01 and B02, were generated to show each stage of the unsecured exchange.  Successful completion of this test partially validated AVO-3_214 as well as provided some processing overhead information for AVO-3_403.

    - Operational Scenario 4

An instance of an authentication level 1 baseline ground router initiated an authentication level 1 unsecured exchange with an authentication level 2 compliant router. Trace files for each router, B01 and B02, was generated to show each stage of the unsecured exchange.  Successful completion of test partially validated AVO-3_214 as well as provided some processing overhead information for AVO-3_403.

  - Validation Assumptions

    - Because actual certificates have not yet been implemented, pseudo-certificates were used instead.  A router’s public key, computed with the ATN user elliptic curve domain parameters, was signed with a CA private key, computed with the ATN CA elliptic curve domain parameters, and sent as a certificate path.  Furthermore, since a CA was not available, each router was given access to a key generated over the ATN CA elliptic curve domain parameters in order to sign their own public key.

    - The MacKey was generated using the AKAS operation described in 8.5.4.2, but entity U was taken to mean the initiator and entity V, the responder.  Because the order of the concatenation is critical in generating a session key, this convention was adopted.

- Defect Report

  - FAA_IDRP_FVE_100-01

5.8.3.2.10.3.1.7.6	Requirement does make any allowance for point compression.   Perhaps to allow future migration the following enhancement of requirement 5.8.3.2.10.3.1.7.4 would be beneficial.

| Algorithm ID | Public Key Representation ID | Public Key |
| --- | --- | --- |
| Octet 1 and 2 | Octet 3 (0x04, 0x02, 0x03) | Octets 4…45 (X¦¦Y) or 4…24 (X) |

Where the Public Key Representation ID is encoded as hexadecimal 04 if no point compression is used, or hexadecimal 03 or 02 if point compression is used.  If the Public Key Representation is hexadecimal 04 then the public key is found in octets 4…45, however if the Public Key Representation is hexadecimal 03 or 02 then the public key is found in octets 4…24 (and computation must be performed by the receiver to retrieve the y-coordinate of the public key).

NOTE: Certicom’s patents encumber point compression, but perhaps allowing compression, thus saving bandwidth, would be wise.  However it seems uncompressed keys should be used by default unless there is foreknowledge that both sides support key compression.

  - FAA_IDRP_FVE_100-02

5.8.3.2.10.3.1.8.4	The Figure 5.8-8 and note show that there can be n CA certificates, each 127-octets long, but in fact the certificates are extensible so boxes for the certificates should probably be broken into length and value boxes for each individual certificate.  Also, concluding from W1S2F1702, the CA hierarchy of cross certification implies that the certificate path will contain at most one CA certificate.  Thus the certificate path would contain at most an ATN router certificate and a CA certificate.

  - FAA_IDRP_FVE_100-03

5.8.3.2.10.3.2.1	This requirement, under authentication level 2, should be somewhat analogous to requirement 5.8.3.2.10.1.4, under authentication level 1, but in comparison is lacking in description.  Suggested addition (analogue of clause 7.7.1 of ISO/IEC 10747):

- Generating a Validation Pattern:

Under authentication level 2 communication, the contents of the Validation Pattern field that is included in an outbound BISPDU (except the OPEN PDU which uses the method cited in 5.8.3.2.10.1.4) shall be generated by applying the ATN Keyed Message Authentication Code Generation Primitive (specified in 8.5.6.2.1) to the input data stream that consists of the contents of the entire BISPDU with all bits of the Validation Pattern field initially set to 0.  The output of this step is a 10-octet long keyed message authentication code, which shall be placed in the Validation Pattern field of the BISPDU.

b) Checking the Validation Pattern of an inbound BISPDU:

The contents of the Validation Pattern field of an inbound BISPDU shall be checked by applying the algorithm of 8.5.6.2.1 to the contents of the inbound BISPDU with its Validation Pattern set to all zeros.  Call this quantity the “reference pattern”.

If the “reference pattern” matches the contents of the Validation Pattern field of the inbound BISPDU, then the BISPDU’s keyed hash is correct; otherwise, it is incorrect.

NOTE: The Validation Pattern field is 16-octets long, but the ATN Keyed Message Authentication Code Generation Primitive generates a 10-octets long code.  Thus the 10-octets long code is placed in the low order portion of the Validation Pattern field with the remaining octets filled with zeros.

  - FAA_IDRP_FVE_100-04

8.5.4.2	As per assumption 3.3.2 of this report, references to U and V without defining them are ambiguous.  Consider defining U to be the initiator and V to be the responder.

- FVE_100 Results and Analysis

  - AVO-3 210

Verify that ground BISs and air-ground BISs will interoperate for the secure exchange of IDRP information. The provisions to authenticate IDRP exchanges with the peer BIS across a ground-ground path will be verified.

In the tests conducted two ground BISs were able to interoperate for both secured and unsecured exchanges provided the basic assumptions given under the Validation Strategy are accepted.  The resulting output files from the tests in the Appendix provides a trace of the secure exchanges.

  - AVO-3 211

Verify that compliant airborne and air-ground BISs supporting authentication of IDRP exchanges in the air-to-ground direction will interoperate.

Not tested.

  - AVO-3 212

Verify that compliant airborne BISs and air-ground BISs supporting the optional mutual authentication of IDRP exchanges are each able to authenticate IDRP PDUs received from the peer BIS.

Not tested.

  - AVO-3 213

Verify that compliant airborne BISs and air-ground BISs supporting the option to request and to attach a security certificate to an IDRP OPEN-PDU will interoperate.

Not tested.

  - AVO-3 214

Verify that a compliant airborne BIS, air-ground BIS and ground BIS will interoperate for the unsecured exchange of routing information with a peer BIS implemented in accordance with the current baseline Sub-Volume V provisions.

Tests conducted in which a ATN Security compliant ground BIS served as both a initiator and responder to a current baseline Sub-Volume V BIS verified that ATN Security implementations do not preclude authentication level 1 exchanges, i.e. an ATN Security compliant ground BIS was backwards compatible.

5.3	AVO-3 400

Evaluate the ground BIS and air-ground BISs capability to authenticate IDRP exchanges with the peer BIS across a ground-ground path.

Tests showed that a ground BIS was able to authenticate IDRP exchanges with the peer BIS across a ground-ground path.  Output in the form of trace files showed that both BIS calculated the same session key and subsequent exchanges tag with the HMAC were verifiable.

5.4	AVO-3 401

Evaluate the compliant airborne and air-ground BISs capability for authentication of IDRP exchanges in the air-to-ground direction.

Not tested.

5.5	AVO-3 402

Evaluate the compliant airborne BISs and air-ground BISs capability for the mutual authentication of IDRP exchanges in the air-to-ground direction.

Not tested.

5.6	AVO-3 403

Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between ground and air-ground BISs.

CPU usage*:

Under the current test configuration (Pentium III 800Mhz with 512MB RAM) with the cryptographic library prototype, the ATN Cryptographic Primitives, added CPU computation overhead of 10ms ± 10ms for key generation and signing.   However because a real-time UNIX was not used, the most accurate conclusion that can be made is that the computation time took less than 20 ms. The signature verification process required more CPU resources and took an average of 20ms ± 10 ms.

Bandwidth usage*:

For ground-ground exchanges the bandwidth increase for the OPEN PDU was 10 octets plus the number of octets used in the Certificate Path Parameter Value, which is variable. The pseudo-certificates used in the tests were 103-octets long (43-octets for the uncompressed public key and 60-octets for the signature).  This is not an exact figure as the certificate path will likely have two compressed certificates.

Note - * ATN Security compliant routers operating under authentication level 2 did not introduce significant overhead when compared to baseline authentication level 1 routers.

5.7	AVO-3 404

Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between airborne and air-ground BISs.

Not tested.

- Conclusions

Through the operational tests conducted over the ground-ground secured and unsecured IDRP exchanges, the William J. Hughes Technical Center achieved level e validation of AVO-3_210, AVO-3_214, AVO-3_400, and AVO-3_403. During implementation and testing a few minor defects/omissions in the Sub-Volume V and VIII requirements were found and reported in section 4 of this report.

- Appendix

  - Operational Scenario 1:

    - Output for B01

(See attached file, TEST1_IDRP_COMPLIANT_B01.auth2)

    - Output for B02

(See attached file, TEST1_IDRP_COMPLIANT_B02.auth2)

  - Operational Scenario 2

    - Output for B01

(See attached file, TEST2_IDRP_COMPLIANT_B01.auth1)

    - Output for B02

(See attached file, TEST2_IDRP_COMPLIANT_B02.auth1)

  - Operational Scenario 3

    - Output for B01

(See attached file, TEST3_IDRP_COMPLIANT_B01_INITIATOR.auth1)

    - Output for B02

(See attached file, TEST3_IDRP_BASELINE_B02_RESPONDER.auth1)

  - Operational Scenario 4

    - Output for B01

(See attached file, TEST3_IDRP_BASELINE_B01_INITIATOR.auth1)

    - Output for B02

(See attached file, TEST3_IDRP_COMPLIANT_B02_RESPONDER.auth1)