ATNP WG-1/SG-2

WP-2006

Aeronautical Telecommunications Network Panel (ATNP)

WG-1 — Systems Planning and Concept Working Group

SG-2 — Security Sub-group

Meeting 20

Toulouse, France

17-20 July 2000

Version 0.6A

Working Paper

Validation Report for the

Aeronautical Telecommunications Network (ATN)

Public Key Infrastructure (PKI) Laboratory

Prepared By: Honeywell Advanced Systems Technology Group

For

NASA Ames Research Center

SUMMARY

This report addresses the PKI Laboratory Validation program which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report on the planned activities and coverage of Sub-Volume VIII validation objectives and Sub-Volume VIII requirements.

DOCUMENT REVISION LOG

| Revision | Description | Date |
| --- | --- | --- |
| 0.1 | DRAFT – Sections 1, 2, and 4 | 03 February 2000 |
| 0.2 | DRAFT – Added detail to sections 4.3.1 and 6 | 01 March 2000 |
| 0.3 | DRAFT – Added detail to sections 3 and 6 | 04 April 2000 |
| 0.4 | First Distribution to WG1SG2 for Meeting 19 – Atlantic City | 09 May 2000 |
| 0.5 | Incorporate Comments fromWG1SG2 Meeting 19 | 15 May 2000 |
| 0.6 | Draft Distribution to WG1SG2 for SV8 Ad Hoc Meeting – Ft. Lauderdale; added detail to section 7, Results and Analysis | 19 June 2000 |
| 0.6A | Second Distribution to WG1SG2 for Meeting 20 – Toulouse, France. | 17-20 July 2000 |

TABLE OF CONTENTS

1.	Introduction	1

1.1.	Document Scope	1

1.2.	Document Overview	1

1.3.	Reference Documents	2

1.4.	Dependencies on External Standards	2

1.4.1.	PKI	2

1.4.2.	ASN.1/PER	2

1.5.	Terminology	2

2.	PKI Laboratory Validation Obectives	3

2.1.	Relationship to ATN Validation Objectives	3

2.2.	Relationship to SARPS Requirements	5

2.3.	High-Level PKI Laboratory Objectives	5

Table 2. Validation Objectives and Means of PKI Laboratory Validation Program	6

Table 3.  Validation Means	6

3.	PKI Laboratory Architecture	7

4.	PKI Laboratory Validation Strategy	9

4.1.	Overall Strategy	9

4.2.	Assumptions and Constraints	9

4.3.	Test Scenarios	9

4.3.1.	PKI Laboratory Independent Test Scenarios	10

4.3.1.1.	Key Management and Distribution	10

4.3.1.1.1.	Test Scenarios	10

4.3.1.1.2.	Expected Output	10

4.3.1.2.	ATN PKI Certificates	10

4.3.1.2.1.	Test Scenarios	10

4.3.1.2.2.	Expected Output	10

4.3.1.2.3.	Test Scenarios	15

4.3.1.2.3.1.	Certificates for CA, Key Certification Signing Use, AP-Title Subject	15

4.3.1.2.3.2.	Certificates for non-CA, Digital Signature Use, AP-Title Subject	16

4.3.1.2.3.3.	Certificates for non-CA, Key Agreement Use, AP-Title Subject	16

4.3.1.2.3.4.	Certificates for non-CA, Key Agreement Use, NET Subject	16

4.3.1.2.3.5.	Certificates for non-CA, Digital Signature Use, AMHS Subject	17

4.3.1.3.	ATN Compressed Certificates	18

4.3.1.3.1.	Test Scenarios	18

4.3.1.3.2.	Expected Output	18

4.3.1.3.3.	Test Scenarios	21

4.3.1.3.3.1.	Compressed CA Certificates	21

4.3.1.3.3.2.	Compressed Ground Certificates	21

4.3.1.3.3.3.	Compressed Router Certificates	21

4.3.1.4.	ATN Certificate Revocation Lists	22

4.3.1.4.1.	Test Scenarios	22

4.3.1.4.2.	Expected Output	22

4.3.1.4.3.	Test Scenarios	22

4.3.2.	PKI Laboratory with NASA Ames Entrust System Support Test Scenarios	25

4.3.2.1.	ATN Cryptographic Setting	25

4.3.2.1.1.	Test Scenarios	25

4.3.2.1.2.	Expected Output	25

4.3.2.2.	Interoperability of Independent Implementations	25

4.3.3.	Protocol Validation Efforts with PKI Laboratory Support Test Scenarios	26

5.	PKI Laboratory Environment	27

6.	Defect Report	29

6.1.	FVO2, FVO3 and FVO4 Defects Found During Test Scenario Preparation	29

6.1.1.	Defects addressed during WG1SG2 Meeting 19, Atlantic City	29

7.	Results and Analysis	32

8.	Conclusions	33

Validation Report for the

Aeronautical Telecommunications Network (ATN)

Public Key Infrastructure (PKI) Laboratory

# Introduction

## Document Scope

This report addresses the PKI Laboratory Validation program which is one of many programs established to support the validation of the ATN Security Provisions, Document 9705, Sub-Volume VIII.  The purpose of this document is to report on the planned activities and coverage of Sub-Volume VIII validation objectives and Sub-Volume VIII requirements.

## Document Overview

This document is organized as follows:

- Section 1 – Introduction

The introduction identifies the scope of the document, summarizes the document organization, provides references to applicable documents, identifies dependencies on external standards, and defines terminology used throughout the document.

- Section 2 – PKI Laboratory Validation Objectives

This section documents the relationship to high-level ATN validation objectives, describes the high-level PKI Laboratory objectives, and provides a cross-reference to specific SARPS requirements.

- Section 3 – PKI Laboratory Architecture

This section describes the architecture of the PKI Laboratory with respect to the ATN communications environment.

- Section 4 – PKI Laboratory Validation Strategy

This section defines the strategy for achieving the validation objectives, including assumptions, constraints and test scenarios.

- Section 5 – PKI Laboratory Environment

This section describes the PKI Laboratory environment including tools and techniques.

(Ed. note: The following sections are the "report" part of this document; they'll be used to capture defects and corrective actions, results and analysis, and conclusions.)

- Section 6 – Defect Report

- Section 7 – Results and Analysis

- Section 8 – Conclusions

## Reference Documents

Draft ATN SARPS Technical Provisions, ICAO Document 9705 Sub-Volume VIII, W1WP1708.

Appendix I, Doc 9705 – Sub-Volume VIII (Security), Validation Report, ATNP/3

## Dependencies on External Standards

### PKI

- ITU-T X.509

### ASN.1/PER

- ISO 8825-2

## Terminology

| ASN | Abstract Syntax Notation |
| --- | --- |
| ATN | Aeronautical Telecommunications Network |
| ATNP | Aeronautical Telecommunications Network Panel |
| CA | Certificate Authority |
| CM | Context Management |
| COTS | Commercial Off-The-Shelf |
| CRL | Certificate Revocation List |
| FVO | Functional Validation Objective |
| ICAO | International Civil Aviation Organization |
| IDRP | Inter-Domain Routing Protocol |
| PER | Packed Encoding Rules |
| PKI | Public Key Infrastructure |
| SARPs | Standards and Recommended Practices |
| SVO | System Validation Objective |
| TVO | Technical Validation Objective |
| ULCS | Upper Layer Communication Services |

# PKI Laboratory Validation Obectives

## Relationship to ATN Validation Objectives

The primary goal of the PKI Laboratory Validation program is to address the PKI portion of several Sub-Volume VIII Validation Objectives by conducting test scenarios defined in this document and documenting the objective, measurable results.  These primary Validation Objectives include FVO1, FVO6, TVO5, and TV08 as defined in Table 1.  A number of other, secondary Validation Objectives related to PKI will be addressed in a more subjective manner, during the conduct of the test scenarios defined in this document.  These secondary validation objectives are FVO2, FVO3, FVO5, TVO3, TVO4, TVO6, and TVO7.  Validation Objectives FVO7, TVO1, TVO3, and TVO4 are potential candidates for coverage by the PKI laboratory Validation program whereby the laboratory can be used to support other TBD validation programs as defined in section 4.3.3. The PKI Laboratory Validation program does not address Validation Objectives SVO1, SVO2, and SVO3.

Table 1.  High-level Sub-Volume VIII Validation Objectives Allocated to the PKI Laboratory Program

| Sub-Volume VIII Validation Objective | Description | Validation Objective Allocated to PKI Laboratory Program |
| --- | --- | --- |
| SVO 1 | To determine which system level requirements are satisfied by the Sub-Volume VIII requirements. | None. |
| SVO 2 | Validate that the Sub-Volume VIII requirements trace to other SARPS sub-volumes, where applicable. | None. |
| SVO 3 | Validate that Sub-Volume VIII includes provision for backward compatibility with prior versions of peer ATN implementations that do not incorporate security services. | None. |
| FVO 1 | Validate that Sub-Volume VIII supports implementation of local security policies and practices, within the boundaries of SARPs, as determined by States/Organizations. | Validate that the SARPs supports implementation of local PKI policies and practices, within the boundaries of the SARPs, as determined by States/Organizations. |
| FVO 2 | Validate that Sub-Volume VIII requirements are complete. | Validate that the Sub-Volume VIII PKI requirements are complete. |
| FVO 3 | Validate that Sub-Volume VIII requirements are unambiguous. | Validate that the Sub-Volume VIII PKI requirements are unambiguous. |
| FVO 4 | Validate that Sub-Volume VIII requirements are consistent. | Validate that the Sub-Volume VIII PKI requirements are consistent. |
| FVO 5 | Determine if there are any Sub-Volume VIII requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. | Determine if there are Sub-Volume VIII PKI requirements that would have no effect if removed. Note: Interpret this VO to mean that there are no requirements in Sub-Volume VIII that are not necessary for the intended functionality, or to achieve migration to future versions. It is not meant to eliminate possible duplicate statements of requirements. |
| FVO 6 | To determine if provision has been made to ensure that Sub-Volume VIII are implementation independent. | To determine if provision has been made to ensure that Sub-Volume VIII PKI requirements are implementation independent. |
| FVO 7 | To determine if Sub-Volume VIII includes provision for security services necessary for all security users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports security services necessary all security users. |
| TVO 1 | Validate that Sub-Volume VIII includes provisions for both mobile and fixed ATN users. | Validate that the Sub-Volume VIII includes provision for a PKI that supports both mobile and fixed ATN users. |
| TVO 2 | Validate that Sub-Volume VIII minimizes air-ground security related protocol overhead. | Validate that the Sub-Volume VIII includes provision for a PKI that supports minimal air-ground overhead to meet required security level. |
| TVO 3 | Validate that Sub-Volume VIII supports the security provisions of the ATN Upper Layer Communications Service (ULCS). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Upper Layer Communication Services (ULCS). |
| TVO 4 | Validate that Sub-Volume VIII supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). | Validate that the Sub-Volume VIII includes provision for a PKI that supports the security provisions of the ATN Inter-Domain Routing Protocol (IDRP). |
| TVO 5 | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. | Validate that independent implementations built in accordance to Sub-Volume VIII will be able to interoperate. |
| TVO 6 | To determine if the ATN security solution has any unacceptable behaviour. | To determine if the ATN security solution has any unacceptable behaviour. |
| TVO 7 | To determine if provision for future migration has been addressed. | To determine if provision for future migration has been addressed. |
| TVO 8 | To determine if the functionality described in Sub-Volume VIII is implementable. | To determine if the functionality described in Sub-Volume VIII is implementable. |

## Relationship to SARPS Requirements

The following individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements are to be validated by the PKI Laboratory effort.

8.3.1.1.4	Singular requirement regarding Public Key Certificates

8.3.1.2 [all]	Key Management and Distribution

8.3.1.9 [all]	Role of ATN Directory Servers within the ATN Security Framework

8.4.1 [all]	Certificate Policy

8.4.2 [all]	Certificate Practice Statement

8.4.3.1 [all]	ATN PKI Certificate Format – General Format

8.4.3.2 [all]	ATN PKI Certificate Format – ATN Certificates to be Signed

8.4.3.3 [all]	ATN PKI Certificate Format – Compressed Certificates

8.4.3.4 [all]	ATN PKI Certificate Format – ATN Certificate Path

8.4.4 [all]	ATN PKI CRL Format

8.4.5 [all] 	ATN PKI Certificate and CRL Validation

8.4.6 [all]	ATN PKI ASN.1 Module

8.5.2 [all]	ATN Cryptographic Setting

## High-Level PKI Laboratory Objectives

Table 2 identifies individual Sub-Volume VIII requirements or complete sections of Sub-Volume VIII requirements cross-referenced to specific Validation Objectives that are to be addressed by the PKI Laboratory effort.  Table entries indicate the validation means to be applied to the intersection of the requirement(s) and validation objective as defined in Table 3.

## Table 2. Validation Objectives and Means of PKI Laboratory Validation Program

| Requirements Grouping | Sub-Volume VIII Requirements | SVO |  |  |  | FVO |  |  |  |  |  |  | TVO |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 1 |  | 2 | 3 | 4 | 5 | 6 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| ATN Security Strategy |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Backward Compatibility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ULCS Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CM Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other Applications Security Services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Key Management and Distribution | 8.3.1.2, 8.3.1.9 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Certificate Authority Architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN PKI Certificates | 8.3.1.1.4, 8.4.1, 8.4.2, 8.4.3.1, 8.4.3.2, 8.4.6 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Compressed Certificates | 8.4.3.3, 8.4.3.4 |  |  |  | c |  | g | g | g | g | c | * | * | g | * | * | b | c | g | c |
| ATN Certificate Revocation Lists | 8.4.4 |  |  |  | c |  | g | g | g | g | c | * | * |  | * | * | b | c | g | c |
| ATN Cryptographic Setting | 8.5.2 |  |  |  |  |  | b | b | b | b | b |  |  |  |  |  | b | b | g | b |
| ATN Key Agreement Scheme (AKAS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Digital Signature Scheme (ADSS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Keyed Message Authentication Code Scheme (AMACS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Auxiliary Cryptographic Primitives and Functions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ATN Security Object (SSO) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note: “*” See section 4.3.3.

## Table 3.  Validation Means

| Code | Description |
| --- | --- |
| a | Two or more independently developed interoperating implementations validated by two or more states/organizations. |
| b | Two or more independently developed interoperating implementations validated by one state/organization. |
| c | One implementation validated by more than one state/organization. |
| d | One implementation validated by one state/organization. |
| e | Partial implementation validated by one state/organization. |
| f | Simulation, analysis using tools e.g. compiler, modeling tools. |
| g | Analysis and inspection. |

# PKI Laboratory Architecture

Figure 1.  PKI Validation Laboratory PKI Architecture

RA

CA

Sub CA

X.500 Directory Server

Peer Logic

Baltimore Technologies

Entrust

CA

RA

Legend:    CA = Certificate Authority

Sub CA = Subordinate Certificate Authority

RA = Registration Authority

This page intentionally left blank.

# PKI Laboratory Validation Strategy

## Overall Strategy

The overall strategy for the PKI Laboratory Validation effort is to validate selected ATN Validation Objectives and specific ATN Technical Provisions, ICAO Document 9705, Sub-Volume VIII requirements using Commercial Off the Shelf PKI Systems (FVO6).

## Assumptions and Constraints

TBD.

## Test Scenarios

The PKI Laboratory test scenarios fall into three major categories.  First category of tests scenarios are those which can be preformed with the PKI Laboratory resources and personnel independently.  The second category are those which the PKI Laboratory resources and personnel lead but require the resources and / or support of other agencies.  The third category are those which are primary conducted by other agencies but where the PKI Laboratory can play a support role.

### PKI Laboratory Independent Test Scenarios

#### Key Management and Distribution

##### Test Scenarios

Numerous test scenarios will be described in detail to address several CA architectures, for example, where a state permits aircraft operating entities to operate CAs, or where one commercial CA operates a CA on behalf of several states (FVO1).

##### Expected Output

The results of these test scenarios will be printed copies of certificates received a test CA representing one states’ CA from various test CAs representing CAs at differing hierarchical levels in another state. These printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08).  Any unacceptable behavior noted during the tests will be documented (TVO6).  The printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

#### ATN PKI Certificates

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPs requirements listed in Table 2 (FVO1).  The structure of an ATN Certificate is shown in Figure 1.

##### Expected Output

The results of these test scenarios will be electronic format, ASN.1 PER, and printed, human readable, copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any defects will be documented in section 6.

| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| --- | --- | --- | --- | --- | --- |
| version |  | v3 | INTEGER | 2 | 0, 1 |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| signature | Algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| issuer | AttributeType | X.501 Distinguished Name | OBJECT IDENTIFIER | X.501 Distinguished Name Types | Non-X.501 Distinguished Name |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| validity | NotBefore | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ or YYMMDDhhmm-HHMM | Non-YYMMDDhhmmZ or Non-YYMMDDhhmm-HHMM |
|  | NotAfter | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time after notBefore) | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time equal to or before notBefore) |
| subject | AttributeType | X.501 distinguished name for CA, Empty otherwise | OBJECT IDENTIFIER | X.501 Distinguished Name Types / Empty | Empty / Non-X.501 Distinguished Name Types |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| subjectPublicKeyInfo | Algorithm | Id-ecPublicKey | OBJECT IDENTIFIER | 1.2.840.10045.2.1 |  |
|  | Parameters | sect163r2 (for non-CAs) sect233r1 (for CAs) | OBJECT IDENTIFIER | 1.3.132.0.15 1.3.132.0.27 | Other OIDs |
|  | SubjectPublicKey | ECPoint | BIT STRING | 04¦¦xQ¦¦yQ | Non- 04¦¦xQ¦¦yQ |
| issuerUniqueID |  |  |  | Omitted | Any |
| subjectUniqueID |  |  |  | Omitted | Any |
| Extensions |  |  |  |  |  |
| AuthorityKeyIdentifier | ExtnID | id-ce-authorityKeyIdentifier | OBJECT IDENTIFIER | 2.5.29.35 | Other OIDs |
|  | Critical | Not Critical | BOOLEAN | FALSE | TRUE |
|  | ExtnValue - keyIdentifier |  | OCTET STRING | 0100 plus 60 bits SHA-1 hash of the subject Public Key | Other than Valid Values |
|  | ExtnValue – authorityCertIssuer | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
|  | ExtnValue - authorityCertSerialNumber |  | INTEGER | Any | Non-Integer |
| KeyUsage | extnID | id-ce-keyUsage | OBJECT IDENTIFIER | 2.5.29.15 | Other OIDs |
|  | critical | Critical | BOOLEAN | TRUE | FALSE |
|  | extnValue - Key Usage |  | BIT STRING digitalSignature (0) nonRepudiation (1) keyEncipherment (2) dataEncipherment (3) keyAgreement (4) keyCertSign (5) cRLSign (6) encipherOnly (7) decipherOnly (8) | 000000001 000010000 001100000 | All other combinations |
| SubjectAltName | extnID | id-ce-subjectAltName | OBJECT IDENTIFIER | 2.5.29.17 | Other OIDs |
|  | critical | Critical | BOOLEAN | TRUE | FALSE |
| General Names | extnValue - SubjectAltName | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 3: AMHS Address (See Note) 7:NET OCTET STRING SIZE (20..21) (See Note) 8:AP-Title (See Note) | 0: 1: 2: 4: 5: 6: |
| IssuerAltName | extnID | id-ce-issuerAltName | OBJECT IDENTIFIER | 2.5.29.18 | Other OIDs |
|  | critical | Critical | BOOLEAN | TRUE | FALSE |
| General Names | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Algorithm OID | algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | parameters | NULL | NULL | NULL | Non-NULL |
| Signature Value | signatureValue | r | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | s | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | Note: |  |  |  |  |
|  | AP-Title (Airborne) | 1.3.27.1.Integer(1..(2**24-1)).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | AP-Title (Ground) | 1.3.27.2.Integer(1..?).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | NET (Fixed AINSC) | 470027+01(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile AINSC) | 470027+41(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Fixed ATSC) | 470027+81(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile ATSC) | 470027+C1(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | AMHS Address | TBD |  |  |  |

Figure 2.  ATN Certificate Structure

This page intentionally left blank.

##### Test Scenarios

There are three certificate parameters that form the basis for the five major types of ATN Certificates:

- Is the certificate for CA or non-CA use

- Is the subject of the certificate a AP-Title, NET entity, or AMHS address

- Is the certificate used for Key Certification Signing, Digital Signature, or Key Agreement

Figure 2 illustrates the valid ATN combinations that form the basis for the test scenarios.

|  | Certificate Type |  |  |
| --- | --- | --- | --- |
| Subject | Key Certification Signing and CRL Signing | Digital Signature | Key Agreement |
| CA / AP-Title |  |  |  |
| Non-CA / AP-Title |  |  |  |
| Non-CA / NET |  |  |  |
| Non-CA / AMHS |  |  |  |
| Notes: 1. CA / NET is not a valid subject for ATN certificates. 2. Valid types apply to both standard and compressed ATN certificates |  |  |  |

Figure 3.  Valid ATN Certificates

###### Certificates for CA, Key Certification Signing Use, AP-Title Subject

This is an example of a certificate issued to State CAs by other State CAs or a certificate issued to AOE CAs by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 1, with the following specific values:

issuer							{x.501 Distinguished Name}

subject							{x.501 Distinguished Name}

subjectPublicKeyInfo

Parameters					1.3.132.0.27

AuthorityKeyIdentifier

extnValue – authorityCertIssuer		8: 1.3.27.2.x.x.xx.xxx

extnValue – authorityCertSerialNumber	x..x

KeyUsage

extnValue – KeyUsage			001100000

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Digital Signature Use, AP-Title Subject

This is an example of a certificate issued to ground CMA, aircraft CMA, AMHS, or other ground entities for their signing keys by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 1, with the following specific values:

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

AuthorityKeyIdentifier

extnValue – authorityCertIssuer		8: 1.3.27.2.x.x.xx.xxx

extnValue – authorityCertSerialNumber	x..x

KeyUsage

extnValue – KeyUsage			000000001

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Key Agreement Use, AP-Title Subject

This is an example of a certificate issued to ground CMA, aircraft CMA, or other ground entities for their key agreement keys by State CAs.

This test scenario utilizes the ATN Certificate described in Figure 1, with the following specific values:

serialNumber						x..x

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

AuthorityKeyIdentifier

extnValue – authorityCertIssuer		8: 1.3.27.2.x.x.xx.xxx

extnValue – authorityCertSerialNumber	x.x

KeyUsage

extnValue – KeyUsage			000010000

SubjectAlternateName

extnValue – SubjectAltName			8: 1.3.27.2.x.x.xx.xxx

Issuer Alternate Name

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Key Agreement Use, NET Subject

This is an example of a certificate issued to ground routers or aircraft routers for their key agreement keys by State CAs.

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

AuthorityKeyIdentifier

extnValue – authorityCertIssuer		8: 1.3.27.2.x.x.xx.xxx

extnValue – authorityCertSerialNumber	x..x

KeyUsage

extnValue – KeyUsage			000010000

SubjectAlternateName

extnValue – SubjectAltName

0: 470027+81xxxxxx00xxxxxxxxxxxxxxxxxxxxxxxx

IssuerAlternateName

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

###### Certificates for non-CA, Digital Signature Use, AMHS Subject

This is an example of a certificate issued for AMHS digital signature key by State CAs.

issuer							{x.501 Distinguished Name}

subject							empty

subjectPublicKey Info

Parameters					1.3.132.0.15

AuthorityKeyIdentifier

extnValue – authorityCertIssuer		8: 1.3.27.2.x.x.xx.xxx

extnValue – authorityCertSerialNumber	x..x

KeyUsage

extnValue – KeyUsage			000000001

SubjectAlternateName

extnValue – SubjectAltName			3:

IssuerAlternateName

extnValue – issuerAltName			8: 1.3.27.2.x.x.xx.xxx

#### ATN Compressed Certificates

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of compressed certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPs requirements listed in Table 2 (FVO1).  The structure of an ATN Compressed Certificate is shown in Figure 3.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| --- | --- | --- | --- | --- | --- |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| validity | NotBefore | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ or YYMMDDhhmm-HHMM | Non-YYMMDDhhmmZ or Non-YYMMDDhhmm-HHMM |
|  | NotAfter | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time after notBefore) | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time equal to or before notBefore) |
| subjectPublicKeyInfo | SubjectPublicKey | ECPoint | BIT STRING | 04¦¦xQ¦¦yQ | Non- 04¦¦xQ¦¦yQ |
| SubjectAltName | extnValue - SubjectAltName | General Names | [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 3: AMHS Address (See Note) 7:NET OCTET STRING SIZE (20..21) (See Note) 8:AP-Title (See Note) | 0: 1: 2: 4: 5: 6: |
| IssuerAltName | extnValue - issuerAltName | General Names | CHOICE [0] Othername [1] IA5String [2] IA5String [3] ORAddress [4] Name [5] EDIPartyName [6] IA5String [7] OCTET STRING [8] OBJECT IDENTIFIER | 8:AP-Title (See Note) | 0: 1: 2: 3: 4: 5: 6: 7: |
| Signature Value | signatureValue | r | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | s | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | Note: |  |  |  |  |
|  | AP-Title (Airborne) | 1.3.27.1.Integer(1..(2**24-1)).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | AP-Title (Ground) | 1.3.27.2.Integer(1..?).Integer(0, 2).Integer(0..10),Integer(1-(2**8-1)) |  |  | See ICAO Doc 9705 Section 4.3 |
|  | NET (Fixed AINSC) | 470027+01(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile AINSC) | 470027+41(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Fixed ATSC) | 470027+81(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | NET (Mobile ATSC) | 470027+C1(3 Octets)00(3 Octets)(2 Octets)(6 Octets)(1 Octet) |  |  | See ICAO Doc 9705 Section 5.4 |
|  | AMHS Address | TBD |  |  |  |

Figure 4.  ATN Compressed Certificate Structure

##### Test Scenarios

###### Compressed CA Certificates

This test consists of generating a compressed certificate version of the certificate described in section 4.3.1.2.3.1.

###### Compressed Ground Certificates

This test consists of generating compressed certificate versions of the certificates described in section 4.3.1.2.3.2 and 4.3.1.2.3.3.

###### Compressed Router Certificates

This test consists of generating compressed certificate versions of the certificates described in section Error: Reference source not found and 4.3.1.2.3.4.

#### ATN Certificate Revocation Lists

##### Test Scenarios

Numerous test scenarios will be described in detail to address the generation of compressed certificates for a variety of values for each certificate field as well as for a variety of policies and practices within the boundaries of the SARPS requirements listed in Table 2 (FVO1). The structure of an ATN Certificate Revocation List is shown in Figure 4.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of certificates generated in accordance with the variables defined in the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

##### Test Scenarios

TBD

| Title | ASN.1 Name | ASN.1 Production Sequence | Tag | Valid Values | Invalid Values |
| --- | --- | --- | --- | --- | --- |
| version |  | v3 | INTEGER | 2 | 0, 1 |
| signature | Algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | Parameters | NULL | NULL | NULL | Non-NULL |
| issuer | AttributeType | X.501 Distinguished Name | OBJECT IDENTIFIER | X.501 Distinguished Name | Non-X.501 Distinguished Name |
|  | AttributeValue | ANY DEFINED BY AttributeType | Type Dependent | Type Dependent | Type Dependent |
| thisUpdate | Time | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ or YYMMDDhhmm-HHMM | Non-YYMMDDhhmmZ or Non-YYMMDDhhmm-HHMM |
| nextUpdate | Time | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time after thisUpdate) | YYMMDDhhmmZ YYMMDDhhmm-HHMM (any time equal to or before thisUpdate) |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| revocationDate | Time | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ or YYMMDDhhmm-HHMM | Non-YYMMDDhhmmZ or Non-YYMMDDhhmm-HHMM |
| crlExtentions |  |  |  | absent |  |
| … |  |  |  |  |  |
| serialNumber | CertificateSerialNumber |  | INTEGER | Any | Non-Integer |
| revocationDate | Time | Coordinated Universal Time to minutes or Local time with offset from Coordinated Universal Time | UTCTime | YYMMDDhhmmZ or YYMMDDhhmm-HHMM | Non-YYMMDDhhmmZ or Non-YYMMDDhhmm-HHMM |
| crlExtentions |  |  |  | absent |  |
| Algorithm OID | algorithm | ecdsa-with-SHA1 | OBJECT IDENTIFIER | 1.2.840.10045.4.1 | Other OIDs |
|  | parameters | NULL | NULL | NULL | Non-NULL |
| Signature Value | signatureValue | R | INTEGER | 1 to n-1 | Other than 1 to n-1 |
|  | signatureValue | S | INTEGER | 1 to n-1 | Other than 1 to n-1 |

Figure 5.  ATN Certificate Revocation List Structure

This page intentionally left blank.

### PKI Laboratory with NASA Ames Entrust System Support Test Scenarios

#### ATN Cryptographic Setting

##### Test Scenarios

Numerous test scenarios will be described in detail to utilize the ATN cryptographic settings in the PKI system.  Proper functionality will be verified by having keys generated and certificates signed by a CA implemented on one COTS PKI system verified by a CA implemented on a PKI system implemented on a different COTS system.

##### Expected Output

The results of these test scenarios will be electronic format and printed copies of keys and certificates generated in accordance with the test scenario.  These electronic format and printed copies will be examined to validate that they conform to the functionality defined by specified Sub-Volume VIII requirements (TV08). Any unacceptable behavior noted during the tests will be documented (TVO6).  Both the printed copies of the certificates and validation results will be documented in Section 7 and any defects documented in Section 6.

All of the remaining validation objectives for this section will be performed by Inspection during the preparation for and conduct of the test scenarios.  Any findings will be documented in section 6.

#### Interoperability of Independent Implementations

(Ed. note: This section will contain a subset of test scenarios from the section 4.3.1 but specify the use of two different PKI systems within the test scenario architecture.)

### Protocol Validation Efforts with PKI Laboratory Support Test Scenarios

There are no firm plans to execute any test scenarios in this section, however, it likely that ATN Certificates, Compressed Certificates and CRLs generated under proceeding PKI Laboratory test scenarios could be of value in validation efforts requiring these types of data as inputs.  The variety of certificates and CRLs generated under the PKI test scenarios might provide a range of inputs to more fully exercise these security services than might otherwise be possible.

# PKI Laboratory Environment

(Ed. note: This section of the report will be completed upon analysis of the scope required by the test scenarios described in section 4 and architecture defined in section 3.)

This page intentionally left blank.

# Defect Report

## FVO2, FVO3 and FVO4 Defects Found During Test Scenario Preparation

### Defects addressed during WG1SG2 Meeting 19, Atlantic City

The following Sub-volume VIII requirements were identified as incomplete, ambiguous or inconsistent during the preparation of the test scenarios for this report and addressed, as noted during WG1SG2 Meeting 19, Atlantic City.

- 8.4.3	The subsection numbering, starting at 8.4.3 should be restructured to follow the major and minor components of the certificate as shown in Figure 2.  [Accept: Tom McP. to review paragraph indentation.]

- 8.4.3.1.3.3	Suggest adding a reference to section 8.5.4.2.1 5) and 7) for the values of r and s respectively to reduce ambiguity and for completeness.  [Accept]

- 8.4.3.2.5	The requirement does not state how the values for type and value are established.  There are a great number of Distinguished Name types.  Recommend that SV VIII define which are to be used within ATN.  It is possible that the AE Title or NET could be added to the distinguished name, thereby eliminating the need for those two certificate extensions.  For example, AE Name could be put in AttributeType “Organizational Role” and NET could be put into AttributeType “Device”.  Other AttributeTypes such as “Country”, Organization”, and “Organizational Unit” could be made mandatory to make the certificates human readable.  In the case of compressed certificates, only the “Organizational role” or “Device” AttributeType would be mandatory.  [Accept: Tom McP. to review directory tree specification and incorporate DN ordering constraints into SV8.]

- 8.4.3.2.7.1	It is not clear how the values for subject are established.  Recommend adding a reference to 8.4.3.2.5 where the syntax for X.501 Distinguished Name is defined.  [Accept.]

- 8.4.3.2.8.1	Is SEC1, Standards for Efficient Cryptography Group, a valid reference for the OIDs defined in 8.4.3.2.8.2 through 8.4.3.2.8.5?  Provide the correct reference for the OID secids modules used in 8.4.3.2.8.5.  [Reject: Draft SV8 Ver 1.0.2 (WP1911) changed to reflect X9.62 with ATN labels.]

- 8.4.3.2.8.4	This appears to be an incomplete OID.  For example, should atnPI-arc be atnPKI-arc?  Also, the OBJECT IDENTIFIER for secids modules is not defined.   [Reject: Draft SV8 Ver 1.0.2 (WP1911) changed to reflect X9.62 with ATN labels.]

- 8.4.3.2.8.5	There is not a reference to how the octet string for ECPoint is derived.  There does not appear to be any equations in section 8.5 defining this.  The recommendation is to treat this like the base point as defined in 8.5.2.3.1.5 and providing a reference to values for the ECPoint, by adding the following to section 8.4.3.2.8.5:

The value of ECPoint  =  04||xQ||yQ, as defined in ANSI X9.62 - 1998, section 4.3.6, where 04 indicates that point compression is not being used.  Values are as defined in ANSI X9.62 - 1998, section 5.2.1.  [Accept: Text from SEC to be added as note to SV8.]

- 8.4.3.2.9	Is “omitted” valid ASN.1 terminology?  Should this say “empty” to be consistent with other requirements such as 8.4.3.2.7.2?  [Reject: "omitted" is correct per PKIX.]

- 8.4.3.2.10	Is “omitted” valid ASN.1 terminology?  Should this say “empty” to be consistent with other requirements such as 8.4.3.2.7.2?  [Reject: "omitted" is correct per PKIX.]

- 8.4.3.2.11.1.3	Recommend adding the reference to extnValue to this requirement to make it unambiguous. [Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.]

- 8.4.3.2.11.1.4	Recommend merging with 8.4.3.2.11.1.5 as this is not a meaningful stand-alone requirement.  Recommend adding a reference to section 8.5.6.3 that specifies how the value is determine.   [Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.]

- 8.4.3.2.11.1.5	Recommend a requirement explicitly stating that authorityCertIssurer and authorityCertSerialNumber are not used in ATN since these are listed as optional fields in X.509.  [Accept: Text to be added stating that these are omitted.]

- 8.4.3.2.11.2.3	Recommend adding the reference to extnValue to this requirement to make it unambiguous.   [Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.]

- 8.4.3.2.11.3.3	Recommend adding the reference to extnValue to this requirement to make it unambiguous.   [Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.]

- 8.4.3.2.11.3.5	Recommend adding a reference to Sub-Volume IV, section 4.3 to clarify how an AP-Title is structured to be consistent with the reference to NET in section 8.4.3.2.11.3.8.  [Accept: Add reference to SV9.]

- 8.4.3.2.11.3.7	Recommend adding a reference to the ICAO assigned OIDs that are valid for use as a type-id value.  [Reject:  "otherName" replaced with IP address octet string.]

- 8.4.3.2.11.4.3	Recommend adding the reference to extnValue to this requirement to make it unambiguous. [Reject: No longer valid with Draft SV8 Ver 1.0.2 (WP1911) changes.]

- 8.4.3.3.2.1	Recommend deleting “…” as it doesn’t appear to be meaningful.  [Reject: Valid ASN syntax.]

- 8.4.3.3.2.1	Recommend clarifying how the values of subjectAETitle and issuerAETitle are specified.  [Accept:  Add reference to SV9.]

- 8.4.3.3.3.1	Recommend deleting “…” as it doesn’t appear to be meaningful.  [Reject: valid ASN syntax.]

- 8.4.3.3.3.1	Recommend clarifying how the values of subjectAETitle and issuerAETitle are specified. [Accept:  Add reference to SV9.]

- 8.4.3.3.4.1	Recommend deleting “…” as it doesn’t appear to be meaningful.  [Reject: valid ASN syntax.]

- 8.4.3.3.4.1	Recommend clarifying how the values of subjectNET and issuerAETitle are specified. [Accept:  Add reference to SV9.]

- Recommendation	Sub-Volume VIII does not specify a cross-certification request protocol or procedures.  With the number CAs that will need to cross-certify, specifying a protocol and possibly a procedure would be beneficial to the smooth operation of the PKI.  [Reject: Guidance issue. Eurocontrol does not want to add requirements that necessitate an electronic solution.]

[In addition,  SG2 recommended that encoding rules specified in SV8 be changed as follows: DER for standard certificates and CRLs and PER for compressed certificates.  This facilitates use of COTS PKI applications, which support DER, for standard certificate and CRLs, which are not sent over the air.  Compressed certificates must be PER encoded since these are sent over the air.]

# Results and Analysis

Based on the validation tests performed to date and questions raised during ATNP WG1SG2 Meeting 19, Honeywell surveyed Baltimore Technologies and Entrust to determine if the specific Sub-Volume VIII requirements can be met by commercial-off-the-shelf products.  The following table contains the results of the survey:

| Requirement | Baltimore Technologies | Entrust |
| --- | --- | --- |
| Issuer Alternate Name | Scheduled for 2nd quarter 2001, however, will not likely support being set as critical | Supported |
| Subject Alternate Name | Supported, however, cannot be set as critical | Modifiable to permit any documented or undocumented syntax by using the E-mail (subjectAltName): field |
| Key Usage | Supported | Supported |
| User Specified Elliptic Curve Parameters | Not available, not currently scheduled, but being considered | Can be specified by providing detailed parameters, not covered by Entrust’s validation efforts |
| Using OIDs within Distinguished Names | Not answered | Not recommended |

# Conclusions