# -DRAFT (version 2) - 17 November 1998-

# ATNP WG2 Response to AMCP WGA on AMSS Issue

1.	BACKGROUND

This communiqué from WG2 of the ATNP was prepared in response of a communiqué received by the panel secretary from the AMCP WGA dated 5 November 1998.    The communiqué from the AMCP WGA stated:

"In updating the current AMSS SARPs, the question has arisen as to whether an ATN-compliant air/ground subnetwork needs explicitly to require the provision of Connectivity Events (Join & Leave) across the ground-side interface, as well as across the air-side interface.    A timely response from ATNP, in time for consideration in the AMCP/WGA/14 meeting (19-29 January 1999) would be appreciated."

This question from the AMCP WGA relates to a prior communiqué from the ATNP WG2 to AMCP dated 26 June 1997 which stated in part:

“For ATN mobile subnetworks (such as AMSS) capable of generating a ‘leave event’, the router will rely on the receipt of the leave event as the primary means of determining connectivity via the mobile subnetwork has been lost.    This will result in an update to the routing tables within the router.    Delays in the subnetwork issuing a leave event can result in the loss of data packets to/from the affected aircraft.    This would have the operational consequence of reduced service availability.    It has been generally recognized that it is more efficient for a subnetwork to employ internal mechanisms to detect loss of aircraft/ground connectivity than the alternative of using the ATN routing protocols for this purpose (i.e., use of inter-domain routing protocol keepalive PDUs).    Working Group 2 of the ATNP encourages the AMCP to review the AMSS SARPs and guidance material to insure that the provisions contained therein include sufficient requirement and guidance to ensure that implementations of the AMSS will be able to generate the leave event in a timely manner after loss of connectivity.    This capability will insure that the operational utility of the services offered by the ATN/AMSS are not compromised.”

- GENERAL REQUIREMENTS

2.1	Provisions of Doc 9705

The ATN SARPs technical provisions are contained in the recently published first edition of ICAO Doc 9705.    As far as mobile subnetworks are concerned the focus of Doc 9705, Sub-Volume V (Internet Communications Service) is on the requirements for the mobile Subnetwork Dependent Convergence Function (SNDCF).    However, paragraph 5.2.5 of Doc 9705 does define a number of requirements for ATN subnetworks.    Section 5.2.5.1 defines requirements applicable to any (i.e., fixed or mobile) ATN subnetwork while para. 5.2.5.2 defines additional requirements specific to ATN mobile subnetworks.    These provisions are generally of a functional nature and do not explicitly require mobile subnetworks to generate join and leave events.    However para. 5.2.5.2.4.1 does require:

“An ATN Mobile Subnetwork shall provide a connection-mode service between SNPAs, with a well-defined start and end to a connection, and with a reliable, sequenced SNSDU transfer over that connection.”

Doc 9705 notes in para. 5.2.5.2.5:

“ATN Mobile Subnetworks may provide a mechanism for detection of change in media connectivity and for the conveyance of this information to connected ATN routers.”

Doc 9705 further requires in para. 5.2.5.2.5.1:

“If a Mobile Subnetwork provides subnetwork connectivity information, the subnetwork shall convey this information to connected subnetwork service users (i.e., connected ATN routers), in order to initiate operation of the internetwork protocols as specified in 5.3.”

  - Provisions of ICAO Manual of ATS Data Link Applications

The ICAO ADS Panel has produced an ICAO Manual of ATS Data Link Applications.    In this document the operational requirement of for the initial data link applications are defined.    The following table and text are excerpted from this document.

| Application | Availability | Integrity | Reliability | Continuity |
| --- | --- | --- | --- | --- |
| DLIC | 99.9% | 10-6 | 99.9% | 99.9% |
| ADS | 99.996% | 10-7 | 99.996% | 99.996% |
| CPDLC | 99.99% | 10-7 | 99.99% | 99.99% |
| FIS | 99.9% | 10-6 | 99.9% | 99.9% |
| AIDC | 99.996% | 10-7 | 99.9% | 99.9% |
| ADS-B | 99.996% | 10-7 | 99.996% | 99.996% |

Table 3A-2:    Application Specific Performance Requirements

“Except in catastrophic situations, no single end-to-end outage should exceed 30 seconds (end-to-End availability may be achieved through provision of alternate communications routings where feasible).”

It is important to note the very demanding requirements for continuity of service and service availability for the CPDLC and ADS (contract) applications.    As noted in the accompanying text (above) the availability may be achieved through provision of alternate communications routing.    Although not explicitly stated, this comment could also be applicable to the requirement for continuity of service.

- REQUIREMENTS FOR JOIN EVENT

  - Doc 9705 Provisions for Join Event and Alternative Procedures

Under Doc 9705 para. 5.3.5.2 (i.e., Air/Ground Route Initiation) there are additional requirements levied on mobile subnetworks related to use of Join Events.    Specifically para. 5.3.5.2.1.1 states

”BIS-BIS communications over a Mobile    Subnetwork shall be either air-initiated or ground-initiated, with one of these two modes of operation selected for all instances of a given subnetwork type.

Note 1.— Three classes of procedures are distinguished by this specification. These are: (a) Air-Initiated i.e. when the Airborne Router initiates the procedure, (b) Ground-Initiated i.e. when the Air/Ground Router initiates the procedure, and (c) Air or Ground-Initiated i.e. when either the Airborne or the Air/Ground Router may initiate the procedure.

Note 2.— Two types of Mobile Subnetworks are also recognised by this specification. These are: (a) those which provide information on the availability of specific Mobile Systems on the subnetwork through the Join Event defined in this section, and (b) those which do not. The latter type are only appropriate to Route Initiation Procedures which are Air-Initiated.

Note 3.— For a given Mobile Subnetwork type, the use of air-initiated or ground-initiated procedures, and the implementation of Join Events is outside of the scope of this specification, and is a matter for the SARPs specified by the relevant ICAO panel.

Note 4.—    The interfaces to all Mobile Subnetworks are assumed to be compatible with ISO/IEC 8208. The ISO/IEC 8208 term Data Terminal Equipment (DTE) is also used in this specification to refer to a system attached to a Mobile Subnetwork.”

Doc 9705 defines in para. 5.3.5.2.3.1 the “airborne router procedures for use of an ISO/IEC 8208 Mobile subnetwork that does not provide information on subnetwork connectivity.    Doc 9705 then goes on to provide in para. 5.3.5.2.3.2 the “airborne router procedures for use with ISO/IEC 8208 Mobile Subnetwork that does provide connectivity information.”

In the case where the mobile subnet does not provide a join event (either air or ground) the following alternative procedure is defined:

5.3.5.2.3.1.1.1    An Airborne Router’s IS-SME shall be configured with a list of subnetwork addresses for each supported Mobile Subnetwork that does not provide information on subnetwork connectivity.

5.3.5.2.3.1.1.2    This list shall include the addresses which are necessary to meet the communication needs of the aircraft.

Note.— In the case of the AMSS, the Airborne Router’s IS-SME will be configured with a list for each GES that the aircraft may use to communicate. Each such list will include the subnetwork addresses (e.g. DTE addresses) of the Air/Ground routers attached to the GES in question through which communications services may be required.

5.3.5.2.3.1.1.3    An Airborne Router’s IS-SME shall continually issue a Call Request to each subnetwork address on each appropriate list with which it does not currently have a subnetwork connection and which is not subject to a back-off period (see 5.3.5.2.3.1.2), in turn.

5.3.5.2.3.1.1.4    The period between each successive Call Request shall be configurable to ensure that the Mobile Subnetwork is not rendered unavailable.

5.3.5.2.3.1.1.5    When a subnetwork connection is successfully established, then the procedures of 5.3.5.2.6 shall be applied to that subnetwork connection. The polling procedure shall continue for the remaining subnetwork addresses on the list, if any.

  - Conclusion on the Use of Join Events

Doc 9705 allows a subnetwork to alternatively::

- generate a Join Event on the aircraft

- generate a Join Event on the ground

- generate a Join Event both on the aircraft and on the ground

- use an alternative to providing support for Join Events

Alternatives a, b, and c will provide an equivalent capability to, in effect, trigger the exchange of routing information between the airborne and air-ground routers.    Alternative (d) above however will not provide the equivalent level of service in that for the case of an AMSS subnetwork, the airborne router would need to have prior knowledge of every possible AMSS GES and could fail to establish connections with any new GES or GES with revised network addresses.    Also the polling procedure defined as an alternative to the use of Join Events would introduce additional delays in establishment of new routes potentially resulting in reduced service availability and continuity.    This could limit the ability of the overall communications service to satisfy the end-to-end requirements for service availability and continuity as established by the ADS Panel.    Thus this alternative would be most appropriate for use within a very constrained environment, such as might be the case for operational trials with a static ground subnetwork topology.

It is the conclusion of ATNP WG2 that mobile subnetworks intended to support safety related ATSC services (e.g., CPDLC, ADS, etc.) should generate a Join Event either on the aircraft, on the ground or both.

Thus in direct response to one aspect of the question from the AMCP WGA, it is not necessary to generate the Join Event on the ground if it is generated on the aircraft.

- REQUIREMENTS FOR LEAVE EVENT

  - Doc 9705 Provisions for Leave Event and Alternative Procedures

Doc 9705 describes in para. 5.3.5.2.13 the requirements for “air/ground route termination.”    It is noted that there are the following two procedures by which a Leave Event can be generated:

“The ‘Leave Event’ is defined to signal when subnetwork connectivity with a remote ATN Router over a Mobile Subnetwork ceases to be available. This event may be generated by (a) the subnetwork itself using mechanisms outside of the scope of this specification, or (b) the SNDCF when it receives a clear indication from the subnetwork reporting either a network or a user initiated call clearing. The Leave Event is always reported to the IS-SME.”

Thus a mobile subnetwork using ISO/IEC 8208 protocol, such as AMSS, could either generate a leave event or clear the 8208 call when air/ground connectivity is lost.

Doc 9705 also defines the following additional mechanism to allow a router to detect the case where air/ground connectivity has in fact been lost, but the mobile subnet has neither issued a Leave Event nor cleared the subnetwork connection:

“5.3.5.2.13.2	When a Mobile Subnetwork does not provide a    network generated Clear Indication (e.g. to indicate that an aircraft has left the range of the Mobile Subnetwork, or when some other communication failure occurs, etc.), an ATN Router shall maintain a “watchdog” timer for each affected subnetwork connection and clear each such subnetwork connection once activity has ceased for a configurable period.

5.3.5.2.13.3	When such a “watchdog” timer expires, this shall be reported as a “Leave Event” for that subnetwork connection.”

Note that the use of the watchdog timer mechanism can be considered a means of detecting a failure within the subnetwork, and is not to be considered as a primary means of monitoring the status of subnetwork connectivity.

  - Conclusion on the Use of Leave Events

Doc 9705 requires mobile subnetworks to either issue Leave Events or to clear subnetwork connections when that mobile subnetwork can no longer provide connectivity between the local and remote ATN routers (i.e., between the peer airborne and the air-ground routers).    Unlike the requirement for the Join Event, this requirement applies equally to both the ground and airborne cases.    In order for both the airborne and the air-ground routers to be able to make a valid routing decision they must know when a mobile subnetwork can no longer provide connectivity to the peer router.    Otherwise, the overall service availability and continuity will be compromised since this could result in the router continuing to send packets to a subnetwork that no longer provides the needed connectivity while there is in fact another subnetwork available that does offer the desired connectivity.

In direct response to one aspect of the question from the AMCP WGA, it is the conclusion of ATNP WG2 that mobile subnetworks are required to generate either a Leave Event or a ‘call clear’ (e.g. subnetwork generates an ISO 8208 ‘clear request’ resulting in a ‘clear indication’ at the router) to both the airborne and the air-ground routers.    Of these two alternatives the use of Leave Events is preferred as it may result in a more timely update of the router’s internal routing information data base.

Furthermore, WG2 would like to elaborate on the issue from the previous communiqué from ATNP WG2 to AMCP:

“Working Group 2 of the ATNP encourages the AMCP to review the AMSS SARPs and guidance material to insure that the provisions contained therein include sufficient requirement and guidance to ensure that implementations of the AMSS will be able to generate the leave event in a timely manner after loss of connectivity.    This capability will insure that the operational utility of the services offered by the ATN/AMSS are not compromised.”

There are two aspects to this latter topic related to the latency of issuing the Leave Event.    In this context, latency is considered to be the time from when the subnetwork is no longer capable of transferring data packets over an existing connection until the time the Leave Event is actually issued.    The first case of interest is the latency of the Leave Event in the absence of any internet traffic.    That is how fast can the mechanisms solely internal to the subnetwork detect the loss of connectivity and then issue the Leave Event.    The second case is the latency of the Leave Event upon receipt of a packet from the router.    This is the case where the subnetwork, through its internal mechanisms, has not yet detected that connectivity has been lost and it is only after receipt of a packet(s) from the router that the subnetwork is able to determine the loss of connectivity.