ATNP/WG2-10
Flimsy #2
Issue 1.0, 8-Oct-96

AVO Coverage Summary

This flimsy summarises the AVOs for which no exercise cross-references have been established in submitted WP340. These gaps should be revisited in the light of newly added contributions (i.e. US and SITA/VDL) and other coverage assessments available.

AVO_111	Verify that Mode S subnetworks have been implemented for support of ATN

communications and demonstrated to be SARPs compliant

AVO_113	Verify that VHF subnetworks have been implemented for support of ATN communications

and demonstrated to be SARPs compliant

AVO_114	Verify that CIDIN subnetworks have been implemented for support of ATN

communications and demonstrated to be SARPs compliant

AVO_203	Verify that two compliant ATN End Systems supporting different protocol profiles

(support of ATN recommendations) interoperate and provide the Transport Service.

Several exercises are needed to investigate different transport and network options.

The exercises should address: TPDU size negotiations including use of new parameter

'preferred maximum TPDU size', use/non-use of checksum, extended format and

associated data transfers, selective acknowledgement (both transfer formats),

request for acknowledgement (both transfer formats).

AVO_312	Verify that the ATN can be designed to accommodate normal traffic and peak traffic.

The traffic expectations are still to be defined. It is proposed that validation

exercises make assumptions on what will be these traffic expectations for a given

region of the ATN (e.g. Europe) and evaluate the possible ATN designs.

AVO_409	Evaluate the reliability of the IDRP transport mechanism (number of retransmissions,

transmission errors)

AVO_423	Show that the above holds with the simulation of many aircraft simultaneously.

(Show that when there is a change in the route to an aircraft, the time taken between

the loss of communication and the establishment of a replacement communications

path neither results in the loss of a transport connection between the ground system

and aircraft, nor does the transit delay increase beyond an acceptable minimum QoS.)

AVO_429	Evaluate the impact on IDRP of additional subnetwork connections between an

air/ground and an airborne router, and the handover from one air/ground router to

another.

AVO_435	Verify that once the applied load on the ATN exceeds its design limits that network

performance degrades gracefully, rather than catastrophically. In particular, verify

that degraded performance is experienced in strict priority order, with performance

degraded first for lower priority  data, and that higher priority transport

connections are not lost before lower priority transport connections following the

same or  a similar route.

This objective may be satisfied by simulation of an example ATN Island.

AVO_442	Evaluate the effects of specific protocol options or implementation strategies on

the end-to-end QoS. In particular:

- effect of the transport acknowledgement strategy on the measured QoS

- effect of transport timer values

- effect of selected TPDU size

- effect of subnetwork maximum SNSDU size

AVO_443	Evaluate the impact of the traffic load on the QoS.

AVO_445	Evaluate the impact of transport parameters tuning on the QoS and performances.

AVO_470	Evaluate the performance of receiver based congestion management over each class of

air/ground subnetwork, when an adjacency is supported by more than one class of

air/ground subnetwork simultaneously and when no subnetwork preference is given i.e.

when an NPDU may go over any of the available subnetwork connections. The

evaluation should aim to determine the conditions by which the required QoS is

maintained even when congestion occurs.

AVO_451	Verify that high priority data have a higher probability of achieving the expected

QoS. This should be verified for various level of traffic up to congested

conditions. Verify that, in congested situations, the ATN delivers application

messages by taking into account the priority of the message.

AVO_452	Evaluate the QoS discrimination between high and low priority data under the various

congestion management strategies.

AVO_456	Evaluate the probability of use of the cancellation procedure in Air-ground

communications.