Use of Boolean Opators in APRL Tables

This flimsy addresses the inconsistent use of symbols to represent the boolean operators in the Air/Ground route initiation APRL tables in section 5.3.5.2.14. The amended tables below  use ‘and’ and ‘or’ the boolean operators.

#### 5.3.5.2.14 APRL for Air/Ground Route Initiation

##### 5.3.5.2.14.1 General

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| njSubnet | Support of Subnetworks that do not provide a Join Event | 5.3.5.2 | O.1 |
| jSubnet | Support of Subnetworks that do provide a Join Event | 5.3.5.2 | O.1 |
| giSubnet | Support of Ground-Initiated Subnetworks | 5.3.5.2 | O.2 |
| aiSubnet | Support of Air-Initiated Subnetworks | 5.3.5.2 | O.2 |
| agSubnet | Support of Air or Ground-Initiated Subnetworks | 5.3.5.2 | O.2 |
| fsSubnet | Support of Subnetworks that support Fast Select | - | O |
| noIDRP-a | Support of optional non-use of IDRP by Airborne BIS | 5.3.5.2.12.3 | O |
| noIDRP-ag | Support of optional non-use of IDRP by Air/Ground BIS | 5.3.5.2.12.2 | M |
| lvSubnet | Support of Subnetworks that provide a Leave Event | 5.3.5.2.13 | M |

##### 5.3.5.2.14.2 Airborne Router - Subnetwork Connection Responder

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| respAR-ar | Response to incoming Call Request | 5.3.5.2.2 | giOragSubnet: M |
| valCR-ar | Validation of incoming Call Request | 5.3.5.2.2 | giOragSubnet:O |
| RespISH-ar | Generation of ISH PDU | 5.3.5.2.6 | giOragSubnet: M |
| ISHinCC-ar | Encoding ISH PDU in Call Accept User Data | 5.3.5.2.6 | RespISH-ar fsSubnet: O |
| negNoIDRP-ar | Transmission of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.6 | noIDRP-a:M |
| negIDRP-ar | Transmission of ISH PDU with SEL field of NET set to zero | 5.3.5.2.6 | ^ noIDRP-a:M |
| autoRoute-ar | Inference of available routes from received NET of A/G Router | 5.3.5.2.12.3 | noIDRP-a:M |
| initIDRP-ar | IDRP startup procedures - Invoke activate action | 5.3.5.2.10 | ^ noIDRP-a:M |
| supISH-ar | Suppression of multiple ISH PDUs | 5.3.5.2.10 | ^ noIDRP-a: O |
| valNET-ar | Validation of received NET | 5.3.5.2.7 | ^ noIDRP-a: O |

giOragSubnet: giSubnet  agSubnet

##### 5.3.5.2.14.3 Airborne Router - Subnetwork Connection Initiator

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| polling-ai | Procedures for polling a list of subnet addresses | 5.3.5.2.3.1 | pollReq: M |
| backoff-ai | Backoff Procedure | 5.3.5.2.3.1 | pollReq: M |
| connect-ai | Connect on receipt of Join Event | 5.3.5.2.3.2 | EventDrvn: M |
| ValJoin-ai | Validation of Join Event | 5.3.5.2.3.2 | EventDrvn: O |
| SendISH-ai | Generation of ISH PDU | 5.3.5.2.6 | EventDrvn pollReq:M |
| ISHinCR-ai | Encoding of ISH PDU in Call Request | 5.3.5.2.6 | SendISH-ar fsSubnet: O |
| negNoIDRP-ai | Transmission of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 | noIDRP-a:M |
| negIDRP-ai | Transmission of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 | ^ noIDRP-a:M |
| autoRoute-ai | Inference of available routes from received NET of A/G Router | 5.3.5.2.12.3 | noIDRP-a:M |
| initIDRP-ai | IDRP startup procedures - listenForOpen set to true | 5.3.5.2.10 | ^ noIDRP-a:M |
| supISH-ai | Suppression of multiple ISH PDUs | 5.3.5.2.10 | ^ noIDRP-a: O |
| valNET-ai | Validation of received NET | 5.3.5.2.7 | ^ noIDRP-a: O |

pollReq: aiSubnet  njSubnet

EventDrvn: jSubnet  (aiSubnet ^orSubnet)

##### 5.3.5.2.14.4 Air/Ground Router - Subnetwork Connection Responder

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| respAR-agr | Response to incoming Call Request | 5.3.5.2.2 | aiOragSubnet: M |
| valCR-agr | Validation of incoming Call Request | 5.3.5.2.2 | aiOragSubnet:O |
| emgncy-agr | Emergency Procedures | 5.3.5.2.2.2 | M |
| RespISH-agr | Generation of ISH PDU | 5.3.5.2.6 | aiOragSubnet: M |
| ISHinCC-agr | Encoding ISH PDU in Call Accepted User Data | 5.3.5.2.6 | RespISH-agr fsSubnet: O |
| negNoIDRP-agr | Receipt of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 | M |
| negIDRP-agr | Receipt of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 | M |
| autoRoute-agr | Inference of available routes from received NET of Airborne Router | 5.3.5.2.12.2 | M |
| initIDRP-agr | IDRP startup procedures - Invoke activate action | 5.3.5.2.10 | M |
| supISH-agr | Suppression of multiple ISH PDUs | 5.3.5.2.10 | O |
| valNET-agr | Validation of received NET | 5.3.5.2.7 | O |

aiOragSubnet: aiSubnet  agSubnet

##### 5.3.5.2.14.5 Air/Ground Router - Subnetwork Connection Initiator

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| connect-agi | Connect on receipt of Join Event | 5.3.5.2.4 | goOragSubnet: M |
| ValJoin-agi | Validation of Join Event | 5.3.5.2.4 | connect-agi: O |
| SendISH-agi | Generation of ISH PDU | 5.3.5.2.6 | connect-agi: M |
| ISHinCR-agi | Encoding of ISH PDU in Call Request | 5.3.5.2.6 | Send-ISH-agi fsSubnet: O |
| negNoIDRP-agi | Receipt of ISH PDU with SEL field of NET set to FEh | 5.3.5.2.8 | M |
| negIDRP-agi | Receipt of ISH PDU with SEL field of NET set to zero | 5.3.5.2.8 | M |
| autoRoute-agi | Inference of available routes from received NET of Airborne Router | 5.3.5.2.12.2 | M |
| initIDRP-agi | IDRP startup procedures - listenForOpen set to true | 5.3.5.2.10 | M |
| supISH-agi | Suppression of multiple ISH PDUs | 5.3.5.2.10 | O |
| valNET-agi | Validation of received NET | 5.3.5.2.7 | O |

goOragSubnet: giSubnet  agSubnet

##### 5.3.5.2.14.6 Termination Procedures

| Item | Description | ATN SARPs Reference | ATN Support |
| --- | --- | --- | --- |
| lvEvent | Processing of Leave Event | 5.3.5.2.13 | M |
| Watchdog | Watchdog Timer | 5.3.5.2.13 | M |
| ConfigWD | Configurability of Watchdog for Subnetwork Characteristics | 5.3.5.2.13 | O |
| conLeave | Processing of a per connection Leave Event | 5.3.5.2.13 | M |
| subnetLeave | Processing of a persubnetwork Leave Event | 5.3.5.2.13 | M |