ATNP/WG2/IDG/WP 2-4

31 August 1999

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

Working Group 2

ICS Drafting Group

2nd Meeting

Munich, Germany

1 - 3 September 1999

List of 3rd Edition ICS Enhancements

Working Paper

Presented by Klaus-Peter Graf

Summary

This paper provides a labelled list of ICS enhancements which are currently progressed by the IDG for inclusion into the 3rd edition of Sub-Volume 5 of Doc 9705. The list is open for further entries in the case that further ICS enhancements will be identified.

The label associated with each enhancement will be used to track the SARPs paragraph numbers which have been revised, deleted or added in the baseline of the 3rd edition in response to the associated enhancement.

# Introduction

Table 1 lists the proposed set of enhancements for the 3rd edition of Sub-Volume 5 of the ATN SARPs. The table has been built based on the work programme of the ICS Drafting Group (IDG) as outlined in ATNP WG2/18 Flimsy 8 and on the decisions made by the 1st IDG meeting held in Fairfax (28 – 30 June 1999).

The items listed in Table 1 represent the current set of enhancements pursued by the IDG for the 3rd edition of Sub-Volume 5. Additional enhancements may be required for the 3rd edition and will be added to the table as appropriate.

Each enhancement is identified by a unique label (i.e. ICS3-xx) which is used to track the modifications made in the SARPs baseline document, i.e. Doc 9705 2nd Edition, when preparing the 3rd edition of the ICS SARPs. Tracking of the SARPs paragraph numbers which have been revised, deleted or added by the 3rd edition enhancements is considered essential in order to ensure traceability of requirements of existing implementation projects and to support the introduction of the 3rd edition changes into existing implementations in a controlled process.

# Enhancements for 3rd Edition of ICS SARPs

| Label | Enhancement for 3rd Edition of ICS SARPs | Supporting Material | Status | Work Item |
| --- | --- | --- | --- | --- |
| ICS3-01 | Requirements for mobile subnetworks to issue Join and Leave Events within given latency intervals | WP484, WP485, Flimsy 17-6, Flimsy IDG1-1 | SARPs updated | 1 |
| ICS3-02 | Requirement for IS-SME to respect quarantine time before processing next Join event from mobile subnetworks having issued a Leave event |  | Text from TW expected | 2 |
| ICS3-03 | Use of adaptive retransmission timers in the Connection Oriented Transport Protocol | WP512, IDG1/WP5 | SARPs updated | 3 |
| ICS3-04 | Route selection based on required support for additional mobile subnetwork types and additional criteria (e.g. service provider selection) | WP521, Flimsy 18-7 | Text from TW expected | 6 |
| ICS3-05 | Requirements for ICS-related systems management consistent with the ATN systems management concept (i.e. draft Sub-Volume 6) | WP497, WP501, WP524, Draft Sub-Volume 6 | Text from JM expected | 9, 18 |
| ICS3-06 | Requirements for enhanced IDRP security consistent with the ATN security architecture (i.e. draft Sub-Volume 8). | WP488, WP492, WP493, WP518, WP522, Draft Sub-Volume 8 | Text from TM expected | 10 |
| ICS3-07 | Deletion of the ATN NSAP address compression algorithm (ACA) |  | SARPs updated | 11 |
| ICS3-08 | Suppression of re-advertisement of routes in the case of changing mobile subnetwork connectivity | WP520 | SARPs updated | 14 |
| ICS3-09 | Conveyance of data link capability parameter in air/ground ISH PDU exchanges to signal protocol capabilities which are beyond 1st edition of Doc 9705 between air/ground and airborne routers in a backwards compatible manner | WP471, WP520 | SARPs updated | 15, 17 |
| ICS3-10 | Improved handling of the loss of TPDUs experienced during changes in subnetwork connectivity | WP486 | To be reviewed at IDG/2 | 3 |
| ICS3-11 | Potential use of IDRP adaptive retransmission timers to better cope with changing subnetwork performance |  | On hold; waiting for further input | 5 |
| ICS3-12 | Potential amendment of existing mobile SNDCF and/or development of additional mobile SNDCFs to include new subnetwork types | WP459, WP491, Flimsy 18-4 and Flimsy 18-5. | On hold; waiting for input from AMCP | 7 |
| ICS3-13 | Potential QoS-related enhancements to IDRP based on recent work such as that of the IETF “QoS Routing Mechanisms and OSPF extensions” | WP409 | On hold; waiting for further input | 8 |
| ICS3-14 | Potential enhancements to the DEFLATE algorithm to allow for the dynamic negotiation for the use of pre-stored dictionaries |  | On hold; waiting for further input | 12 |
| ICS3-15 | Potential additional mobile SNDCF option to allow maintenance of DEFLATE history window when changing ground stations or air/ground routers respectively | IDG1/WP6 | On hold until more info on ICS3-12 is available | 12 |
| ICS3-16 | Potential use of additional security tags to introduce new routing capabilities in a backwards compatible manner |  | To be reviewed at IDG/2 | 17 |
| ICS3-17 | Potential efficiency enhancements to the DEFLATE algorithm | IDG1/Flimsy 3 | Delayed beyond ATNP/3 | 12 |

Table 1: List of Enhancements for 3rd Edition of ICS SARPs

The last column of the above table establishes the cross-reference between the ICS enhancements and the high-level work programme item (as documented in ATNP WG2/18 Flimsy 8 and ATNP WG2/IDG WP2) under which the relevant enhancement is proposed.

# Recommendation

IDG members are invited to review the above table and to propose modifications and amendments as appropriate.