THIS ATTACHMENT TO WP424 CONTAINS THE PDR RAISED TO RESOLVE THE PROBLEM DESCRIBED IN SECTION 3 -  LREF DIRECTORY MANAGEMENT.

NB: The Proposed SARPsamendment below has revision marks for ease of use in WG2, the original PDR has no revision marks.

--------------------------------------------------------------------------------------------------------------------------------

Title: 					LREF Directory Management

PDR Reference:				97100048

Originator Reference: 			UK-SV5-01

SARPs Document Reference: 		ICS SARPs, Section 5.7.6.3.4.5

Status: 					SUBMITTED

PDR Revision Date: 			< >

PDR Submission Date:			29/10/97

Submitting State/Organisation: 		NATS, UK

Submitting Author Name: 			Cardwell, B

Submitting Author E-mail Address: 	cardwellb@natsgcs.co.uk

Submitting Author Supplemental 		Tel: +44-1293-576 401

Contact Information:			Fax: +44-1293-576 399

SARPs Date:				ICAO V2.1, Oct. 97

SARPs Language: 			English

Summary of Defect:

There is scope for non-interoperability in Section 5.7.6.3.4.5 of the ICS

SARPs (Incoming SNDCF Error Report).  The relevant SARPs are reproduced

below.

Para 5.7.6.3.4.5.1   On receipt of an SNDCF Error Report, the virtual

circuit shall be reset (see 5.7.6.3.7), unless the reason is "compressed

PDU with unrecognized local reference".

Para 5.7.6.3.4.5.2   In this case, the directory entry corresponding to the

local reference returned in the SNDCF Error Report shall be reset to the

ununsed state.

Note: Section 5.7.6.3.7 (Call Reset Provisions) states the actions required

on RECEIPT of a Reset Indication.

1)  When para 5.7.6.3.4.5.1 applies it not clear that it is also necessary

to clear the local LREF directory associated with that virtual circuit as

well as initiating the Reset procedure.

2)  Para 5.7.6.3.4.5.2 can be mis-read, it could mean "when 5.7.6.3.4.5.1

applies" or "when the SNDCF Error Report gives the reason 'compressed PDU

with unrecognized local reference'".  [It means the latter]

Assigned SME:				Sub-Volume V SME

Proposed SARPs amendment:

To resolve point 1)

Change para 5.7.6.3.4.5.1 to

On receipt of an SNDCF Error Report, the virtual circuit shall be reset

(see 5.7.6.3.7) and the directory associated with the virtual circuit

cleared to its initial state, unless the reason is "compressed PDU with

unrecognized local reference".

To resolve point 2)

Change Para 5.7.6.3.4.5.2 to

In this caseOn receipt of an SNDCF Error Report with reason "compressed PDU with

unrecognized local reference", the directory entry corresponding to the

local reference returned in the SNDCF Error Report shall be reset to the

ununsed state.

SME Recommendation to CCB:		<    >

CCB Decision:				<    >