# PDRs against SV5 Edition 2 and the need for validation

The change control log for SV5 Edition 3 indicates that the following PDRs have been incorporated in addition to the Enhancements that have been incorporated in the SV5 Ed 3 Validation Report.

| PDR Reference | Title | Validation Impact |
| --- | --- | --- |
| PDR 99 07 0001 | ICAO 9705 Edition 2 Editorial Errors | Editorial – no validation required |
| PDR 99 07 0004 | Remove Jitter on IDRP Timers for Airborne BIS | Clarification – Jitter made optional for airborne routers – assuming negotiation is clear, no validation is required. |
| PDR 99 07 0005 | ATSC Class of Locally Originated Routes | Clarification – validated by inspection, unlikely to require further validation but it is a change to route advertisement process. |
| PDR 99 09 0001 | Over-specification of ARS Address Field Assignment | Clarification – relaxed specification of ARS field to local policy – no validation required. |
| PDR 99 09 0002 | Extension Capability of Mobile SNDCF Header | Clarification - Impact on Interoperability: The SARPs amendment proposed in this PDR will provide for the necessary long-term backwards compatibility mechanism when introducing new SNDCF capabilities in future SARPs versions. The SARPs amendment is specified in a way that it will be backwards compatible with implementations based on Edition 1 and 2 of Doc 9705. This backwards compatibility is achieved by a mechanism which requires routers implementing this PDR to fall back to version 1 of the SNDCF protocol (specified in Edition 1 and 2 of Doc 9705), when recognising (from the diagnostic code of a received call clear packet) that the peer router has not implemented this PDR. ## Validation is complete as part of FRAVI |
| PDR 99 10 0003 | LREF Compression and CLNP ECHO NPDUs | Clarification – enables transmission of CLNP ECHO PDUs over a/g link when LREF is in use – as implemented in TAR/ProATN \ validated already |
| PDR 99 10 0004 | ISO/IEC 8208 Non-Standard Default Packet Size Facility | Clarification - removes mandatory use of the ISO/IEC 8208 non-standard default packet size facility – no validation required |
| PDR 99 10 0005 | Reservation of Unassigned/Undefined Values | Clarification - reserves parameter bits for future use – no validation required |
| PDR M0 04 0001 | Incorrect/Duplicated ATSC Class Security Tag Requirement | Clarification - Deletion of duplicated requirement – no validation necessary |
| PDR M0 04 0002 | Potential mis-delivery of CLNP packets | Backwards compatible change – both the negotitation mechanism and the 32-bit checksum algorithm should be validated. |
| PDR M0 06 0001 | Editorial | Editorial – no validation required |