ATNP/WG2/Flimsy 02

Aeronautical Telecommunication Network Panel (ATNP)

Working Group 2

Meeting 21

July 11 –14, 2000

Limerick, Ireland

Proposed Changes to Sub-Volume V

for

IDRP Authentication

Presented by Tom McParland

Summary

The attached flimsy contains proposed changes to Sub-Volume V for IDRP authentication in response to PDR M0020018 and for alignment with Sub-VolumeVIII.

1.   	PDR M0020018

The current status of PDR M0020018 as documented in the SME 5 (Internet Communications Service) Status Report (WG2/WP 586) is SUBMITTED.  The PDR defect summary is as follows:

The SV 5 is not specific on the procedure to be followed by a third edition compliant A/G or Airborne BIS (supporting Authentication type 2) when the routing is initiated over a mobile subnetwork with a remote BIS that does not support Authentication type 2.

As a resolution to this PDR, it is recommended that requirement 5.8.3.2.10.1.3 have the following text appended:

“; otherwise, the OPEN PDU shall be discarded without any further action.”

Accordingly, the complete requirement would read as follows:

5.8.3.2.10.1.3	When an Airborne or Air/Ground Router, which has signalled type 2 authentication during the ISO/IEC 9542 ISH exchange, receives an OPEN PDU with the Authentication Code field set to 1, it shall process the OPEN PDU only if permitted to do so by local policy; otherwise, the OPEN PDU shall be discarded without any further action.

2. 	Alignment with Sub-Volume VIII.

Due to restructuring of Sub-Volume VIII, certain requirement sections, which are referenced by Sub-Volume V, have been moved or re-titled.  Therefore, the following changes are necessary:

1.  Re:  5.8.3.2.10.2.6.2 – Change reference to ATN Key Agreement Scheme from “8.5.3” to “8.5.4”

2.  Re:  5.8.3.2.10.2.6.4 – Change name of “ATN Keyed Message Authentication Code Primitive” to “ATN Keyed Message Authentication Code Generation Primitive”

3.  Re:  5.8.3.2.10.2.6.4 – Change reference from “8.5.5.3” to “8.5.6.2.1”

4.  Re:  5.8.3.2.10.2.6.5 – Change reference from “8.5.5.4” to “8.5.6.2.2”

5.  Re:  5.8.3.2.10.3.2.1 – Change name of “ATN Keyed Message Authentication Code Primitive” to “ATN Keyed Message Authentication Code Generation Primitive”

6.  Re:  5.8.3.2.10.3.2.1 – Change reference from “8.5.5.3” to “8.5.6.2.1”