WG2-22 Flimsy #3

Revision a

Changes to the maximum values of the COTP Timer Value Ranges (Table  5.5-1)

# Introduction

The minimum and  nominal values  the COTP timers T1, R, L and W in Table 5.5-1 are currently determined based on the relationships defined in clause 12.2.1.1 of ISO/IEC 8073, and based on an estimate of the suitable values for timers MRL, MLR, ERL, ELR, AL, AR , N and I.

For consistency reasons, it is proposed that the maximum values of T1, R, L and W in the Table 5.5-1 be determined in the same way.

# Proposed Changes

Replace the values in the rightmost column of the Table 5.5-1 as follows :

- Maximum value of T1 : replace 300 by 701 seconds

- Maximum value of R : Replace 2710 by 6310 seconds

- Maximum value of L : Replace 3000 by 7910 seconds

- Maximum value of W : Replace 6000 by 5500 seconds

As a result of these changes, the rightmost column of the Table 5.5-1 should be as follows:

|  | ### Maximum Value |
| --- | --- |
| (MRL, MLR) | 600 |
| (ERL, ELR) | 150 |
| (AL, AR) | 400 |
| (T1) | 300 701 |
| (R) | 2710 6310 |
| (N) | 10 |
| (L) | 3000 7910 |
| (I) | 6000 |
| (W) | 6000 5500 |

# Background

## The new proposed maximum value for T1 (i.e. 701 seconds)  has been obtained by applying the formula « T1 = ELR + ERL + AR + x » with :

ELR = ERL = 150

AR = 400

x = 1

The new proposed maximum value for R (i.e. 6310 seconds)  has been obtained by applying the formula « R = T1*(N-1) + x » with :

T1 = 701

N = 10

x =1

## The new proposed maximum value for L (i.e.  7910)  has been obtained by applying the formula « L = = MLR + MRL + R + AR » with

MLR = MRL = 600

R = 6310

AR = 400

The new proposed maximum value for W (i.e.  5500 seconds)  has been  obtained by applying the formula « W = I - ELR  – offset »

I = 6000

ELR  = 150

Offset = 350

Note: The offset is an unanticipated delay the value of which must exceed the ELR  value. 350 seconds is considered to be a suitable value for this parameter.