Aeronautical Telecommunication Network Panel (ATNP)

Working Group 2

Meeting 22

August 20 – 23, 2000

Berlin, Germany

Change to 5.8.3.2.10.1.3 to split two ‘shall’ statements

- Introduction.

A change proposed at the WG2-21 meeting has caused a paragraph to contain two shall statements.  This Flimsy proposes a further change to correct this error.

- Proposed Changes.

###### 5.8.3.2.10.1.3	When an Airborne or Air/Ground Router, which has signalled type 2 authentication during the ISO/IEC 9542 ISH exchange,  receives an OPEN PDU with the Authentication Code field set to 1, it shall process the OPEN PDU only if permitted to do so by local policy.

5.8.3.2.10.1.4	When an Airborne or Air/Ground Router, which has signalled type 2 authentication during the ISO/IEC 9542 ISH exchange,  receives an OPEN PDU with the Authentication Code field set to 1, and local policy does not permit it to process the OPEN PDU, then the OPEN PDU shall be discarded without any further action.

###### 5.8.3.2.10.1.5	When an authentication code of 1 is specified in the Authentication Code field of the OPEN BISPDU that initiated a BIS-BIS connection, then, an ATN Router shall generate a validation pattern according to clause 7.7.1 of ISO/IEC 10747, for each BISPDU that it sends over that connection, and similarly validate the validation pattern of all received BISPDUs on such a connection.

Note.— The use of ISO/IEC 10747 type 2 authentication is under consideration for specification in future versions of these SARPs.

###### 5.8.3.2.10.1.6	The type 1 authentication code shall be generated according to the MD4 specification published in RFC 1320.

NO FURTHER CHANGES.