ATN Systems, Incorporated Requirements Definition Team

Protocol Implementation Conformance Statements for Intermediate Systems Inter-Domain Routing Protocol

This page is intentionally left blank

Table Of Contents

1. Scope of this Document

2. References

3. Conventions

4. Content of this Document

5. ASI ISs IDRP PICSs

5.1 ATN Specific Protocol Requirements

5.2 IDRP General

5.3 IDRP Update Send Process

5.4 IDRP Update Receive Process

5.5 IDRP Decision Process

5.6 IDRP Receive

5.7 Peer Entity Authentication

5.8 IDRP CLNS Forwarding

5.9 IDRP Receive Process

5.10 IDRP Optional Transitive Attributes

5.11 Generating Well-Known Discretionary Attributes

5.12 Propagating Well-Known Discretionary Attributes

5.13 Receiving Well-Known Discretionary Attributes

5.14 IDRP Timers

This page is intentionally left blank

# Scope of this Document

This document presents a draft version of the Protocol Implementation Conformance Statements (PICSs) of ATN Systems, Inc. (ASI) intended implementation of the Inter-Domain Routing Protocol (IDRP) of ATN Intermediate Systems (ISs). These PICS state conformance of ASI ISs IDRP to the provisions of the ICAO CNS/ATM-1 draft SARPs for Internet Communication Services (Version 3.0, 7/08/95) for IDRP Protocol.

It should be noted that the PICSs contained in this document are not self-sufficient to procure an ATN-compliant implementation of ISs.

# References

Reference  1. 	CNS/ATM-1 Package SARPs and Guidance Material  V3.0 	07 August 1995

PART V-Internet Communication Service

Reference  2	 ISO/IEC 10747:1993	Information Processing Systems - Telecommunications 		and Information Exchange between Systems - Protocol for exchange of inter-domain 		routeing information among intermediate systems to support forwarding of ISO 		8473 PDUs (IDRP) [c0 6090]

# Conventions

The PICSs provided in this document use the following conventions and symbols:

M		[M]andatory - the capability must be supported

MO		Mandatory to implement but optional for use

O		[O]ptional - The capability may optionally be supported

O.<n>		[O]ptional, but support of at least one of the group of options labelled by the

numeral <n> is required

X or P		prohibited/precluded i.e. the capability must not be supported.

OX or OP	Optional to implement but precluded for use

<pred>:		Condition item symbol, including predicate identification

^		Logical negation, applied to a condition item’s predicate

<r>		Receive aspects of an item

<s>		Send aspects of an item

Y		[Y]es, indicates that ASI implementation must support the item

N		[N]o, indicates that there is no requirement for the ASI implementation

to support the item.

N/A		Not Applicable

# Content of this Document

This document contains the Protocol Implementation Conformance Statements for the IDRP protocol as specified for the ASI ground and airborne ATN Intermediate Systems.

# ASI ISs IDRP PICSs

## ATN Specific Protocol Requirements

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Index | Item | CNS/ATM-1 SARPs Ref | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ATNIDRP1 | Encoding and use of the Security Path Attribute | 8.3.1.2, 8.3.1.3 | M | M | M | Y | Y | Y |
| ATNIDRP2 | Additional procedures for non-use of the minRouteAdvertisementInterval Timer | 8.3.1.5 | M | M | - | Y | Y | N |
| ATNIDRP3 | Support of Route Aggregation and Route Information Reduction | 8.3.2.2 | O | O | - | Y | Y | N |
| ATNIDRP4 | Support for Route Merging Procedures | 8.3.1.6 | M | M | - | Y | Y | N |

## IDRP General

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. |  | ISO Status |  | G-G Router |  | A/G Router |  | Airborne Router |  | ASI GGIS Support |  | ASI AGIS Support |  | ASI Airborne IS Support |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BASIC [a11 a 0940] | Are all basic BIS functions implemented? | 12.1 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| MGT [a11 a 0950] | Is this system capable of being managed by the specified management information? | 11 |  | M |  | O |  | O |  | O |  | N |  | N |  | N |  |
| VER [a11 a 0960] | Does this BIS support Version Negotiation? | 7.8 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| RTSEP [a11 a 0970] | Does this BIS support ROUTE_SEPARATOR attribute? | 7.12.1 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| HOPS [a11 a 0980] | Does this BIS support the RD_HOP_COUNT attribute? | 7.12.13 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| PATH [a11 a 0990] | Does this BIS support the RD_PATH attribute? | 7.12.3 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| CAPY [a11 a 1000] | Does this BIS support the Capacity Attribute? | 7.12.15 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| FSM [a11 a 1010] | Does this BIS manage BIS-BIS connections according to the BIS FSM description? | 7.6.1 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| FCTL [a11 a 1020] | Does this BIS provide flow control? | 7.7.5 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| SEQNO [a11 a 1030] | Does this BIS provide sequence number support? | 7.7.4 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| INTG1 [a11 a 1040] | Does this BIS provide Data integrity using authentication type 1? | 7.7.1 |  | O.1 |  | M |  | M |  | M |  | Y |  | Y |  | Y |  |
| INTG2 [a11 a 1050] | Does this BIS provide Data integrity using authentication type 2? | 7.7.2 |  | O.1 |  | O |  | O |  | O |  | N |  | N |  | N |  |
| INTG3 [a11 a 1060] | Does this BIS provide Data integrity using authentication type 3? | 7.7.3 |  | O.1 |  | O |  | O |  | O |  | N |  | N |  | N |  |
| ERROR [a11 a 1070] | Does this BIS handle error handling for IDRP? |  | 7.20 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |
| RIBCHK [a11 a 1080] | Does this BIS operate in a "fail-stop" manner with respect to corrupted routing information? |  | 7.10.2 |  | M |  | M |  | M |  | M |  | Y |  | Y |  | Y |

## IDRP Update Send Process

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

|  | Item | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INT [a11 a 1090] | Does the BIS provide the internal update procedures? | 7.17.1 | M | M | M | O | Y | Y | N |
| RTSEL [a11 a 1100] | Does this BIS support the MinRouteAdvertisementInterval Timer? | 7.17.3.1 | M | M | M | O | Y | Y | N |
| RTORG [a11 a 1110] | Does this BIS support the MinRDOriginationInterval Timer? | 7.17.3.2 | M | M | M | M | Y | Y | Y |
| JITTER [a11 a 1120] | Does this BIS provide jitter on its timers? | 7.17.3.3 | M | M | M | M | Y | Y | Y |

## IDRP Update Receive Process

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INPDU [a11 a 1130] | Does the BIS handle inbound BISPDUs correctly? | 7.14 | M | M | M | M | Y | Y | Y |
| INCONS [a11 a 1140] | Does this BIS detect inconsistent routing information? | 7.15.1 | M | M | M | M | Y | Y | Y |

## IDRP Decision Process

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TIES [a11 a 1150] | Does this BIS break ties between candidate routes correctly? | 7.16.2.1 | M | M | M | M | Y | Y | Y |
| RIBUPD [a11 a 1160] | Does this BIS update the Loc-RIBs correctly? | 7.16.2 | M | M | M | M | Y | Y | Y |
| AGGRT [a11 a 1170] | Does this BIS support route aggregations? | 7.18.2.1, 7.18.2.2, 7.18.2.3 | O | ATNIDRP3:M | ATNIDRP3:M | - | Y | Y | N |
| LOCK [a11 a 1180] | Does this BIS provide interlocks between its Decision Process and the updating of the information in its Adj-RIBs-In? | 7.16.4 | M | M | M | M | Y | Y | Y |

## IDRP Receive

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RCV [a11 a 1200] | Does the BIS process incoming BISPDUs and respond correctly to error conditions? | 7.14, 7.20 | M | M | M | M | Y | Y | Y |
| OSIZE [a11 a 1210] | Does this BIS accept incoming OPEN PDUs whose size in octets is between MinBISPDULength and 3000? | 6.2,7.20 | M | M | M | M | Y | Y | Y |
| MXPDU [a11 a 1220] | Does the BIS accept incoming UPDATE, IDRP ERROR and RIB REFRESH PDUs whose size in octets is between minBISPDULength and maxBISPDULength? | 6.2,7.20 | M | M | M | BISREF: OX ^BISREF:M | Y | Y | BISREF: N ^BISREF:Y |

BISREF: if RIB REFRESH PDU then true else false

## Peer Entity Authentication

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AUTH [a11 a 1280] | Does this BIS correctly authenticate the source of a BISPDU? | 7.7.2 | O | M | M | M | Y | Y | Y |

Note.— Only support for an Authentication Code 1 is required [a11 n 0240].

## IDRP CLNS Forwarding

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSRCRT [a11 a 1230] | Does the BIS correctly handle 8473 NPDUs that contain a partial source route? | 8 | M | O | O | O | N | N | N |
| DATTS [a11 a 1240] | Does the BIS correctly extract the NPDU-derived Distinguishing Attributes from an 8473 NPDU? | 8.2 | M | M | M | M | Y | Y | Y |
| MATCH [a11 a 1250] | Does the BIS correctly match the NPDU-derived Distinguishing Attributes with the corresponding FIB-Atts? | 8.3 | M | M | M | M | Y | Y | Y |
| EXTF [a11 a 1260] | Does the BIS correctly forward NPDUs with destinations outside its own routing domain? | 8.4 | M | M | M | M | Y | Y | Y |
| INTF [a11 a 1270] | Does the BIS correctly forward NPDUs with destinations inside its own routing domain? | 8.1 | M | M | M | M | Y | Y | Y |

## IDRP Receive Process

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RCV | Does the BIS process incoming BISPDUs and respond correctly to error conditions? | 7.14, 7.20 | M | M | M | M | Y | Y | Y |
| OSIZE | Does this BIS accept incoming OPEN PDUs whose size in octets is between MinBISPDULength and 3000? | 6.2,7.20 | M | M | M | M | Y | Y | Y |
| MXPDU | Does the BIS accept incoming UPDATE, IDRP ERROR and RIB REFRESH PDUs whose size in octets is between minBISPDULength and maxBISPDULength? | 6.2,7.20 | M | M | M | M | Y | Y | Y |

## IDRP Optional Transitive Attributes

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MEXIT [a11 a 1290] | Does this BIS support use of the MULTI-EXIT DISC attribute? | 7.12.7 | O | O | O | O | Y | Y | Y |

## Generating Well-Known Discretionary Attributes

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTG [a11 a 1300] | Does the BIS support generation of the EXT_INFO attribute? | 7.12.2 | O | O | O | O | N | N | N |
| NHRS [a11 a 1310] | Does the BIS support generation of the NEXT_HOP attribute in support of route servers? | 7.12.4 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| NHSN [a11 a 1320] | Does the BIS support generation of the NEXT_HOP attribute to advertise SNPAs? | 7.12.4 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| DLI [a11 a 1330] | Does the BIS support generation of the DIST_LIST_INCL attribute? | 7.12.5 | O | O | O | O | Y | Y | Y |
| DLE [a11 a 1340] | Does the BIS support generation of the DIST_LIST_EXCL attribute? | 7.12.6 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| TDLY [a11 a 1350] | Does the BIS support generation of the TRANSIT DELAY attribute? | 7.12.8 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| RERR [a11 a 1360] | Does the BIS support generation of the RESIDUAL ERROR attribute? | 7.12.9 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| EXP [a11 a 1370] | Does the BIS support generation of the EXPENSE attribute? | 7.12.10 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |
| LQOSG [a11 a 1380] | Does the BIS support generation of the LOCALLY DEFINED QOS attribute? | 7.12.11 | O | OX | OX | OX | N | N | N |
| HREC [a11 a 1390] | Does the BIS support generation of the HIERARCHICAL RECORDING attribute? | 7.12.12 | O | OX | OX | OX | N | N | N |
| SECG [a11 a 1400] | Does the BIS support generation of the SECURITY attribute? | 7.12.14 | O | M | M | M | Y | Y | Y |
| PRTY [a11 a 1410] | Does the BIS support generation of the PRIORITY attribute? | 7.12.16 | O | O | IDRPAG:OX ^IDRPAG:O | O | N | N | N |

IDRPAG: if Air/Ground adjacency then true else false

## Propagating Well-Known Discretionary Attributes

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTGP [a11 a 1420] | Does the BIS support propagation of the EXT_INFO attribute? | 7.12.2 | M | M | M | - | Y | Y | N |
| NHRSP [a11 a 1430] | Does the BIS support propagation of the NEXT_HOP attribute in support of route servers? | 7.12.4 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |
| NHSNP [a11 a 1440] | Does the BIS support propagation of the NEXT_HOP attribute to advertise SNPAs? | 7.12.4 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |
| DLIP [a11 a 1450] | Does the BIS support propagation of the DIST_LIST_INCL attribute? | 7.12.5 | O | M | M | - | Y | Y | N |
| DLEP [a11 a 1460] | Does the BIS support propagation of the DIST_LIST_EXCL attribute? | 7.12.6 | O | M | IDRPAG:OX ^IDRPAG:M | - | Y | IDRPAG: N ^IDRPAG:Y | N |
| TDLYP [a11 a 1470] | Does the BIS support propagation of the TRANSIT DELAY attribute? | 7.12.8 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |
| RERRP [a11 a 1480] | Does the BIS support propagation of the RESIDUAL ERROR attribute? | 7.12.9 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |
| EXPP [a11 a 1490] | Does the BIS support propagation of the EXPENSE attribute? | 7.12.10 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |
| LQOSP [a11 a 1500] | Does the BIS support propagation of the LOCALLY DEFINED QOS attribute? | 7.12.11 | O | OX | OX | - | N | N | N |
| HRECP [a11 a 1510] | Does the BIS support propagation of the HIERARCHICAL RECORDING attribute? | 7.12.12 | O | OX | OX | - | N | N | N |
| SECP [a11 a 1520] | Does the BIS support propagation of the SECURITY attribute? | 7.12.14 | O | M | M | - | Y | Y | N |
| PRTYP [a11 a 1530] | Does the BIS support propagation of the PRIORITY attribute? | 7.12.16 | O | O | IDRPAG:OX ^IDRPAG:O | - | N | N | N |

## Receiving Well-Known Discretionary Attributes

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EXTR [a11 a 1540] | Does the BIS recognise upon receipt the EXT_INFO attribute? | 7.12.2 | M | M | M | M | Y | Y | Y |
| NHRSR [a11 a 1550] | Does the BIS recognise upon receipt the NEXT_HOP attribute ? | 7.12.4 | M | M | M | O | Y | Y | N |
| DLIR [a11 a 1560] | Does the BIS recognise upon receipt the DIST_LIST_INCL attribute? | 7.12.5 | M | M | M | M | Y | Y | Y |
| DLER [a11 a 1570] | Does the BIS recognise upon receipt the DIST_LIST_EXCL attribute? | 7.12.6 | M | M | M | O | Y | Y | N |
| TDLYR [a11 a 1580] | Does the BIS recognise upon receipt the TRANSIT DELAY attribute? | 7.12.8 | M | M | M | O | Y | Y | N |
| RERRR [a11 a 1590] | Does the BIS recognise upon receipt the RESIDUAL ERROR attribute? | 7.12.9 | M | M | M | O | Y | Y | N |
| EXPR [a11 a 1600] | Does the BIS recognise upon receipt the EXPENSE attribute? | 7.12.10 | M | M | M | O | Y | Y | N |
| LQOSR [a11 a 1610] | Does the BIS recognise upon receipt the LOCALLY DEFINED QOS attribute? | 7.12.11 | O | O | O | O | N | N | N |
| HRECR [a11 a 1620] | Does the BIS recognise upon receipt the HIERARCHICAL RECORDING attribute? | 7.12.12 | M | M | M | O | Y | Y | N |
| SECR [a11 a 1630] | Does the BIS recognise upon receipt the SECURITY attribute? | 7.12.14 | M | M | M | M | Y | Y | Y |
| PRTYR [a11 a 1640] | Does the BIS recognise upon receipt the PRIORITY attribute? | 7.12.16 | M | M | M | O | Y | Y | N |

## IDRP Timers

The ASI Intermediate-Systems shall implement the features marked “Y” in the table.

| Item | Description | ISO 10747 Ref. | ISO Status | G-G Router | A/G Router | Airborne Router | ASI GGIS Support | ASI AGIS Support | ASI Airborne IS Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ta [a11 a 1650] | KeepAlive time | 7.6.5 | M | M | M | M | Y | Y | Y |
| Tr [a11 a 1660] | Retransmission (tr) timer | 7.6.5 | M | M | M | M | Y | Y | Y |
| Tmr [a11 a 1670] | maxRIBIntegrityCheck timer | 7.10.2 | M | M | M | M | Y | Y | Y |
| Tma [a11 a 1680] | MinRouteAdvertisement timer | 7.17.3.1 | M | M | M | O | Y | Y | N |
| Trd [a11 a 1690] | MinRDOriginationInterval timer | 7.17.3.2 | M | M | M | M | Y | Y | Y |
| Tcw [a11 a 1700] | closeWaitDelay timer | 7.6.4 | M | M | M | M | Y | Y | Y |

The interpretation of this Item is that mandatory compliance requires that access to the MO is provided via a Systems Management Agent. Remote Systems Management is not required for CNS/ATM-1 Package and hence it is not reasonable to require mandatory support for this requirement.