ATNP/WG 1 IP355

ATNP/WG 2 WP355

07 October 1996

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL (ATNP)

WORKING GROUPS 1 and 2

Addressing Guidance for Diverse Ground-Based Subnetworks

Presented by

Leon Sayadian

(Federal Aviation Administration)

Prepared by

Kelly Kitchens

SUMMARY This paper is a continuation of WP290, 24-28 June 1996, Paragraph 2.7 (Naming and Addressing) and provides guidance for integrating different subnetwork addressing structures into the ATN.

1.0  BACKGROUND

The interconnection of the various multinational data networks into an Air Traffic

Management internet has been defined in the ICAO  Aeronautical Telecommunications

Network (ATN) Standards and Recommended Practices (SARPs)[1].  The networks that

will be integrated into the ATN use a newer technology or were in use prior to the

development of the SARPs.  These networks, once integrated into the ATN will become

subnetworks, which are made up of  a variety of  protocols and address structures.  In

addition, emerging network technologies that are just beginning their deployment cycle

must be able to interface to the ATN.  Therefore, consideration should be given on how to

integrate various address structures into the ATN.  This paper makes recommendations

on how to deal with the diverse address structures and successfully integrate them into the

ATN.

2.0  DISCUSSION

In order to efficiently implement the global ATN, member states are going to implement

the ICAO ATN protocols with existing subnetworks and use the ATN routing network to

tie these subnetworks together to form the ATN, Figure 1.  These subnetworks use a

variety of popular protocols that have different addressing mechanisms.  These

addressing mechanisms contain different  lengths as well as different semantics (e.g.,

hierarchical, flat addresses).  Presently, the draft ATN SARPs looks only at the

subnetwork elements using the protocols defined in the SARPs and does not make

recommendations to integrate existing subnetworks.  This lack of integration will cause

interoperability problems in the ATN.  In order to avoid this situation, addressing

standards for the most common subnetwork protocols and the most popular emerging

protocols are reviewed in respect to the ATN.

o o

dd

Figure 1 ATN Subnetwork Address Integration

The most commonly implemented network protocols that should be considered are:

Internet Protocol Suite

Ethernet Local Area Networks

X.25 Packet Switched Networks

In addition to these existing protocols, the following data transmission technologies are

gaining in popularity and will eventually achieve wide scale deployment and should also

be considered.

Asynchronous Transfer Mode

Fiber Distributed Data Interface

Integrated Service Digital Network

Frame Relay

Therefore, to ensure a coherent data network environment, consideration should be given

to implementation/integration of  these various addressing schemes into the ATN.  The

SARPs defines the Network Service Access Point (NSAP), which is one side of the

problem, but it must further define the standard subnetwork profiles in order to ensure

interoperability at the subnetwork level, and an interface rich environment that promotes

access to the ATN domain.

2.1  Internet Protocol Suite  (IPS)

Implementation of addressing in an IPS environment is done in accordance with RFC-

791, Internet Protocol[2].  Implementation of addressing in the ATN is performed in

accordance with ISO 6523-ICD-Information Technology-Structure for Identification of

Organizations[3], and ISO 8348 AD2, Information Processing Systems-Data

Communications-Network Service Definitions: Addendum 2 Network Layer

Addressing,[4].  The two addressing structures are incompatible and must be

accommodated using an encapsulation or conversion technique.  This will allow

communication between IPS based subnetworks and the ATN.

2.2  Ethernet Local Area Networks

Integration of Ethernet subnetworks are described in Volume VI of the draft SARPs,

Section 5.7. Refer to this document for information on integrating an Ethernet

subnetwork.

2.3  X.25 Packet Switched Network

Implementation of X.25 are described in Volume VI of the draft SARPS, Section 5.4.

Refer to this document for information on integrating an X.25 subnetwork.

2.4  Asynchronous Transfer Mode (ATM)

Addressing support for ATM is defined in User Network Interfaces (UNI) 3.0/3.1[5].

The address format is based on the OSI syntax for Network Service Access Point (NSAP)

addresses.  However, it is not as defined and allocated by ISO.  Despite the similar

structure, they are better described as private ATM network addresses and are used to

identify subnetwork points of attachment.  The 20-byte ATM addresses are designed for

use within private ATM networks.  There are three different formats: NSAP Encoded

E.164[6], Data Country Code (DCC) Format, and International Code Designator (ICD)

Format.  Implementation of ATM subnetworks will require an address conversion process

in order to convert from the ATM defined NSAP to the ATN defined NSAP.

2.5  Fiber Distributed Data Interface (FDDI)

The physical layer and data link interface for FDDI is covered in ISO 9314-1, Fiber

Distributed Data Interface[7].  However, FDDI is not a network level protocol and does

not have an address like ISDN, X.25, B-ISDN, or Frame Relay.  Instead FDDI should be able to use any of the higher level protocol address schemes.  Use ISO 9542 and

addendums, End System to Intermediate System Routing Exchange Protocol[8] in

conjunction with ISO 8473[9], the protocol for providing the connectionless-mode

network service for the address resolution functionality.

2.6  Integrated Service Digital Network (ISDN)

The network layer for ISDN is covered in ISO 9574, Information Technology-

Telecommunications and Information Exchange Between Systems-Provision of the OSI

Connection-mode Network Service by Packet Mode Terminal Equipment Connected to

an Integrated Services Digital Network (ISDN)[10].  ISDN specifies an information

element (IE) to carry address information.  This IE is the called/calling party number.

Inside of this IE are two more subfields.  One is the type of number and the other is

numbering plan.

The Type of number lists either:

unknown,

international,

national,

network specific,

subscriber, or

abbreviated number.

The numbering plan lists one of the following:

unknown,

ISDN/telephony (E.163/E.164),

data (X.121),

telex (F.69),

national standard,

private.

Implementation of addressing is performed in accordance with ISO 8348, Addendum 2,

Information processing systems-Data communications-Network service definitions.  The

numbering plan used is ISDN/telephony (E.163/164).

2.7  Frame Relay

Frame relay uses the same format as ISDN (E.163/164).  Address integration is done in

accordance with ISO 8348, Addendum 2, Information processing systems-Data

communications-Network service definitions[4].

∙	2.8  Recommendation

Develop standards profiles and procedures to integrate these and other protocols into the

ATN end systems in order to allow communications over other types of subnetworks.

LIST OF REFERENCES

ICAO CNS/ATM-1 Package SARPs and Guidance Material

RFC-791 Internet Protocol, J. Postel September 1991

ISO 6523 1984: Data Interchange-Structure for the Identification of Organizations

ISO/IEC 8348 AD2:  1988 Information Processing Systems-Data Communications-

Network Service Definition:  Addendum 2 Network Layer Addressing.

ATM Panel Recommendation 1993: Universal Network Interface 3.0/3.1

E.164 1991: Numbering Plan for the ISDN Era

ISO 9314-1 1989: Fibre Distributed Data Interface (FDDI)-Part 1-Physical Layer

Protocol (PHY)

ISO/IEC 9542 1988: Information Processing Systems-Telecommunications and

Information Exchange between Systems-ES to IS Routing Exchange Protocol

ISO 8473 1988: Information Processing Systems-Data Communications-Protocol for

Providing the Connectionless-Mode Network Service

ISO 9574 1989: Information Technology-Telecommunications and Information

Exchange Between Systems-Provision of the OSI Connection-mode Network

Service by Packet Mode Terminal Equipment Connected to an Integrated Services

Digital Network (ISDN)