Extension of Section 2.4 of Guidance Material

Background

During the review of chapter 2 of the guidance material, it was agreed to extend the guidance material by adding an additional paragraph providing guidance on the (high level) concept of mobile route initiation.

This flimsy provides such additional text and proposes its inclusion in version 1.5 of the GM.

Proposal

Add the following paragraph after 2.4.3 (Mobile Users):

#### 2.4.3.1	Route Initiation

The establishment of a communications path between BISs in any two Routing Domains is known as “Route Initiation”. These procedures apply to the establishment of both, ground/ground as well as air/ground communications. However, different from the first one (which is static), Route Initiation for mobile users is dynamic, has to follow ICAO specified procedures and is initiated by the mobile subnetwork. The air/ground subnetwork first has to signal the availability of any newly established subnetwork connection  (e.g. an aircraft entering coverage of  a Mode S subnetwork) to its attached BIS (more specifically, to its subnetwork-dependent conversion function, SNDCF).

This Route Initiation is either air initiated (i.e. VDL, AMSS) or ground initiated (Mode S). Detailed guidance about the Route Initiation is contained in chapter 5.10 and chapter 7.3.