FLIMSY 7   - 27/06/96

ATNP/WG2-9

Objective of the proposed SNDCF change : To describe in the Fast Select processing that the SNDCF should verify whether an NPDU has been transmitted in the additional User Data field -

This flimsy references the revised Chapter 7 (26/6/96) -

On page 5.7-6 , section 5.7.6.2.1.6.1 (Calling DTE procedure , Fast Select Facility in Use)

Add a paragraph

5.7.6.2.1.6.1.3 : If the length indicator in the SNDCF header of the received Call Confirm packet exceeds two  and if the User Data field is a recognized NPDU SPI, then the User Data field contains an NPDU and the calling SNDCF shall pass this NPDU in an SN-UNIT-DATA indication to the appropriate ATN SN-service User.

On page 5-7, section 5.7.6.2.2.2 (Call Acceptance with the Fast Select Facility in Use )

Add a paragraph

5.7.6.2.2.2.3  If the length indicator in the SNDCF header of  the received Incoming Call packet exceeds seven (or nine if Local Reference Compression is offered) and if the User Data field is a recognized NPDU SPI, then the User Data field contains an NPDU and the called SNDCF shall pass this NPDU in an SN-UNIT-DATA indication to the appropriate ATN SN-service User.