Flimsy #5
25 June 1996

Rev. No.

Use of the Mobile SNDCF for Ground to Ground Communications

# Introduction

During the review of  WP291, it was agreed that 5.7.6.1.4 was inappropriatly located. The recommendation applies to ground subnetworks, and is more appropriate for 5.7.5.

# Proposed Change

It is therefore proposed to move 5.7.5.1.4 to become 5.7.5.2. In order to place it in context, an explanatory note should precede the new 5.7.5.2., and 5.7.5.1 needs also to be amended to acknowledge the possibility of the use of the Mobile SNDCF.

5.7.5.1 Over ISO/IEC 8208 General Topology Subnetworks, the subnetwork service described in 5.7.2 shall be provided using either the SNDCF for ISO/IEC 8208 General Topology Subnetworks as specified in ISO/IEC 8473-3, or the Mobile SNDCF specified below in 5.7.6.

Note. — Although most ATN Ground Systems are generally expected to use the ISO/IEC 8473-3 specified SNDCF over ISO/IEC 8208 General Topology Subnetworks, Ground Systems may specify the use of the Mobile SNDCF, in order to improve bandwidth utilisation over fixed ISO/IEC 8208 subnetworks.

5.7.5.2 Recommendation.— Implementations using the Mobile SNDCF as specified in 5.7.6,  and the LREF Compression Procedure for Ground/Ground communications, should also use the LREF optional local reference cancellation mechanism.