Flimsy #1
24 June 1996

Rev. No.

Amending the Definition the Default Route to all Mobiles

# Introduction

During a review of the draft ATN SARPs Sub-Volume 5, it was agreed that the definition in 5.2.2.7.1.(b) of the default route to  all mobiles was deficient in that it did  not recognise that this route must have two NSAP Address Prefixes in its NLRI. This is because there are two Network Addressing Domains for airborne systems.

# Proposed Text

The following is the proposed new text of 5.2.2.7.1.(b):

- has in its destination two NSAP address prefixes. One of these is the NSAP Address Prefix that is common to all AINSC Airborne Systems and only AINSC Airborne Systems, and the other is the NSAP Address Prefix that is common to all ATSC Airborne Systems and only ATSC Airborne Systems.