# Network and Transport Addressing Specification

## Introduction

Note 1.— The ATN Internet Addressing Plan defines an OSI Network Service Access Point (NSAP) address structure which can support efficient internet routing procedures, and which conforms to common abstract syntax, semantic and encoding rules throughout the ATN OSI environment.

Note 2. — In general, where reference is made in this chapter to delegation of administrative responsibility by ICAO to States or organisations, it is expected that the practical effect of this delegation is that the respective States or organisations assume full administrative duties related to the delegated responsibilities. This means, for example, that if ICAO delegates to one or more States or organisations the responsibility for allocation, assignment and general administration of particular segments of the ATN address space, then those States or organisations must place into operation the necessary administrative structure to carry out the delegated allocation, assignment and administration activities. After having carried out the delegated administration of these field values, the State or organisation is then obliged to inform ICAO on a mutually agreed basis of administrative actions taken, so that ICAO may fulfill its responsibility as the ATN addressing authority in terms of publication and communication of this information for use by the civil aviation community. It is also important to note that a State or organisation may request delegation by ICAO of direct responsibility for its own administrative address space, if and when that State or organisation wishes to commence its own administrative activities. Finally, it is important to note that the role of ICAO in this area is one of international coordination, advice and consultation in order to ensure orderly and efficient operation of the global aspects of the ATN. Thus, ICAO may be expected to provide counsel to States and organisations having assumed such delegated responsibilities, in order to ensure that address administration is carried out in a manner that supports the orderly and efficient global operation of the ATN Internet

### Addressing Plan Scope

#### The ATN Internet Addressing Plan shall be used by ATN End Systems and Intermediate Systems.

Note.— The ATN Internet Addressing Plan serves the needs of a variety of aeronautical data communication user groups, including ATSC and AINSC users.

### Addressing Plan Applicability

#### Note.— The ATN Internet Addressing Plan defines the Network and Transport Layer addressing information to be utilized by ATN End Systems, and by ATN Intermediate Systems.

## Network Layer Addressing

### General

##### ATN NSAP addresses shall provide unambiguous identification of NSAPs and network entities located in ATN End Systems and Intermediate Systems operating within the ATN environment.

Note. — When allocating NSAP addresses, it is important to consider the impact of address assignment strategies on the quantity and frequency of exchange of routing information. In other words, while it is a given that the unambiguous identification of Network entities is the primary goal of the assignment of Network addresses, it is also true that without due caution, both routing information exchange efficiency and the resulting effectiveness of routing aggregation may be adversely affected. These effects must be considered when performing the administrative functions of address assignment and field value allocation.

#### Conformance to ISO Standards

##### ATN NSAP addresses shall conform to ISO/IEC 8348.

#### ISO Address Format Definition

##### All ATN NSAP addresses shall use an ISO 6523-ICD IDI format and a binary abstract syntax DSP.

Note. —  This requires an AFI value of [47].

#### Subnetwork Independence

Note . — The ATN NSAP Addressing Plan doesnot depend upon particular subnetwork address syntax, semantics or encoding techniques.

Note.— This facilitates the internetworking of End Systems and Intermediate Systems operated by various end-users and service providers within the aeronautical domain, while allowing these end-users and service providers to retain local control of local End System and subnetwork operations.

#### Address Mapping

##### ATN End Systems and Intermediate Systems shall perform any mapping of ATN NSAP addresses to the SNPA addresses required for onward forwarding of CLNP packets.

Note.— For this reason, only ATN Host Computers (i.e. End Systems) and Routers (i.e. Intermediate Systems) need evaluate the contents of an ATN NSAP address.

#### Definition of a Network Entity Title

Note 1.— A Network Entity Title (NET) is the unique name of a Network Entity (NE) contained in an End System (ES) or in an Intermediate System (IS). It is used to unambiguously identify a given NE. An End- or Intermediate System may comprise multiple NEs, in which case each will be identified by a unique NET.

Note 2.— NETs are assigned from the same addressing space as Network Service Access Point (NSAP) addresses. The authority which is responsible for allocating addresses from a given address space to NSAPs, may choose also to allocate NET’s following the same procedures and rules it observes in the allocation of NSAPs.

Note 3.— NETs and NSAP addresses are syntactically indistinguishable; any value that the responsible authority is permitted to allocate as an NSAP address may be allocated as a NET.

Note 4.— The ATN NSAP Addressing Plan mandates specific values for the Selector (SEL) field for two types of NEs contained in ATN Intermediate Systems, as given in 5.

#### Network Addressing Domains

Note . — A Network Addressing Domain is a subset of a Global Network Addressing Domain consisting of all of the NSAP addresses that are assigned by one or more addressing authorities, as defined in ISO/IEC 8348.

##### An ATSC Network Addressing Domain shall be a Network Adressing Domain administered by an ATSC authority.

##### An AINSC Network Addressing Domain shall be a Network Adressing Domain administered by a member of the Aeronautical Industry.

##### ATN End Systems or Intermediate Systems located on-board general aviation aircraft shall belong to an ATSC Network Addressing Domain, whereas ATN systems installed on-board commercial aircraft shall belong to an AINSC Network Addressing Domain.

### Abstract Syntax Provisions

#### General

##### The Domain Specific Part (DSP) of the ATN NSAP Addresses shall be represented in the binary abstract syntax.

Note 1.— The Initial Domain Part (IDP) is specified in ISO/IEC 8348 to have a decimal abstract syntax.

Note 2.— For the purpose of administration and assignment of ATN NSAP Addresses, the NSAP address format includes nine fields, two of which comprise the IDP (AFI and IDI) and the remaining seven of which comprise the DSP.

Note 3 . — Certain DSP fields of ATN NSAP addresses are derived from existing ICAO and IATA registration schemes which do not specify them in a binary abstract syntax.  Rules for the translation of their native abstract syntaxes to the required binary abstract syntax are provided in this specification.

Format Definition

Figure 5.4-1 ATN NSAP Address Format

##### The ATN NSAP address format shall comprise nine fields, as specified in Figure 5.4-1.

#### Field Abstract Syntax

##### The address field abstract syntax shall be as specified in Table 5.4-1.

#### Field Syntax Construction

##### All fields shall be right-justified, and shall be filled with leading zero digits (for decimal or hexadecimal syntax fields) or leading ''@'' characters (for

alphanumeric syntax fields) as required to produce fields of specified fixed lengths.

#### Reserved Values in Address Fields

##### Address field values specified as "reserved" shall not be used until assigned by future versions of the specification.

Encoding Provisions

Note.— This clause specifies general provisions applicable to the encoding rules of the ATN NSAP Addressing Plan.

#### Field Encoding Definition

##### The address field encoding shall be as specified in Table 5.4-1.

#### Fill Bits

Unspecified leading bits in all encoded fields shall be filled with the binary value zero.

Table 5.4-1 ATN NSAP Address Field Characteristics

| Address Field Name | Address Field Size | Field Abstract Syntax | Digit/Character Count | Field Encoding |
| --- | --- | --- | --- | --- |
| AFI | 1 Octet | Decimal | 2 Digits | BCD |
| IDI | 2 Octets | Decimal | 4 Digits | BCD |
| VER | 1 Octet | Hexadecimal | 2 Digits | Binary |
| ADM | 3 Octets | Hexadecimal | 6 Digits | Binary |
| RDF | 1 Octet | Hexadecimal | 2 Digits | Binary |
| ARS | 3 Octets | Hexadecimal | 6 Digits | Binary |
| LOC | 2 Octets | Hexadecimal | 4 Digits | Binary |
| SYS | 6 Octets | Hexadecimal | 12 Digits | Binary |
| SEL | 1 Octet | Hexadecimal | 2 Digits | Binary |

#### Field Size

##### All encoded fields shall be fixed in size, as specified in Table 5.4-1.

#### Encoding of Decimal Syntax

##### The BCD numeric digit set shall comprise the ten digits ''0'' through ''9''. BCD field digits shall be represented with 4-bit binary numbers ranging in encoded decimal values from [0] through [9].

#### Encoding of Hexadecimal Syntax

##### The hexadecimal digit set shall comprise the sixteen digits ''0'' through ''9'', and ''a'' through ''f''. Hexadecimal field digits shall be represented with 4-bit binary numbers ranging in encoded decimal values from [0] through [15].

#### Encoding of Alphanumeric Syntax

##### The alphanumeric character set shall comprise the International Alphabet No. 5 (IA-5) character set, and shall be encoded according to IA-5 encoding rules.

#### Encoding of Identifier Syntax

##### The identifier meta-character set shall comprise the list of all three-character identifiers composed from the character set including only:

- the decimal numerals ''0'' through ''9'', with decimal ordinal values [0] through [9];

- the symbol ''@'', with decimal ordinal value [10]; and,

- the upper-case alphabet ''A'' through ''Z'', with decimal ordinal values [11] through [37].

##### The resulting list of identifier metacharacters shall comprise a range in abstract syntax from ''000'' through ''ZZZ'', with an ordinal range in encoded decimal values from [0] through [50652].

Note 1.— This construct may be viewed as a three-digit radix-37 number.

Note 2.— This encoding mechanism is provided to support the optional mapping of certain IATA-defined character-based identifiers into binary address fields.

### Address Field Provisions

Note 1.— This clause specifies the detailed requirements for each of the nine fields which form the ATN NSAP address.

Note 2.— Field value specifications are enclosed in square brackets (i.e. [ ]), with syntax as noted (i.e. hexadecimal, decimal, alphanumeric or identifier).

#### Authority/Format Identifier

Note.— The Authority/Format Identifier (AFI) field specifies the authority responsible for allocating values for the IDI field, the format of the IDI field, and the abstract syntax representation used for the Domain Specific Part (DSP) of the NSAP address.

##### Format

###### The AFI field shall contain the two-digit decimal representation of the ATN NSAP Authority and Format Identifier.

##### Administration

Note.— The AFI field value for an ATN NSAP address has been selected by ICAO from the set of values specified by ISO.

##### Range

###### The AFI field value for an ATN NSAP address shall be [47], expressed as a decimal number.

##### Encoding

###### The AFI field shall be encoded using BCD rules, as defined in 5..

#### Initial Domain Identifier

Note . — The Initial Domain Identifier (IDI) field introduces the abstract syntax and semantics of the DSP portion of the ATN NSAP address.

##### Format

###### The IDI field shall contain the four-digit decimal representation of the International Code Designator which defines an ICAO standardized ATN NSAP DSP format.

##### Administration

Note.— The IDI value of an ATN NSAP Addresses is registered with ISO.

##### Range

###### The IDI field value for an ATN NSAP address shall be [0027], expressed as a decimal number.

##### Encoding

###### The IDI field shall be encoded using BCD rules, as defined in 5..

#### Version Identifier

Note . — Beside the version number of the ATN NSAP Addressing Plan which is applicable to the ATN NSAP address, the Version Identifier (VER) field identifies the type of the addressed object in terms of whether it belongs to a fixed or Mobile system, and whether it belongs to an ATSC or AINSC addressing domain.

##### Format

###### The VER field shall contain the two-digit hexadecimal version number of the ATN NSAP Addressing Plan applicable to the ATN NSAP address which represents

- the network addressing domain type (ATSC or AINSC), and

- whether the addressed object is contained in a mobile or fixed ATN system.

##### Administration

Note.— The VER field value for an ATN NSAP address is defined by this specification and is administered, if required, through future versions of this specification.

##### Range

###### The VER field value for Version 1 of an AINSC ATN NSAP Address in fixed systems, expressed as a two-digit hexadecimal number, shall be [01].

###### The VER field value for Version 1 of the AINSC ATN NSAP Address in Mobile systems, expressed as a two-digit hexadecimal number, shall be [41].

###### The VER field value for Version 1 of an ATSC ATN NSAP Address in fixed systems, expressed as a two-digit hexadecimal number, shall be [81].

###### The VER field value for Version 1 of an ATSC ATN NSAP Address in Mobile systems, expressed as a two-digit hexadecimal number, shall be [c1].

Note.— VER field values in the range [02 - 3f] may be allocated to fixed AINSC systems and VER field values in the range [42 - 7f] to Mobile AINSC systems in future versions of this specification. VER field values in the range [82 - bf] may be allocated to fixed ATSC systems and VER field values in the range [c2 - ff] to Mobile ATSC systems in future versions of this specification.

These values shall be reserved until they are assigned.

###### The VER field shall be encoded using binary rules, as defined in 5..

#### Administration Identifier

Note . — The Administration Identifier (ADM) field specifies the organization responsible for administration of an ATN Network Addressing Domain.

##### Format

###### General

The format of the ADM field shall be as given in 5. or 5., based on whether the organization responsible for administration of the associated ATN Network Addressing Domain is an ATSC organization or an AINSC organization.

###### ATSC Format

The ATSC ADM field shall contain the six-digit hexadecimal code for an ATSC Network Addressing Domain identifier.

This field value shall be derived from the set of three-character alphanumeric symbols representing a three-character ISO country designator as defined in ISO 3166.

###### AINSC Format

The AINSC ADM field shall contain the six-digit hexadecimal code for an AINSC Network Addressing Domain identifier.

Recommendation.— This field value should be derived from the set of three-character alphanumeric symbols representing an IATA Airline or Aeronautical Stakeholder Designator.

##### Administration

###### General

ADM field values shall be administered by the authority given in 5. or 5., based on whether the associated ATN Network Addressing Domain is an ATSC domain or an AINSC domain.

###### ATSC Administration

Note. — ATSC ADM values are assigned by this specification..

###### AINSC Administration

AINSC ADM values shall be assigned and administered by IATA.

Note.— Authority may be further delegated to aeronautical industry authorities as required.

##### Range

###### General

The range of ADM field values shall be determined as given in 5. or 5., based on whether the associated ATN Network Addressing Domain is an ATSC domain or an AINSC domain.

###### ATSC Range

The ATSC ADM field values shall be in the range [000000] - [ffffff], represented as hexadecimal numbers.

The ATSC ADM field shall contain the IA-5 encoding of valid three-character alphanumeric ISO Country Code values, as specified in ISO 3166.

For this purpose, the ATSC ADM field shall be represented using upper-case alphabetic characters.

###### AINSC Range

The AINSC ADM field values shall be in the range [000000] - [ffffff], represented as hexadecimal numbers.

Recommendation.— The AINSC ADM field should contain the IA-5 encoding of valid three-character alphanumeric IATA Airline or Aeronautical Stakeholder Designator values, as specified in the IATA Airline Coding Directory. For this purpose, the AINSC ADM field should be represented using upper-case alphabetic characters and decimal numerals.

##### Encoding

###### The ADM field shall be encoded using alphanumeric rules, as defined in 5..

#### Routing Domain Format

##### The RDF field value shall be [00] expressed as a two-digit hexadecimal number. Other RDF field values shall be reserved.

##### The RDF field shall be encoded using binary rules, as defined in 5..

#### Administrative Region Selector

Note . — The Administrative Region Selector (ARS) field identifies the routing domain of the addressed network entity or NSAP.

Note 1.— In the case of a fixed (i.e. ground-based) network entity, the ARS field will indicate the fixed routing domain within which that network entity is located.

Note 2.— In the case of a Mobile (i.e. aircraft-based) network entity, the ARS field will indicate the identity of the Mobile (i.e. aircraft) routing domain within which that network entity is located.

##### Format

###### General

The format of the ARS field shall be as given in 5., 5., or 5., based on the authority responsible for the associated ATN Administrative Domain, and based on the value of the VER field.

###### ATSC-Fixed Format

The ATSC-Fixed ARS field shall contain the six-digit hexadecimal code for an ATN routing domain, when a VER value of [81] is present.

Note.— An ATSC administration may opt to derive six-digit hexadecimal ATSC-Fixed ARS field value syntax from the IA-5 encoding of the three least-significant characters of a four-character alphabetic symbol representing an appropriate ICAO Location Indicator.

###### AINSC-Fixed Format

The AINSC-Fixed ARS field shall contain the six-digit hexadecimal code for an ATN routing domain, when a VER value of [01] is present.

Note.— An AINSC administration may opt to derive six-digit hexadecimal AINSC-Fixed ARS field value syntax from the IA-5 encoding of a three-character alphabetic symbol representing an appropriate IATA Location Identifier.

###### Mobile Format

The Mobile-ARS field shall contain a six-digit hexadecimal number representing the 24-bit ICAO Aircraft Identifier, when a VER value of [41] or [c1] is present.

##### Administration

###### General

ARS field values shall be administered by the authority given in 5., 5., or 5., based on the value of the VER field.

###### ATSC-Fixed Administration

ATSC ARS values shall be assigned and administered by ICAO.

Note.— Authority may be further delegated to State authorities as required.

###### AINSC-Fixed Administration

AINSC ARS values shall be assigned and administered by IATA.

Note.— Authority may be further delegated to aeronautical industry authorities as required.

###### Mobile Administration

Mobile ARS values shall be assigned and administered by ICAO.

Note.— Authority may be further delegated to State authorities as required.

##### Range

###### ARS field values shall be in the range [000000]-[ffffff], expressed as hexadecimal numbers.

##### Encoding

###### The ARS field shall be encoded using binary rules, as defined in 5..

##### Default Value

###### If an ATN administrative domain chooses not to allocate fixed ARS field values (i.e. chooses not to partition the administrative domain into routing domains), then the value of the ARS field for all fixed addresses within that ATN administrative domain shall be [000001].

###### No default ARS field value shall be defined or allowed for Mobile routing domains.

#### Location Identifier

Note . — The Location Identifier (LOC) field may be used for partitioning the address space administered by a single ATN routing domain authority into more than one routing area.

##### Format

###### General

The format of the LOC field shall be as given in 5., 5. or 5., based on the authority responsible for the associated ATN Network Addressing Domain, and based on the value of the VER field.

###### ATSC Format

The ATSC-Fixed LOC field shall contain the four-digit hexadecimal code for an ATN routing area identifier, when an ''ATSC'' ADM value is present in combination with a VER value of [81].

Note.— An ATSC administration may opt to derive four-digit hexadecimal ATSC LOC field value syntax from the three least-significant characters of a four-character identifier meta-character representing appropriate ICAO Location Indicators, encoded using ordinal rules defined in 5..

###### AINSC Format

The AINSC-Fixed LOC field shall contain the four-digit hexadecimal code for an ATN routing area identifier, when an ''AINSC'' ADM value is present in combination with a VER value of [01].

Note.— An AINSC administration may opt to derive four-digit hexadecimal AINSC LOC field value syntax from a three-character identifier meta-character representing appropriate IATA Location Identifiers, encoded using ordinal rules defined in 5..

###### Mobile Format

The Mobile-LOC field shall contain the four-digit hexadecimal code for an ATN routing area identifier, when a VER value of [41] or [c1] is present.

##### Administration

###### LOC field values shall be administered by the authority designated in the ADM field.

##### Range

###### LOC field values shall be in the range [0000]-[ffff], expressed as hexadecimal numbers.

##### Encoding

###### The LOC field shall be encoded using binary rules, as defined in 5..

##### Default Value

###### If an ATN routing domain administration chooses not to allocate LOC field values (i.e. chooses not to partition the routing domain into routing areas), then the value of the LOC field for all addresses within that ATN routing domain shall be [0001].

#### System Identifier

Note . — The System Identifier (SYS) field identifies the End System or Intermediate System containing the addressed Network entity or NSAP.

##### Format

###### The format of the SYS field shall be determined by the authority designated in the ADM field.

##### Administration

###### The SYS field shall be administered by the authority designated in the ADM field.

##### Range

###### SYS field values shall be in the range [000000000000]-[ffffffffffff], expressed as hexadecimal numbers.

##### Encoding

###### The SYS field shall be encoded using binary rules, as defined in 5..

#### NSAP Selector

Note . — The NSAP Selector (SEL) field identifies the End System or Intermediate System network entity or network service user process responsible for originating or receiving Network Service Data Units (NSDUs).

##### Format

###### The SEL field shall contain the two-digit hexadecimal representation of a network entity or network service user protocol selector.

##### Administration

###### SEL field values in the range [01-fd] shall be assigned and administered by the authority designated in the ADM field.

Note. — Authority may be further delegated by the designated authority as required.

##### Range

###### Valid SEL field values shall be in the range [00-ff].

###### The SEL field value for an Intermediate System network entity shall be [00], except for the case of an airborne iIntermediate System implementing the procedures for the otional non-use of IDRP.

###### In that case, the SEL field value shall be [fe].

###### The SEL field value [ff] shall be reserved.

Note. — SEL field values in stand-alone End Systems (i.e. in End Systems not co-located with Intermediate Systems) are not constrained.

##### Encoding

###### The SEL field shall be encoded using binary rules, as defined in 5..

## Transport Layer Addressing

### General

Note 1.— This section provides requirements on the format of ATN TSAP addresses. An ATN TSAP address is an NSAP address and a TSAP selector.

Note 2.— The requirements in this section apply to the administration of transport addresses local to an ATN End System. They do not apply to all systems in a global OSI Environment. An ATN System may allow remote transport addresses to obey different standards, e.g. when interworking with a non-ATN system is required.

### ATN TSAP Selector

#### An ATN TSAP selector shall be either one or two octets in length.

### Format

#### The TSAP Selector field shall be interpreted as an unsigned binary integer.

### Administration

#### The TSAP Selector field shall be administered on a local basis.

### Range

#### Valid ATN TSAP Selector field values shall be in the range [0 -65535].

### Encoding

#### The TSAP Selector field shall be encoded using binary rules, as defined in 5..

#### Recommendation.— TSAP selector values in the range [0 -255] should be encoded using one octet, higher values should be encoded using two octets.