FLIMSY  ___

Working Group 2/8

Brussels, Belgium @ Eurocontrol

PROVISIONS  FOR  FAST SELECT CONTROL

IN ICAO AMSS SARPS  AND THE INMARSAT

SATELLITE SUBNETWORK

WG-2 requested to provide information on the availability of the  Fast Select Functionality on the Inmarsat network.

There are provisions in the system to meet this requirement, at the user’s request. However, it should be noted that this parameter is only provided in order to allow interworking with X.25 networks, and is not a standard parameter in the OSI Network Service Interface.  It is only present when requested by an SNS-user  interworking with such a network  and  requiring these specific services for a particular connection.

The Fast Select Control parameter is used to impose controls on the transfer of user data during connection establishment and release  and on whether or not the connection may enter the data-transfer state.

Current edition of the ICAO Annex 10, Vol. III, Part 1 Chapter 4 contains provisions to allow Fast Select Control for the AMSS users, when the  aeronautical mobile-satellite service is installed and maintained in operations as an aid to the air traffic services, however, this facility is not mandatory (Section 4.7).

The ATN Sub Volume V SARPS should take these provisions accordingly.