ATNP/WG-2/Flimsy 10 &12

April 24, 1996

Brussels, 4/22/96 - 4/26/96

WP/270 was presented at the WG-2/8 meeting at Brussels.  This paper recommended some changes to the SNDCF requirements specified in the CNS/ATM-1 SARPs such that VDL and ATN SNDCFs can be combined.  WG-2 agreed to the concepts proposed in the WP/270 but  expressed some concerns with the use of Fast Select over multiple public data networks connected via X.75 gateways.

Flimsies 10 and 12 are prepared to specifically address some of these issues and make necessary change proposals to Package-1 SARPs.

Use of Fast Select

AMCP mandated the use of Fast Select over the VDL mobile subnetwork to minimize bandwidth requirement during connection setup.  WG-2 acknowledged that bandwidth conservation is a prime objective but due to practical limitations, it may not be possible to use Fast Select over all subnetworks.  As such, use os Fast Select is only mandated when supported by the underlying subnetwork connecting the source and destination DTEs.  The characteristics of the subnetworks must be known apriori by the DTEs during configuration time which will permit the SNDCF to exercise the appropriate option.

Compression Noegotiation without Fast Select

If Fast Select is not available, negotiating compression techniques become difficult due the absence of Sync. Points.  If data PDUs are used to exchage compression options, the receiving SNDCF can not determine when to exercise the uncompression algorithm.  To simplify the compression negotiation without Fast Select, WG-2 agreed that the called DTE should accept the call if it supports the proposed compression technique.  Otherwise, the called DTE will clear the call with specific diagnostic code.  This code will allow the calling DTE to determine the clearing cause and reestablish the VC by selecting the right compression option.  A new diagnostic code is intrduced in this flimsy for this purpose.

Maintaining Local Reference

Due to high rate of ground station handoffs in VDL, air/gound VCs are establinhed every 10 to 12 minutes.  If local  reference tables are maintain for each VC, the compression efficiency will reduce significantly.  By using a spare bit in the SNDCF header, the local reference tables can be consolidated for all VCs between the same calling and called DTE pairs.  VDL requires that a new subnetwork connection be established before clearing the old VC which permits the sharing of compression tables between VCs.  If the SNDCF does not support this option over a specific subnetwork, such as AMSS, then the M/I bit can be set to 0 by the called SNDCF, thereby refusing the local reference sharing among VCs.

Attached are appropriate pages  of the SARPs with revision marks to identify the changes recommended for the above.  WG-2 is invited to review and approve these proposals.