Flimsy #16
26 April 1996

Rev. No.

Change Proposal to enable the "Weak" Interpretation of ATSC Class

# Introduction

Following the acceptance of WP282, the following Change Proposal (based on WP266) has been agreed by WG2.

# Proposed Changes to 3.2.1.2.1.2

3.2.1.2.1.2 ATSC Class Specified

Note 1.— This case corresponds to Traffic Type and Associated Routing Policy Security Tag values 000 10000 to 000 10111 inclusive.

If the NPDU contains a CLNP Header Security Parameter in the globally unique format, and encodes:

- security related information according to Chapter 6 under the ATN Security Registration Identifier,

- a traffic type of ATN Operational Communications - Air Traffic Service Communications, and

- a requirement to route the NPDU over a route of a specified ATSC Class,

then the NPDU shall be forwarded over a selected route to the NPDU’s destination that contains a security path attribute comprising the ATN Security Registration Identifier and security information that comprises:

- An Air/Ground Subnetwork Security Tag that has “ATN Operational Communications - Air Traffic Services Communications” in its set of permissible Traffic Types, or

- no Air/Ground Subnetwork Security Tag

and,

an ATSC Class Security Tag indicating:

- support of the required class, or a higher class, or

- if no such route is available then, the route with the highest ATSC Class available is chosen..

Note 2.— ATSC Class “H” is the lowest and Class “A” is the highest.

If no such route can be found then the NPDU shall be discarded.

If multiple routes are available which meet or exceed the required ATSC Class, then the route with the lowest relative cost shall be selected.

# Proposed Change to 8.3.1.6.3

Delete the note in this section.

# Proposed Change to Table 6-1

- For ATN Operational Communications - ATSC, change “only follows” to “prefers”.

- Delete Security Tag Value 000 00011 and Semantics from the Table.

# Proposed Change to 2.7.1.1

Delete the 2nd list item (b)

# Proposed Change to 3.2.1.2.2

Delete all.