ATNP/WG2/
 WP171/
14 September 1995

Review of Route Initiation APRL (ATNP/WG2 Action 5/6)

SUMMARY

In response to action 5/6, a review of the Route Initiation APRLs has been undertaken. This has resulted in the identification of several defects and mis-alignment with the current SARPs for Route Initiation, requiring changes to the APRLs. This paper provides a proposed replacement APRL that fixes the identified defects.

C&R Ref.

Presented By Henk Hof

Prepared by Tony Whyman

Rev. No.

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP TWO

Banff 9.10.95-13.10.95

DOCUMENT CONTROL LOG

| SECTION | DATE | REV. NO. | REASON FOR CHANGE OR REFERENCE TO CHANGE |
| --- | --- | --- | --- |
|  | 14-Sep-95 | Issue 1.0 |  |

TABLE OF CONTENTS

1. Introduction

2. Review Comments

3. Proposed APRL

3.1 APRL for Air/Ground Route Initiation

3.1.1 General

3.1.2 Airborne Router - Connection Responder

3.1.3 Airborne Router - Connection Initiator

3.1.4 Air/Ground Router - Connection Responder

3.1.5 Air/Ground Router - Connection Initiator

3.1.6 Termination Procedures

# Introduction

At the ATNP/WG2 Rome Meeting, the APRL provided in appendix 10 for Route Initiation was reviewed, and deficiencies noted. An action was then accepted by Eurocontrol to provide a more detailed review. The result of this review is contained in this paper, together with a proposed updated APRL.

# Review Comments

- Generally, the APRL in A1.13 of Draft 2.1 has been overtaken by events. The Rome meeting introduced a third mode of operation (symmetric) and the editing work done for appendix 6 had made significant changes to the structure of the requirements text and which needs to be reflected in the APRL.

- Emergency Mode is not included in the APRL

- The APRL assumes that the Leave Event is only appropriate to networks that also provide a Join Event. This is not true.

- ISH in Call Confirm is only appropriate to networks that provide Fast Select and when the Mobile SNDCF is employed. This dependency needs to be made clear.

- Validation of Call Requests is not included in the APRL.

- Optional non-use of IDRP is not included in the APRL.

- Recommendation on ISH PDU Suppression missing.

# Proposed APRL

## APRL for Air/Ground Route Initiation

### General

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| njSubnet | Support of Subnetworks that do not provide Join Event | 3.5.2 | O.1 |
| jSubnet | Support of Subnetworks do provide a Join Event | 3.5.2 | O.1 |
| giSubnet | Support of Ground Initiated Subnetworks | 3.5.2 | O.2 |
| aiSubnet | Support of Air Initiated Subnetworks | 3.5.2 | O.2 |
| agSubnet | Support of Air or Ground Initiated Subnetworks | 3.5.2 | O.2 |
| fsSubnet | Support of Subnetworks that support Fast Select | - | O |
| noIDRP | Support of optional non-use of IDRP | 3.5.2.11 | O |
| lvSubnet | Support of Subnetworks that provide a Leave Event | 3.5.2.12 | M |

### Airborne Router - Connection Responder

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| respAR-ar | Response to incoming Call Requests | 3.5.2.1 | giOragSubnet: M |
| valCR-ar | Validation of incoming Call Request | 3.5.2.1 | giOragSubnet:O |
| RespISH-ar | Generation of ISH PDU | 3.5.2.5 | giOragSubnet: M |
| ISHinCC-ar | Encoding ISH PDU in Connect Confirm User Data | 3.5.2.5 | RespISH-ar & fsSubnet: O |
| negNoIDRP-ar | Transmission of ISH with SEL field set to 0FEh | 3.5.2.5 | noIDRP:M |
| negIDRP-ar | Transmission of ISH with SEL field set to zero | 3.5.2.5 | ¬noIDRP:M |
| autoRoute-ar | Inference of available routes from A/G Router NET | 3.5.2.11.2 | noIDRP:M |
| initIDRP-ar | IDRP startup procedures - ListenForOpen set to true | 3.5.2.9 | ¬noIDRP:M |
| supISH-ar | Suppression of multiple ISH PDUs | 3.5.2.9 | ¬noIDRP: O |
| valNET-ar | Validation of received NET | 3.5.2.6 | ¬noIDRP: O |

giOragSubnet: giSubnet ^ agSubnet

### Airborne Router - Connection Initiator

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| polling-ai | Procedures for polling a list of subnet addresses | 3.5.2.2.1 | pollReq: M |
| backoff-ai | Backoff Procedure | 3.5.2.2.1.1 | pollReq: M |
| connect-ai | Connect on receipt of Join Event | 3.5.2.2.2 | EventDrvn: M |
| ValJoin-ai | Validation of Join Event | 3.5.2.2.2 | EventDrvn: O |
| SendISH-ai | Generation of ISH PDU | 3.5.2.5 | EventDrvn ^ pollReq:M |
| ISHinCR-ai | Encoding of ISH PDU in Connect Request | 3.5.2.5 | SendISH-ar & fsSubnet: O |
| negNoIDRP-ai | Transmission of ISH with SEL field set to 0FEh | 3.5.2.7 | noIDRP:M |
| negIDRP-ai | Transmission of ISH with SEL field set to zero | 3.5.2.7 | ¬noIDRP:M |
| autoRoute-ai | Inference of available routes from A/G Router NET | 3.5.2.11.2 | noIDRP:M |
| initIDRP-ai | IDRP startup procedures - listenForOpen set to false | 3.5.2.9 | ¬noIDRP:M |
| supISH-ai | Suppression of multiple ISH PDUs | 3.5.2.9 | ¬noIDRP: O |
| valNET-ai | Validation of received NET | 3.5.2.6 | ¬noIDRP: O |

pollReq: aiSubnet & njSubnet

EventDrvn: jSubnet & (aiSubnet ^ agSubnet)

### Air/Ground Router - Connection Responder

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| respAR-agr | Response to incoming Call Requests | 3.5.2.1 | aiOragSubnet: M |
| valCR-agr | Validation of incoming Call Request | 3.5.2.1 | aiOragSubnet:O |
| emgncy-agr | Emergency Procedures | 3.5.2.1.1 | M |
| RespISH-agr | Generation of ISH PDU | 3.5.2.5 | aiOragSubnet: M |
| ISHinCC-agr | Encoding ISH PDU in Connect Confirm User Data | 3.5.2.5 | RespISH-agr & fsSubnet: O |
| negNoIDRP-agr | Receipt of ISH with SEL field set to 0FEh | 3.5.2.7 | M |
| negIDRP-agr | Receipt of ISH with SEL field set to zero | 3.5.2.7 | M |
| autoRoute-agr | Inference of available routes from Airborne Router NET | 3.5.2.11.1 | M |
| initIDRP-agr | IDRP startup procedures - ListenForOpen set to true | 3.5.2.9 | M |
| supISH-agr | Suppression of multiple ISH PDUs | 3.5.2.9 | ¬noIDRP: O |
| valNET-agr | Validation of received NET | 3.5.2.6 | ¬noIDRP: O |

aiOragSubnet: aiSubnet ^ agSubnet

### Air/Ground Router - Connection Initiator

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| connect-agi | Connect on receipt of Join Event | 3.5.2.3 | goOragSubnet: M |
| ValJoin-agi | Validation of Join Event | 3.5.2.3 | connect-agi: O |
| SendISH-agi | Generation of ISH PDU | 3.5.2.5 | connect-agi: M |
| ISHinCR-agi | Encoding of ISH PDU in Connect Request | 3.5.2.5 | Send-ISH-agi & fsSubnet: O |
| negNoIDRP-agi | Receipt of ISH with SEL field set to 0FEh | 3.5.2.7 | M |
| negIDRP-agi | Receipt of ISH with SEL field set to zero | 3.5.2.7 | M |
| autoRoute-agi | Inference of available routes from Airborne Router NET | 3.5.2.11.1 | noIDRP:M |
| initIDRP-agi | IDRP startup procedures - listenForOpen set to false | 3.5.2.9 | ¬noIDRP:M |
| supISH-agi | Suppression of multiple ISH PDUs | 3.5.2.9 | ¬noIDRP: O |
| valNET-agi | Validation of received NET | 3.5.2.6 | ¬noIDRP: O |

goOragSubnet: giSubnet ^ agSubnet

### Termination Procedures

| Item | Description | ATN SARPs Reference | CNS/ATM Package-1 Support |
| --- | --- | --- | --- |
| lvEvent | Processing of Leave Event | 3.5.2.12 | M |
| Watchdog | Watchdog Timer | 3.5.2.12 | M |
| ConfigWD | Configurability of Watchdog for Subnetwork Characteristics | 3.5.2.12 | O |
| conLeave | Processing of a per connection Leave Event | 3.5.2.12 | M |
| subnetLeave | Processing of a per Subnetwork Leave Event | 3.5.2.12 | M |