# Transport Service and Protocol Specification

## General

- The COTP shall be used to provide an end-to-end reliable data transfer service between transport service users on two ATN ESs [a8 t 0010].

- In ATN ESs, the implementation of the COTP shall conform to ISO 8073:1992 and the mandatory requirements given in this chapter [a8 t 0020].

- The CLTP shall be used to provide a CL data transfer service between TS-users on two ATN ESs [a8 t 0030].

- In ATN ESs, the implementation of the CLTP shall conform to ISO 8602:1987 and the mandatory requirements given in this chapter [a8 t 0040].

Note.— The transport protocols specified for use in ATN ESs provide both CO and CL communication services.  The implementation and use of a particular mode of the transport layer service depends on the requirements of the application(s) supported by a given ATN ES. [a8 n 0005]

### References

1.	ISO/IEC 8073:1992 Information Technology - Telecommunications and Information Exchange between Systems - Open Systems Interconnection - Protocol for providing the Connection-mode Transport Service

2.	ISO/IEC 8602:1987 Information Processing Systems -  Open Systems Interconnection - Protocol for providing the Connectionless-mode Transport Service.

### Internet Service Description

Note 1. - When the TS-USER requires use of the connection mode transport protocol the TS-USER will provide the following information to the TS-PROVIDER on a per Transport Connection basis:

a)	called and calling TSAP address;

b)	whether or not the expedited data option is required;

c)	the required residual error rate (RER) to determine whether or not the transport checksum is required;

d)	the Application Service Priority to be mapped into the resulting CLNP NPDUs according to Table 2-2;

e)	the ATN Traffic Type, i.e.

- ATN Operational Communications;

- ATN Administrative Communications;

- General Communications;

- Systems Management Communications.

In the case where the Traffic Type specified is ATN Operational Communications the TS-USER will additionally provide the Sub-type, i.e. Air Traffic Services Communications (ATSC) or Aeronautical Operational Control Communications (AOCC).

In the case of  the ATSC sub-type the TS-USER will further specify the required Class of Communications Service from Class A to Class H.

In the case of the AOCC sub-type the TS-USER will further specify the subnetwork preference (including no preference).

The ATN Traffic Types and their associated Sub-types are specified in Chapter 6, Table 6-1.

Note 2. - The TS-USER is not required to specify any other Transport Service Quality of Service parameters.

### Transport Service Access Point Addresses

- A TSAP address shall comprise two elements, a NSAP address and a TSAP selector [a8 t 0050].

- The NSAP address and the TSAP selector shall conform to the provisions in Chapter 4 [a8 t 0060].

### Network Service Specification

1.  The COTP shall operate using the CLNS as specified in Chapter 6 [a8 t 0070].

Note.— TPDUs are sent via the N-UNITDATA Request primitive.

### Exchange of Transport-Selector parameters

Note 1. — TSAP Selectors are transmitted in Calling and Called Transport-Selector parameters in COTP, and in Source and Destination Transport-Selector parameters in CLTP.

5.1.5.a. The transport entity shall support Transport-Selector parameters to accomodate the ATN TSAP selector syntax and encoding requirements as specified in Chapter 4 [a8 t 0071].

Recommendation.—  The transport entity should support remote Transport-Selector parameters of variable size from 0 up to 32 octets using any encoding and any value. [a7 r 0055]

Note 2. — The absence of a Calling and Called Transport-Selector assumes the Network Address alone unambiguously defines the Transport Address.

5.1.5.b. In COTP, on receipt of CR TPDU, the absence of a Calling or Called Transport-Selector shall be treated as equivalent to a zero length Calling or Called Transport-Selector [a8 t 0072]. The absence of a Calling or Called Transport-Selector in a received CC TPDU shall indicate that Calling or Called Transport-Selector is equivalent to the corresponding parameter specified in the sent CR TPDU [a8 t 0073]. When present in a received CC TPDU, Calling and Called Transport-Selector parameters shall be identical in length and value to the corresponding parameter specified in the sent CR TPDU [a8 t 0074].

5.1.5.c. In CLTP, on receipt of UD TPDU, the absence of a Source or Destination Transport-Selector shall be treated as equivalent to a zero length Source or Destination Transport-Selector [a8 t 0075].

## Connection Mode Transport Layer Operation

### Connection Mode Transport Protocol Overview

ATN ESs shall implement the ISO 8073 Class 4 transport protocol in order to provide CO communications over the ATN internetwork [a8 t 0080].

Note.— The ATN COTS model conforms to the model defined in ISO 8072 [a8 n 0020].

### Connection Mode Transport Quality of Service

#### Connection Mode Transport Priority

- The transport layer shall allow a TS-user to specify priority in the range [0 - 14]. [a8 t 0090]

- The transport layer shall not alter the proposed TC priority specified by the TS-user [a8 t 0100].

- The transport layer shall treat all connections without expressed priority as being at the default priority  [a8 t 0110]

- The default priority shall be the lowest priority [14]. [a8 t 0120]

- When a TS-user specifies a transport priority, the relationship between this transport priority and the network priority shall be as specified in Table 2-2. [ new]

#### Connection Mode Transport Security

1.	The transport layer shall allow a TS-user to specify security in accordance with the four Security Types applicable to the ATN and their respective sub-types, as defined in Chapter 6 (Table 6-1):

- ATN Operational Communications;

- ATN Administrative Communications;

- General Communications; and,

- ATN Systems Management Communications [a8 t 0130].

2.	The transport layer shall not alter the TC security specified by the TS-user [a8 t 0140].

3.	The transport layer shall treat all connections without expressed security as default security [a8 t 0150].

4.	The default security shall be the lowest security (i.e. General Communications). [a8 t 0160]

### Connection Mode Transport Service Primitives

Note 1. — For the purpose of describing the notional interfaces between different OSI protocol layers, each protocol layer is assumed to provide a service to the next higher protocol layer.  The assumed service provided by the OSI transport layer to its user is described in ISO 8072.

Note 2. — ATN Applications may specify their use of the COTP implemented in ATN ESs using the TS specified in ISO 8072 including use of, priority, and security parameters as specified in this specification.

Note 3. — There is no requirement to implement ISO 8072 as a software interface [a8 n 0040].

### Connection Mode Transport APRL

#### Mandatory and Optional Functions

Note. — The requirements for the COTP are provided in the form of an ATN Profile Requirements List (APRL).  The APRL has been prepared using the PICS proforma provided with ISO 8073.  The APRL conventions are described in Part 1 of the CNS/ATM-1 Package SARPs.

An implementation of the ISO 8073 Transport Protocol shall be used in an ATN ES if and only if its PICS is in compliance with the APRL provided with these SARPs [a8 t 0180].

##### Protocol Implementation

###### Classes Implemented

The ATN COTP shall implement the features marked "M" in the table.

| Index | Class | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| C0 | Class 0 | 14.2 | O.1 | O |
| C1 | Class 1 | 14.4 | C0:O | O |
| C2 | Class 2 | 14.2 | O.1 | O |
| C3 | Class 3 | 14.3 | C2:O | O |
| C4 | Class 4 operation over CONS | 14.3 | C2:O | O |
| C4L [a8 a 0060] | Class 4 operation over CLNS | 14.3 | C2:O | M |

###### Specific ATN Requirements.

The CNS/ATM-1 ATN COTP shall implement the features marked "M" in the table.

| Index | Feature | CNS/ATM-1 Package Support |
| --- | --- | --- |
| ATN1 [a8 a 0070] | Initiating CR TPDU? | M |
| ATN2 [a8 a 0080] | Responding to CR TPDU? | M |
| ATN3 [a8 a 0090] | Extended TPDU Numbering | O |
| ATN4 [a8 a 0100] | Acceptance of Non-use of Checksum? | M |
| ATN5 [a8 a 0110] | Use of Concatenation? | O |
| ATN6 [a8 a 0120] | Use of Selective Acknowledgement? | O |
| ATN7 [a8 a 0130] | Use of Request of Acknowledgment? | O |
| ATN8 [a8 a 0140] | Reduction of Credit Window | O |
| ATN9 [a8 a 0150] | ER TPDU Transmission? | O |
| ATN10 [a8 a 0160] | Use of Called Transport-Selector Parameter in CR TPDU? | M |
| ATN11 [a8 a 0170] | Use of Calling Transport-Selector Parameter in CR TPDU? | M |
| ATN12 [a8 a 0180] | Use of TPDU Size Parameter in CR TPDU? | O |
| ATN13 [a8 a 0190] | Use of the Additional Option Selection Parameter in CR TPDU? | M |
| ATN14 [a8 a 0200] | Use of the Priority Parameter in CR TPDU? | O |
| ATN15 [a8 a 0210] | Use of the Acknowledgment Timer Parameter in CR TPDU? | O |
| ATN16 [a8 a 0220] | Use of Preferred Maximum TPDU Size Parameter in CR TPDU? | O |
| ATN17 [a8 a 0230] | Use of Inactivity Time Parameter in CR TPDU? | O |
| ATN18 [a8 a 0240] | Use of Called Transport-Selector Parameter in CC TPDU? | M |
| ATN19 [a8 a 0250] | Use of Calling Transport-Selector Parameter in CC TPDU? | M |
| ATN20 [a8 a 0260] | Use of TPDU Size Parameter in CC TPDU? | O |

| Index | Feature | CNS/ATM-1 Package Support |
| --- | --- | --- |
| ATN21 [a8 a 0270] | Use of the Additional Option Selection Parameter in CC TPDU? | M |
| ATN22 [a8 a 0280] | Use of the Priority Parameter in CC TPDU? | O |
| ATN23 [a8 a 0290] | Use of the Acknowledgment Timer Parameter in CC TPDU? | O |
| ATN24 [a8 a 0300] | Use of Preferred Maximum TPDU Size Parameter in CC TPDU? | O |
| ATN25 [a8 a 0310] | Use of Inactivity Time Parameter in CC TPDU? | O |
| ATN26 [a8 a 0320] | 1024 octets as the minimum preferred maximum TPDU size in a CR TPDU? | O |
| ATN27 [a8 a 0330] | 1024 octets as the minimum preferred maximum TPDU size in a CC TPDU? | O |
| ATN28 [a8 a 0340] | 1024 octets as the largest value of the maximum TPDU size parameter in a CR TPDU with preferred class 4? | O |
| ATN29 [a8 a 0350] | 1024 octets as the largest value of the maximum TPDU size parameter which may be sent in a CC TPDU when class 4 is selected? | O |
| ATN30 [a8 a 0360] | Congestion Avoidance Measures? | O |
| ATN31 [a8 a 0370] | Quality of Service Mapping? | O |
| ATN32 [a8 a 0380] | Timer Settings? | O |

##### Initiator/Responder Capability for Protocol Classes 0-4

The ATN transport protocol shall implement the features marked " M" in the table [a8 t 0190].

| Index |  | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| IR1 [a8 a 0390] | Initiating CR TPDU | 14.5 a) | O.2 | M |
| IR2 [a8 a 0400] | Responding to CR TPDU | 14.5 a) | O.2 | M |

Note. — ISO 8073 requires that at least one of these options shall be implemented.

##### Supported Functions

###### Supported Functions for Class 4 (C4 OR C4L::).

Mandatory Functions for Class 4.

The ATN COTP shall implement the features marked "M" in the table.

| Index | Function | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| T4F1 [a8 a 0410] | TPDU transfer | 6.2 | M | M |
| T4F2 [a8 a 0420] | Segmenting | 6.3 | M | M |
| T4F3 [a8 a 0430] | Reassembling | 6.3 | M | M |
| T4F4 [a8 a 0440] | Separation | 6.4 | M | M |
| T4F5 [a8 a 0450] | Connection establishment | 6.5 | M | M |
| T4F6 [a8 a 0460] | Connection refusal | 6.6 | M | M |
| T4F7 [a8 a 0470] | Data TPDU numbering (normal) | 6.10 | M | M |
| T4F8 [a8 a 0480] | Retention and acknowledgement of TPDUs (AK) | 6.13.4.1 | M | M |
| T4F9 [a8 a 0490] | Explicit flow control | 6.16 | M | M |
| T4F10 [a8 a 0500] | Checksum | 6.17 | M | M |
| T4F11 [a8 a 0510] | Frozen references | 6.18 | M | M |
| T4F12 [a8 a 0520] | Retransmission on time-out | 6.19 | M | M |
| T4F13 [a8 a 0530] | Resequencing | 6.20 | M | M |
| T4F14 [a8 a 0540] | Inactivity control | 6.21 | M | M |

Mandatory Functions for Operation over Connectionless Network Service.

The ATN COTP shall implement the features marked "M" in the table.

| Index | Function | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| T4F23 [a8 a 0550] | Transmission over CLNS | 6.1.2 | M | M |
| T4F24 [a8 a 0560] | Normal release when operating over CLNS (explicit) | 6.7.2 | M | M |
| T4F25 [a8 a 0570] | Association of TPDUs with transport connections when operating over CLNS | 6.9.2 | M | M |
| T4F26 [a8 a 0580] | Expedited data transfer when operating over CLNS (Network normal) | 6.11.2 | M | M |
| T4F27 [a8 a 0590] | Treatment of protocol errors when operating over CLNS | 6.22.2 | M | M |

ISO 8073 Optional Functions.

The ATN COTP shall implement the features marked "Predicate:M" in the table if the predicate is true, i.e. the ATN recommendation has been followed.

| Index | Feature | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| T4F28 [a8 a 0600] | Data TPDU numbering (extended) | 6.10 | O | ATN3:M |
| T4F29 [a8 a 0610] | Non-use of checksum | 6.17 | O | ATN4:M |
| T4F30 [a8 a 0620] | Concatenation | 6.4 | O | ATN5:M |
| T4F31 [a8 a 0630] | Retention and acknowledgement of TPDUs Use of selective acknowledgement | 6.13.4.3 | O | ATN6:M |
| T4F32 [a8 a 0640] | Retention and acknowledgement of TPDUs Use of request acknowledgement | 6.13.4.2 | O | ATN7:M |

T4F30::  The transport entity shall not concatenate TPDUs from TCs with different transport priorities or different security types [a8 t 0200].

T4F31::  Recommendation.— The Selective Acknowledgement should be used for conservation of bandwidth by preventing retransmission of correctly received out-of-sequence TPDUs.

T4F32::  Recommendation.— The Request of Acknowledgement should be used to reduce AK traffic

##### Supported TPDUs

The ATN COTP shall implement the features marked "M" in the table.

| Index | TPDUs |  |  | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- | --- | --- |
| ST1 [a8 a 0650] | CR | supported on transmission | 13.1 | IR1:M | M |
| ST2 [a8 a 0660] | CR | supported on receipt | 13.1 | IR2:M | M |
| ST3 [a8 a 0670] | CC | supported on transmission | 13.1 | IR2:M | M |
| ST4 [a8 a 0680] | CC | supported on receipt | 13.1 | IR1:M | M |
| ST5 [a8 a 0690] | DR | supported on transmission | 13.1 | IR2:M | M |
| ST6 [a8 a 0700] | DR | supported on receipt | 13.1 | IR1:M | M |
| ST7 [a8 a 0710] | DC | supported on transmission | 13.1 | C4L:M | M |
| ST8 [a8 a 0720] | DC | supported on receipt | 13.1 | C4L:M | M |
| ST9 [a8 a 0730] | DT | supported on transmission | 13.1 | M | M |
| ST10 [a8 a 0740] | DT | supported on receipt | 13.1 | M | M |
| ST11 [a8 a 0750] | ED | supported on transmission | 13.1 | C4L:M | MO |
| ST12 [a8 a 0760] | ED | supported on receipt | 13.1 | C4L:M | MO |
| ST13 [a8 a 0770] | AK | supported on transmission | 13.1 | C4L:M | M |
| ST14 [a8 a 0780] | AK | supported on receipt | 13.1 | C4L:M | M |
| ST15 [a8 a 0790] | EA | supported on transmission | 13.1 | C4L:M | MO |
| ST16 [a8 a 0800] | EA | supported on receipt | 13.1 | C4L:M | MO |
| ST19 [a8 a 0810] | ER | supported on receipt | 13.1 | M | M |

Note.— The following table states for which classes, if any, ER TPDU is supported on transmission:

| Index | Class | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| SER4L [a8 a 0820] | Class 4 over CLNS | 6.22.2 | O | ATN9:M |

##### Supported Parameters of Issued TPDUs

###### Parameter Values for CR TPDU (C4L::)

If the additional options selection parameter is issued in a CR TPDU it is mandatory that: [a8 t 0210]

| Index |  | ISO/IEC 8073 Reference |
| --- | --- | --- |
| ICR1 [a8 a 0830] | Bits 8 to 7 shall be set to zero | 13.3.4 g) |

If the preferred class in the CR is 2,3 or 4:

| Index |  | ISO/IEC 8073 Reference | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| ICR2 [a8 a 0840] | Is class 0 always offered as an alternative class? | 14.4 | O | X |

###### Supported parameters for Class 4 TPDUs (C4L::)

Optional Parameters for a Connection Request TPDU.

The ATN COTP shall implement the features marked "M" in the table.

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4CR7 [a8 a 0850] | Called Transport-Selector | 13.3.4 a) | O | M |
| I4CR8 [a8 a 0860] | Calling Transport-Selector | 13.3.4 a) | O | M |
| I4CR9 [a8 a 0870] | TPDU size | 13.3.4 b) | O | ATN12:M |
| I4CR10 [a8 a 0880] | Version Number | 13.3.4 d) | O | O |
| I4CR11 [a8 a 0890] | Protection parameters | 13.3.4 e) | O | O |
| I4CR12 [a8 a 0900] | Additional option selection | 13.3.4 g) | O | M |
| I4CR13 [a8 a 0910] | Throughput | 13.3.4 k) | O | O |
| I4CR14 [a8 a 0920] | Residual error rate | 13.3.4 m) | O | O |
| I4CR15 [a8 a 0930] | Priority | 13.3.4 n) | O | ATN14:M |
| I4CR16 [a8 a 0940] | Transit delay | 13.3.4 p) | O | O |
| I4CR17 [a8 a 0950] | Acknowledgement time | 13.3.4 j) | O | ATN15:M |
| I4CR18 [a8 a 0960] | Preferred maximum TPDU size | 13.3.4 c) | O | ATN16:M |
| I4CR19 [a8 a 0970] | Inactivity timer | 13.3.4 r) | O | ATN17:M |

I4CR9::  Recommendation.— The transport layer should propose a TPDU size of 1024 octets or more [a8 r 0040]

Recommendation.— The transport layer should use the TPDU size parameter rather than the preferred maximum TPDU size parameter [a8 r 0045].

Optional Parameters for a Connection Confirm TPDU.

Note. — According to ISO, the following parameters are optional if a CC TPDU is issued in class 4:

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4CC6 [a8 a 0980] | Called Transport-Selector | 13.4.4 | O | M |
| I4CC7 [a8 a 0990] | Calling Transport-Selector | 13.4.4 | O | M |
| I4CC8 [a8 a 1000] | TPDU size | 13.4.4 | O | ATN20:M |
| I4CC9 [a8 a 1010] | Protection parameters | 13.4.4 | O | O |
| I4CC10 [a8 a 1020] | Additional option selection | 13.4.4 | O | M |
| I4CC11 [a8 a 1030] | Acknowledgement time | 13.4.4 | O | ATN23:M |
| I4CC12 [a8 a 1040] | Throughput | 13.4.4 | O | O |
| I4CC13 [a8 a 1050] | Residual error rate | 13.4.4 | O | O |
| I4CC14 [a8 a 1060] | Priority | 13.4.4 | O | ATN22:M |
| I4CC15 [a8 a 1070] | Transit delay | 13.4.4 | O | O |
| I4CC16 [a8 a 1080] | Preferred maximum TPDU size | 13.4.4 | I4CR18:O | ATN24:M |
| I4CC17 [a8 a 1090] | Inactivity timer | 13.4.4 | O | ATN25:M |

I4CC10:: Note.— The support of T4F26 implies that the Additional Options Selection parameter is mandatory. [a8 n 0056]

Optional Parameter for a Disconnect Request TPDU.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4DR4 [a8 a 1100] | Additional information | 13.5.4 a) | O | O |

Mandatory Parameter for a Data TPDU

Note.— According to ISO, the following parameter is mandatory in a DT TPDU if request of acknowledgement has been selected.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4DT4 [a8 a 1110] | ROA | 13.7.3 b) | T4F32:M | T4F32:M |

Optional Parameter for an Acknowledgement TPDU

Note.— According to ISO, an AK TPDU containing flow control information will be transmitted if an AK TPDU is received under the conditions specified in ISO 8073 12.2.3.9.  The following parameter is mandatory if an AK TPDU is issued in Class 4.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4AK4 [a8 a 1120] | Flow control confirmation | 13.9.4 c) | O | M |

Use of the Subsequence Number Parameter in the Acknowledgement TPDU

Note.— According to ISO, if an implementation can reduce credit and does so in the manner outlined in ISO 8073 12.2.3.8.2 then subsequence number in AK is mandatory.

| Index | Supported parameters | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4AK5 [a8 a 1130] | Subsequence number | 13.9.4. b) | O | ATN8:M |

Use of the Selective Acknowledgement Parameter in the Acknowledgement TPDU

Note.— According to ISO, the following parameter is optional in an AK TPDU if selective acknowledgement has been negotiated.

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4AK6 [a8 a 1140] | Selective acknowledgement parameters | 13.9.4. d) | T4F31:O | T4F31:O |

Optional Parameters for an Error TPDU

| Index | Supported parameter | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| I4ER3 [a8 a 1150] | Invalid TPDU | 13.12.4 a) | O | O |

##### Supported parameters for received TPDUs

Note.— ISO 8073 requires implementations to be capable of receiving and processing all possible parameters for all possible TPDUs, depending upon the class and optional functions implemented.

###### TPDUs in Class 4 (C4L::)

Note.— According to ISO, if use of checksum has been selected then it is mandatory to process a checksum parameter in the following TPDUs:

The ATN COTP shall implement the features marked "M" in the table.

| Index | TPDU | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| R4CCch [a8 a 1160] | CC TPDU | 13.4.4 | M | M |
| R4DRch [a8 a 1170] | DR TPDU | 13.5.4 b) | M | M |
| R4DCch [a8 a 1180] | DC TPDU | 13.6.4 | M | M |
| R4DTch [a8 a 1190] | DT TPDU | 13.7.4 | M | M |
| R4EDch [a8 a 1200] | ED TPDU | 13.8.4 | M | M |
| R4AKch [a8 a 1210] | AK TPDU | 13.9.4 a) | M | M |
| R4EAch [a8 a 1220] | EA TPDU | 13.10.4 | M | M |
| R4ERch [a8 a 1230] | ER TPDU | 13.12.4 b) | M | M |

##### User Data in Issued TPDUs

###### Class 4 (C4 or C4L::)

The ATN COTP shall implement the features marked "M" in the table.

| Index | User Data | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| D4ICR [a8 a 1240] | User data of up to 32 octets in a CR with preferred class 4 | 13.3.5 | M | M |
| D4ICC [a8 a 1250] | User data of up to 32 octets in a CC | 13.4.5 | M | M |
| D4IDR [a8 a 1260] | User data of up to 64 octets in a DR | 13.5.5 | M | M |

##### User Data in Received TPDUs

The transport layer shall be able to receive the following:

| Index | User Data | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| DRCC [a8 a 1270] | 32 octets of user data in a CC TPDU | 13.4.5 | IR1:M | IR1:M |
| DRDR [a8 a 1280] | 64 octets of user data in a DR TPDU | 13.5.5 | IR1:M | IR1:M |
| DRCR [a8 a 1290] | 32 octets of user data in a CR TPDU | 13.3.5 | IR2:M | IR2:M |

##### Negotiation

Note.— If an option is not returned in the CC, it is considered to have been refused. This allows compatible negotiation between versions of the ISO 8073 transport protocol [a8 n 0057]

###### Class Negotiation - Initiator

| Index | Feature | ISO/IEC 8073 References | CNS/ATM-1 Package Supported Value |
| --- | --- | --- | --- |
| NC [a8 a 1300] | The preferred class in the CR TPDU may contain any of the classes supported by the implementation | 6.5.5 j) | Class 4 |

Note 1.— Negotiation of other protocol classes is out of scope.  If this is the only profile supported then it is not possible to negotiate any other protocol class. [a8 n 0058]

Note 2.— The table below specifies valid alternative classes

| Index | Preferred class | ISO/IEC 8073 References | ISO Allowed values | CNS/ATM-1 Package Supported values |
| --- | --- | --- | --- | --- |
| NAC5 [a8 a 1310] | Class 4 over CLNS | 6.5.5 j) | None | None |

NAC5::  Note.— The class cannot be negotiated since Class 4 is the only class allowed over CLNS. [a8 n 0059]

###### Class negotiation - responder side

| Index | Preferred class | ISO/IEC 8073 References | ISO Allowed responses | CNS/ATM-1 Package Supported values |
| --- | --- | --- | --- | --- |
| RC4 [a8 a 1320] | What classes can you respond with if CR proposes only class 4? | 6.5.4 j) Table 3 | 2,4 or connection refused depending on classes supported | 4 |
| RC4a [a8 a 1330] | What classes can you respond with if CR proposes class 4 as preferred class and the alternative class parameter is present? | 6.5.4 j) Table 3 | 0,1,2,3,4 or connection refused depending on classes supported and coding of alternative class | 4 |

Note.— This table does not preclude connection refusal for other reasons [a8 n 0060].

###### TPDU Size Negotiation

| Index | TPDU size | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| TS1 [a8 a 1340] | If maximum TPDU size is proposed in a CR TPDU then the initiator shall support all TPDU sizes from 128 octets to the maximum proposed, as defined in ISO 8073 | 14.6 e) | I4CR9:M | I4CR9:M |
| TS2 [a8 a 1350] | If the preferred maximum TPDU size parameter is used in a CR TPDU then the initiator shall support all TPDU sizes, except 0, that are multiples of 128 octets up to the preferred maximum proposed | 14.6 e) | I4CR18:M | I4CR18:M |

| Index | TPDU size | ISO/IEC 8073 References | ISO Allowed values | CNS/ATM-1 Package Supported values |
| --- | --- | --- | --- | --- |
| TS3 [a8 a 1370] | What is the largest value of the preferred maximum TPDU size parameter in a CR TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |
| TS4 [a8 a 1380] | What is the largest value of the preferred maximum TPDU size parameter in a CC TPDU? | 14.6 e) | any multiple of 128 octets | any multiple of 128 octets |

TS3, TS4::  Note.— An implementation of the transport layer can support a preferred maximum TPDU size larger than 1024 octets [a8 n 0070].

TS3, TS4:: Recommendation.- 1024 octets is the recommended minimum maximum-TPDU size [a8 r 0050]

| Index | TPDU size | ISO/IEC 8073 References | Allowed Values | CNS/ATM-1 Package Supported Values |
| --- | --- | --- | --- | --- |
| T4S1 [a8 a 1390] | What is the largest value of the maximum TPDU size parameter in a CR TPDU with preferred class 4? | 14.6 e) | One of 128, 256, 512, 1024, 2048, 4096, 8192 | One of 128, 256, 512, 1024, 2048, 4096, 8192 |
| T4S2 [a8 a 1400] | What is the largest value of the maximum TPDU size parameter which may be sent in the CC TPDU when class 4 is selected? | 14.6 e) | 128, 256, 512, 1024, 2048, 4096, 8192 | 128, 256, 512, 1024, 2048, 4096, 8192 |

TS3, TS4, T4S1, T4S2:: Recommendation.- The supported TPDU size of 1024 octets is recommended to support efficient transmission of anticipated application data exchanges [a8 r 0060].

TS3, TS4, T4S1, T4S2:: Note.- A given transport implementation may support a smaller TPDU size [a8 n 0080].

###### Use of Extended Format.

| Index | Extended format | ISO/IEC 8073 References | ISO Allowed Values | CNS/ATM-1 Package Supported Value |
| --- | --- | --- | --- | --- |
| NEF3 [a8 a 1410] | What formats can you propose in the CR TPDU in class 4? | 6.5.5 n) | normal, extended | normal,extended |
| NEF6 [a8 a 1420] | What formats can you select in CC when extended has been proposed in CR in class 4? | 6.5.5 n) | normal, extended | normal,extended |

NEF3:: Recommendation.— Implementations of the ATN transport layer should propose use of normal format in the CR TPDU [a8 r 0070].

NEF3:: Note.- Because the increased TPDU size resulting from use of extended data TPDU numbering may be more inefficient, this option should be used on a TC only when absolutely required [a8 n 0090].

NEF3, NEF6:: Note.— This table does not preclude proposal of the extended format.

###### Expedited data Transport service

The ATN COTP shall implement the feature marked "M" in the table.

Note. — Use of the feature is optional [a8 n 0100].

| Index | Expedited data | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Supported values |
| --- | --- | --- | --- | --- |
| TED1 [a8 a 1430] | Is the expedited data indication supported in CR and CC TPDU? | 6.5.5 r) | M | MO |

TED1::  Note.— Expedited data is proposed using the Additional Options Parameters in the CR and CC TPDUs [a8 n 0110].

###### Non-use of Checksum (C4L and T4F29::).

| Index | Non-use of checksum | ISO/IEC 8073 References | ISO Allowed Values | CNS/ATM-1 Package Supported Values |
| --- | --- | --- | --- | --- |
| NUC1 [a8 a 1440] | What proposals can you make in the CR? | 6.5.5 p) | non-use, use | non-use, use |
| NUC2 [a8 a 1450] | What proposals can you make in CC when non-use of checksum has been proposed in CR? | 6.5.5 p) | non-use, use | non-use, use |

NUC1:: Note.— A transport layer is able to propose either use or non-use of checksum in a CR TPDU.

NUC2:: Note.— The term ”non-use” means that the transport layer may respond accepting non-use of checksum. A transport layer may also respond with use of checksum if non-use has been proposed.

NUC2:: Recommendation.— The transport layer should accept non-use of checksum when proposed in a CR TPDU [a8 r 0080].

###### Use of selective acknowledgement

| Index | Selective Acknowledgement | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| USA1 [a8 a 1460] | Is use of selective acknowledgement proposed in CR TPDUs ? | 6.5.5 s) | O | ATN6:M |
| USA2 [a8 a 1470] | Is use of selective acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 s) | O | ATN6:M |

###### Use of Request Acknowledgement

| Index | Request of Acknowledgement | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| ROA1 [a8 a 1480] | Is use of request of acknowledgement proposed in CR TPDUs ? | 6.5.5 t) | O | ATN7:M |
| ROA2 [a8 a 1490] | Is use of request of acknowledgement selected in a CC when it has been proposed in a CR ? | 6.5.5 t) | O | ATN7:M |

##### Error Handling

Note.— Using Class 4 over CLNS, a TPDU with an invalid checksum will be discarded.

###### Action on Detection of a Protocol Error

| Index | Item | ISO/IEC 8073 | Values |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | References | ISO Allowed | CNS/ATM-1 Package Supported |
| PE4L [a8 a 1500] | Class 4 over CLNS | 6.22.2 | ER, DR, Discard | ER, DR, Discard |

PE0-PE3:: Note.— N/A

PE4L::  Note.— The choice of action (DR, Discard) is an implementation choice and may depend on the type of error encountered [a8 n 0130].

###### Actions on receipt of an invalid or undefined parameter in a CR TPDU

The ATN COTP shall implement the features marked "M" in the table.

| Index | Event | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| RR1 [a8 a 1510] | A parameter not defined in ISO 8073 shall be ignored | 13.2.3 | M | M |
| RR2 [a8 a 1520] | An invalid value in the alternative protocol class parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR3 [a8 a 1530] | An invalid value in the class and option parameter shall be treated as a protocol error | 13.2.3 | M | M |
| RR4 [a8 a 1540] | On receipt of the additional option selection parameter bits 8 to 7, and bits 6 to 1 if not meaningful for the proposed class, shall be ignored | 13.3.4 g) | M | M |
| RR6 [a8 a 1550] | On receipt of the class option parameter bits 4 to 1 if not meaningful for the proposed class shall be ignored | 13.3.3 h) | M | M |

| Index | Event | ISO/IEC 8073 | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Reference | ISO Allowed | CNS/ATM-1 Package Supported |
| RR7 [a8 a 1560] | A parameter defined in ISO 8073 (other than those covered above) and having an invalid value | 13.2.3 | Ignore, Protocol Error | Ignore, Protocol Error |

RR7::  Note.— The choice of action (Ignore, Protocol error) is an implementation choice and may depend on the type of error encountered [a8 n 0140].

###### Actions on receipt of an invalid or undefined parameter in a TPDU other than a CR TPDU

The ATN COTP shall implement the features marked "M" in the table.

| Index | Event | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| U11 [a8 a 1570] | A parameter not defined in ISO 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U12 [a8 a 1580] | A parameter which has an invalid value as defined in ISO 8073 shall be treated as a protocol error | 13.2.3 | M | M |
| U13 (class 4 only) [a8 a 1590] | A TPDU received with a checksum which does not satisfy the defined formula shall be discarded | 6.17.3 | M | M |

##### Class 4 Timers and Protocol Parameters

The ATN COTP shall implement the features marked "M" in the table.

| Index |  | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| TA1 [a8 a 1600] | T1 (Local Retransmission) | 12.2.1.1.4 | M | M |
| TA2 [a8 a 1610] | N (Maximum Transmission) | 12.2.1 | M | M |
| TA3 [a8 a 1620] | IL (Inactivity Time) | 12.2.1.1.7 | M | M |
| TA4 [a8 a 1630] | W (Window Update) | 12.2.1 | M | M |
| TA5 [a8 a 1640] | L (Frozen Reference Time) | 12.2.1.1.6 | M | M |

| Index |  | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| ATN-TA1 [a8 a 1650] | R (Persistence) | 12.2.1.1.5 | O | O |
| ATN-TA2 [a8 a 1660] | MLR (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA3 [a8 a 1670] | MRL (NSDU Lifetime) | 12.2.1.1.1 | O | O |
| ATN-TA4 [a8 a 1680] | ELR (Maximum Transmission Delay) | 12.2.1.1.2 | O | O |
| ATN-TA5 [a8 a 1690] | ]ERL (Maximum Transmission Delay) | 12.2.1.1.2 | O | O |
| ATN-TA6 [a8 a 1700] | AL (Acknowledgement Time) | 12.2.1.1.3 | O | ATN15:M |
| ATN-TA7 [a8 a 1710] | AR (Acknowledgement Time) | 12.2.1.1.3 | O | ATN15:M |
| ATN-TA8 [a8 a 1720] | IR (Inactivity Time) | 12.2.1.1.7 | O | ATN17:M |

Note. — According to ISO, the following applies to an implementation under test:

| Index |  | ISO/IEC 8073 References | ISO Status | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- |
| OT9 [a8 a 1730] | Does IUT support optional timer TS2 when operating in class 4? | 6.22.2.3 | O | O |

#### Encoding of Transport Protocol Data Units

The encoding of TPDUs shall conform to ISO 8073 for the COTP [a8 t 0360].

### Use of the ATN Network Service

Note.— This section specifies how the COTP operates over the CLNS provided by the ATN network layer.

#### General

The COTP shall use the network service primitives described in Chapter 6 for the transmission and reception of TPDUs [a8 t 0370].

Note.— The way the parameters are exchanged between the transport entity and the Network Service is a local matter [a8 n 0150].

#### Use of the N-UNITDATA Request

5.2.5.2.a. The transport layer shall use the N-UNITDATA Request primitive, as defined in ISO 8073, to transmit TPDUs [a8 t 0380].

5.2.5.2.b. The length indication given to the network service shall be equal to the length of the TPDU(s). [a8 t 0390]

Note.— The maximum size of each TPDU is restricted to the locally defined maximum NSDU size [a8 n 0160].

##### NS-user-data

Note.— Transport entities transmit TPDUs as NS-user-data of the N-UNITDATA Request primitive [a8 n 0170].

##### Network Service Access Point Addresses

Note.- The transport layer has knowledge of the source and destination address parameters only as octet strings [a8 n 0180].

##### Network Quality of Service

The COTP shall use the network QOS parameters as defined in the sections below [a8 t 0400].

###### Network Layer Priority.

5.2.5.2.3.1.a. The COTP shall use the network priority parameter to indicate the relative priority of a NSDU [a8 t 0410].  When a transport priority has been specified, the value of network priority shall be determined based on the transport connection priority, as defined in Table 2-2 [a8 t 0420].

5.2.5.2.3.1.b. If the transport layer supports levels of TC priority greater than 14, TPDUs associated with the TC shall be transmitted using a network priority level of zero [a8 t 0430].

Note.— As specified in ISO 8073, the transport layer priority level zero is highest [a8 n 0185].  ISO 8473 specifies zero as the lowest network priority and fourteen as the highest [a8 n 0186]. Table 2-2 provides a suitable mapping between these two schemes for use by ATN systems.

###### Network Layer Security.

The COTP shall use the security label provided in the T-CONNECT request as the value of the N-UNITDATA security parameter [a8 t 0460].

#### Use of the N-UNITDATA Indication

The transport layer shall be capable of receiving TPDUs from the ATN network service using the N-UNITDATA indication primitive, as defined in ISO 8073 [a8 t 0470].

##### NS-user-data

Note.— Transport entities receive TPDUs as NS-user-data of the N-UNITDATA Indication primitive. [a8 n 0190].

##### Network Service Access Point Addresses

Note.— The source and destination address parameters supplied to the COTP by the CLNS via the N-UNITDATA primitive supports the ATN NSAP addressing plan as specified in Chapter 4. [a8 n 0195].

##### Network Quality of Service

Note.— There are no requirements for the receiving transport entity to perform any actions based on the other network QOS parameters contained in the N-UNITDATA Indication primitive [a8 n 0200].

### Transport Layer Congestion Avoidance

Note.—  ATNP/WG2 recognizes the need for congestion management in the ATN.  Specific algorithms for transport layer congestion management are not at present defined for the CNS/ATM-1 Package, therefore no requirement has been placed in these SARPs.  However, specific requirements may be added at the sixth WG2 meeting (October 1995) based on the validation results presented at that meeting.  This will not preclude the use of Explicit Flow Control for congestion avoidance which is a required element of COTP.

## Connectionless Mode Transport Protocol Operation

### Connectionless Mode Transport Protocol Overview

ATN ESs shall implement the ISO 8602 transport protocol in order to provide CL communications over the ATN internetwork [a8 t 0580].

Note.— The ATN CLTS model conforms to the service model defined in ISO 8072/AD1.

### ATN Connectionless Mode Transport Quality of Service Parameters

Recommendation.— The transport layer should support the dynamic selection of checksums on a per TSDU basis

#### Priority

The transport entity providing the connectionless mode transport service shall allow a TS-user to specify TSDU priority in the range [0-14].

Note.— The CLTP itself does not support a priority field in the TPDU.

#### Security

The transport entity providing the connectionless mode transport service shall use the TSDU security label provided in the T-UNITDATA request as the value of the N-UNITDATA security parameter.

Note.— The CLTP itself does not support a security parameter field in the TPDU.

### Connectionless Mode Transport Service Primitives

Note 1. — For the purposes of describing the notional interfaces between different OSI protocol layers, each protocol layer is assumed to provide a service to the next higher protocol layer.  The assumed service provided by the OSI transport layer to its user is described in ISO 8072.

Note 2.- ISO 8072 limits CL user-data to a maximum of 63 488 octets per TSDU [a8 n 0251].

Note 3. — There is no requirement to implement ISO 8072 as a software interface [a8 n 0255].

#### T-UNITDATA Request

The source and destination Transport Addresses shall conform to the ATN Transport Layer Addressing provisions as specified in Chapter 4 [a8 t 0630].

#### T-UNITDATA Indication

Note. — All of the associated parameter values are equal to the values passed to the TS provider via the T-UNITDATA Request primitive, except possibly the QOS parameter values.

### Connectionless Mode Transport APRL

Note. — The requirements for the CLTP are provided in the form of an APRL.  The APRL has been prepared using the PICS proforma provided with ISO 8602.

Before use in an ATN ES, the transport protocol implementor shall provide a completed PICS for the ISO transport protocol implementation, using the PICS proforma provided with ISO 8602, including any extensions to that proforma provided with this material. [a8 t 0631]  An implementation of the ISO 8602 Transport Protocol shall be used in an ATN ES if and only if its PICS is in compliance with the APRL provided with these SARPs. [a8 t 0632]

#### Protocol Implementation

The CLTP shall operate over the CLNS provided by the ATN network layer, according to the provisions in 5.3.5. [a8 t 0640]

#### Encoding of Transport Protocol Data Units

The encoding of TPDUs shall conform to ISO 8602 for the CLTP [a8 t 0650].

Use of the ATN Network Service

#### ATN Connectionless Transport Protocol APRL

The ATN CLTP shall implement the features marked "M" in the table.

| Item |  | Protocol Function Support |  | ISO 8602 Clause |  | ISO Status |  | CNS/ATM-1 Package Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NS [a8 a 1740] |  | Network service selection |  | 5.3.2.2 |  | M |  | M |
| AM [a8 a 1750] |  | Address mapping |  | 5.3.2.3 |  | M |  | M |
|  |  | PDU Support |  |  |  |  |  |  |
| UD1 [a8 a 1760] |  | Unitdata PDU supported on transmission |  | 6.1.3 |  | M |  | M |
| UD2 [a8 a 1770] |  | Unitdata PDU supported on reception |  | 6.1.3 |  | M |  | M |
|  |  | Parameters of the Unitdata PDU on Transmission |  |  |  |  |  |  |
| TpTc [a8 a 1780] |  | <t> TPDU UD Checksum |  | 6.2.4.1 |  | O |  | M |
| TpTs [a8 a 1790] | <t> TPDU UD Source Transport-Selector |  | 6.2.4.1 |  | M |  | M |  |
| TpTd [a8 a 1800] |  | <t> TPDU UD Destination Transport-Selector |  | 6.2.4.1 |  | M |  | M |
| TpTu [a8 a 1810] |  | <t> TPDU UD User Data |  | 6.2.4.1 |  | O |  | M |
|  |  | Parameters of the Unitdata PDU on Reception |  |  |  |  |  |  |
| TpRc [a8 a 1820] |  | <r> TPDU UD Checksum |  | 6.2.4.2 |  | M |  | M |
| TpRs [a8 a 1830] |  | <r> TPDU UD Source Transport-Selector |  | 6.2.4.2 |  | M |  | M |
| TpRd [a8 a 1840] |  | <r> TPDU UD Destination Transport-Selector |  | 6.2.4.2 |  | M |  | M |
| TpRu [a8 a 1850] |  | <r> TPDU UD User Data |  | 6.2.4.2 |  | M |  | M |
|  |  | Service Support |  |  |  |  |  |  |
| CL [a8 a 1860] |  | Connectionless Mode Network Service |  | 6.2 |  | M |  | M |

Note.— This section specifies how the CLTP operates over the CLNS provided by the ATN network layer.

#### General

The CLTP shall use the NS primitives described in Chapter 6 for the transmission and reception of TPDUs [a8 t 0660].

Note.— The method of parameter exchange between the TE and the NS is a local matter [a8 n 0260].

#### Use of the N-UNITDATA Request

The transport layer shall use the N-UNITDATA Request Primitive, as defined in ISO 8602, to transmit TPDUs [a8 t 0670].

##### NS-user-data

The length indicated shall be no different from the length of the TPDU(s). [a8 t 0680]

Note.— TEs transmit TPDUs as NS-user-data of the N-UNITDATA Request primitive [a8 n 0270].

##### Network Service Access Point Addresses

Note. — The transport layer has knowledge of source and destination address parameters only as octet strings [a8 n 0280].

##### Network Quality of Service

The transport entity providing the connectionless mode transport service shall use the network QOS parameters as defined in the sections below [a8 t 0690].

###### Network Layer Priority

The transport entity providing the connectionless mode transport service shall use the network priority parameter to indicate the relative priority of an NSDU. [a8 t 0700]  The NSDU priority shall be determined from the TSDU priority, using the mapping given in Table 2-2. [a8 t 0701

###### Network Layer Security

The transport entity providing the connectionless mode transport service shall use the security label provided in the T-UNITDATA request as the value of the N-UNITDATA security parameter [a8 t 0720].

#### Use of the N-UNITDATA Indication

Note.- Following ISO 8602, the transport layer receives TPDUs from the network-layer-provided N-UNITDATA Indication primitive [a8 n 0290].

##### Network Service Access Point Addresses

Note.— The source and destination address parameters supplied to the CLTP by the NS via the N-UNITDATA Indication primitive supports the ATN NSAP addressing plan as specified in Chapter 4. [a8 n 0300]

##### Network Quality of Service

To meet the ISO 8072 service specification, the transport entity providing the connectionless mode transport service shall translate application service priority to network layer priority, and shall perform the inverse operation on reception, using the mapping shown in Table 2-2. [a8 t 0750]

## Implementation

Recommendation. The following timers and variables should be configurable on a TC basis:

- the local retransmission timer (T1),

- the acknowledgement timer (AL),

- the window update timer (W),

- the inactivity timer (I),

- the frozen reference time (L),

- the maximum retransmission number (N),

- the persistence timer (R) [a8 r 0140]

.

5. Transport Service and Protocol Specification

5.1 General

5.1.1 References

5.1.2 Internet Service Description

5.1.3 Transport Service Access Point Addresses

5.1.4 Network Service Specification

5.1.5 Exchange of Transport-Selector parameters

5.2 Connection Mode Transport Layer Operation

5.2.1 Connection Mode Transport Protocol Overview

5.2.2 Connection Mode Transport Quality of Service

5.2.2.1 Connection Mode Transport Priority

5.2.2.2 Connection Mode Transport Security

5.2.3 Connection Mode Transport Service Primitives

5.2.4 Connection Mode Transport APRL

5.2.4.1 Mandatory and Optional Functions

5.2.4.1.1 Protocol Implementation

5.2.4.1.1.1 Classes Implemented

5.2.4.1.1.2 Specific ATN Requirements.

5.2.4.1.2 Initiator/Responder Capability for Protocol Classes 0-4

5.2.4.1.3 Supported Functions

5.2.4.1.3.1 Supported Functions for Class 4 (C4 OR C4L::).

5.2.4.1.3.1.1 Mandatory Functions for Class 4.

5.2.4.1.3.1.2 Mandatory Functions for Operation over Connectionless Network Service.

5.2.4.1.3.1.3 ISO 8073 Optional Functions.

5.2.4.1.4 Supported TPDUs

5.2.4.1.5 Supported Parameters of Issued TPDUs

5.2.4.1.5.1 Parameter Values for CR TPDU (C4L::)

5.2.4.1.5.2 Supported parameters for Class 4 TPDUs (C4L::)

5.2.4.1.5.2.1 Optional Parameters for a Connection Request TPDU.

5.2.4.1.5.2.2 Optional Parameters for a Connection Confirm TPDU.

5.2.4.1.5.2.3 Optional Parameter for a Disconnect Request TPDU.

5.2.4.1.5.2.4 Mandatory Parameter for a Data TPDU

5.2.4.1.5.2.5 Optional Parameter for an Acknowledgement TPDU

5.2.4.1.5.2.6 Use of the Subsequence Number Parameter in the Acknowledgement TPDU

5.2.4.1.5.2.7 Use of the Selective Acknowledgement Parameter in the Acknowledgement TPDU

5.2.4.1.5.2.8 Optional Parameters for an Error TPDU

5.2.4.1.6 Supported parameters for received TPDUs

5.2.4.1.6.1 TPDUs in Class 4 (C4L::)

5.2.4.1.7 User Data in Issued TPDUs

5.2.4.1.7.1 Class 4 (C4 or C4L::)

5.2.4.1.8 User Data in Received TPDUs

5.2.4.1.9 Negotiation

5.2.4.1.9.1 Class Negotiation - Initiator

5.2.4.1.9.2 Class negotiation - responder side

5.2.4.1.9.3 TPDU Size Negotiation

5.2.4.1.9.4 Use of Extended Format.

5.2.4.1.9.5 Expedited data Transport service

5.2.4.1.9.6 Non-use of Checksum (C4L and T4F29::).

5.2.4.1.9.7 Use of selective acknowledgement

5.2.4.1.9.8 Use of Request Acknowledgement

5.2.4.1.10 Error Handling

5.2.4.1.10.1 Action on Detection of a Protocol Error

5.2.4.1.10.2 Actions on receipt of an invalid or undefined parameter in a CR TPDU

5.2.4.1.10.3 Actions on receipt of an invalid or undefined parameter in a TPDU other than a CR TPDU

5.2.4.1.11 Class 4 Timers and Protocol Parameters

5.2.4.2 Encoding of Transport Protocol Data Units

5.2.5 Use of the ATN Network Service

5.2.5.1 General

5.2.5.2 Use of the N-UNITDATA Request

5.2.5.2.1 NS-user-data

5.2.5.2.2 Network Service Access Point Addresses

5.2.5.2.3 Network Quality of Service

5.2.5.2.3.1 Network Layer Priority.

5.2.5.2.3.2 Network Layer Security.

5.2.5.3 Use of the N-UNITDATA Indication

5.2.5.3.1 NS-user-data

5.2.5.3.2 Network Service Access Point Addresses

5.2.5.3.3 Network Quality of Service

5.2.6 Transport Layer Congestion Avoidance

5.3 Connectionless Mode Transport Protocol Operation

5.3.1 Connectionless Mode Transport Protocol Overview

5.3.2 ATN Connectionless Mode Transport Quality of Service Parameters

5.3.2.1 Priority

5.3.2.2 Security

5.3.3 Connectionless Mode Transport Service Primitives

5.3.3.1 T-UNITDATA Request

5.3.3.2 T-UNITDATA Indication

5.3.4 Connectionless Mode Transport APRL

5.3.4.1 Protocol Implementation

5.3.4.2 Encoding of Transport Protocol Data Units

5.3.4.3 ATN Connectionless Transport Protocol APRL

5.3.5 Use of the ATN Network Service

5.3.5.1 General

5.3.5.2 Use of the N-UNITDATA Request

5.3.5.2.1 NS-user-data

5.3.5.2.2 Network Service Access Point Addresses

5.3.5.2.3 Network Quality of Service

5.3.5.2.3.1 Network Layer Priority

5.3.5.2.3.2 Network Layer Security

5.3.5.3 Use of the N-UNITDATA Indication

5.3.5.3.1 Network Service Access Point Addresses

5.3.5.3.2 Network Quality of Service

5.4 Implementation