ATNP/WG2-WP/203
22-Jan-96

Validation Tool Descriptions

(Information Paper)

TABLE OF CONTENTS

SUMMARY

This document contains the validation tool descriptions submitted so far by WG2 members. The descriptions are made using the tool description template distributed to atn-internet-technical-list in November 95 (action 6/17).

Prepared by Jean-Pierre Briand

Presented by Henk Hof

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL

WORKING GROUP 2

Brisbane, Australia,  5-9 February 1996

1. Introduction

2. Comments on description template

Attachment 1: Prototype Implementations

Attachment 2: Simulation Models

# Introduction

This document contains the validation tool descriptions submitted so far by WG2 members. The descriptions are made using the tool description template distributed to atn-internet-technical-list in November 95 (action 6/17). This document will contribute to the ATN Validation Report. WG2 members are invited to submit additional descriptions of tools they intend to use within their organisation and provide comments on the current template.

# Comments on description template

To date, no specific comments were received on the description template itself. However, its use brought up a number of issues which can be summarised as follows:

a) tool description is meant to describe "classes" of tools, it is not suitable at present to describe real topologies/platforms by describing "how many systems", "actual capacities", etc. This has been solved in the European validation initiative by having a separate description of configurations in terms of actual capacities. This configuration description makes reference to tool descriptions.

b) There has been some difficulty in filling checkboxes, especially in CNS/ATM-1 Specifics section. It should be clear that this description cannot replace a compliance statement (i.e. PICS) an that the ultimate reference should be that compliance statement. The main use of this tool description is classification.

c) adoption of WORD Forms features seems to be a failure since all forms which were received were unprotected by their authors (probably to add their own comments and notes to the existing form). Unfortunately, when unprotected, a form cannot be protected again without erasing its content! Either some tutorial is provided on how to use forms, or the template is reissued without form features.

Attachment 1: Prototype Implementations

- ADS-Europe

- DEMISIS

- EURATN

- TAR D

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | ADS-Europe |  |
| Full Name |  |  |
| Category | PROTOTYPE IMPLEMENTATION |  |
| Description | Ground ADS application over ATN (interface at transport level) |  |
| Contact Point and/or Supplier | STNA, Martine Blaize, tel: (33) 62 14 58 88 e-mail: blaize_martine@ccmail.dgac.fr or CENA, Jean-Francois Grout tel: +33 62 25 95 36 e-mail: grout@cenatls.cena.dgac.fr |  |
| Tool Version and Date |  |  |
| Supporting Hardware | SUN Sparc 20 |  |
| Supporting Operating System and/or Software | SUNOS 4.3.1-U1 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS Not Applicable |  |
| Protocols | ISO 8073 ISO 8602 but not used ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation ATN Priority ATN Security Other: ADS application (A745-2) + « basic CMA » (log on message only) |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| V24/V28 | DB 25 (1) |  |
| - Notes |  |  |
| - Interoperable ADS/ATN avionics are being installed on commercial aircraft (A310 and B747) - Next version of ground and airborne software may support additional Package 1 functions |  |  |

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | DEMISIS |  |
| Full Name | DEMISIS (Development/Modification of ISO 10747, ISO 8473 and the SNDCF for ISO 8208 Subnetworks) |  |
| Category | PROTOTYPE IMPLEMENTATION |  |
| Description | DEMISIS is a configurable ATN Intermediate System/ End System. It can be configured to operate as various types of ATN routers (see below) and End Systems. This release conforms to the ATN Manual 2.0, but currently does not comply to the Draft CNS/ATM-1 SARPs. The upgrade is planned for 1996. DEMISIS was developed by ESG for EUROCONTROL. |  |
| Contact Point and/or Supplier | DFS Deutsche Flusicherung Dr. Andreas Herber Tel: +49 69 6335 263 Fax: +49 69 6335 219 Email: herber@se.dfs.com |  |
| Tool Version and Date | Release dfs-1.1a-TP4, 951123 |  |
| Supporting Hardware | HP 9000/700 series |  |
| Supporting Operating System and/or Software | HP-UX 9.0 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS |  |
| Protocols | ISO 8073 ISO 8602 ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: ISO 10589 |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation (currently, only ATN Manual compliant) ATN Priority ATN Security |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| ISO 8802-3 LAN | As per workstation configuration |  |
| X.25 | As per workstation configuration |  |
| - Notes |  |  |
| forseen to be upgraded to conform to Draft CNS/ATM-1 SARPs |  |  |

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | EURATN |  |
| Full Name | EURopean ATN |  |
| Category | PROTOTYPE IMPLEMENTATION |  |
| Description | EURATN is an experimental ATN network, including systems that can be configured as ES, IS, G/G BIS, A/G BIS and Airborne BIS, a real Data-3 satellite link and ground X.25 WANs and Ethernet LANs. |  |
| Contact Point and/or Supplier | STNA/8CA, Jean-Michel Crenais, tel: (33) 62 14 54 88 e-mail: crenais_jean-michel@ccmail.dgac.fr or CENA, Thomas Kircher tel: +33 62 25 95 57 e-mail: kircher@cenatls.cena.dgac.fr |  |
| Tool Version and Date | Release 1.2, 11 Sep 95 |  |
| Supporting Hardware | SUN Sparc IPX |  |
| Supporting Operating System and/or Software | SUNOS 4.3.1-U1 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS Not Applicable |  |
| Protocols | ISO 8073 ISO 8602 ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: ––––– |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation ATN Priority at transport and network level, not supported at X.25 level (would require a SOLARIS porting) ATN Security Other: ––––– |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| ISO 8802-3 LAN | As per workstation configuration |  |
| X.25 | As per workstation configuration |  |
| AMSS satellite link (Data-3) |  |  |
| - Notes |  |  |
| Next version of EURATN (Package-1 Upgrade) will add the following Package-1 Specifics: ATN Security ATN Routing Policies |  |  |

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | TAR D |  |
| Full Name | Trials ATN Router Release D |  |
| Category | PROTOTYPE IMPLEMENTATION |  |
| Description | TAR is a configurable ATN Intermediate System. It can be configured to operate as various types of ATN routers (see below). The release D supports the ATN security scheme and the ATN IDRP profiles as specified in CNS/ATM-1 SARPs. TAR was developed by Telegenics for EUROCONTROL. |  |
| Contact Point and/or Supplier | EUROCONTROL Henk HOF Tel: +32 2 729 3329 Fax: +32 2 729 3783 Email: hof.henk@eurocontrol.be |  |
| Tool Version and Date | Release D, 20 Nov 95 |  |
| Supporting Hardware | HP 9000/700 series |  |
| Supporting Operating System and/or Software | HP-UX 9.0 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS Not Applicable |  |
| Protocols | ISO 8073 ISO 8602 ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: ISO 10589 |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation ATN Priority ATN Security Other: |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| ISO 8802-3 LAN | As per workstation configuration |  |
| X.25 | As per workstation configuration |  |
| - Notes |  |  |

Attachment 2: Simulation Models

- CLNP Model

- IDRP Model

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | CLNP Model |  |
| Full Name | Connectionless Network Protocol Model |  |
| Category | SIMULATION MODEL |  |
| Description | The CLNP Model is a detailed simulation model of the Connectionless Network Protocol. The model implements static FIBs support. Protocol functions are controlled by configuration parameters. Version 2.0 will enable the simulation of CLNP congestion situations. |  |
| Contact Point and/or Supplier | EUROCONTROL Henk HOF Tel: +32 2 729 3329 Fax: +32 2 729 3783 Email: hof.henk@eurocontrol.be |  |
| Tool Version and Date | Version 1.0, October 1994 Version 2.0, to be delivered |  |
| Supporting Hardware | as supported by OPNET Release 2.5 |  |
| Supporting Operating System and/or Software | OS as supported by OPNET Release 2.5 Software: OPNET Release 2.5 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS Not Applicable |  |
| Protocols | ISO 8073 ISO 8602 ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation ATN Priority ATN Security Other: ATN Internetwork Service and Protocol Guidance |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| - Notes |  |  |

| - Tool Identification |  |  |
| --- | --- | --- |
| Name | IDRP Model |  |
| Full Name | IDRP Convergence Model |  |
| Category | SIMULATION MODEL |  |
| Description | The IDRP Convergence Model was developped to study the propagation within the ground network of routes to mobiles. The forwarding Process, and the interface to ISO 8473 are not modelled. ATN Transit Routing policies of backbone, off-backbone transit routing domains, and end routing domains can be simulated. Route merging and route aggregation in general are not supported. |  |
| Contact Point and/or Supplier | CENA, Thomas Kircher, tel (33) 62 25 95 57, e-mail: kircher@cenatls.cena.dgac.fr |  |
| Tool Version and Date | Version 3.0, December 1995 |  |
| Supporting Hardware | as supported by OPNET Release 2.5 |  |
| Supporting Operating System and/or Software | OS as supported by OPNET Release 2.5 Software: OPNET Release 2.5 |  |
| - CNS/ATM-1 SARPs Scope |  |  |
| ATN Systems | End System Intra-domain Intermediate System Ground-ground BIS Air-ground BIS Airborne BIS Not Applicable |  |
| Protocols | ISO 8073 ISO 8602 ISO 8473 ISO 9542 ISO 10747 ISO 8802 SNDCF ISO 8208 SNDCF ISO 8208 Mobile SNDCF Other: |  |
| CNS/ATM-1 Specifics | ATN Addressing ATN Routing Policy Air-Ground Route Initiation ATN Priority ATN Security Other: |  |
| - Connectivity Information |  |  |
| Type | Connector Type and Number | Notes |
| - Notes |  |  |
| Joint Ownership by CENA and EUROCONTROL |  |  |