| FRAVI | j June, 2000 |
| --- | --- |

French ATN Validation Initiative

-

Summary validation report

Stéphane Tamalet

TABLE OF CONTENTS

1.	Introduction	1

1.1	Purpose of the document	1

1.2	Structure of the document	1

1.3	References	1

2.	The French ATN Validation Initiative	3

2.1	Introduction	3

2.2	Initiative Reference & Title	3

2.3	Type	3

2.4	Responsible State/Organisation	3

2.5	Contact Point	3

2.6	Validation tools involved	3

2.7	Validation Period	4

2.8	Objectives of the French ATN Validation Initiative	4

2.8.1	General objectives of FRAVI	4

2.8.2	Detailed Objectives	4

2.8.2.1	General	4

2.8.2.2	AVOs covered by the French DGAC initiative	5

2.8.2.3	AVOs not covered by the French DGAC initiative	6

2.9	Validation strategy	6

3.	Implementation status and report	8

3.1	Implementation status	8

3.2	Findings	8

4.	Validation exercizes results report	10

4.1	Introduction	10

4.2	AVE_100 results	10

4.3	AVE_101 results	11

4.4	AVE_240 results	11

4.5	AVE_250 results	11

4.6	AVE_251 results	11

4.7	AVE_203 results	11

4.8	AVE_204 results	12

4.9	AVE_210 results	12

4.10	AVE_211 results	12

4.11	AVE_214 results	12

5.	Conclusion	14

# Introduction

## Purpose of the document

This report presents the results of the French ATN Validation Initiative (FRAVI) that have been obtained in the period from December 1999 to June 2000. It summarises the outcomes of the validation exercises, lists the ATN Validation Objectives that have been covered, and finally expresses the level of confidence of the French DGAC on the quality and correctness of the Sub-Volume V.

## Structure of the document

This document comprises the following sections:

- Chapter 1 is the introductory section

- Chapter 2 gives an overview of the French Initiative, describes the tools used for the validation, and list the objectives

- Chapter 3 provides the status of the implementation of the draft 3rd edition ICS enhancements, and reports on the potential defects raised by the development team.

- Chapter 4 summarizes the results of the validation initiative

- Chapter 5 is the conclusion of this report

## References

| REF1 | Proposed Draft third Edition of Doc 9705 Sub-Volume 5 (10 December 99) |
| --- | --- |
| REF2 | Proposed Doc 9705, Sub-Volume V (ICS) 3rd Edition Validation Report |
| REF3 | FRAVI - ATN Internet SARPs Validation Plan - URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/fravplan.zip |
| REF4 | FRAVI - AVE 100 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_100.zip |
| REF5 | FRAVI - AVE 101 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_101.zip |
| REF6 | FRAVI - AVE 240 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_240.zip |
| REF7 | FRAVI - AVE 250 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_250.zip |
| REF8 | FRAVI - AVE 251 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_251.zip |
| REF9 | FRAVI - AVE 203 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_203.zip |
| REF10 | FRAVI - AVE 204 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_204.zip |
| REF11 | FRAVI - AVE 210 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_210.zip |
| REF12 | FRAVI - AVE 211 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_211.zip |
| REF13 | FRAVI - AVE 214 Result Report URL: http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi/res_214.zip |

# The French ATN Validation Initiative

## Introduction

The French ATN Validation Initiative (FRAVI) is the validation programme undertaken by the French DGAC to contribute toward the validation of the third edition of Doc 9705 Sub-Volume V.

The principle of FRAVI is to implement and test a number of the Package 2 ICS enhancements on the operational ATN End System of the DNA, and on the Sofréavia/Airtel-ATN ProATN router. FRAVI will verify first the correct behaviour of a Package 2 implementation and secondly the backwards compatibility between Package 2 and Package 1 implementations of the ICS SARPs.

## Initiative Reference & Title

FRAVI: French DGAC ATN Validation Initiative

## Type

Experimentation/pre-operational implementation

## Responsible State/Organisation

French DGAC

## Contact Point

| State/Organisation | Contact Details |
| --- | --- |
| French DGAC | Mrs Christine Ricci STNA/8CA 1, avenue Dr Grynfogel BP 1084 31055 Toulouse Cedex France Tel: +33 (0)5 62 14 54 82 Fax: +33 (0)5 62 14 54 02 e-mail: ricci_christine@stna.dgac.fr |

## Validation tools involved

The experiments are conducted on the STNA ATN test laboratory. The STNA ATN test laboratory consists of multiple SUN SOLARIS workstations running ATN software, and which can be interconnected in multiple ways through X.25 and/or Ethernet subnetworks according to the test scenarios requirements. A maximum of 4 workstations is available for validation purpose. However, for test configurations that require more than 4 systems, it is possible to run multiple instances of the ATN system software on each of the available workstations, and to interconnect each running instance of an ATN system through either real or simulated LANs, X.25 WANs and mobile subnetworks. This permits to set-up and test large network topology.

The ATN software environment consists of the version 2.1 and the beta Version 3 of the ProATN air/ground BIS software, and the version 1.3.3 of the DGAC End System.

The version 2.1 of the ProATN air/ground BIS is compliant with the second edition of Doc 9705 Sub-Volume V. This version of the system is used for testing the backward interoperability of the beta 3.0 version with ATN systems compliant with the baseline Doc 9705 Sub-Volume V.

The beta version 3.0 of the ProATNair/ground BIS is compliant with the draft third edition of Doc 9705 Sub-Volume V, at the following exception: the procedures specified for generating and verifying digital signatures and used for the authentication of the IDRP routing information have not been implemented.

The DGAC End System is an ATN End System that is operationally used within the French CAUTRA system, and provides transport and internetworking services to CAUTRA applications. The system is compliant with the draft third edition of Doc 9705 Sub-Volume V at the exception of the support of the ATN Transport Congestion Avoidance mechanism.

## Validation Period

The validation of the draft third edition of Doc 9705 Sub-Volume V spans over a period of 7 month from December 1999 to June 2000.

## Objectives of the French ATN Validation Initiative

### General objectives of FRAVI

The French DGAC validation initiative mainly aims at demonstrating that:

- the requirements of the third edition of Doc 9705 Sub-Volume V are implementable,

- compliant ATN systems are interoperable,

- compliant ATN systems are interoperable with ATN systems that comply with the baseline edition of Doc 9705 Sub-Volume V,

- The draft third edition of Doc 9705 Sub-Volume V satisfies the user requirements.

The French DGAC validation initiative also covers some validation objectives related to ATN properties and performances.

### Detailed Objectives

#### General

The overall strategy for the validation of the draft third edition of Doc 9705 Sub-Volume V has been defined by ATNP/WG2, on the basis of its past experience on the validation of the baseline edition of Doc 9705 Sub-Volume V. The starting point of the validation process is the definition by ATNP/WG2 of a common unique set of ATN Validation Objectives (AVOs). AVOs are statements which express the various verifications and evaluations required in order to declare related part of the draft third edition of Doc 9705 Sub-Volume V as validated.

The coverage of AVOs by the French DGAC validation initiative is described in the following sections. AVOs that have been covered by the time of ATNP/3.

#### AVOs covered by the French DGAC initiative

| AVO Number | Description |
| --- | --- |
| AVO-3_101 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to ground End Systems have been implemented and demonstrated to be compliant. |
| AVO-3_102 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to airborne End Systems have been implemented and demonstrated to be compliant. |
| AVO-3_103 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to ground-ground Boundary Intermediate Systems have been implemented and demonstrated to be compliant. |
| AVO-3_104 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to air-ground Boundary Intermediate Systems have been implemented and demonstrated to be SARPs compliant. |
| AVO-3_105 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to airborne Boundary Intermediate Systems supporting IDRP have been implemented and demonstrated to be compliant. |
| AVO-3_106 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to airborne Boundary Intermediate Systems not supporting IDRP have been implemented and demonstrated to be compliant. |
| AVO-3_201 | - Verify that two compliant ATN End Systems interoperate and provide Connection-Oriented Transport Service to Transport Service users. |
| AVO-3_202 | - Verify that two ATN End Systems supporting different protocol profiles (i.e., support of ATN recommendations) interoperate and provide the Transport Service |
| AVO-3_460 | Show the enhanced COTP provisions for dynamic re-transmission timers result in an improved ability to quickly recover to changes in transit delay by the underlying internet service or a non-uniform delay in the delivery of TPDUs. |
| AVO-3_461 | Show that the revised COTP ACK timer setting results in improved recovery from the loss of TPDUs. |
| AVO-3_240 | - Verify compliant airborne and air-ground BISs will interoperate with a peer BIS has been implemented according to the baseline Doc 9705 Sub-Volume V and implementing the ACA compression mechanism. |
| AVO-3_109 | Verify that all requirements associated with Doc 9705 Sub-Volume V Third edition enhancements pertaining to routing architecture and routing policy have been implemented and demonstrated to be SARPs compliant. This includes ATN system aspects and associated procedures. |
| AVO-3_203 | Verify that compliant airborne and air-ground BISs implementing the enhancements to the mobile SNDCF(s) to deal with changing subnetwork connectivity will interoperate |
| AVO-3_204 | Verify that a compliant airborne BIS or air-ground BIS implementing the enhancements to the mobile SNDCF(s) to deal with changing subnetwork connectivity will interoperate with a peer BIS implementing the mobile SNDCF according to the existing baseline Sub-Volume V provision. |
| AVO-3_210 | Verify that ground BISs and air-ground BISs will interoperate for the secure exchange of IDRP information. The provisions to authenticate IDRP exchanges with the peer BIS across a ground-ground path will be verified. |
| AVO-3_211 | Verify that compliant airborne and air-ground BISs supporting authentication of IDRP exchanges in the air-to-ground direction will interoperate. |
| AVO-3_213 | Verify that compliant airborne BISs and air-ground BISs supporting the option to request and to attach a security certificate to an IDRP OPEN-PDU will interoperate. |
| AVO-3_214 | Verify that a compliant airborne BIS, air-ground BIS and ground BIS will interoperate for the unsecured exchange of routing information with a peer BIS implemented in accordance with the current baseline Sub-Volume V provisions. |
| AVO-3_220 | Verify compliant airborne and air-ground BISs suppress the re-advertisement of routes in the case of changing mobile subnetwork connectivity. |
| AVO-3_221 | - Verify that a compliant airborne or air-ground BIS supporting the mechanisms to suppress the re-advertisement of routes in the case of changing mobile subnetwork connectivity will interoperate with a peer BIS that has been implemented according to the baseline Doc 9705 Sub-Volume V. |
| AVO-3_230 | - Verify compliant airborne and air-ground BISs using a parameter in ISH PDU to signal protocol capabilities which are beyond first edition of Doc. 9705 will interoperate. |
| AVO-3_231 | - Verify that a compliant airborne or air-ground BIS supporting the use of a parameter in ISH PDU to signal protocol capabilities which are beyond first edition of Doc. 9705 will interoperate with a peer BIS that has been implemented according to the baseline Doc 9705 Sub-Volume V. |
| AVO-3_250 | - Verify compliant airborne and air-ground BISs supporting negotiation of pre-stored Deflate dictionaries will interoperate. |
| AVO-3_251 | - Verify a compliant airborne or air-ground BISs supporting negotiation of pre-stored Deflate dictionaries will interoperate with a peer BIS that has been implemented according to the baseline Doc 9705 Sub-Volume V. |
| AVO-3_252 | - Verify compliant airborne and air-ground BISs supporting a mobile SNDCF option to allow maintenance of Deflate history window when changing ground stations will interoperate. |
| AVO-3_253 | - Verify a compliant airborne or air-ground BISs supporting a mobile SNDCF option to allow maintenance of Deflate history window when changing ground stations will interoperate with a peer BIS that has been implemented according to the baseline Doc 9705 Sub-Volume V. |
| AVO-3_300 | - Verify the ability of the ATN service to ensure a rapid fall back to another mobile subnetwork in case of service interruption on the default mobile subnetwork, consistent with the declared ATSC traffic class supported by the default mobile subnetwork |
| AVO-3_301 | - Verify that a perturbated default sub-network (experiencing fading on the r.f. channel) has no impact on the ATN service except for increase in average end-to-end transit delay and/or fall back to another mobile subnetwork. |
| AVO-3_410 | - Show that compliant airborne and air-ground BISs implementing the enhancements to the mobile SNDCF(s) to deal with changing subnetwork connectivity minimise routing updates when the mobile subnetwork is experiencing rapid changes in connectivity. |
| AVO-3_420 | - Show compliant airborne and air-ground BISs suppress the re-advertisement of routes in the case of changing mobile subnetwork connectivity. |
| AVO-3_451 | - Evaluate the performance advantage provided by the use of a pre-stored Deflate dictionary compared to the dynamic construction of the dictionary, as per the standard Deflate mechanism specified by the baseline Doc 9705 Sub-Volume V |
| AVO-3_452 | - Evaluate the performance advantage provided by allowing the maintenance of Deflate history window when changing ground stations as compared to a Deflate algorithm that has been implemented according to the baseline Doc 9705 Sub-Volume V. |

#### AVOs not covered by the French DGAC initiative

| AVO Number | Description |
| --- | --- |
| AVO-3_107 | Verify that the VDL Mode 3 subnetwork and the mobile SNDCF interaction has been implemented for support of ATN communications and demonstrated to be SARPs compliant. |
| AVO-3_108 | Verify consistency of the Mobile SNDCF provisions versus the behaviour of VDL Mode 2, VDL Mode 3, AMSS, HFDL and Mode S subnetworks in the issuance of Join and Leave Events under conditions of rapid fading conditions on the r.f. link. |
| - AVO-3_212 | Verify that compliant airborne BISs and air-ground BISs supporting the optional mutual authentication of IDRP exchanges are each able to authenticate IDRP PDUs received from the peer BIS. |
| AVO-3_400 | Evaluate the ground BISs and air-ground BISs capability to authenticate IDRP exchanges with the peer BIS across a ground-ground path. |
| AVO-3_401 | Evaluate the compliant airborne and air-ground BISs capability for authentication of IDRP exchanges in the air-to-ground direction. |
| AVO-3_402 | Evaluate the compliant airborne BISs and air-ground BISs capability for the mutual authentication (option) for IDRP exchanges. |
| AVO-3_403 | Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between ground and air-ground BISs. |
| AVO-3_404 | Evaluate the security information exchange and processing overhead for the secure exchange of IDRP routing information, between airborne and air-ground BISs. |

## Validation strategy

The principle of the initiative is first to develop an ATN system that complies with third edition of Doc 9705 Sub-Volume V, in order to demonstrate feasibility. The system is developed by a team of persons who have not participated to the production of the SARPs. This provides further level of confidence that the SARPs are unambiguous. During implementation, the STNA experts participating to the ATNP/WG2 consider any requests for clarification, or questions raised by the development team. Those issues requiring correction to the Sub-Volume V, and/or provision of additional guidance are reported to the WG2 and/or to the CCB under the form of PDRs or of Working Papers.

Once a draft 3rd edition ICS enhancement is implemented, the focus is directed on the testing of the new functionality and the coverage of the associated AVOs. The AVOs coverage is achieved through the performance of a number of ATN Validation Exercizes (AVEs) . The problems detected with the validation exercizes are reported to the WG2 and/or to the CCB under the form of "Potential 3rd Edition Defect Reports" (P3DRs) or of Working Papers. The result of each AVE is documented in a separate AVE result report made available on the ATNP archive.

The FRAVI summary report is produced on the basis of the outcomes of the implementation phase and of the AVE result reports.

# Implementation status and report

## Implementation status

The following table summarizes the current status of the implementation of the ICS-3 (draft third edition of Doc 9705 Sub-Volume V) enhancements on the beta version 3.0 of the ProATN air/ground BIS.

| Label | Enhancement for 3rd Edition of ICS SARPs | Implementation status |
| --- | --- | --- |
| ICS3-01 | Requirements for mobile subnetworks to issue Join and Leave Events within given latency intervals | Completed |
| ICS3-02 | Requirement for IS-SME to respect quarantine time before processing next Join event from mobile subnetworks having issued a Leave event | Completed |
| ICS3-03 | Use of adaptive retransmission timers in the Connection Oriented Transport Protocol | Completed |
| ICS3-06 | Requirements for enhanced IDRP security consistent with the ATN security architecture (i.e. draft Sub-Volume 8). | Partial Implementation (see below) |
| ICS3-07 | Deletion of the ATN NSAP address compression algorithm (ACA) | Completed |
| ICS3-08 | Suppression of re-advertisement of routes in the case of changing mobile subnetwork connectivity | Completed |
| ICS3-09 | Conveyance of data link capability parameter in air/ground ISH PDU exchanges to signal protocol capabilities which are beyond 1st edition of Doc 9705 between air/ground and airborne routers in a backwards compatible manner | Completed |
| ICS3-11 | Revised COTP acknowledgement (ACK) timer setting | Completed |
| ICS3-13 | Potential enhancements to the DEFLATE algorithm to allow for the dynamic negotiation for the use of pre-stored dictionaries | Completed |
| ICS3-14 | Potential additional mobile SNDCF option to allow maintenance of DEFLATE history window when changing ground stations or air/ground routers respectively | Completed |

Status of the implementation of the version 3.0 of the ProATN Air-Ground BIS

At the exception of ICS3-06, all draft third edition ICS enhancements have been implemented.

The enhancement ICS3-6 has been partially implemented. The software implements all procedures and options related to the negotiation of the use of mutual or single authentications on air-ground and ground-ground IDRP connections. However the implementation of the ASVDP, AKDF , AMACP, and AMAVP procedures that are specified in Sub-Volume 8 is not planned at the moment.

## Findings

No major deficiency has been identified on the ICS enhancements. The implementation of ICS3 enhancement allowed raising 13 potential defects on the third edition of of Doc 9705 Sub-Volume V. However, these defects have all been classified as "minor/clarification". Most of them are requests for clarification aiming at removing potential ambiguities from the text.

All these defects have been reported to the SubVolume V Subject Matter Expert and have been resolved by the Working Group2 by amending the draft third edition, or thanks to the production of additional Guidance Material. These defects are listed by title in the table below. The associated P3DR forms are available on the ATNP archive.

| P2DR number | Title |
| --- | --- |
| M0020010 | Processing of received Deflate Maintenance Parameter |
| M0020011 | Issues on the concept of Subnetwork Connection Group |
| M0020012 | Bit 0 of the ISH Data link Capability |
| M0020013 | TP4 retransmission timer on the first RTT sample |
| M0020014 | Valid/invalid round trip time sample in TP4 |
| M0020015 | Error condition for deflate compressor window |
| M0020016 | Use of received security info by Airborne BIS |
| M0020017 | Interoperability problem due to the suppression of ACA |
| M0020018 | Interoperability with a peer BIS that does not support authentication type 2 |
| M0020019 | BIS behaviour in case of certificate path validation failure |
| M0020020 | A/G BIS access to delivery service |
| M0020021 | Encoding of Random Variable Parameter value |
| M0020022 | Length of Certificate Path Parameter |

An issue was raised regarding the enhancement ICS3-08 which aims at resolving the problem of the regular re-advertisement of IDRP routes that may result from changing mobile connectivity when an aircraft has more than one adjacency with the same A/G router via different mobile subnetworks. It was observed that the problem of the re-advertisement of routes was not totally solved with the changes made in the draft third edition of the SARPs. Further changes, which would allow to fix definitively the problem, were proposed in a Working Paper presented at the ATNP/WG2 Internet Drafting Group. At the time this report is produced, these changes are still under consideration.

The implementation of the draft 3rd edition enhancements to the mobile SNDCF (ICS3_13 and ICS3_14) led the ProATN A/G BIS team to deeply investigate the mechanisms of the deflate compression. This work allowed discovering that the ProATN A/G BIS did not comply with all the baseline SARPs requirements on the Deflate, and would not be able to interoperate with baseline SARPs compliant routeurs if deflate compression is used. It was felt that the ProATN A/G BIS could not be the only router concerned by this non-conformance issue, and a document describing the problem that existed in the ProATN A/G BIS implementation was distributed to the ATN systems developers and to the WG2. The discussion of this problem is in progress.

A number of Editorial corrections were proposed, and logged in an addendum/Corrigendum to the draft third edition of Doc 9705 until their incorporation in the final draft.

Finally, the validation exercizes allowed verifying that the changes introduced in the Draft third edition of the SubVolume V enhance the performance of the ATN Internet Communications Service, taking note of the fact that the procedures for the generation and verification of digital signatures that have been standardized for the authentication of the IDRP routing information have not been implemented nor tested in this initiative.

# Validation exercizes results report

## Introduction

FRAVI consists of the 10 ATN Validation Exercizes (AVE) listed by title in the following table. These Validation Exercizes are specified in [REF3]. Each exercize comprised multiple Validation Tests. For each Validation Exercize, a separate 'AVE Result report' has been produced. These Result Reports are made available to the ATN community via the ATNP archive at the following URL

http://www.tls.cena.fr/atnp/wg2/val-Ed3/fravi

| AVE name | AVE title | AVE Result report |
| --- | --- | --- |
| AVE_100 | Non-regression tests | REF4 |
| AVE_101 | Validation of the enhancements to the ATN transport protocol | REF5 |
| AVE_240 | Validation of the deletion of the ACA mechanism | REF6 |
| AVE_250 | Validation of the enhancements to the mobile SNDCF | REF7 |
| AVE_251 | Validation of the enhancements to the mobile SNDCF (backward interoperability) | REF8 |
| AVE_203 | Validation of the enhancements to deal with changing subnetwork connectivity | REF9 |
| AVE_204 | Validation of the enhancements to deal with changing subnetwork connectivity (backward interoperability) | REF10 |
| AVE_210 | Validation of the secure and unsecure exchange of IDRP information over a ground-ground path | REF11 |
| AVE_211 | Validation of the secure and unsecure exchange of IDRP information over an air-ground path | REF12 |
| AVE_214 | Validation of the IDRP security enhancements (backward interoperability) | REF13 |

The following sections provide a summary of the results of these exercizes.

## AVE_100 results

All ProATN Air/Ground BIS non-regression tests were successfully replayed in a simulated environment with a beta version 3.0 of the ProATN Software

The ProATN A/G BIS non-regression tests allow the automatic testing of most of the different functions supported by the ProATN A/G BIS software, including its behaviour when faced to numerous different communication failure conditions, and its operation in many different network configurations.

The success of these tests gives insurance that:

- the implementation of the draft third edition ICS enhancements within an ATN system, does not compromise the correct execution of the basic functions of that ATN system;

- baseline edition-compliant ATN systems can be transparently replaced in an ATN network by third-edition-compliant ATN systems without negative effects.

## AVE_101 results

This validation exercize allowed verifying that the draft third edition enhancements to the transport protocol perform well, allow interoperability with baseline edition compliant systems and contribute to provide extremely better end to end performances to the Transport Service end users.

Two minor defects were found in the draft third specification of these enhancements. These defects were reported as P3DRs to the WG2 and resolved.

## AVE_240 results

This validation exercize allowed demonstrating that a third edition-compliant airborne (resp. air/ground) BIS is capable to interoperate with a baseline edition-compliant air-ground (resp.airborne) BIS implementing the ACA compression mechanism.

One potential backward interoperability problem was identified in the case where the X.25 fast select facility is not available over the mobile subnetwork. This problem was reported as a P3DR to the WG2 experts and has been resolved

## AVE_250 results

This validation exercize allowed verifying that the draft third edition enhancements to the mobile SNDCF can be implemented, do not introduce interoperability problems between Edition 3 compliant systems, and enhance the level of compression achieved for mobile communications.

Areas where the draft 3rd edition of the SubVolume V was not specific enough were detected during implementation of these draft third Edition enhancements. Requests and suggestion for clarifications were issued under the form of P3DRs, and handled by the ATNP/WG2 Internet Drafting Group.

During the test, the ProATN A/G BIS was discovered to be non-conformant to the baseline SARPs with regard to the Deflate compression. It was felt that the ProATN A/G BIS could not be the only router concerned by this non-conformance issue, and a document describing the problem that existed in the ProATN A/G BIS implementation was distributed to the ATN systems developers and to the WG2. The discussion of this problem is in progress.

## AVE_251 results

The success of this validation exercize demonstrates that the draft third edition enhancements to the mobile SNDCF do not introduce interoperability problems between Edition 3 compliant systems and baseline edition compliant systems. The issues encountered in the context of this validation exercize were the same as the ones already documented in the context of AVE_250.

## AVE_203 results

This validation exercize focused on the validation of the draft third edition enhancements to deal with changing mobile subnetwork connectivity. Thanks to this exercize, it was demonstrated that these enhancements can be implemented, do not introduce interoperability problems between Edition 3 compliant systems and contribute to reduce the amount of routing information exchanged over the mobile subnetworks. However, as already mentioned in section 3.2, it was observed that a further level of optimization regarding the problem of the re-advertisement of IDRP routes over mobile subnetworks is possible and additional changes have been proposed.

One minor issue has been raised during implementation. It has been documented under the form of a P3DR on Edition 3 and has been resolved by the WG2 Internet Drafting Group.

## AVE_204 results

This validation exercize allowed verifying that a third edition-compliant airborne (resp. air/ground) BIS, implementing the enhancements ICS-01 (for mobile subnetwork to issue Join and Leave Events within given latency intervals), ICS3-02 (for IS-SME to respect quarantine time of the Join events) and ICS3-08 (suppression of re-advertisement of routes in the case of changing mobile subnetwork connectivity) can interoperate without trouble with an air/ground BIS (resp. airborne BIS) that is compliant with the baseline edition of Doc 9705 Sub-Volume V.

The issues encountered in the context of this validation exercize were the same as the ones already documented in the context of AVE_203.

## AVE_210 results

This success of this validation exercize gives insurance that third Edition 3 compliant BISs will interoperate without trouble on the ground for the secure exchange of IDRP information with other 3rd edition compliant BIS, as well as for the unsecure exchange of IDRP information with other BISs that do not support or use the IDRP Authentication Type 2.

It must however be noted that the authentication procedures (ASVDP, AKDF, AMACP and AMAVP) have not been implemented and tested in the context of this validation exercize. As a consequence, the enhancements for the secure exchange of IDRP information between ground BISs cannot be considered as totally validated by this validation exercise.

## AVE_211 results

This validation exercize focused on the validation of the draft third edition procedures and options for the negotiation of the use of authentication type 2 on air-ground IDRP connections (i.e. over mobile connection). Thanks to this exercize, it has been demonstrated that these procedures and options can be implemented, work correctly, and that Edition 3 compliant systems interoperate when these procedures and options are used.

Here again however, it must be noted that the authentication procedures (ASVDP, AKDF, AMACP and AMAVP) have not been implemented and tested in the context of this validation exercize. As a consequence, the enhancements for the secure exchange of IDRP information over mobile subnetworks cannot be considered as totally validated by this validation exercise.

Areas where the draft 3rd edition of the SubVolume V was not specific enough were detected during implementation. Requests and suggestion for clarifications were issued under the form of P3DRs, and handled by the ATNP/WG2 Internet Drafting Group.

## AVE_214 results

This validation exercize complemented AVE_211. It allowed verifying that the draft third edition procedures and options for the negotiation of the use of authentication type 2 on air-ground IDRP connections do not introduce interoperability problems between Edition 3 compliant systems and baseline edition compliant systems.

The issues encountered in the context of this validation exercize were the same as the ones already documented in the context of AVE_211.

# Conclusion

As a result of the successful incorporation of the draft third edition enhancements to the ICS SARPs into the ProATN A/G BIS, and considering the success of the validation exercizes, the STNA is in position to express its confidence regarding the quality, and the validity of the SubVolume V of the third edition of Doc 9705.

With the exception of the procedures of generation and verification of digital signatures used for the authentication of the IDRP routing information, all draft 3rd edition enhancements have been implemented, and tested. It has been verified that these enhancements do not compromise the correct execution of the baseline functions of the ATN systems, and that interoperability between ATN systems is maintained. No major deficiency has been identified.

The implementation of the enhancements and the first validation exercizes allowed the detection of some areas in the specification where clarifications were required. This resulted in the production of a number of defect reports the resolution of which is in progress. However, in general, the third edition of Doc 9705 SubVolume V was found consistent and unambiguous.

Finally, the validation exercizes allowed verifying that the changes introduced in the Draft third edition of the SubVolume V enhance the performance of the ATN Internet Communications Service, taking note of the fact that the procedures for the generation and verification of digital signatures that have been standardized for the authentication of the IDRP routing information have not been implemented nor tested in this initiative.