ATNP/WG3/WP 19-40

4 August 2000

AERONAUTICAL TELECOMMUNICATION NETWORK PANEL

WORKING GROUP 3 (APPLICATIONS AND UPPER LAYERS)

Berlin, Germany  21 August – 25 August 2000

Agenda Item 5 : PICS and OICS

PICS/OICS Guidance Material

Prepared by: Mike Harcourt

SUMMARY

This paper provides the guidance material for the PICS/OICS Proforma tables.

TABLE OF CONTENTS

1.	Introduction	1

1.1	Purpose and Scope	1

1.2	Document Structure	1

1.3	References	1

2.	PICS/OICS Overview	2

2.1	History and Development of PICS/OICS	2

2.2	PICS/OICS Relationship to Standards and Requirements	2

2.3	PICS/OICS Relationship to ASN.1	4

2.3.1	ASN.1 Introduction	4

3.	PICS/OICS Proformas	6

3.1	PICS/OICS Relationship to ICAO ATN SARPs	6

3.2	General Format	6

3.2.1	File Structure	6

3.2.2	Information Section	7

3.2.2.1	PICS/OICS Identification (Table I-1)	7

3.2.2.2	Supplier and Implementation Identification (Table I-2)	7

3.2.2.3	Application Protocol Identification (Table I-3)	8

3.2.3	Configurations and Services Section	8

3.2.3.1	Application Protocol Options (Table S-1)	8

3.2.3.2	Application Conformant Configurations (Table S-2)	8

3.2.3.3	Supported Service Primitives (Table S-3)	9

3.2.3.4	Service Tables	9

3.2.4	Messages Section	10

3.2.5	Parameters Section	10

3.2.6	Column Headings	12

3.2.7	Notes and Conditional Status	13

3.2.8	Formatting and Colours	13

3.3	Classification Scheme	15

3.3.1	Overview	15

3.3.2	ASN.1 Encoding and Decoding	15

3.3.3	Operational Classification	15

3.3.3.1	ADSP Profile/Operational Profile Status	15

4.	Guidance for Defining Operational Profiles	17

5.	Guidance for Completing a Proforma	17

6.	Application Specific Guidance	18

6.1	Introduction	18

6.2	Guidance for ADS	18

6.2.1	Introduction	18

6.2.2	Proforma Tables	18

6.3	Guidance for ADS Report Forwarding	21

6.3.1	Introduction	21

6.3.2	Proforma Tables	21

6.4	Guidance for CM	22

6.4.1	Introduction	22

6.4.2	Proforma Tables	22

6.5	Guidance for CPDLC	25

6.5.1	Introduction	25

6.5.2	Proforma Tables	25

6.5.2.1	Table M-4: Uplink Message Elements	26

6.5.2.2	Table M-5: Uplink Messages Permitted Operational Responses	26

6.5.2.3	Table M-6: Downlink Message Elements	26

6.5.2.4	Table M-7: Downlink Messages Permitted Operational Responses	26

6.5.2.5	Table M-8: Compound Message Parameter Elements	26

6.5.2.6	Parameter Tables	27

6.5.3	Profiles	27

6.5.4	Guidance for Implementers	27

6.6	Guidance for FIS	29

6.6.1	Introduction	29

6.6.2	Proforma Tables	29

# Introduction

## Purpose and Scope

This document provides the guidance material for all aspects of PICS/OICS.  It provides a detailed description of the format and layout of PICS/OICS proformas, guidance for producing new PICS/OICS proformas and operational profiles, and guidance for completing a proforma.

Its intended readership is anyone who needs an understanding of the PICS/OICS proformas; this includes the writers of new PICS/OICS proformas, the writers of operational profiles and systems implementers who need to complete the proformas.

A number of PICS/OICS proformas for ATN applications are being developed by ATNP WG3/SG2:

- ADS Airborne, Ground and Report Forwarding ASEs PICS/OICS Proformas

- CM Airborne and Ground ASEs PICS/OICS Proformas

- CPDLC Airborne and Ground ASEs PICS/OICS Proformas

- FIS Airborne and Ground ASEs PICS/OICS Proformas

## Document Structure

The reminder of this document is divided into sections which cover the following topics:

Section 2:	An introduction to PICS/OICS explaining how they have evolved and their relationship to base standards.

Section 3:	Describes the proforma format and classification scheme.

Section 4:	Gives guidance to the writers of Operational Profiles.

Section 5:	Gives guidance on completing a proforma.

Section 6:	Contains the application specific guidance material.

## References

- ICAO Manual of Air Traffic Services (ATS) Data Link Applications, Doc 9694 (First Edition)

- ICAO Manual of Technical Provisions for the Aeronautical Telecommunication Network (ATN). Doc 9705-AN/956 (Second Edition – 1999)

- ISO 9646-7 OSI Conformance testing methodology and framework – Implementation Conformance Statements.

# PICS/OICS Overview

## History and Development of PICS/OICS

The need to have a mechanism for conformance testing and capturing the conformance of implementations of international data communications standards was first recognised and developed by the International Standards Organisation (ISO) during the 1970s and 1980s.  This resulted in a series of ISO standards, ISO 9646 OSI Conformance Testing Methodology and Framework.  One of this series “Implementation Conformance Statements” [3] was used as the basis for the development of the ATN PICS/OICS.

The ATN context requires two levels of conformance to be addressed:

- Conformance at the protocol and encoding level.  This corresponds directly to the ISO protocol conformance statements and uses the same name, Protocol Implementation Conformance Statement or PICS.

- The second is conformance at an operational level, hence Operational Implementation Conformance Statement or OICS.

A conformance statement sets out in tabular form a precise analysis of the various services and protocol elements as defined in base standards, in the ATN context the SARPs are the base standards, reference [1].  A completed conformance statement provides a structured basis determining an implementations conformance to a standard or profile, and can provide the basis for comparison of implementations in order to determine whether there are any interoperability issues.

The proformas have been developed to meet not only the need for conformance to the SARPs and the global operational profile in the “ADSP Profile” but also to allow differing requirements for operation in different areas to be classified in an “Operational Profile”.

The ADSP profile identifies and classifies:

- the services which must be implemented for a minimum level of interoperability;

- the selection of options within those services for added functionality;

- the protocol elements which must be implemented for a minimum level of interoperability;

- the selection of optional protocol elements which must be implemented for optional services;

- the restrictions and constraints that apply to the use of services and protocol elements

An Operational Profile not only takes into account the ATN profile but also adds specific interpretations and constraints which are applicable to an implementation for operation within a specified area.

## PICS/OICS Relationship to Standards and Requirements

The PICS/OICS proformas are derived through a series of requirements, specifications and interactions which are modelled in Figure 2-1; specifications are shown in boxes and the interactions are shown by arrows.  The dotted lines around the profile specification boxes indicates that the specification may only exist as part of the proforma.  The figure shows the derivation of both the ADSP Profile and an Operational Profile and indicates the body responsible for the specification or proforma.

Figure 2-1: Relationships between Requirements, Specifications, Proformas and OICS/PICS

## PICS/OICS Relationship to ASN.1

### ASN.1 Introduction

ASN.1 or Abstract Syntax Notation One is a language for describing structured information; typically data to be transferred across a communications system.  It is widly used in the specification of communications protocols hence its use in the ATN application SARPs.

The structure of a typical ATN application's ASN.1 is illustrated in Figure 2-2.  Starting at the top level which corresponds to the message exchanged between the instances of an application, the ASN.1 types are decomposed into "Constructors" which decompose further and "Primitives" or "Simple Types" which do not decompose.  Constructors are either a list of possible choices , only one of which may be used in any given instance of communication, or a sequence of elements, which must be present in an instance of communication unless an element is classified as optional.  Constraints may also be applied:

- Constructor Types – in the form of the number of instances of a type that can be repeated, for example there is a limit in ADS of the number of extended projected profile points that may be sent by an aircraft in a report;

- Primitive Types – in the form of field lengths or value ranges, for example an aircraft flight identification in CPDLC must be in the range 2 to 8 characters.

An element is classified at the highest level where it appears.  In Figure 2-2, a mandatory element is shown as M and an optional element is shown as O.  Note that the classification is applied at the higher node at the start of the arrow in the figure and not at the lower node.  This is because nodes may be reached from several different branches which have different classifications, see Constructor-3 as an example.

Figure 2-2: ASN.1 Hierarchy Model

The type 'Top Level – A' contains two elements, 'Simple Type – i' and 'Constructor – 1'.  'Constructor – 1', in turn, contains two elements, 'Simple Type – i' and 'Simple Type – ii'.

The corresponding ASN.1 for Message A could be written as:

MessageA ::= SEQUENCE

{

firstElement		[0] STi

secondElement	[1] Cons1 OPTIONAL

}

Cons1 ::= SEQUENCE

{

elementOne	[0] STi  OPTIONAL

elementTwo	[1] STii

}

STi ::= INTEGER(0..255)

STii ::= IA5String (SIZE(1..80))

The combination of the mandatory and optional elements in the ASN.1 hierarchy determine what can be sent in a given instance of communication, for example message A in Figure 2-2 has the following possible combinations:

- TL-A + ST-i

- TL-A + ST-i + [C-1 + ST-ii]

- TL-A + ST-i + [C-1 + ST-i + ST-ii]

Note that in 3. the ST-i element appears twice.  This illustrates the reason why the classification is at the previous node in the hierarchy since ST-i is both mandatory and optional.

# PICS/OICS Proformas

## PICS/OICS Relationship to ICAO ATN SARPs

The PICS/OICS proformas are derived from Sub-Volumes II and III of ICAO Doc 9705; not all of an applications requirements can be expressed in the PICS/OICS proforma, either because the requirements are defining the dynamic behaviour of the application ASE or because the requirement is an essential part of implementing an application.  General statements such as the support for version numbers in Chapter 2 and the decoding/encoding rules in Chapter 3 are examples of the requirements which are not covered.

For each ATN application the PICS/OICS coverage of the requirements in the SARPs chapters is indicated in the following table:

| SARPs Chapter | PICS/OICS Coverage |
| --- | --- |
| Chapter 1 (Introduction) | No requirements |
| Chapter 2 (General Requirements) | Not covered |
| Chapter 3 (Abstract Service) | Complete |
| Chapter 4 (Formal Definitions of Messages) | Complete |
| Chapter 5 (Protocol Definition) | Partial (Technical Timers) |
| Chapter 6 (Communications Requirements) | Not covered |
| Chapter 7 (User Requirements) | Not covered |
| Chapter 8 (Subsetting Rules) | Complete |

## General Format

### File Structure

The proformas consist of a number of tables some of which are common to all ATN applications and some of which are specific to an application.  The common tables are discussed in Section 3.2.2 below.  The tables are implemented as spreadsheets with each table held in a separate spreadsheet.

For Air/Ground applications there are two proformas one for the airborne system and one for the ground system.  For Ground/Ground applications there are also two proformas one for the initiating system and one for the receiving system.  Each proforma is held in a separate file.

Within each file the tables are grouped together in sections, with each table and spreadsheet/worksheet identified by section identifier and table number within section.  The sections and identifiers are:

| I | Information Section. Contains the reference information about profiles and implementations. |
| --- | --- |
| S | Configurations and Services Section. Contains the proforma for operational configurations, protocol options, ASE abstract services and timers |
| M | Messages Section. Contains the proforma for the top level message elements. |
| P | Parameters section. Contains the proforma of all the remaining protocol elements. |

Examples of section and table identification are shown in the rest of this section.

### Information Section

The tables described below are common to all proformas; their main purpose is to capture reference information about a profiles and implementations.

#### PICS/OICS Identification (Table I-1)

The table is divided into three areas.  The first two contain the dates and reference or serial numbers for a completed PICS and/or OICS which are completed by implementers.  The third section contains details of the profile to which the PICS/OICS applies.  This section is completed by the profile authority as part of the process of creating a profile specific proforma.

An example of a completed table including entries in all three sections is shown in Figure 3-3.

| Ref No | PICS Identification | Implementation PICS |
| --- | --- | --- |
| 1 | Date of Statement | 1 April 1999 |
| 2 | PICS Serial Number | P/99/12345/001 |
|  | OICS Identification | Implementation OICS |
| 3 | Date of Statement | 1 April 1999 |
| 4 | OICS Serial Number | O/99/12345/001 |
|  | Profile Identification | Profile Details |
| 5 | Profile Name | European ATN Trails |
| 6 | Version | 1.0 |
| 7 | Profile Authority Name | ATC Europe |
| 8 | Profile Applicability | Western Europe |
|  | (Areas, Countries, Organisations etc where the |  |
|  | profile can be applied) |  |
| 9 | Date of effect | 1 January 1999 |
| 10 | Other Information | The data contained in this table and the other |
|  |  | examples in this section is shown in green and |
|  |  | italics to indicate that it is not real data but has |
|  |  | been invented as an example for this document. |

Figure 3-3: PICS/OICS Identification Example

#### Supplier and Implementation Identification (Table I-2)

This table contains the name and contact details of a supplier and the details of the implementation to which a completed PICS/OICS applies.  An example is shown in

| Ref No | Supplier Information | Supplier Details |
| --- | --- | --- |
| 11 | Organization Name | ATN Implementers Ltd |
| 12 | Contact Name(s) | J Smith |
| 13 | Address | ATNI House |
|  |  | Airport Road |
| 14 | Telephone Number | +44 1234 567890 |
| 15 | Telex Number | - |
| 16 | Fax Number | +44 1234 567910 |
| 17 | E-mail Address | jsmith@atni.co.uk |
| 18 | Other Information |  |
|  | Implementation Information | Implementation Details |
| 19 | Implementation Name |  |
| 20 | Implementation Version |  |
| 21 | Hardware Name |  |
| 22 | Hardware Version |  |
| 23 | Operating System Name |  |
| 24 | Operating System Version |  |
| 25 | Special Configuration |  |
| 26 | Other Information |  |

#### Application Protocol Identification (Table I-3)

This table lists the base standards, protocol version, amendments and defect reports which apply to the classification of the ATN and/or profile requirements for a specific application; together with the information for an implementation in a completed proforma.

### Configurations and Services Section

#### Application Protocol Options (Table S-1)

This table details the protocol options and associated predicates for a specific application.  The S-1 table shown below is taken from the ADS Ground proforma:

| Source: | Chapter 8 - Subsetting Rules | ADSP | Profile | IMP | Associated |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ref No | Protocol Option | Profile | Status | Support | Predicate | Notes |
| S-1.2 | ADS-ground-ASE | M |  |  | ADS/ground |  |
| S-1.3 | Demand contract supported | C.1 |  |  | G-DC-FU |  |
| S-1.4 | Event and Emergency contracts supported | C.1 |  |  | G-EC-FU |  |
| S-1.5 | Periodic and Emergency contracts supported | C.1 |  |  | G-PC-FU |  |

If an implementation supports, for example, event contracts then the G-EC-FU predicate becomes true.  In other tables conditions are resolved from the True/False status of the predicates.

#### Application Conformant Configurations (Table S-2)

This table details the configurations that may be supported for a specific application. The S-2 table shown below is taken from the ADS Ground proforma:

| Source: | Chapter 8 - Subsetting Rules |  |  | ADSP | Profile | IMP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ref No | List of Configurations: Ground ASE |  |  | Profile | Status | Support | Notes |
| S-2.1 | I. | ADS/ground + G-DC-FU | C.1 |  |  | Demand Contract Only |
| S-2.2 | II. | ADS/ground + G-EC-FU | C.1 |  |  | Event and Emergency Contracts Only |
| S-2.3 | III. | ADS/ground + G-PC-FU | C.1 |  |  | Periodic and Emergency Contracts Only |
| S-2.4 | IV. | ADS/ground + G-DC-FU + G-EC-FU | C.1 |  |  | Demand, Event and Emergency Contracts Only |
| S-2.5 | V. | ADS/ground + G-DC-FU + G-PC-FU | C.1 |  |  | Demand, Periodic and Emergency Contracts Only |
| S-2.6 | VI. | ADS/ground + G-EC-FU + G-PC-FU | C.1 |  |  | Periodic, Event and Emergency Contracts Only |
| S-2.7 | VII. | ADS/ground + G-DC-FU + G-EC-FU + G-PC-FU | C.1 |  |  | Demand, Periodic, Event and Emergency Contracts |

The supported configuration is determined by the protocol options supported in S-1.  For example, an implementation that only supports Event contracts can only be conformant to configuration II.

#### Supported Service Primitives (Table S-3)

This table details the support for the primitives of an application's abstract service. . The S-3 table shown below is taken from the ADS Ground proforma:

| Source: | Chapter 8 - Subsetting Rules | Sender (req, [cnf]) |  |  |  |  | Receiver (ind, [rsp]) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ADSP | Profile | IMP | ADSP | Profile | IMP |  |
| Ref No | Service Primitives | Profile | Status | Support | Profile | Status | Support | Notes |
| S-3.1 | ADS-demand-contract | C.1 |  |  | — | — | — | See S-4 |
| S-3.2 | ADS-event-contract | C.2 |  |  | — | — | — |  |
| S-3.3 | ADS-periodic-contract | C.3 |  |  | — | — | — |  |
| S-3.4 | ADS-report | — | — | — | M |  |  |  |
| S-3.5 | ADS-cancel | C.4 |  |  | — | — | — |  |
| S-3.6 | ADS-cancel-all-contracts | C.5 |  |  | — | — | — |  |
| S-3.7 | ADS-emergency-report | — | — | — | C.7 |  |  |  |
| S-3.8 | ADS-modify-emergency-contract | C.6 |  |  | — | — | — |  |
| S-3.9 | ADS-cancel-emergency-contract | — | — | — | C.7 |  |  |  |
| S-3.10 | ADS-user-abort | M |  |  | M |  |  |  |
| S-3.11 | ADS-provider-abort | — | — | — | M |  |  |  |

| C.1 | If Demand contract (G-DC-FU) supported then M else — |
| --- | --- |
| C.2 | If Event contract (G-EC-FU) supported then M else — |
| C.3 | If Periodic contract (G-PC-FU)supported then M else — |
| C.4 | If ((Periodic (G-PC-FU) or Event (G-EC-FU) contract supported) and Cancel supported then M else — |
| C.5 | If ((Periodic (G-PC-FU) or Event (G-EC-FU) contract supported) and Cancel all contracts supported then M else — |
| C.6 | If Modify emergency contract supported then M else — |
| C.7 | If Periodic (G-PC-FU) or Event (G-EC-FU) contract supported then M else — |

The supported service primitives are determined by the protocol options supported in S-1.  For example, an implementation that only supports Event contracts will resolve conditions C.2 and C.7 to M and may also resolve C.4, C.5 or C.6 to M depending on the support for those services.

#### Service Tables

The remaining tables in this section detail each abstract service.  Conformance for the support of the parameters of an abstract service is phrased in terms of the capability of an implementation to support the parameters since the SARPs do not specify or constrain how an abstract service is implemented.  Where the values of a parameter are specified using an ASN.1 type definition the ASN.1 Description column of the table contains a reference to the table containing the type definition.

The example service table shown below is of the ADS Event contract for a ground system:

| Source: | Chapter 3 - Abstract Service | OICS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Operational Use |  |  |  |  |  |  |  |  |  |  |
|  |  | ADSP | Profile |  |  | Implementation |  |  | ASN.1 Description |  |
| Ref No | Operational Elements | Profile | Status | Cons | Status | Cons |  | Notes |
| S-5.1 | Capability of the ground system to specify an Aircraft Address |  |  |  |  |  |  |  |  |
| S-5.1.1 | Aircraft Address = Bit String SIZE(24) | C.1 |  | — |  | — |  |  |
| S-5.2 | Capability of the ground system to request a Class of Communication Service |  |  |  |  |  |  |  |  |
| S-5.2.1 | One from the abstract values: 'A', 'B', 'C', 'D', 'E', 'F', 'G', 'H' | C.1 |  |  |  |  |  |  |
| S-5.3 | Capability of the ground system to specify the terms of an event contract |  |  |  |  |  |  |  |  |
| S-5.3.1 | Contract Details | C.1 |  |  |  |  | See EventContract (Table P-3) |  |
| S-5.4 | Capability of the ground system to understand a Reply from an Aircraft |  |  |  |  |  |  |  |  |
| S-5.4.1 | Negative Acknowledgement | C.1 |  |  |  |  | See NegativeAcknowledgement (Table P-10) |  |  |
| S-5.4.2 | Noncompliance Notification | C.1 |  |  |  |  | See NoncomplianceNotification (Table P-10) |  |  |
| S-5.4.3 | Positive Acknowledgement = NULL | C.1 |  |  |  |  |  |  |
| S-5.5 | Capability of the ground system to specify its own ICAO Facility Designation |  |  |  |  |  |  |  |  |
| S-5.5.1 | ICAO facility designation = IA5 String SIZE(4..8) | C.1 |  |  |  |  |  |  |

| C.1 | If Event contract (G-EC-FU)supported then M else — |
| --- | --- |

The condition C.1 determines if the whole table is in or out of scope.  If an implementation does not support Event contracts the table is not completed.

### Messages Section

The tables in the section detail the PDUs and top level messages exchanged by the application peers. The example message table, M-1, shown below is part of the table for an ADS ground system showing the messages received from an aircraft.

### Parameters Section

The tables in the section detail the protocol elements exchanged in PDUs or messages.  The example parameters table, P-3, shown below is part of the table for an ADS Event Contract request for the ground sysytem.

Table M-1: ADS Messages (top level) [Part only]

| Source: | Chapter 4 - ASN.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Send |  |  |  |  |  |  |  |  | Receive |  |  |  |  |  |  |  |  |  |  |
|  |  | ADSP | Profile |  |  | Implementation |  |  | ADSP | Profile |  |  | Implementation |  |  | ASN.1 Protocol Elements |  |
| Ref No | Operational Elements | Profile | Status | Cons | Status | Cons | Profile | Status | Cons | Status | Cons |  | Notes |
| M-1.1 | ADS Aircraft PDUs |  |  |  |  |  |  |  |  |  |  | ADSAircraftPDUs ::= CHOICE |  |
| M-1.1.1 | ADS cancel emergency PDU | — | — | — | — | — | C.1 |  |  |  |  | [0] NULL |  |
| M-1.1.2 | ADS demand report PDU | — | — | — | — | — | C.2 |  |  |  |  | [1] ADSDemandReport |  |
| M-1.1.3 | ADS emergency report PDU | — | — | — | — | — | C.1 |  |  |  |  | [2] ADSEmergency |  |
| M-1.1.4 | ADS event report PDU | — | — | — | — | — | C.3 |  |  |  |  | [3] ADSEventReport |  |
| M-1.1.5 | ADS negative acknowledgement PDU | — | — | — | — | — | M |  |  |  |  | [4] NegativeAcknowledgement |  |
| M-1.1.6 | ADS noncompliance notification PDU | — | — | — | — | — | M |  |  |  |  | [5] NoncomplianceNotification |  |
| M-1.1.7 | ADS periodic report PDU | — | — | — | — | — | C.4 |  |  |  |  | [6] ADSPeriodicReport |  |
| M-1.1.8 | ADS positive acknowledgement PDU | — | — | — | — | — | C.5 |  |  |  |  | [7] PositiveAcknowledgement |  |
| M-1.1.9 | ADS provider abort PDU | — | — | — | — | — | M |  |  |  |  | [8] AbortReason |  |
| M-1.1.10 | Use of Extensibility | — | — | — | — | — | M |  |  |  |  | Use of Extensibility |  |

Table P-3: ADS Event Contract Request [Part only]

| Source: | Chapter 4 - ASN.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Send |  |  |  |  |  |  |  |  | Receive |  |  |  |  |  |  |  |  |  |  |
|  |  | ADSP | Profile |  |  | Implementation |  |  | ADSP | Profile |  |  | Implementation |  |  | ASN.1 Protocol Elements |  |
| Ref No | Operational Elements | Profile | Status | Cons | Status | Cons | Profile | Status | Cons | Status | Cons |  | Notes |
| P-3.1 | Event Contract Request |  |  |  |  |  |  |  |  |  |  | EventContract ::= SEQUENCE |  |
| P-3.1.1 | Lateral Change | C |  |  |  |  | — | — | — | — | — | [0] LateralChange (OPTIONAL) |  |
| P-3.1.2 | Vertical Rate Change | C |  |  |  |  | — | — | — | — | — | [1] VerticalRateChange (OPTIONAL) |  |  |
| P-3.1.3 | Level Range | C |  |  |  |  | — | — | — | — | — | [2] LevelRange (OPTIONAL) |  |
| P-3.1.4 | Way point change | C |  |  |  |  | — | — | — | — | — | [3] NULL (OPTIONAL) |  |
| P-3.1.5 | Air Speed Change | C |  |  |  |  | — | — | — | — | — | [4] AirSpeedChange (OPTIONAL) |  |  |
| P-3.1.6 | Ground Speed Change | C |  |  |  |  | — | — | — | — | — | [5] GroundSpeedChange (OPTIONAL) |  |  |
| P-3.1.7 | Heading change | C |  |  |  |  | — | — | — | — | — | [6] DegreesDirection (OPTIONAL) |  |  |
| P-3.1.8 | Extended Projected Profile | C |  |  |  |  | — | — | — | — | — | [7] ExtendedProjectedProfileRequest (OPTIONAL) |  |  |
| P-3.1.9 | FOM change | C |  |  |  |  | — | — | — | — | — | [8] NULL (OPTIONAL) |  |
| P-3.1.10 | Track angle change | C |  |  |  |  | — | — | — | — | — | [9] DegreesDirection (OPTIONAL) |  |  |
| P-3.1.11 | Level Change | C |  |  |  |  | — | — | — | — | — | [10] LevelChange (OPTIONAL) |  |
| P-3.1.12 | Use of Extensibility | X |  |  |  |  | — | — | — | — | — | Use of Extensibility |  |

### Column Headings

The layout of the column headings used in the parameter tables is shown in  Figure 3-4.  The table is divided into columns as follows:

Column 1:	Source: Contains a reference to the Chapter in the SARPs [2] from which the content of the table has been derived.

Ref No.  Contains the reference number which is a unique identifier for each information field, service or protocol element in the PICS/OICS for an application.  The reference number format is T-n.a.b.c....  Where T-n is the table identifier and a.b.c... is the hierarchical identifier for the fields within this table.  For example the reference number of the North component of Latitude Direction (taken from table P-12, Lattitude Longitude in CPDLC) is P-12.1.1.2.1; the table below shows how the top down hierarchy is applied to the reference.

| P-12.1. | Latitude Longitude |
| --- | --- |
| P-12.1.1 | Latitude |
| P-12.1.2 | Longitude |
| P-12.1.1. | Latitude |
| P-12.1.1.1 | Latitude Type |
| P-12.1.1.2 | Latitude Direction |
| P-12.1.1.2 | Latitude Direction |
| P-12.1.1.2.1 | North |
| P-12.1.1.2.2 | South |

Column 2:	Operational Elements.  Contains the name of the item from an operational aspect.  If the item contains a value, the units, permitted range of values and resolution as specified for the ATN Profile are also shown.  For example the value of minutes of latitude or longitude used in CPDLC are expressed in 100ths of a minute using the MinutesLatLon protocol element; the corresponding entry in the table is:

MinutesLatLon  (0-59.99)/0.01

Columns 3-7 and 8-12 are repeated, the first set is for sending and the second is for receiving.

Column 3/8:	ADSP Profile.  Contains the required support of the item by an implementation for conformance to the ADSP Profile.

Column 4/9:	Profile Status.  Contains the required support of the item by an implementation for conformance to a Profile.  If the proforma is for a profile the details of the profile are given in the PICS/OICS identification table, see Section 3.2.2.1.

Column 5/10:	Profile Cons.  Contains the constraints which apply to a Profile where they differ from those of the ADSP Profile.  These then become the constraints against which implementers complete the conformance tables.

Column 6/11:	Implementation Status.  Contains the support for the item by an implementation in a completed proforma.

Column 7/12:	Implementation Cons.  Contains the constraints which apply to an implementation where they differ from those of either the ADSP Profile or Operational Profile.

Column 13:	ASN.1 Protocol Elements.  Contains the ASN.1 protocol elements.  A type name is always followed by the ASN.1 type and if an element is an ASN.1 basic type this is followed by any constrains, for example the entry for the ASD Intermediate Intent type is:

IntermediateIntent

SEQUENCE SIZE(0..7) OF SEQUENCE

INTEGER (1..8000)

DegreesDirection

Level

ProjectionTime

Column 14:	Notes.  The column may be used to add additional notes about the item.  The note is identified by a letter in the column; all the notes are found after the end of the table.

### Notes and Conditional Status

Any Notes or Conditional Status descriptions are listed after the end of the table.  A conditional status is written as C or C.n  or P.n, where n is a number used to identify the condition within a table.  The C signifies that at least one element in the list must be selected.  The C.n signifies an SARPs derived conditional and P.n signifies an operational profile derived conditional.  A conditional statement is written in the form of an "if" statement which resolves the element status according to whether the condition is true or false:

IF (condition) THEN (status if condition is true) ELSE (status if condition is false)

Complex conditions may include brackets ( ) and the Boolean AND or OR operators.

An example of a conditional description from ADS is:

C.1  If (Periodic (G-PC-FU) or Event (G-EC-FU) contract supported) then M else —

### Formatting and Colours

In the tables item names and identifiers are show in bold type; the elements that make up a item are shown in normal type.  The names of ASN.1 enumerated types are shown in italic type.

Colours are used as follows:

Blue		Column 1 reference numbers

Red		Profile classification (columns 4, 5, 9, 10), plus any profile conditionals or notes.

Light grey	Element is not applicable in this context.

Figure 3-4: Column Headings

## Classification Scheme

### Overview

The status of each service and element or item is given a classification based on the following general rules:

- An item is Mandatory if in at least one circumstance it has to be used.

- An item is Optional if there are no circumstances in which it must be used.

- An item is Conditional if it has to be used when a given circumstance or set of circumstances are true.  A conditional status applies relative to where the item is within the hierarchy of the service or protocol definition.  The elements that are below a conditional in the hierarchy are classified as if the conditional item is supported.

- If a condition only resolves to Optional or Not Used, the item is not considered to be Conditional but is made Optional or Not Used.  Only if there is at least one circumstance in which an item must be used is it classified as conditional.

### ASN.1 Encoding and Decoding

There are no columns in the proformas for classifying the encoding and decoding of ASN.1 because this is always covered by general requirements in Chapter 4 of all application SARPs which mandate that an ASE shall be able to encode all PDUs that it sends and decode all PDUs that it receives.

Operationally this means that for sending an element must be encoded according to the ASN.1 definition and encoding rules.  For receiving this means decoding an element according to the ASN.1 definition and encoding rules; even if an element is not supported operationally an implementation must have the capability to recognise the element and processes it as an exception.

### Operational Classification

#### ADSP Profile/Operational Profile Status

The operational status of an item is indicated in either the ADSP Profile or Profile Status columns 3 and 4 in  Figure 3-4

M Mandatory	the capability is required to be implemented in accordance with the operational requirements of either the SARPs or the Operational Profile.

Both the sender and receiver must support the capability.

O Optional		the capability may be implemented, and if it is implemented it is required to conform to the operational requirements of either the SARPs or Operational Profile.

Optional here does not necessarily have the same meaning as optional in the ASN.1 definition; it is a true choice as to whether the capability is ever supported for sending whereas optional in ASN.1 only refers to an instance of communication.  An optional element in ASN.1 may become mandatory for operational support because it needs to be sent in at least once.

The choice for support of an optional element only applies to the sender of the element; the receiver must always be able to receive an optional element.

C Conditional	the requirement on the capability depends on the selection of other optional or conditional items.  C is used for conditions specified by the SARPs.  The C on its own with no number suffix is used in a list to signify that at least one of the options in the list of C’s must be supported.

C.x Conditional	the requirement on the capability depends on the selection of other optional or conditional items.  The x is a number which refers to a condition described in notes at the end of the table.

P.x Conditional	the requirement on the capability depends on the selection of other optional or conditional items.  P is intended for use in operational profiles where a condition is not the same as the SARPs.  The x is a number which refers to a condition described in notes at the end of the table.

X Prohibited		there is a requirement NOT to use this capability in this context.  If used it should be treated as a protocol violation by the receiver.

— Not applicable	in the given context either the SARPs or Operational Profile make it impossible to use this element or capability.

I Out of scope	the element or capability is outside of the scope of the given context.  If used the behaviour of the receiving entity is undefined, unless an operational profile defines how this is handled.

This is intended for use in operational profiles, where for example there is no requirement for support of a particular capability but it is not necessary to prohibit its inclusion by an implementation as a potential capability for use in other profiles.

# Guidance for Defining Operational Profiles

Operational Profiles provide a mechanism for specifying the requirements that are needed for a operating within a specific airspace.  Typically a profile is used to define the minimum subset required for interoperability within the airspace.

For services and elements that are classified as either optional or conditional, it should be possible to resolve many of these in defining a profile.  Where a condition remains and is the same as the ADSP Profile the “C.x” form should be used in the profile column; if it is not the same the “P.x” form should be used

If an element is not required to be supported by a profile, consideration should be given as to whether it is classified as X (Prohibited) or I (Out of scope).  It is recommended that out of scope is used unless it is absolutely necessary to prohibit the use.

# Guidance for Completing a Proforma

The Information (I-1 to I-3) and Configuration and Services Section (S-1 to S-n) tables are completed first.  The first three S tables, S-1 Protocol Options, S-2 Conformant Configurations and S-3 Supported Service Primitives set the high level options and predicates that determine how the remainder of the tables are completed.  Where a function or facility is not supported at a high level implementers are not required to complete any of the associated lower level tables.  For example, an airborne CM implementation that does not support the CM Update service is not required to complete Table S-5 CM Update Service.

The Messages and Parameters tables must be completed hierarchically starting from a top level message type and following the ASN.1 structure down in sequence.  Once a no answer is entered, implementers need go no further down the hierarchy.

The supplier of an implementation completes a proforma by filling in the Implementation Status and Cons columns, 6 and 7, and 11 and 12 in  Figure 3-4, indicating support or non-support as follows:

Y or y		The implementation supports the item

N or n		The implementation does not support the item or only implements some aspects of the item.  In the latter case the supplier should indicate in a note the degree of support for the item.

—		The item is not applicable in this context

Where a supplier indicates non-support and the reason for non-support includes a constraint which differs from the ATN or Operational Profile, the Implementation Cons column 7 in  Figure 3-4 is used to specify the actual constrains of the implementation.

When completing a proforma it is important to remember that the status of an element applies to the whole life of the profile and not to an instance of communication; this means that for components of ASN.1 Choice or Sequence with optional components may have mandatory components because they must be supported in at least one instance of communication.

# Application Specific Guidance

## Introduction

This section contains guidance material for the PICS/OICS for each of the ATN applications.  The information is in addition to the general guidance contained in Sections 1 to 5 of this document.

## Guidance for ADS

### Introduction

The ADS application uses the standard set of proforma tables.

### Proforma Tables

The same list of tables are used for both the airborne and ground ADS implementations:

Information Section

Table I-1: PICS/OICS Identification

Table I-2: Supplier and Implementation Identification

Table I-3: ADS Protocol Identification

Configuration and Services Section

Table S-1: ADS Protocol Options

Table S-2: ADS Operational Configurations

Table S-3: Supported ADS Service Primitives - Airborne or Ground ASE

Table S-4: ADS Demand Contract Service

Table S-5: ADS Event Contract Service

Table S-6: ADS Periodic Contract Service

Table S-7: ADS Report Service

Table S-8: Table S-8: ADS Cancel and ADS Cancel All Services

Table S-9: ADS Emergency Report Service

Table S-10: ADS Modify Emergency Contract Service

Table S-11: ADS User Abort Service

Table S-12: ADS Provider Abort Service

Table S-13: ADS Timers

Messages Section

Table M-1: ADS Messages (top level)

Parameters Section

Table P-1: ADS Cancel Contract

Table P-2: ADS Demand Contract Request

Table P-3: ADS Event Contract Request

Table P-4: ADS Modify Emergency Contract

Table P-5: ADS Periodic Contract Request

Table P-6: ADS Demand Report

Table P-7: ADS Emergency Report

Table P-8: ADS Event Report

Table P-9: ADS Periodic Report

Table P-10: ADS Acknowledgements and Noncompliance Notification

Table P-11: Common Components in ADS Reports

### Completing an ADS Proforma

The Information section tables, I-1 to I-3, are completed first.  The Configuration and Services tables, S-1 to S-13, are completed in order; S-4, S-5, S-6, S-8, S-9 and S-10 are completed for the contract types and services supported by an implementation.  The Messages table, M-1, is completed by all implementations and the Parameters tables P-10 and P-11.  The other Parameters tables P-1 to P-9 are completed for the contract and report types supported by an implementation.

The relationship between the proforma tables is shown in Figure 6-5.

Figure 6-5: ADS Proforma Structure

## Guidance for ADS Report Forwarding

### Introduction

The ADS Report Forwarding (RF) application uses the standard set of proforma tables.

### Proforma Tables

The same list of tables are used for both the initiating and and responding ADS RF implementations:

Information Section

Table I-1: PICS/OICS Identification

Table I-2: Supplier and Implementation Identification

Table I-3: ADS RF Protocol Identification

Configuration and Services Section

Table S-1: ADS RF Protocol Options

Table S-2: ADS RF Conformant Configurations

Table S-3: Supported ADS RF Service Primitives – Initiating or Responding ASE

Table S-4: ADS Start Forward Service

Table S-5: ADS Forward Report Service

Table S-6: ADS End Forward Service

Table S-7: ADS User Abort Service

Table S-8: ADS Provider Abort Service

Table S-9: ADS RF Timers

Messages Section

Table M-1: ADS RF Top Level Messages

Parameters Section

Table P-1: ADS Report

Table P-2: ADS Emergency Report

### Completing an ADS RF Proforma

The Information section tables, I-1 to I-3, are completed first.  The Configuration and Services tables, S-1 to S-9, are completed in order; S-4, S-5, S-6, S-8, S-9 and S-10 are completed for the contract types and services supported by an implementation.  The Messages table, M-1, and the Parameters tables P-1 and P-2 are completed for all implementations.

The relationship between the proforma tables is shown in Figure 6-6.

Figure 6-6: ADS RF Proforma Structure

## Guidance for CM

### Introduction

The CM application uses the standard set of proforma tables.

### Proforma Tables

The same list of tables are used for both the airborne and ground CM implementations:

Information Section

Table I-1: PICS/OICS Identification

Table I-2: Supplier and Implementation Identification

Table I-3: CM Protocol Identification

Configuration and Services Section

Table S-1: CM Protocol Options

Table S-2: CM-ASE Conformant Configurations

Table S-3: Supported CM Service Primitives – Airborne or Ground ASE

Table S-4: CM Logon Service

Table S-5: CM Update Service

Table S-6: CM Contact Service

Table S-7: CM End Service

Table S-8: CM User Abort Service

Table S-9: CM Provider Abort Service

Table S-10: CM Forward Service (Initiator) (Ground CM only)

Table S-11: CM Forward Service (Responder) (ground CM only)

Table S-12: CM Technical Timers

Messages Section

Table M-1: CM Messages (top level)

Parameters Section

Table P-1: CM Message Components

### Completing A CM Proforma

The Information section tables, I-1 to I-3, are completed first.  The configuration and services tables, S-1 to S-12, are completed in order; S-5 and S-6 are only completed if Update or Contact are supported.  S-10 and S-11 are only present in the ground proforma and are only completed if Forwarding is supported.  The relationship between the proforma tables is shown in Figure 6-7.

Figure 6-7: CM Proforma Structure

## Guidance for CPDLC

### Introduction

The CPDLC application differs from the other ATN applications in several important ways that affect how the PICS/OICS proforma is defined. In CPDLC it is the content of the uplink and downlink message that determine the majority of options and choices.  The same data types are used different messages but the options used within the types may also be different depending on the message context.  In addition some data types are used several times within the same message, again with the possibility of selecting different options.

For these reasons the CPDLC PICS/OICS sections for tables and parameters are structured to allow conformance to be specified based on groups of messages or a single message.  This enables the PICS/OICS to be completed by either a profile writer or implementers starting from a list of the supported uplink and downlink messages.

### Proforma Tables

The Information and, Configuration and Services sections are standard; the Messages and Parameters sections contain the non-standard tables which are specific to CPDLC.  In the list of tables given below those shown in italics are the CPDLC specific tables.  The Parameters section tables include a list of the CPDLC message level ASN.1 types included in each table.

Information Section

Table I-1: PICS/OICS Identification

Table I-2: Supplier and Implementation Identification

Table I-3: CPDLC Protocol Identification

Configuration and Services Section

Table S-1: CPDLC Protocol Options

Table S-2: CPDLC-ASE Conformant Configurations

Table S-3: Supported CPDLC Service Primitives - Airborne ASE

Table S-4: Air Initiated CPDLC Start Service Parameters

Table S-5: Ground Initiated CPDLC Start Service Parameters

Table S-6: CPDLC Message Service Parameters

Table S-7: CPDLC End Service Parameters

Table S-8 : DSC Start Service Parameters

Table S-9: DSC End Service Parameters

Table S-10: Forward (Initiator) Service Parameters

Table S-11: Forward (Responder) Service Parameters

Table S-12: CPDLC User Abort Service Parameters

Table S-13: CPDLC Provider Abort Service Parameters

Table S-14: CPDLC Timers

Messages Section

Table M-1: Ground Generated Messages

Table M-2: Aircraft Generated Messages

Table M-3: Uplink and Downlink Messages Common Elements

Table M-4: Uplink Message Elements

Table M-5: Uplink Messages Permitted Operational Responses

Table M-6: Downlink Message Elements

Table M-7: Downlink Messages Permitted Operational Responses

Table M-8: Compound Message Parameter Elements

Table M-9: Ground Forward Initiator

Table M-10: Ground Forward Responder

Parameters Section

Table P-1: Airport Parameter

Table P-2: Altimeter Parameter Uplink

Table P-3: Clearance Type Parameter - Downlinked

Table P-4: Degrees Parameter

Table P-5: Departure Clearance Parameter

Table P-6: Direction Parameter

Table P-7: Distance Parameter

Table P-8: Distance Specified Direction Parameter

Table P-9: Error Information Parameter

Table P-10: Frequency Parameter

Table P-11: Hold Clearance Parameter

Table P-12: Latitude Longitude Parameter

Table P-13: Leg Type Parameter

Table P-14: Level Parameter

Table P-15: Position Parameter

Table P-16: Position Report Parameter

Table P-17: Procedure Name Parameters

Table P-18: Published Identifier Parameter

Table P-19: Route Clearance Parameter

Table P-20: Route Information Parameter

Table P-21: Runway Parameter

Table P-22: Speed Parameter

Table P-23: Speed Type Parameter

Table P-24: Time Parameter

Table P-25: Unit Name Parameter

Table P-26: Vertical Rate Parameter

#### Table M-4: Uplink Message Elements

The table has standard column headings as shown in  Figure 3-4 but in the Notes column for those messages that have parameters there is a reference to the Parameter table containing the message parameters.

#### Table M-5: Uplink Messages Permitted Operational Responses

The table shows all of the messages that are permitted in response to those uplink messages which require a response, i.e. those having a “Y” Response Attribute.

#### Table M-6: Downlink Message Elements

The table has standard column headings as shown in  Figure 3-4 but in the Notes column for those messages that have parameters there is a reference to the Parameter table containing the message parameters.

#### Table M-7: Downlink Messages Permitted Operational Responses

The table shows all of the messages that are permitted in response to those downlink messages which require a response, i.e. those having a “Y” Response Attribute.

#### Table M-8: Compound Message Parameter Elements

This table contains all of the top level message ASN.1 types that are combinations of other types.

#### Parameter Tables

These tables are referenced either from the top level parameter in a message or from other tables.  Each type is defined only once, where a type is used as an element within another type the table containing the type definition is referenced in the notes column.

When completing proforma, implementers are required to copy the tables and complete separately for each time a type is used differently.  This may include different messages or different uses within the same message.

The tables have the standard column headings as shown in  Figure 3-4.

### Profiles

A profile may specify a subset of the CPDLC messages which must be supported; this can result in some parameter tables being no longer applicable because none of the parameters they contain are required.  In these cases it is recommended that the initial part of the table containing the top level message parameters is retained, indicating that none of the parameters are applicable, but that the detailed part may be deleted.

### Guidance for Implementers

The parameter tables, as presented in the PICS/OICS, contain only one copy of the proforma for each set of message parameters.  There maybe circumstances when it is not possible for implementers to complete only one copy of a proforma.  For example, an ASN.1 type containing choices may use a different set of options depending on the message or the type may be used more than once within a message with different options depending on the context.  In such circumstances implementers are required to complete additional copies of the proforma as follows:

- Make a copy of the blank proforma.

- Complete the copy of the proforma for each different message parameter/choice combination.

- Add notes listing the context and all of the uplink and downlink messages to which the completed proforma applies.

#### Completing a CPDLC Proforma

The Information section tables, I-1 to I-3, are completed first.  The Configuration and Services tables, S-1 to S-14, are completed in order; S-4, S-5, S-8, S-9, S-10 and S-11 are completed for the services supported by an implementation.  The Messages tables, M-1, M-2 and M-3 are completed.  Tables M-4, M-6 and M-8 are completed for the Uplink and Downlink messages supported by an implementation.  The Message Response tables, M-5 and M-7, are completed from the answers given in M-4 and M-6.  M-9 and M-10 are only completed from ground systems that support forwarding.

From each supported message in M-4 and M-6, the hierarchy is followed down completing the P tables as required.

The relationship between the proforma tables is shown in Figure 6-8.

Figure 6-8: CPDLC Proforma Structure

## Guidance for FIS

### Introduction

The FIS application uses the standard set of proforma tables.

### Proforma Tables

The same list of tables are used for both the airborne and ground FIS implementations:

Information Section

Table I-1: PICS/OICS Identification

Table I-2: Supplier and Implementation Identification

Table I-3: FIS Protocol Identification

Configuration and Services Section

Table S-1: FIS Protocol Options

Table S-2: FIS-ASE Conformant Configurations

Table S-3: Supported FIS Service Primitives

Table S-4: FIS-demand-contract Service Parameters

Table S-5: FIS-update-contract Service Parameters

Table S-6: FIS-report Service Parameters

Table S-7: FIS-cancel-contracts Service Parameters

Table S-8: FIS-cancel-update-contract Service Parameters

Table S-9: FIS-provider-abort Service Parameters

Table S-10: Technical Timers

Messages Section

Table M-1: FIS Messages (Top Level)

Table M-2: FIS Messages (Second Level)

Table M-3: ATIS Messages (Top Level)

Parameters Section

Table P-1: Altimeter Setting

Table P-2: Cloud

Table P-3: Date & Time

Table P-4: Present Weather

Table P-5: Runway

Table P-6: Surface Wind

Table P-7: Visibility

Table P-8 Miscellaneous

This introduction is only intended to provide enough background to enable an understanding of the relationship between ANS.1 and the PICS/OICS proforma.

The proformas have been developed using Microsoft Excel, each PICS/OICS Proforma is a “Workbook” containing “Worksheets” for each table.

If an implementation does not support a conditional element, the implementation status column entry becomes no ('n' or 'N') and the implementer need go no further down the hierarchy.

Prohibiting the use of an element has implications for testing.