ATNP/WG3/WP -54

AERONAUTICAL TELECOMMUNICATIONS NETWORK PANEL (ATNP)

working group OF THE WHOLE

August 2000

Berlin, Germany

Agenda Item 4.3

Changes to Sub-volume IX

Prepared by WG3/SG3

Summary

During validation of the ASN.1 modules for Subvolumes IV and VIII, it was noted that there was an OBJECT IDENTIFIER allocation conflict.

The ATN Security Services requires the use of  a set of the same OBJECT IDENTIFIERS in both Subvolumes IV and VIII.  To remove the OID conflicts, WG1/SG2 and WG3/SG3 agreed to place those OIDs that are common to both Subvolumes IV and VIII in the ATNObjectIdentifiers ASN.1 module of Subvolume IX.

One of the original purposes of Subvolume IX was to collect all top-level OIDs to prevent OID conflicts.  This scope is  expanded to include other, lower-level OIDs for the resolution of OID conflicts between the Subvolumes.

This paper presents changes to Subvolume IX to accommodate this expanded scope.  Subvolumes IV and VIII have already been modified to align with the changes included herein.

### Introduction

During validation of the ASN.1 modules for Subvolumes IV and VIII, it was noted that there was an OBJECT IDENTIFIER allocation conflict.

The ATN Security Services requires the use of  a set of the same OBJECT IDENTIFIERS in both Subvolumes IV and VIII.  To remove the OID conflicts, WG1/SG2 and WG3/SG3 agreed to place those OIDs that are common to both Subvolumes IV and VIII in the ATNObjectIdentifiers ASN.1 module of Subvolume IX.

One of the original purposes of Subvolume IX was to collect all top-level OIDs to prevent OID conflicts.  This scope is  expanded to include other, lower-level OIDs for the resolution of OID conflicts between the Subvolumes.

This paper presents changes to Subvolume IX to accommodate this expanded scope.  Subvolumes IV and VIII have already been modified to align with the changes included herein.

### Required Changes to Subvolume IX text

| Paragraph | Change |
| --- | --- |
| 9.1.2 | Add the following statement as the last sentence of the paragraph. “OIDs which are used by two or more Sub-volumes may be specified here.” |
| 9.2.1.1 Note 3. | Remove the last sentence: “Similarly, IATA has obtained an ICD of “iata(19)”; values assigned under the IATA name space are out of scope.” |
| Figure 9-1 | Remove the iata(19) arc from the figure. |
| 9.2.1.3 | Change the title of the paragraph from “Top Level ICAO Identifier ASN.1” to “ATN Object Identifiers ASN.1”. |
| Global | Change “subvolume” to “sub-volume”. |

### Required Changes to Subvolume IX ASN.1

Replace the ASN.1 module in 9.2.1.3 with the following.

ATNObjectIdentifier { iso(1) identified-organization(3) icao(27) atn(0)

objectIdentifiers(0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- EXPORTS everything

icao-arc		OBJECT IDENTIFIER ::= { iso(1) identified-organization(3)

icao(27) }

-- Root of the ICAO OBJECT IDENTIFIER space

icao-atn		OBJECT IDENTIFIER ::= { icao-arc atn(0) }

-- General ATN

icao-atn-end-system-air

OBJECT IDENTIFIER ::= { icao-arc atn-end-system-air(1) }

-- ATN aircraft end system

icao-atn-end-system-ground

OBJECT IDENTIFIER ::= { icao-arc atn-end-system-ground(2) }

-- ATN ground end system

icao-atn-ac		OBJECT IDENTIFIER ::= { icao-arc atn-ac(3) }

-- ATN application context names

icao-atn-sm		OBJECT IDENTIFIER ::= { icao-arc atn-sm(4) }

-- ATN system management

icao-atn-security-requirements

OBJECT IDENTIFIER ::= { icao-arc

atn-security-requirements(5) }

-- ATN security

icao-atn-certificate-authority

OBJECT IDENTIFIER ::= { icao-arc

atn-certificate-authority(6) }

-- ATN certificate authority

icao-atn-directory

OBJECT IDENTIFIER ::= { icao-arc atn-directory(7) }

-- ATN Directory

icao-atn-amhs	OBJECT IDENTIFIER ::= { icao-arc atn-amhs(8) }

-- ATN AMHS

--

-- ATN security OIDs shared between Subvolumes IV and VIII

--

secids		OBJECT IDENTIFIER ::= { icao-atn-security-requirements }

-- Categories of information object --

modules		OBJECT IDENTIFIER ::= { secids 1 }

secExchTypes	OBJECT IDENTIFIER ::= { secids 2 }

-- Security ASN.1 modules in Subvolume IV --

securityExchanges	OBJECT IDENTIFIER ::= { modules 1 }

abstractSyntax	OBJECT IDENTIFIER ::= { modules 2 }

-- Security ASN.1 module in Subvolume VIII --

atnPKI		OBJECT IDENTIFIER ::= { modules 3 }

-- SECURITY-EXCHANGE identifiers --

atn-establish	OBJECT IDENTIFIER ::= { secExchTypes 1 }

atn-protect-sign	OBJECT IDENTIFIER ::= { secExchTypes 2 }

END -- ATN OID definitions