PART VI

AIR  TRAFFIC  SERVICES  INTERFACILITY

DATA  COMMUNICATION

(AIDC)

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

# APPLICATION OVERVIEW

## Introduction

### The ATS Interfacility Data Communications (AIDC) application exchanges ATC information between ATSUs in support of ATC functions, including notification of flights approaching an Flight Information Region (FIR) boundary, coordination of boundary crossing conditions, and transfer of control.

### The AIDC application will use ATN.

### Appendix A to this chapter provides guidance to assist in the transition from character-based, inter-facility data interchange to bit-oriented AIDC message exchanges.

### The following abbreviations and definitions are used in this application:

C-ATSU:  The ATS unit currently responsible for control (i.e., the controlling ATS unit).

D-ATSU:  A downstream ATS unit is defined as an ATS unit responsible for control of an airspace region through or near which the aircraft will progress at some future time.

R-ATSU:  The ATS unit that will assume control responsibility (i.e., the receiving ATS unit).

ATSU1:	Used to denote an arbitrary ATS unit.

ATSU2:	Used to denote an arbitrary ATS unit, other than ATSU1, and

A dialogue is a sequence of one or more related message exchanges.

## Operational Concept

### Phases of Coordination

#### The AIDC message set is based upon the concept of phases of coordination.  Each flight, from an individual ATSU’s perspective, progresses sequentially through a series of phases.  Each flight, as it proceeds from one FIR to another transitions through three phases of coordination.  The phases encompassing AIDC are listed in sequential order as follows:

- Notify Phase,  in which the trajectory and any changes may be conveyed to one ATSU from the C-ATSU prior to coordination using a Notification dialogue;

- Coordinate Phase,  in which the flight's trajectory is coordinated between two or more ATSUs when the flight approaches a common boundary using Coordination dialogue; and

- Transfer Phase,  in which communications and executive control authority is transferred from one ATS Unit to another using a Transfer dialogue.

#### Once a flight is airborne, the trajectory may be changed by the C-ATSU.  The trajectory and any changes may be sent to affected D-ATSUs using a Notification dialogue.  A flight does not actually have to enter an FIR for notification to be relevant; it need only pass within the lateral separation minima associated with an FIR.  Notification messages are transmitted in advance of actual coordination with a D-ATSU.

#### The coordination phase begins when the flight is a parameter time or distance from a common boundary between FIRs controlled by two or more ATSUs.  A flight does not actually have to enter an FIR for coordination to be required: it need only pass within the lateral separation minima associated with an FIR.  The cleared flight trajectory and boundary crossing conditions (if the flight actually crosses the boundary) are negotiated between the ATSUs using an initial Coordination dialogue until mutually agreed upon conditions are obtained.  A re-Coordination dialogue(s) provides a mechanism for modifying an existing coordination, and may be invoked while the aircraft remains in the border areas of two or more FIRs.

#### Transfers of communications and control authority normally occur when the flight is about to enter a new FIR.  However, these may occur at any point subsequent to initial coordination subject to approval of both ATSUs.  Transfer dialogues are employed to perform these functions.

#### A unique subset of AIDC messages is associated with each phase of coordination.

#### Several messages are included within the AIDC message set to support simultaneous coordination between three or more ATSUs

#### Other AIDC messages support ancillary ATC data exchanges between ATSUs, including:

- Transfer of surveillance data, including ADS and radar reports; and

- Exchange of free-text messages.

## AIDC Application Features

### The AIDC application will use the ATN to ensure that ATC data are exchanged in a reliable and timely manner between ATSUs.

### The AIDC service itself also provides several features that support reliable and timely data exchange:

- Application Response:  An ATSU receiving a message must acknowledge receipt of that message to the ATSU which transmitted it.  This acknowledgment indicates whether the message was accepted for further processing (the AppAccept message), or the message was rejected due to errors (the AppError message).  Such an acknowledgment is referred to as an Application Response, and is performed by the ATSU receiving the message.  Non-receipt of the Application Response within the required time will result in exception processing.

- Operational Response:  Many AIDC messages require an operational response.  For example, a CoordinateInitial message, which describes the aircraft's current cleared flight trajectory and boundary crossing information, requires either a CoordinateAccept ,acceptance of the trajectory as proposed, or a CoordinateNegotiate ,negotiation of the coordination conditions, response.  Such a response is referred to as an Operational Response, and may be generated by the ATC automation system or via human interaction.  Non-receipt of an Operational Response within the required time will result in exception processing.

- Application Status Monitoring:  A warning is issued to an ATSU if the ATN detects that a existing AIDC link with another ATSU has failed.

## Regional Adaptation

### Regional adaptation of the AIDC application may be accomplished by mutual agreement.

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

CHAPTER 1 APPENDIX A

TRANSITION

1A.1	This material is provided as guidance to assist in the transition from character-based, inter-facility data interchange to bit-oriented AIDC message exchange

1A.2	The relationship between the AIDC message set and existing messages currently in use within different ATS regions is expressed in the following table.

| AIDC Message | Related Existing ATS Messages |
| --- | --- |
| NotifyNotify | ABI (Europe, NAT & A/P) |
| CoordinateInitial | CPL (ICAO, NAT & A/P) EST (ICAO & A/P) PAC, ACT & RAP (Europe) |
| CoordinateNegotiate | CDN (ICAO, NAT, Europe & A/P) |
| CoordinateAccept | ACP (ICAO, NAT, Europe & A/P) LAM (Europe) |
| CoordinateReject | REJ (NAT & A/P) RJC (Europe) |
| CoordinateCancel | MAC (Europe, NAT & A/P) |
| CoordinateReady | RDY (NAT) |
| CoordinateCommit | CMT (NAT) |
| CoordinateRollback | ROL (NAT) |
| CoordinateUpdate | CUR (NAT) unsolicited CDN, REV & RRV (Europe) |
| CoordinateStandby | SBY (Europe & NAT) |
| TransferInitiate | TIM (Europe) |
| TransferConditionsProposal | HOP (Europe) |
| TransferConditionsAccept | ROF (Europe) |
| TransferRequest | ROF (Europe) |
| TransferControl | TOC (NAT & A/P) |
| TransferControlAssume | AOC (NAT & A/P) |
| TransferComm | COF (Europe) |
| TransferCommAssume | MAS (Europe) |
| GeneralPoint | point INF (Europe) |
| GeneralExecData | SDM (Europe) |
| FreetextEmergency | EMG (NAT & A/P) |
| FreetextGeneral | MIS (NAT & A/P) |
| AppStatus | ASM (NAT) |
| AppAccept | LAM (ICAO, NAT, Europe & A/P) |
| AppError | LRM (NAT & A/P) |

Table 1A-1:  Relationship Between AIDC Messages and Existing ATS Messages

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

# GENERAL REQUIREMENTS

## Message Handling

### Message Transmission

#### The AIDC application requires:

- Confirmation that a message has been received (by transmission of an AppAccept or AppError); and

- Messages to be delivered in the order they are sent.

### Message Urgency Attributes

#### When an ATSU queues received AIDC messages, messages with the highest Urgency type will be placed at the beginning of the queue.

#### When an ATSU queues received AIDC messages, messages with the same Urgency type will be queued in order of receipt.

### Message Urgency.  AIDC messages will be assigned one of the following urgency attributes:

- Normal;

- Urgent; or

- Distress.

#### Freetext messages have an urgency attribute of either Normal or Distress.  Surveillance data transfer messages have an urgency attribute of Urgent.  All other AIDC messages have an urgency attribute of Normal.

### AIDC Message Priority

#### The priority for all AIDC messages will be “normal priority flight safety messages” as determined by the ATN Internet Protocol Priority categorization.

### Time Requirements

#### Wherever time is used in the AIDC application, it will be accurate to within 1 second of UTC.

#### Time stamping will be mandatory for all messages.  The time stamp will consist of the date (YYMMDD) and time (HHMMSS).  The time stamp will be the time the message is dispatched by the originating user.

### Message Identification

#### Every AIDC message will contain a unique ID, which may be used to reference the message.

#### An AIDC message may also contain a reference to a previously sent AIDC message.  This reference shall contain the ID of the referenced message, along with an identification of the source of the referenced message. (i.e., a unique identifier for the ATS Unit which transmitted the referenced message).

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

# AIDC FUNCTIONAL CAPABILITIES

## Dialogues

### A dialogue is a sequence of one or more related message exchange sequences.  All exchanges of AIDC messages between ATSUs may be viewed as dialogues.  AIDC dialogues support the following functions:

- Notification;

- Coordination;

- Transfer;

- General Information Interchange; and

- Surveillance Data Transfer.

### Most dialogues consist of one message exchange sequence.  However, coordination and transfer dialogues involve multiple messages exchange sequences.  The structure and sequence of AIDC dialogues is discussed further in Chapter 5 of this part.

## Operating method

### A dialogue is typically initiated when one of several events, such as the following, occurs:

- proximity to an airspace boundary;

- controller action;

- a change in a flight's cleared trajectory affecting a D-ATSU; or

- surveillance data update.

### Confirmation that an ATSU has received a message is accomplished by that ATSU transmitting an AppAccept or AppError response to any received message, other than an AppAccept or AppError.

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

# MESSAGE DESCRIPTIONS

## Introduction

### The tables below describe in general terms the contents of each AIDC message.

| Message | Purpose | Message Contents |
| --- | --- | --- |
| NotifyNotify | Updates the information a D-ATSU maintains on a flight that is expected to enter its area of interest at some future time. | Aircraft identification Flight rules Type of flight Aircraft model Wake turbulence category CNS equipment, including data link equipage Boundary estimate data (boundary fix, crossing time, crossing altitude) Route Other information |

Table 4-1:  AIDC Notification Message Contents

| Message | Purpose | Message Contents |
| --- | --- | --- |
| CoordinateInitial | Begins an initial coordination dialogue between ATSUs. | Aircraft identification Flight rules Type of flight Aircraft model Wake turbulence category CNS equipment, including data link equipage Boundary estimate data (boundary fix, crossing time, crossing altitude) Route Other information |
| CoordinateNegotiate | Used to negotiate the coordination conditions. | Aircraft identification Boundary estimate data (boundary fix, crossing time, crossing altitude), and/or Route |
| CoordinateAccept | Signifies acceptance of the proposed coordination conditions. | Aircraft identification Frequency |
| CoordinateCancel | Notifies a D-ATSU that a flight previously expected to enter its area of interest will no longer do so. | Aircraft identification Boundary estimate data (boundary fix, crossing time, crossing altitude) Other information |
| CoordinateReject | Immediately terminates a coordination dialogue. Any previous coordination conditions shall remain as agreed. | Aircraft identification |
| CoordinateReady | Signals that an ATSU is ready to update its flight database with the agreed upon coordination conditions. | Aircraft identification |
| CoordinateCommit | Causes an ATSU to update its flight database with the agreed upon coordination conditions. | Aircraft identification |
| CoordinateUpdate | Used to initiate the re-negotiation of coordination conditions, after the initial coordination dialogue has been completed. | Aircraft identification Boundary estimate data (boundary fix, crossing time, crossing altitude), and/or Route |
| CoordinateRollback | Causes an ATSU to revert to the previously agreed upon coordination conditions for a flight. | Aircraft identification |
| CoordinateStandby | Extends the coordination time-out values. This is typically needed when a controller or another ATSU must be consulted before responding to a coordination message. | Aircraft identification |

Table 4-2:  AIDC Coordination Message Contents

### CoordinateReady, CoordinateCommit; and CoordinateRollback messages are to be used exclusively with the multi-coordination procedure.  Note that other coordination messages are also employed by this procedure.

| Message | Purpose | Message Contents |
| --- | --- | --- |
| Transfer Initiate | Initiate a transfer of control information and track data. | Aircraft identification Control data, including clearance restrictions Track data (position, time, altitude) |
| Transfer Conditions Proposal | Offer control conditions to an adjacent ATSU. | Aircraft identification Control data, including clearance restrictions |
| Transfer Conditions Accept | Indicates willingness to accept proposed control conditions. | Aircraft identification Frequency |
| Transfer Request | Request transfer of control and communication responsibility. | Aircraft identification Frequency |
| Transfer Control | Indicates that the C-ATSU wishes to relinquish control responsibility. | Aircraft identification Executive data |
| TransferControlAssume | Accept control authority for a flight | Aircraft identification |
| TransferControlReject | Refusal to accept control authority for a flight | Aircraft identification |
| TransferComm | Indicates that C-ATSU is relinquishing communications with a flight | Aircraft identification Executive data Release indicator |
| TransferCommAssume | Indicates that communications with a flight have been established | Aircraft identification |

Table 4-3:  AIDC Transfer Of Control Message Contents

| Message | Purpose | Message Contents |
| --- | --- | --- |
| SurvADS | Transfer ADS data to an adjacent ATS unit. | Aircraft identification ADS data (any of the ADS downlink report blocks) |
| SurvGeneral | Transfer surveillance data to an adjacent ATS unit. | Aircraft identification Track data (position, time, altitude and, if known, ground speed and true track angle) |

Table 4-4:  AIDC Surveillance Message Contents

| Message | Purpose | Message Contents |
| --- | --- | --- |
| GeneralPoint | Point out a flight to an adjacent ATS unit. | Functional Address Aircraft identification Flight rules Type of flight Aircraft model Wake turbulence category CNS equipment, including data link equipage Boundary estimate data (boundary fix, crossing time, crossing level) Route Other information |
| GeneralExecData | Transfer control information to an adjacent ATSU | Aircraft identification Control data, including clearance restrictions Frequency |
| FreetextEmergency | Support free text information exchange in an emergency condition. | Aircraft identification or functional address Free text |
| FreetextGeneral | Support free text information exchange in a non-emergency condition. | Aircraft identification or functional address Free text |

Table 4-5:  AIDC General Information Message Contents

| Message | Purpose | Message Contents |
| --- | --- | --- |
| AppStatus | Determine if the ATC application residing within an adjacent ATSU is active. | Nil |
| AppVersion | Inform another ATSU of the AIDC Service version number being employed. | AIDC version number, |
| AppAccept | Acknowledge acceptance of a received message. | Nil |
| AppError | Signal that a received message contained an error. | Message type Component type Error code Error data |

Table 4-6:  AIDC Application Management Message Contents

PART VI - AIR TRAFFIC SERVICES INTERFACILITY DATA COMMUNICATION

# OPERATIONAL MESSAGE SEQUENCES

## AIDC State Transitions

### There are three principal states associated with a flight corresponding to the phases of flight.  A fourth state, Pre-Notify, represents the initial Coordinate state.  Note that a flight does not, from the AIDC service perspective, have to enter in the Notify state; it may go directly to the Coordinate state without any Notification messages being sent.  See Figure 5-1.

Figure 5-1:  AIDC State Transition Diagram

### Two of the states - Coordinate and Transfer - are further decomposed in Figures 5-2 and 5-3, respectively.  The transfer state is the most complicated of the AIDC states.  It includes an optional backward coordination capability.  This capability is only employed when a flight is in an FIR border region.  See Figure 5-4 for further comments.

Figure 5-2: Coordinate State

Figure 5-3: Transfer State

Figure 5-4:  Transfer Boundary Conditions

## Dialogue Sequences

### The exchange of AIDC messages for each flight is centered around the concept of dialogues, as mentioned in Section 3.1.1.  There is a sequencing order imposed upon dialogues themselves, and upon the messages contained within a dialogue, as described below.

| Dialogue | Sequencing Rules |
| --- | --- |
| Notification | May only occur prior to the initial coordination dialogue. |
| Notification Cancellation | May only occur during the Notification phase of coordination. |
| Initial Coordination | This is the key dialogue which must be successfully completed for each flight. |
| Re-Coordination | May occur any time after the initial coordination dialogue has been successfully completed. |
| Coordination Cancellation | May occur after the initial coordination dialogue has been completed and before the start of a transfer dialogue. |
| Transfer of Executive Conditions | May occur any time after the initial coordination dialogue has been successfully completed. |
| Transfer of Control | May occur any time after the initial coordination dialogue has been successfully completed. |
| Transfer of Communications | May occur any time after the initial coordination dialogue has been successfully completed. |
| Free Text | May occur at any time, but typically after the initial coordination dialogue. |
| Surveillance Data Transfer | May occur at any time, but typically after the initial coordination dialogue. |
| Point-Out | May occur at any time. |
| Executive Information Transfer | May occur at any time after initial coordination is completed. |

Table 5-1:  AIDC Dialogue Sequence.

### Several dialogues identified above consist of multiple AIDC message exchanges (excluding AppAccept and AppError).  The tables below identify the message sequences within these dialogues.  The remaining dialogues consist of a single message exchange sequence only.

| Message | Comments |
| --- | --- |
| CoordinateInitial | Begins the initial coordination of a flight. |
| CoordinateNegotiate | Supports negotiation of coordination conditions between ATSUs. May be exchanged zero or more times. |
| CoordinateAccept | Closes the initial coordination dialogue. All initial coordination dialogues must be closed using this message, indicating that coordination conditions have been mutually agreed upon by the ATSUs involved. |

Table 5-2:  Initial Coordination Dialogue Sequence.

| Message | Comments |
| --- | --- |
| CoordinateUpdate | Begins the re-coordination of a flight. |
| CoordinateNegotiate | Supports negotiation of coordination conditions between ATSUs. May be exchanged zero or more times. |
| either CoordinateAccept or CoordinateReject | Closes the re-coordination dialogue. All re-coordination dialogues must be closed using one of these messages. A CoordinateAccept indicates that coordination conditions have been mutually agreed upon by the ATSUs involved, while a CoordinateReject indicates that the previously agreed upon coordination conditions remain in effect. |

Table 5-3:  Re-Coordination Dialogue Sequence.

| Message | Comments |
| --- | --- |
| TransferInitiate | Initiate control information exchange. |
| TransferConditionsProposal | Proposes executive control conditions (assigned heading, assigned speed, etc.). |
| TransferConditionsAccept | Accept proposed executive control conditions and close dialogue. |

Table 5-4:  Transfer of Executive Conditions Dialogue Sequence.

| Message | Comments |
| --- | --- |
| TransferControl | Initiates a dialogue to transfer Executive Control Authority from C-ATSU to R-ATSU. |
| (either) TransferControlAssume (or) TransferControlReject | Closes the Transfer of Control dialogue. All Transfer of Control dialogues must be closed using one of these messages. A TransferControlAssume indicates that the sending ATSU accepts executive control authority for the flight. A TransferControlReject indicates that the sending ATSU is refusing to accept executive control authority for the flight. |

Table 5-5:  Transfer of Control Dialogue Sequence.

| Message | Comments |
| --- | --- |
| TransferComm | Initiates the transfer of communications. |
| TransferCommAssume | Closes the transfer of communications dialogue and terminates the transfer phase. |

Table 5-6:  Transfer of Communications Dialogue Sequence.

### Every message has an AIDC dialogue associated with it.  For example, the CoordinateInitial message has a corresponding Initial Coordination dialogue.  In addition, there is a Cancel Notification dialogue which is equivalent to the Cancel Coordination (which employs the CoordinateCancel message) dialogue.

## Message Event Triggers and Valid Responses

### The tables below provide additional information on when a message is transmitted, the message source ATSU and message destination ATSU, and the valid response to a received message.

| AIDC Message | Event Trigger | Source/Destination | Valid Response |
| --- | --- | --- | --- |
| NotifyNotify | Defined time from ATS boundary, or change in flight trajectory. | C-ATSU/ ATSU | None required |

Table 5-7:  AIDC Notification Messages

| Received Message | Event Trigger | Source/ Destination | Valid Response |
| --- | --- | --- | --- |
| CoordinateInitial | Proximity to an ATS boundary. | C-ATSU/ D-ATSU | CoordinateAccept CoordinateNegotiate |
| CoordinateNegotiate | Proposed coordination conditions must be changed for some reason. | ATSU1/ ATSU2 | CoordinateNegotiate CoordinateAccept CoordinateReject |
| CoordinateAccept | Coordination conditions are acceptable. | ATSU1/ ATSU2 | None required |
| CoordinateCancel | Change in flight trajectory causes a flight not to enter or fly near an ATS boundary. | C-ATSU/ D-ATSU | None required |
| CoordinateReject | Proposed coordination conditions are unacceptable. | ATSU1/ ATSU2 | None required |
| CoordinateUpdate | Propose revised coordination conditions after termination of an initial coordination dialogue. | ATSU1/ ATSU2 | CoordinateNegotiate CoordinateAccept CoordinateReject |
| CoordinateReady | D-ATSU is ready to accept the proposed coordination conditions. | D-ATSU/ C-ATSU | CoordinateCommit CoordinateRollback |
| CoordinateCommit | C-ATSU has agreement from all affected D-ATSUs on the coordination conditions. | C-ATSU/ D-ATSU | CoordinateAccept |
| Coordinate Rollback | Existing coordination conditions must be changed. | C-ATSU/ D-ATSU | None required |
| Coordinate Standby | Coordination with one or more down-stream ATSUs, or referral to the controller, must be completed before continuing the current co-ordination dialogue. | ATSU1/ ATSU2 | None required |

Table 5-8:  AIDC Coordination Messages

| Received Message | Event Trigger | Source/ Destination | Valid Response |
| --- | --- | --- | --- |
| TransferInitiate | Flight is near an airspace boundary. | C-ATSU/ R-ATSU | none required |
| TransferConditions-Proposal | Manual hand-off initiation by C-ATSU. | C-ATSU/ R-ATSU | none required TransferAccept |
| TransferConditions-Accept | Manual acceptance of proposed transfer conditions by R-ATSU. | R-ATSU/ C-ATSU | none required |
| TransferRequest | Manual request for transfer by R-ATSU | R-ATSU/ C-ATSU | none required |
| TransferControl | Manual or automatic relinquishing of control at C-ATSU. | C-ATSU/ R-ATSU | TransferControl-Assume TransferControlReject |
| TransferControl-Assume | Manual or automatic assumption of control at R-ATSU. | R-ATSU/ C-ATSU | none required |
| TransferControlReject | Manual rejection of control at R-ATSU. | R-ATSU/ C-ATSU | none required |
| TransferComm | Manual relinquishing of communications at C-ATSU. | C-ATSU/ R-ATSU | none required TransferComm-Assume |
| TransferComm-Assume | Aircraft in communications with R-ATSU. | R-ATSU/ C-ATSU | none required |

Table 5-9:  AIDC Transfer Of Control Messages

| Received Message | Event Trigger | Source/ Destination | Valid Response |
| --- | --- | --- | --- |
| SurvADS | ADS Report received. | ATSU1/ ATSU2 | None required |
| SurvGeneral | Track update calculated. | ATSU1/ ATSU2 | None required |

Table 5-10:  AIDC Surveillance Messages

| Received Message | Event Trigger | Source/ Destination | Valid Response |
| --- | --- | --- | --- |
| GeneralPoint | Controller-triggered to some adjacent ATS Unit. | ATSU1/ ATSU2 | None required |
| GeneralExecData | A change in executive control information. | ATSU1/ ATSU2 | None required |
| FreeTextEmergency | Emergency condition. | ATSU1/ ATSU2 | None required |
| FreeTextGeneral | Controller action. | ATSU1/ ATSU2 | None required |

Table 5-11:  AIDC General Information Messages

| Received Message | Event Trigger | Source/ Destination | Valid Response |
| --- | --- | --- | --- |
| AppStatus | No message has been received within a VSP time period from an adjacent ATSU. | ATSU1/ ATSU2 | None required |
| AppVersion | As determined by ATSU1. | ATSU1/ ATSU2 | None required |
| AppAccept | Receipt of valid message by the ATC application at ATSU1. | ATSU1/ ATSU2 | None required |
| AppError | Receipt of an invalid message by the ATC application at ATSU1. | ATSU1/ ATSU2 | None required |

Table 5-12:  AIDC Application Management Messages

-- END --