WG2 WP276

Flimsy 6-6, Rev C

Bruxelles

WG3 Matters of Interest to WG2

1.   WG2 Action 7/44,   CP for the Security Label.

1.1  5.1.2   Internet Service Description

Note 1 -

a) stet

b) stet

c) the required residual error rate (RER) to determine whether or not the transport checksum is required non-use of the transport checksum is allowed.

Flimsy Note -- CNS/ATM-1 air-ground applications require additional lower-layer integrity measures.  Ground-ground applications may achieve low RER with non-use of transport checksum.  Among CNS/ATM-1 applications, all except MHS specify low “RER”.

1.2  6.2.2  The Security Function

Flimsy Note -- Further work with WG2 has indicated that the Security Label, particularly the Security Registration ID, is used in Sub-Volume 5, 6.2.2.1, and ISO/IEC 10747, 6.3.1.14.  The Security Label DR is withdrawn.

2.  WG2 Action 7/45,   DR/CP for ATSC Class Table 2-2.

WG3 forwards WG3 Flimsy 6-8 (WG2 WP282).  This contains complete agreed text on the ATSC Class issue.

3.  Default Traffic Type

If ULA is not passed a traffic type (null parameter value), ULA inserts a ‘No Traffic Type’ Security Tag.  WG3 seeks assurance that any Operational Communications - ATSC path is selectable by the router.

Flimsy Note -- WG2 confirms that Sub-Volume 5, 3.2.1.2.1.2 correctly addresses this case.

4.  Integrity Architecture

Sub-Volume 5, 5.1.2 specifies TS-User selection of RER to trigger DT TPDU checksum on/off.  However, Sub-Volume 5 5.2.4.1.9.6 makes non-use of checksum possible.  A situation exists where ATN TP4 implementations may conformantly negotiate non-use of checksum.  Guidance may be useful to indicate that all CNS/ATM-1 air-ground applications require low RER and hence require transport checksum.  Clearly, ground-ground applications over a reliable subnetwork may request non-use of transport checksum.

Flimsy Note -- WG2 confirms that an air-ground application can always require transport checksum from a conformant implementation.

5.  Editorials

5.1  WG3 has adopted the WG2 APRL conventions.  WG3 clarifies that ISO 9646-7, 9.1.2 allows M/m in PRLs.  All PRL entries will be in capital letters.

5.2.  In Sub-Volume 5, version 4.0, 5.1.2 d) Table 2.2 should read 2.3.

Flimsy Note -- WG2 confirm this is corrected in v4.1.

5.3 In Sub-Volume 5, 2.2.2.3, the Note and Recommendation seem to conflict.

Flimsy Note -- WG2 agrees and took a DR to clarify.  To summarize:  you can have more than one BIS on an aircraft.  The Note says each BIS should be in a separate RD (since there is no intradomain forwarding specified on the aircraft).  The Recommendation then says that any other non-BIS IS must  be in the same RD as one of the BISs in the Note.

6.  Connectionless Communication

Sub-Volume 5 offers the connectionless transport protocol.  There is no connectionless ULA in CNS/ATM-1, so that applications cannot use the connectionless transport protocol.  WG3 notes that a connectionless ULA has been drafted and presented at this WG3 meeting.

Flimsy Note --  WG2 invites such a contribution and notes that CLTP is optional.

7.  Efficient TP4

WG3 experts are aware of ITU-T SG7 work on efficient (‘fast-byte’) transport protocol proposals.  If of interest to WG2, these matters can be pursued, and an IP offered for Munich.

Flimsy Note -- WG2 invites such a contribution.

8. CNS/ATM-2 Planning

WG3 has been producing material for CNS/ATM-2 planning.  WG3 seeks information on such material from WG2 for ATNP/2.

Flimsy Note -- Mr. Cossa is leading such an action for WG2.

9.  (WG1)  WWP Review

WG1 seeks contributions for the WWP document, especially on user requirements for address registration authorities.

Flimsy Note -- Mr. Graf has accepted the action.12.  (WG1)  Sub-Volume 1

WG1 is pleased to announce the availability of Sub-Volume, System Requirements, version  0.2.  Comments and proposed text are solicited from WG2.  Comments should be submitted by 20 May 1996 for the Munich Sub-Volume 1 publication.