WG3 Flimsy 6-13
April 26, 1996

# CPDLC Expression of VHF Frequency Names

# Introduction

Although expression of VHF Frequency in draft Version 3.0 of the CPDLC SARPS (Sub-Volume 2, Part 3) recognizes the new ICAO standards of 8.33 KHz channel spacing, it unfortunately does not fully account for the more complex way in which ”channel names” are expressed as “frequencies” by ICAO.

# Description

Introduction of 8.33 channels has introduced the concept of “wide band” and ”narrow band” channels.  That is, a frequency of 123.45 KHz can be transmitted either as a traditional “wide band” signal, named 123.450, or a “narrow band” signal, arbitrarily named 123.451.

| Frequency | Wide Band Name | Narrow Band Name |
| --- | --- | --- |
| 1XX.X0000 | 1XX.X00 | 1XX.X01 |
| 1XX.X0833 | NA | 1XX.X08 |
| 1XX.X1667 | NA | 1XX.X17 |
| 1XX.X2500 | 1XX.X20 | 1XX.X25 |
| 1XX.X3333 | NA | 1XX.X33 |
| 1XX.X4167 | NA | 1XX.X42 |
| 1XX.X5000 | 1XX.X50 | 1XX.X51 |
| 1XX.X5833 | NA | 1XX.X58 |
| 1XX.X6667 | NA | 1XX.X67 |
| 1XX.X7500 | 1XX.X70 | 1XX.X75 |
| 1XX.X6333 | NA | 1XX.X63 |
| 1XX.X9167 | NA | 1XX.X92 |

Note: 	These channel names are subject to verification.  I think I remember them correctly.  There has been some controversy about the naming of the wide band .025 and .075 frequencies, which has mostly died down.

Therefore, there are 16 separate “channels” for every 100 KHz in the band from 118.000 MHz to 137.000 MHz (137.000 is not a valid frequency), for a total of 3040 channels.  In my opinion, these are best expressed as two separate channel bands, containing 760 wide band channels and 2280 narrow band channels.

# Recommendation

I recommend that the wide band channels be assigned the same numbers found in RTCA 
DO-219 and that the narrow band channels be assign a separate, not necessarily contiguous, block of numbers but all part of the variable_name FrequencyVHF.