# THE AIDC-ASE PROTOCOL DEFINITION

- ¾ This Chapter specifies the AIDC-ASE protocol.

## AIDC-ASE Protocol Description

### Only if requirements are described for an AIDC-ASE primitive when an AIDC-ASE is in a particular state, shall the invocation of that primitive be permitted while the AIDC-ASE is in that state.

### If no requirements are described for the arrival of a PDU when the AIDC-ASE is in a particular state, then exception handling procedures as described in section 6.2 shall apply.

### The CF shall ensure that the sequencing rules for the AIDC-ASE service primitives is maintained.

- ¾ This requirements eliminates the need for the protocol to know which user is the C-ATSU AIDC-User and which is the D-ATSU AIDC-User.

### AIDC-nfy Request Primitive

- ¾ The transition from the IDLE state to the NOTIFY state occurs with the receipt of an AIDC-ucf indication from the remote AIDC-User, with the Reference ID parameter equal to the value stored in vs2.

#### When Invoked

##### It shall be valid to invoke the AIDC-nfy request primitive when the AIDC-ASE protocol machine is in the NULL state, or the NOTIFY state.

#### Action Upon Invocation

##### Upon the receipt of an AIDC-nfy request primitive the AIDC-ASE shall:

###### If in either the IDLE state or the NOTIFY state:

- create an AIDC-nfy-apdu based on the Msg Number parameter, and the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-nfy-apdu as the User Data parameter;

- start the timer t1C; and

- set the variable vs1 = notify and vs2 = Reference ID.

- ¾ The variable vs2  is used to determine when the response from the remote AIDC-User arrives.  It is the responsibility of the AIDC-User to properly manage the Reference ID values to ensure proper operation of the confirmation.

###### If the AIDC-ASE protocol machine is any other state, the AIDC-ASE shall:

- invoke an AIDC-pvd-abrt indication primitive with the Reason parameter set to the abstract value: ProtocolError;

- invoke an AIDC-ABORT request primitive; and

- enter the NULL state.

### AIDC-DATA Indication Primitive with an AIDC-nfy-Req-PDU

- ¾ The AIDC-nfy-Req-PDU can only be received by the D-ATSU AIDC-User.

- ¾ The AIDC-nfy-Req-PDU is only valid if the ASE protocol machine is in either the IDLE state or the NOTIFY state.

- ¾ The transition from the IDLE state to the NOTIFY state occurs with the receipt of an AIDC-ucf indication from the local AIDC-User with the Reference ID parameter equal to the value stored in vr2.

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-nfy-Req-PDU the AIDC-ASE shall:

##### If in the IDLE state or the NOTIFY state:

- invoke an AIDC-nfy indication primitive with the User Data parameter as the AIDC User Data parameter; and

- set the variable vr1 = notify and vr2 = Msg Number.

### AIDC-crd-start Request Primitive

- ¾ The AIDC-crd-start-request is only valid if the ASE protocol machine is in the IDLE state, or the NOTIFY state, or the TRANSFERRED state.

- ¾ The transition from the IDLE state, or the NOTIFY state, or the TRANSFERRED state occurs with the receipt of an AIDC-ucf indication from the local AIDC-User with the Reference ID parameter equal to the value stored in vr2.

#### Upon the receipt of an AIDC-crd-start request primitive the AIDC-ASE shall:

##### If in the IDLE state, NOTIFY state or the TRANSFERRED state:

- create an AIDC-crd-start-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-crd-start-Req-PDU as the User Data parameter;

- start the timer t1C; and

- Set the variable vs1 = coord-start and vs2 = Msg Number.

- ¾ The enforcement of the sequencing of service primitives ensures that the primitive is received after all required logical user confirmations have been received.

- ¾ Since no logical user confirmations relating to primitives other than asynchronous primitives are outstanding, the value of vs1  would be NULL and can be used for verification of proper service interactions.

### AIDC-DATA Indication Primitive with an AIDC-crd-start-Req-PDU

- ¾ The AIDC-crd-start-Req-PDU can only be received by the D-ATSU AIDC-User.

- ¾ The AIDC-crd-start-Req-PDU is only valid if the ASE protocol machine is in the IDLE state, or the NOTIFY state, or the TRANSFERRED state.

- ¾ The transition from the IDLE state, or the NOTIFY state, or the TRANSFERRED state occurs with the receipt of an AIDC-ucf indication from the local AIDC-User with the Reference ID parameter equal to the value stored in vr2.

#### Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-start-Req-PDU the AIDC ASE shall:

##### If in the IDLE state, the NOTIFY state, or the TRANSFERRED state:

- Invoke an AIDC-crd-start indication primitive with the User Data parameter as the AIDC User Data parameter; and

- Set vr1 = coord-start and vr2 = Msg Number.

### AIDC-crd-end Request Primitive

- ¾ The AIDC-crd-end request can be received from either the C-ATSU AIDC-User or the D-ATSU AIDC-User.

- ¾ The AIDC-crd-end-request is only valid if the ASE protocol machine is in the NEGOTIATING state.

- ¾ The transition from the NEGOTIATING state occurs with the receipt of an AIDC-ucf indication from the local AIDC-User with the Reference ID parameter equal to the value stored in vr2.

#### Upon the receipt of an AIDC-crd-end request primitive the AIDC-ASE shall:

##### If in the NEGOTIATING state:

- create an AIDC-crd-end-Req-PDU based on the Result parameter and the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-crd-end-Req-PDU as the User Data parameter;

- start the timer t1C; and

- if the Result parameter has the abstract value of:

  - accept; set variable vs1 = coord-end and vs2 = Msg Number.

  - reject; set variable vs1 = negot and vs2 = reference number.

### AIDC-DATA Indication Primitive with an AIDC-crd-end-Req-PDU

#### Upon the receipt of a AIDC-DATA indication primitive with an AIDC-crd-end-Req-PDU, the AIDC ASE shall:

##### If in the NEGOTIATING state:

- invoke an AIDC-crd-end indication primitive with the Result parameter and AIDC User Data parameter created from the User Data parameter; and

- if the Result parameter has the abstract value of:

  - accept; set the variable vr1 = coord-end and vr2 = Msg Number.

  - reject; set the variable vr1 = negot and vr2 = Msg Number.

### AIDC-crd-ngtt Request Primitive

#### Upon the receipt of an AIDC-crd-ngtt request primitive, the AIDC-ASE shall:

##### If in the NEGOTIATING, or BACKWARD-COORDINATING state:

- create an AIDC-crd-ngtt-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-crd-ngtt-Req-PDU as the User Data parameter;

- start the timer t1C; and

### AIDC-DATA Indication Primitive with an AIDC-crd-ngtt-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-ngtt-Req-PDU the AIDC-ASE shall:

##### If in the NEGOTIATING or BACKWARD-COORDINATING state, invoke an AIDC-crd-ngtt indication primitive with the AIDC User Data parameter as the Coordinate Negotiate Data parameter.

### AIDC-crd-stndby Request Primitive

#### Upon the receipt of an AIDC-crd-stndby request primitive the AIDC-ASE shall:

##### If in the NEGOTIATING or BACKWARD-COORDINATING state:

- create an AIDC-crd-stndby-Req-PDU based on the Coordinate Standby Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-crd-stndby-Req-PDU as the User Data parameter;

- set the standby variable; and

- start the timer t1C.

### AIDC-DATA Indication Primitive with an AIDC-crd-stndby-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-crd-stndby-Req-PDU the AIDC-ASE shall:

##### If in the NEGOTIATING or BACKWARD-COORDINATING state:

- Invoke an AIDC-crd-stndby indication primitive with the User Data parameter as the Coordinate Standby Data parameter; and

- set the standby variable.

### AIDC-tfr-init Request Primitive

#### Upon the receipt of an AIDC-tfr-init request primitive the AIDC-ASE shall:

##### If in the COORDINATED state:

- create an AIDC-tfr-init-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-init-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set variable vs1 = transfer-init and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-init-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-init-Req-PDU the AIDC-ASE shall:

##### If in the COORDINATED state:

- Invoke an AIDC-tfr-init indication primitive with the User Data parameter as the AIDC User Data parameter; and

- set variable vr1 = transfer-init and vr2 = reference number.

### AIDC-tfr-rqst Request Primitive

#### Upon the receipt of an AIDC-tfr-rqst request primitive the AIDC-ASE shall:

##### If in the COORDINATED state, the AIDC-ASE shall:

- create an AIDC-tfr-rqst-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-rqst-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set variable vs1 = transfer-init  and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-rqst-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-rqst-Req-PDU the AIDC-ASE shall:

##### If in the COORDINATED state:

- Invoke an AIDC-tfr-rqst indication primitive with the User Data parameter as the AIDC User Data parameter; and

- set variable vr1 = transfer-init and vr2 = Msg Number.

### AIDC-tfr-prpsl Request Primitive

#### Upon the receipt of an AIDC-tfr-prpsl request primitive the AIDC-ASE shall:

##### If in the COORDINATED state:

- create an AIDC-tfr-prpsl-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-prpsl-Req-PDU as the User Data parameter; and

- start the timer t1C.

### AIDC-DATA Indication Primitive with an AIDC-tfr-prpsl-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-prpsl-Req-PDU the AIDC-ASE shall:

##### If in the COORDINATED state, invoke an AIDC-tfr-prpsl indication primitive with the User Data parameter as the AIDC User Data parameter.

### AIDC-tfr-accept Request Primitive

#### Upon the receipt of an AIDC-tfr-accept request primitive the AIDC-ASE shall:

##### If in the PRE-TRANSFER state:

- create an AIDC-tfr-accept-Req-PDU based on the result parameter and the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-prpsl-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set vs1 = transfer-accept and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-prpsl-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-accept-Req-PDU the AIDC-ASE shall:

##### If in the COORDINATED state:

- Invoke an AIDC-tfr-accept indication primitive with the User Data parameter as the AIDC User Data parameter;

- if the result parameter is set to accept set vr1 = transfer-accept;

- if the result parameter is set to reject set vr1 = transfer-reject; and

- set  vr2 = Msg Number.

### AIDC-tfr-cntrl Request Primitive

#### Upon the receipt of an AIDC-tfr-cntrl request primitive the AIDC-ASE shall:

##### If in the COORDINATED state:

- create an AIDC-tfr-cntrl-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-cntrl-start-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set vs1 = transfer-pending and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-cntrl-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Req-PDU the AIDC-ASE shall:

##### If in the COORDINATED state:

- Invoke an AIDC-tfr-cntrl indication primitive with the AIDC User Data parameter created from the User Data parameter; and

- set vr1 = transfer-pending and vr2 = Msg Number.

### AIDC-tfr-cntrl Response Primitive

#### Upon the receipt of an AIDC-tfr-cntrl-response primitive the AIDC-ASE shall:

##### If in the TRANSFERRING state:

- create an AIDC-tfr-cntrl-response-PDU based on the Result and AIDC User Data parameters;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-cntrl-Response-PDU as the User Data parameter;

- start the timer t1C;

- if the Result parameter has the abstract value:

  - accepted; set vs1 = transfer-accept.

  - rejected; set vs1 = transfer-reject.

- set vs2 = Msg Number.

### AIDC-DATA Indication with an AIDC-tfr-cntrl-Response-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-Response-PDU the AIDC-ASE shall:

##### If in the TRANSFERRING state:

- Invoke an AIDC-tfr-cntrl-confirmation primitive with the Result parameter and the AIDC User Data parameter, created from the User Data parameter;

- if the Result parameter has the abstract value:

  - accepted; set vr1 = transfer-accept.

  - rejected; set vr1 = transfer-reject.

- set vr2 = Msg Number

### AIDC-tfr-comm Request Primitive

#### Upon the receipt of an AIDC-tfr-comm request primitive the AIDC-ASE shall:

##### If in the PRE-TRANSFER state:

- create an AIDC-tfr-comm-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-comm-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set vs1 = transfer-comm and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-comm-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-end-Req-PDU the AIDC-ASE shall:

##### If in the PRE-TRANSFER state:

- Invoke an AIDC-tfr-comm indication primitive with the AIDC User Data parameter created from the User Data; and

- set vr1 = transfer-comm and vr2 = Msg Number.

### AIDC-tfr-comm-assm Request Primitive

#### Upon the receipt of an AIDC-tfr-comm-assm request primitive the AIDC-ASE shall:

##### If in the PRE-TRANSFER state, or the TRANSFERRING state, or the TRANSFER CONDITIONS state:

- create an AIDC-tfr-comm-assm-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-tfr-comm-assm-Req-PDU as the User Data parameter;

- start the timer t1C;

- set the variable transfer; and

- set the variable vs1 = transfer-assume and vs2 = Msg Number.

### AIDC-DATA Indication Primitive with an AIDC-tfr-comm-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-tfr-cntrl-end-Req-PDU the AIDC-ASE shall:

##### If in the TRANSFERRING state, or the PRE-TRANSFER state, ore the TRANSFER CONDITIONS state:

- invoke an AIDC-tfr-comm indication primitive with the AIDC User Data parameter created from the User Data parameter;

- set vr1 = transfer-comm and vr2 = Msg Number.

### AIDC-info-trnsfr Request Primitive

#### Upon the receipt of an AIDC-info-trnsfr request primitive the AIDC-ASE shall:

##### If in the NOTIFY, NEGOTIATING, COORDINATED, PRE-TRANSFER, TRANSFERRING, TRANSFER CONDITIONS, or TRANSFERRED state:

- create an AIDC-info-trnsfr-Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-info-trnsfr-Req-PDU as the User Data parameter; and

- start the timer t1C.

### AIDC-DATA Request Primitive with an AIDC-info-trnsfr-Req-PDU

#### Upon the receipt of a AIDC-DATA request primitive with an AIDC-info-trnsfr-Req-PDU the AIDC-ASE shall:

##### If in the NOTIFY, NEGOTIATING, COORDINATED, PRE-TRANSFER, TRANSFERRING, TRANSFER CONDITIONS, or TRANSFERRED state, invoke an AIDC-info-trnsfr indication primitive with the User Data parameter as the AIDC User Data parameter.

### AIDC-ucf Request Primitive

#### Upon the receipt of an AIDC-ucf request primitive, the AIDC-ASE shall:

- create an AIDC-ucf-Req-PDU based on the Response parameter, the Reference ID parameter, and, if present, the Reason parameter;

- invoke a AIDC-DATA request primitive with the AIDC-ucf-Req-PDU as the User Data  parameter;

- if  vr2 = Reference ID:

  - if the variable vr1 = notify; in state IDLE or NOTIFY, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted; then enter the NOTIFY state and set vr1 = NULL and vr2 = NULL

  - if vr1 = coord-start; in state IDLE or NOTIFY, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the NEGOTIATING state and set vr1 = NULL and vr2 = NULL.

  - if vr1 = coord-end; in state NEGOTIATING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the COORDINATED state and set vr1 = NULL and vr2 = NULL.

  - if vr1 = transfer-init; in state COORDINATED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter PRE-TRANSFER state and set vr1 = NULL and vr2 = NULL.

  - if vr1 = transfer-start , in the state COORDINATED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRING

  - if vr1 = back; in the state TRANSFERRED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state BACKWARD COORDINATING

  - if vr1 = back-end, in the state BACKWARD COORDINATING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state as specified in variable transfer-state.

  - if vr1 = transfer-accept; in the state PRE-TRANSFER, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFER-CONDITIONS.

  - if vr1 = trnsf-accpt; in the state TRANSFERRING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRED.

  - if vr1 = trns-comm; in the state PRE-TRANSFER, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRING.

  - if vr1 = transfer-assume; in the state PRE-TRANSFER or the TRANSFERRING state, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRED.

  - if vr1 = end; and in any state other than IDLE and ENDING, then enter the state ENDING.

  - if vr1 = end; and in the state ENDING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state IDLE.

- if vr2 != Reference ID then remain in the current state.

### AIDC-DATA Indication Primitive with an AIDC-ucf-Req-PDU

#### Upon the receipt of an AIDC-DATA indication primitive with an AIDC-ucf-Req-PDU:

- create an AIDC-ucf-Req-indication based on the Response parameter, the Reference ID parameter, and, if present, the Reason parameter;

- If  vs2 = Reference ID:

  - if vs1 = notify; in state IDLE or NOTIFY, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the NOTIFY state and set vs1 = NULL and vs2 = NULL

  - if vs1 = coord-start; in state IDLE or NOTIFY, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the NEGOTIATING state and set vs1 = NULL and vs2 = NULL.

  - if vs1 = coord-end; in state NEGOTIATING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the COORDINATED state and set vs1 = NULL and vs2 = NULL.

  - if vs1 = transfer-init; in state COORDINATED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter PRE-TRANSFER state and set vs1 = NULL and vs2 = NULL.

  - if vs1 = transfer-start; in the state COORDINATED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRING

  - if vs1 = back; in the state TRANSFERRED, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state BACKWARD COORDINATING

  - if vs1 = back-end; in the state BACKWARD COORDINATING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state as specified in variable transfer-state.

  - if vs1 = transfer-accept; in the state PRE-TRANSFER, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFER-CONDITIONS.

  - if vs1 = trnsf-accpt; in the state TRANSFERRING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRED.

  - if vs1 = trns-comm; in the state PRE-TRANSFER, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRED.

  - if vs1 = transfer-assume; in the state PRE-TRANSFER, or the TRANSFERRING state, or TRANSFER-CONDITIONS state, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state TRANSFERRED.

  - if vs1 = end; and in any state other than IDLE and ENDING, then enter the state ENDING.

  - if vs1 = end; and in the state ENDING, and the Response parameter of the AIDC-ucf request primitive has the abstract value accepted then enter the state IDLE.

- if vs2 != Reference ID then remain in the current state.

### AIDC-end Request Primitive

#### Upon the receipt of an AIDC-end request primitive, the AIDC-AE shall:

##### If in any state other than, END PENDING, ENDING or IDLE:

- create an AIDC-end--Req-PDU based on the AIDC User Data parameter;

- invoke a AIDC-DATA request primitive with the AIDC-end-Req-PDU as the User Data parameter;

- start the timer t1C; and

- set vs1 = end and vs2 = Msg Number.

### AIDC-DATA Request with an AIDC-end-Req-PDU

#### Upon the receipt of a AIDC-DATA request primitive with an AIDC-end-Req-PDU, the AIDC-ASE shall:

##### If in any state other than IDLE, END PENDING or ENDING:

- Invoke an AIDC-end indication primitive with the User Data parameter as the AIDC User Data parameter;

- start the timer tend; and

- set vr1 = end and vr2 = Msg Number.

### AIDC-end Response Primitive

#### Upon the receipt of an AIDC-end response primitive, the AIDC-ASE shall:

##### If in the ENDING state:

- create an AIDC-end-Rsp-PDU based on the Result and Reason  parameters;

- invoke a AIDC-DATA request primitive with the AIDC-end-Rsp-PDU as the User Data parameter;

- stop the timer tend; and

- if the Result parameter has the abstract value:

  - accepted; enter the IDLE state; or

  - rejected; enter the previous state.

### AIDC-usr-abrt Request Primitive

#### Upon the receipt of an AIDC-usr-abrt request primitive the AIDC-ASE shall:

##### If in any state other than the IDLE state invoke a AIDC-ABORT request primitive.

### AIDC-ABORT Indication Primitive

#### Upon the receipt of a AIDC-ABORT indication primitive the AIDC-ASE shall:

##### If in any state other than the IDLE state:

- invoke a AIDC-usr-abrt indication primitive; and

- enter the IDLE state.

### AIDC-P-ABORT indication

#### Upon the receipt of an AIDC-P-ABORT indication primitive the AIDC-ASE shall:

##### If in any state other than the IDLE state:

- invoke an AIDC-pvd-abrt indication primitive with the Reason parameter set to the abstract value communications service failure; and

- enter the IDLE state.

## Exception Handling

### Timer Expiration

#### If a timer expires the AIDC-ASE shall:

- interrupt any current activity;

- invoke AIDC-ABORT request primitive; and

- invoke an AIDC-pvd-abrt indication primitive with the AIDC-pvd-abrt Reason parameter containing the value timer expired.

### Irrecoverable System Error

#### If an AIDC-ASE has an irrecoverable system error, the AIDC-ASE shall:

- interrupt an current activity;

- invoke AIDC-ABORT request primitive; and

- invoke an AIDC-pvd-abrt indication with the AIDC-pvd-abrt Reason parameter set to the abstract value undefined-error.

### Invalid PDU

#### If a PDU received is determined to be invalid, the AIDC-ASE shall:

- invoke an AIDC-ABORT request primitive; and

- invoke an AIDC-pvd-abrt indication primitive, Reason parameter set to the abstract value invalid PDU; and

- enter the IDLE state.

## AIDC Application Timers

- Table 6-1 lists the time constraints related to the AIDC application.  Each time constraint requires a time to be set in the AIDC protocol machine.

### An AIDC-ASE shall measure the time between the initial event and the corresponding final event.

### If the maximum time is exceeded before the final event has occurred, an AIDC-ASE shall take appropriate action as defined in section 6.2.1.

#### The action shall be taken when the maximum time as indicated in Table 6-1 has expired.

Table 6-1:  AIDC-ASE Timers

| AIDC Service | Timer | Timer Value | Timer Start Event | Timer Stop Event |
| --- | --- | --- | --- | --- |
| All services except abort and end services. | t1C | 2 min +/- 10% | Invocation of a AIDC-DATA request primitive with an AIDC APDU. | Reception of a AIDC-DATA indication primitive with an AIDC-ucf-Req-PDU. |
| AIDC-crd-start service, AIDC-tfr-cntrl service and AIDC-tfr-comm service with predicate TRUE. | t2R | 20 min +/- 10% | Reception of a AIDC-DATA indication with an AIDC-ucf-Req-PDU (Response = “accepted”), in response to one of the following primitives: - AIDC-crd-start request, - AIDC-tfr-cntrl request, or - AIDC-tfr-comm request. | Reception of an AIDC-ucf request primitive (Response = “accepted”) invoked in response to one of the following primitives: - AIDC-crd-end indication, - AIDC-crd-ngtt indication, - AIDC-crd-stndby indication, - AIDC-tfr-cntrl response, or - AIDC-tfr-comm-assm indication |
|  | t3M |  |  |  |
| AIDC-crd-start service | t4S | X min +/- 10% | Invocation of a AIDC-DATA request primitive with an AIDC-ucf-Req-PDU (Response = “accepted”) | Reception of an AIDC-ucf request primitive (Response = “accepted”) in response to one of the following primitives: - AIDC-crd-end indication, - AIDC-crd-ngtt indication, or - AIDC-crd-stndby indication |

## State Tables

### The AIDC-ASE shall behave in accordance with the following state tables, which shows diagrammatically the state transitions and actions performed by the AIDC-ASE in response to incoming events.  Incoming events are shown in the first column of the state table.  Each cell in the state table shows:

- optionally, one or more variables, denoted “rstl, vrN, or vsN”, where N is an integer.  The variables are defined as required and take on a value within the state tables.

- the new state that the AIDC-ASE shall enter after the action has been performed

- the action, if any, which the AIDC-ASE shall perform.

### Blank cells indicate error conditions. The error handling shall result in the association being aborted, if one exists, and a notification being given to the AIDC-User.

### In the event of a conflict between the actions implied by the state table and the text in the above sections, the text shall take precedence.

### Variables

rslt	-	Result of confirmed service

vr1	-	variable holding the last received event type for saving

vr2	-	variable holding the last received Msg Number for saving

vs1	-	variable holding the last sent event type for saving

vs2	-	variable holding the last sent Msg Number for saving

### Predicates

c1	-	Logical Confirmation result = Accept

c2 	-	vr2 = Reference ID

c3	-	vs2 = Reference ID parameter

c4	-	vr1 = “notify”

c5	-	vs1 = “notify”

c6 	-	vr1 = “coord-start”

c7	-	vs1 = “coord-start”

c8	-	vr1 = “coord-end”

c9	-	vs1 = “coord-end”

c10	-	vr1 = “trns-init”

c11	-	vs1 = 	“trns-init”

c12	-	vr1 = “trns-start”

c13	-	vs1 = “trns-start”

c14	-	vr1 = “trns-accept”

c15	-	vs1 = “trns-accept”

c16	-	vr1 = “trns-comm”

c17	-	vs1 = “trns-comm”

c18	-	vr1 = “trns-assum”

c19	-	vs1 = “trns-assum”

c20	-	vr1 = “back”

c21	-	vs1 = “back”

c22	-	vr1 = “back-end”

c23	-	vs1 = “back-end”

c24	-	vr1 = “end”

c25	-	vs1 = “end”

| StateÞ Eventß | IDLE | NOTIFY | NEGOTIATING | COORDINATED |
| --- | --- | --- | --- | --- |
| AIDC-nfy-REQ | vs1=“notify” vs2=Msg Number IDLE | vs1=“notify” vs2=Msg Number NOTIFY |  |  |
| rcv AIDC-NTF-REQ-PDU | vr1=“notify” vr2=Msg Number IDLE | vr1=“notify” vr2=Msg Number NOTIFY |  |  |
| AIDC-crd-start-req |  | vs1=“coord-start” vs2=Msg Number NOTIFY |  |  |
| rcv AIDC-crd-start-REQ pdu |  | vr1=“coord-start” vr2=Msg Number NOTIFY |  |  |
| AIDC-crd--end REQ |  |  | + vs1=“coord-end” vs2=Msg Number rslt=“accept” NEGOTIATING - vs1=“coord-end” vs2=Msg Number rslt=“reject” NEGOTIATING |  |
| rcv AIDC-crd-end-REQ pdu |  |  | + vs1=“coord-end” vs2=Msg Number rslt=“accept” NEGOTIATING - vs1=“coord-end” vs2=Msg Number rslt=“reject” NEGOTIATING |  |
| AIDC-crd-ngtt-REQ |  |  | NEGOTIATING |  |
| rcv AIDC-crd-ngtt-REQ pdu |  |  | NEGOTIATING |  |
| AIDC-crd-stnby REQ |  |  | NEGOTIATING |  |
| AIDC-crd-stnby REQ |  |  | NEGOTIATING |  |
| AIDC-tfr-init REQ |  |  |  | vs1=“trns-init” vs2=Reference ID COORDINATED |
| rcv AIDC-tfr-init REQ pdu |  |  |  | vr1=“trns-init” vr2=Reference ID COORDINATED |
| AIDC-tfr-prpsl REQ |  |  |  | vs1=“trns-init” vs2=Reference ID COORDINATED |
| rcv AIDC-tfr-prpsl-REQ pdu |  |  |  | vr1=“trns-init” vr2=Reference ID COORDINATED |

| StateÞ Eventß | IDLE | NOTIFY | NEGOTIATING | COORDINATED |
| --- | --- | --- | --- | --- |
| AIDC-tfr-control- REQ |  |  |  | vs1=“trns-pending” vs2=Reference ID COORDINATED |
| rcv AIDC-tfr-control-REQ pdu |  |  |  | vr1=“trns-pending” vr2=Reference ID COORDINATED |
| AIDC-info-trnsfr REQ | IDLE | NOTIFY | NEGOTIATING | COORDINATED |
| rcv AIDC-info-trnsfr REQ pdu | IDLE | NOTIFY | NEGOTIATING | COORDINATED |
| AIDC-end REQ |  | vs1=“end” vs2 = Msg Number | vs1=“end” vs2 = Msg Number | vs1=“end” vs2 = Msg Number |
| rcv AIDC-end REQ pdu |  | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number |
| AIDC-usr-abrt REQ | IDLE | IDLE | IDLE | IDLE |
| AIDC-ABORT ind | IDLE | IDLE | IDLE | IDLE |

| StateÞ Eventß | IDLE | NOTIFY | NEGOTIATING | COORDINATED |
| --- | --- | --- | --- | --- |
| AIDC-ucf REQ | if c1 & c2 & c4 vr1=NULL vr2=NULL NOTIFY if !c1 & c2 & c4 vr1=NULL vr2=NULL IDLE if !c2 NOTIFY | if c1 & c2 & c6 vr1=NULL vr2=NULL NEGOTIATING if !c1 & c2 & c6 vr1=NULL vr2=NULL NOTIFY if !c2 NOTIFY if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL NOTIFY | if c1& c2 & c8 & rslt = + vr1=NULL vr2=NULL COORDINATED if c1& c2 & c8 & rslt = - vr1=NULL vr2=NULL NEGOTIATING if !c1 & c2 & c8 vr1=NULL vr2=NULL NEGOTIATING if !c2 NEGOTIATING if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL NEGOTIATING | if c1 & c2 & c10 vr1=NULL vr2=NULL PRE-TRANSFER if !c1 & c2 & c10 vr1=NULL vr2=NULL COORDINATED if c1 & c2 & c12 vr1=NULL vr2=NULL TRANSFERRING if !c1 & c2 & c12 vr1=NULL vr2=NULL COORDINATED if !c2 COORDINATED if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL COORDINATED |
| rcv AIDC-ucf-REQ pdu | if c1 & c3 & c5 vs1=NULL vs2=NULL NOTIFY if !c1 & c3 & c5 vs1=NULL vs2=NULL IDLE if !c3 IDLE | if c1 & c3 & c7 vs1=NULL vs2=NULL NEGOTIATING if !c1 & c3 & c7 vs1=NULL vs2=NULL NOTIFY if !c3 NOTIFY if c1 & c2 & c24 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL NOTIFY | if c1& c3 & c9 & rslt = + vs1=NULL vs2=NULL COORDINATED if c1& c3 & c9 & rslt = - vs1=NULL vs2=NULL NEGOTIATING if !c1 & c3 & c9 vs1=NULL vs2=NULL NEGOTIATING if !c3 NEGOTIATING if c1 & c2 & c24 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL NEGOTIATING | if c1 & c3 & c11 vr1=NULL vr2=NULL PRE-TRANSFER if !c1 & c3 & c11 vr1=NULL vr2=NULL COORDINATED if c1 & c3 & c13 vr1=NULL vr2=NULL TRANSFERRING if !c1 & c3 & c13 vr1=NULL vr2=NULL COORDINATED if !c3 COORDINATED if c1 & c2 & c24 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL COORDINATED |

| StateÞ Eventß | BACKWARD COORDIATING | TRANSFERRED | PRE-TRANSFER | TRANSFERRING |
| --- | --- | --- | --- | --- |
| AIDC-crd-start-req |  | vs1 = “back” vs2 = Msg Number |  |  |
| rcv AIDC-crd-start-REQ pdu |  | vr1 = “back” vr2 = Msg Number |  |  |
| AIDC-crd--end REQ | vs1 = “back-end” vs2 = Msg Number |  |  |  |
| rcv AIDC-crd-end-REQ pdu | vr1 = “back-end” vr2 = Msg Number |  |  |  |
| AIDC-crd-ngtt-REQ | BACKWARD-COORDINATING |  |  |  |
| rcv AIDC-crd-ngtt-REQ pdu | BACKWARD-COORDINATING |  |  |  |
| rcv AIDC-crd-stnby REQ pdu | BACKWARD-COORDINATING |  |  |  |
| AIDC-tfr-accept |  |  | vs1 = “trns-accpt” vs2 = Reference ID PRE-TRANSFER |  |
| rcv AIDC-tfr-accept pdu |  |  | vr1 = “trns-accpt” vr2 = Reference ID PRE-TRANSFER |  |
| AIDC-tfr-rqst REQ |  |  | PRE-TRANSFER |  |
| rcv AIDC-tfr-rqst-REQ pdu |  |  | PRE-TRANSFER |  |
| AIDC-tfr-cntrl RSP |  |  |  | + vr1 = “tnsf-accpt” vr2 = Reference ID TRANSFERRING - vr1 = “tnsf-reject” vr2 = Reference ID PRE-TRANSFER |
| rcv AIDC-tfr-cntl-RSP pdu |  |  |  | + vs1 = “tnsf-accpt” vs2 = Reference ID TRANSFERRING - vs1 = “tnsf-reject” vs2 = Reference ID PRE-TRANSFER |
| AIDC-tfr-comm REQ |  |  | vr1 = “tns-comm” vr2 = Reference ID PRE-TRANSFER |  |
| rcv AIDC-tfr-comm REQ PDU |  |  | vs1 = “tns-comm” vs2 = Reference ID PRE-TRANSFER |  |

| StateÞ Eventß | BACKWARD COORDIATING | TRANSFERRED | PRE-TRANSFER | TRANSFERRING |
| --- | --- | --- | --- | --- |
| AIDC-tfr-comm-assum REQ |  |  | vr1 = “tnsf-assum” vr2 = Reference ID PRE-TRANSFER |  |
| rcv AIDC-tfr-comm-assum REQ PDU |  |  | vs1 = “tnsf-assum” vs2 = Reference ID PRE-TRANSFER |  |
| AIDC-info-trnsfr REQ |  |  | PRE-TRANSFER |  |
| rcv AIDC-info-trnsfr REQ pdu |  |  | PRE-TRANSFER |  |
| AIDC-end REQ | vs1=“end” vs2 = Msg Number | vs1=“end” vs2 = Msg Number | vs1=“end” vs2 = Msg Number | vs1=“end” vs2 = Msg Number |
| rcv AIDC-end REQ pdu | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number |
| AIDC-usr-abrt-REQ | IDLE | IDLE | IDLE |  |
| AIDC-ABORT ind | IDLE | IDLE | IDLE |  |

| StateÞ Eventß | BACKWARD COORDIATING | TRANSFERRED | PRE-TRANSFER | TRANSFERRING |
| --- | --- | --- | --- | --- |
| AIDC-ucf REQ | if c1 & c2 & c22 vr 1= NULL vr2 = NULL ???????????? if !c1 & c2 & c22 vr1 = NULL vr2 = NULL BACKWARD-COORDINATING if !c2 BACKWARD-COORDINTAING if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL BACKWARD-COORDINATING | if c1 & c2 & c20 vr 1= NULL vr2 = NULL BACKWARD-COORDINATING if !c1 & c2 & c20 vr1 = NULL vr2 = NULL TRANSFERRED if !c2 TRANSFERRED if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL TRANSFERRED | if c1 & c2 & c14 vr1 = NULL vr2 = NULL TRNSFER-CONDITIONS if !c1 & c2 & c14 vr1 = NULL vr2 = NULL PRE-TRANSFER if c1 & c2 & c16 & vr1 = NULL vr2 = NULL TRANSFERRING if c1 & c2 & c16 & vr1 = NULL vr2 = NULL TRANSFERRED if !c1 & c2 & c16 & vr1 = NULL vr2 = NULL PRE-TRANSFER if !c1 & c2 & c16 & vr1 = NULL vr2 = NULL PRE-TRANSFER if !c2 PRE-TRANSFER if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL PRE-TRANSFER | if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL TRANSFERRING |

| StateÞ Eventß | BACKWARD COORDIATING | TRANSFERRED | PRE-TRANSFER | TRANSFERRING |
| --- | --- | --- | --- | --- |
| rcv AIDC-ucf-REQ pdu | if c1 & c3 & c23 vs 1= NULL vs2 = NULL ???????????? if !c1 & c3 & c23 vs1 = NULL vs2 = NULL BACKWARD-COORDINATING if !c3 BACKWARD-COORDINTAING if c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL BACKWARD-COORDINATING | if c1 & c3 & c21 vr 1= NULL vr2 = NULL BACKWARD-COORDINATING if !c1 & c3 & c21 vr1 = NULL vr2 = NULL TRANSFERRED if !c3 TRANSFERRED if c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL TRANSFERRED | if c1 & c3 & c15 vr1 = NULL vr2 = NULL TRNSFER-CONDITIONS if !c1 & c3 & c15 vr1 = NULL vr2 = NULL PRE-TRANSFER if c1 & c3 & c17 & vr1 = NULL vr2 = NULL TRANSFERRING if c1 & c3 & c17 & vr1 = NULL vr2 = NULL TRANSFERRED if !c1 & c3 & c17 & vr1 = NULL vr2 = NULL PRE-TRANSFER if !c1 & c3 & c17 & vr1 = NULL vr2 = NULL PRE-TRANSFER if !c3 PRE-TRANSFER if c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL PRE-TRANSFER | if c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL TRANSFERRING |

| StateÞ Eventß | ENDING | TRANSFERRING | TRANSFER-CONDITIONS |
| --- | --- | --- | --- |
| AIDC-tfr-comm-REQ |  |  | vr1 = “tns-comm” vr2 = Reference ID TRANSFER-CONDITIONS |
| rcv AIDC-tfr-comm-REQ pdu |  |  | vs1 = “tns-comm” vs2 = Reference ID TRANSFER-CONDITIONS |
| AIDC-tfr-comm-assm REQ |  | vr1 = “tnsf-assum” vr2 = Reference ID TRANSFERRING | TRANSFERRED-PENDING |
| rcv AIDC-tfr-comm-assum REQ pdu |  | vs1 = “tnsf-assum” vs2 = Reference ID TRANSFERRING | TRANSFERRED-PENDING |
| AIDC-info-trnsfr REQ |  | TRANSFERRING | TRANSFER-CONDITIONS |
| rcv AIDC-info-trnsfr-REQ pdu |  | TRANSFERRING | TRANSFER-CONDITIONS |
| AIDC-end-REQ |  | vs1 = “end” vs2 = Msg Number | vs1 = “end” vs2 = Msg Number |
| rcv AIDC-end REQ pdu |  | vr1 = “end” vr2 = Msg Number | vr1 = “end” vr2 = Msg Number |
| AIDC-end-response | vs1 = “end” vs2 = Msg Number |  |  |
| rcv AIDC-end-response pdu | vr1 = “end” vr2 = Msg Number |  |  |
| AIDC-usr-abrt REQ |  | IDLE | IDLE |
| AIDC-ABORT IND |  | IDEL | IDLE |

| StateÞ Eventß | ENDING | TRANSFERRING | TRANSFERR-CONDITIONS |
| --- | --- | --- | --- |
| AIDC-ucf REQ | if c1 & c2 & c24 vr1 = NULL vr2 = NULL IDLE if !c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING | if c1 & c2 & c16 vr1 = NULL vr2 = NULL TRANSFERRED if !c1 & c2 & c16 vr1 = NULL vr2 = NULL TRANSFERRING if !c2 TRANSFERRING if c1 & c2 & c24 vr1 = NULL vr2 = NULL ENDING if !c1 & c2 & c24 vr1 = NULL vr2 = NULL TRANSFERRING | if c1 & c2 & c16 vr1 = NULL vr2 = NULL TRANSFERRING if c1 & c2 & c16 vr1 = NULL vr2 = NULL TRANSFERRED if !c1 & c2 & c16 vr1 = NULL vr2 = NULL PRE-TRANSFER if !c1 & c2 & c16 vr1 = NULL vr2 = NULL PRE-TRANSFER |
| rcv AIDC-ucf REQ pdu | if c1 & c3 & c25 vs1 = NULL vs2 = NULL IDLE if !c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING | if c1 & c3 & c17 vs1 = NULL vs2 = NULL TRANSFERRED if !c1 & c3 & c17 vs1 = NULL vs2 = NULL TRANSFERRING if !c3 TRANSFERRING if c1 & c2 & c24 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL TRANSFERRING | if c1 & c3 & c17 vs1 = NULL vs2 = NULL TRANSFERRING if c1 & c3 & c17 vs1 = NULL vs2 = NULL TRANSFERRED if !c1 & c3 & c17 vs1 = NULL vs2 = NULL PRE-TRANSFER if !c1 & c3 & c17 vs1 = NULL vs2 = NULL PRE-TRANSFER if c1 & c3 & c25 vs1 = NULL vs2 = NULL ENDING if !c1 & c3 & c25 vs1 = NULL vs2 = NULL TRANSFER-CONDITIONS |

This timer is dependant upon the operational standby timer defined when AIDC is implemented between neighbouring states.